3GPP TSG-RAN WG2 Meeting #127	R2-2406606

Maastricht, NL, August 19-23, 2024

Agenda Item:	8.8.4

Source:	THALES

Title:	Discussion on MBS Broadcast service area signaling

Document for:	Decision

# 1	Introduction

The approved work item justifications and objectives for NR NTN Evolution in Rel-19 [1] emphasizes the following:

3) MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service.

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this paper, we discuss how to encode and single the MBS broadcast service area in a NTN cell.

# 2	Discussion

MBS Broadcast session intended service area

From the system architecture point of view, the MBS broadcast session can already be linked to a specific area as defined in [2]-§6.2.3 “Location dependent MBS service”. The MBS session creation is described in [2]-§7.1.1.2. When the location dependent MBS Session (e.g. broadcast) is announced (step 7. in Figure 7.1.1.2-1), the MBS service area information associated to MBS Session ID e.g. TMGI (Temporary Mobile Group Identifier) is announced by the AF (Application Function) to the UEs. The MBS service area information can be Cell ID list, TAI list, geographical are information or civic address. It was already agreed in RAN2#125bis that the intended service 	are can cover the area of more than one NTN cells or a portions thereof. It was also agreed that geographical area will be defined for intended service area and that this information could be a circle (like for TN coverage in Rel-18) or polygons.

Figure 1 - MBS Broadcast location-dependent sessions

In Figure 1 is depicted to the typical architecture for MBS broadcast location-dependent. The MBS session area is defined by the AF, transmitted by the MB-SMF (associated with MBS Session ID) to UE through the NG-RAN. What is new for NTN, is the MBS broadcast service area broadcasting to the UE (illustrated by the colors green, purple, and orange) in the physical cell (blue). The MBS session#1 shows that an intended service area can be larger than a physical cell. The MBS session#2 shows that an intended service area can be smaller than a cell and in this example, it could corresponds to one physical satellite beam belonging to the physical cell. The MBS session#3 is positioned to cover a specific geographical region (e.g. a country) and covers multiple cells.

The intended geographical service area for MBS broadcast could be larger than physical cell, smaller or astride multiple cells

The intended service area could correspond to a satellite physical beam in a cell and therefore best represented by a circle

The intended service area could correspond to a geographical region with borders and therefore best represented by a polygon

Geographical Area coding

CMAS (Commercial Mobile Alert System) is used as Public Warning System (PWS) to deliver multiple, concurrent warning text messages. This emergency alerts broadcast can be either within the whole network or within certain geographical areas, down to the size of a single cell. In SIB8 for CMAS information broadcast, the geographical area can be defined to define where the CMAS message is valid:

SIB8 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningMessageSegmentType ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber INTEGER (0..63), warningMessageSegment OCTET STRING, dataCodingScheme OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 warningAreaCoordinatesSegment OCTET STRING OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... }

Especially, the warningAreaCoordinatesSegment IE defines the geographical coordinate. This information element conveys the description of Warning Area Geometries specified by the Alert Originator to the mobile device to perform geo-fencing on the alert [4]. The use case is very similar to the one of the intended service area for MBS Broadcast where the network wants to indicate to the UE geographical zones associated to a MBS Session. The [3]-§9.3.63 defines the Warning Area Coordinates IE as follow:

The Warning Area Coordinates IE indicates alert area coordinates of a warning message. The eNB and NG-RAN node pass this information to the UE. The UE uses this information to conditionally display (see clause 9.1.3.4.2 and clause 9.1.3.5.2) the warning message contents.

The Warning coordinates are coded as a Tag-Length-Value finite bit string as specified in [4]-§5.2:

Figure 2 - Warning Area Coordinates

Where:

    - Tag (4 bits): Indicates the type of parameter. 0010 for polygon and 0011 for circle.

    - Length (10 bits): Indicates the length of the entire WAC TLV in octets.

    - Value (variable number of octets, up to 1022): The value of the parameter.

This coding offers the possibility to indicate coordinates using polygons or circles to define intended service area. The Warning Area Coordinate for polygon (tag 0010) is given below:

Figure 3 - Warning Area Coordinates polygon

Each vertex adds 24 bits (3 bytes) to the IE size to be added to the 2 bytes of the header. Since the maximum IE’s value size is 1020 bytes, 340 vertices can define a geographical area. A square defined geographical coordinates size is 5 (the first and the last point should match) x 3 bytes + 2 bytes equal 17 bytes.

The coordinates for circles (tag 0011) is given below:

Figure 4 - Warning Area Coordinates circle

The size of a coordinates with circles is 10 bytes.

The others tags are not expected to be used (geo-fencing maximum wait time and CMSP defined use).

SIB8 for CMAS signaling uses the Warning Area Coordinated IE to encode geographical region

In Rel-18 was defined CoverageAreaInfo for TN cell broadcasting in NTN via SIB 25 as follow:

SIB25-r18 ::= SEQUENCE { coverageAreaInfoList-r18 CoverageAreaInfoList-r18 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } CoverageAreaInfoList-r18 ::= SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF CoverageAreaInfo-r18 CoverageAreaInfo-r18 ::= SEQUENCE { tn-AreaId-r18 TN-AreaId-r18, tn-ReferenceLocation-r18 ReferenceLocation-r17, tn-DistanceRadius-r18 INTEGER(0..65535) }

It looks very similar to the WAC for circle. The circle can be sufficient to meet the WID objectives since we can define area smaller or larger than a beam footprint.

The distance radius is coded on 16 bits instead of 20 bits for WAC. The argument to choose CoverageAreaInfo instead of WAC for intended service area coding could not be determined only on the impact of the SIB size whilst the size difference for a circle-type encoding with the two methods are similar.

The main argument to re-use the Warning Area Coordinates coding is the possibility to define either circle or polygon for the intended service area, according to what is provided by the MBS broadcast AF for the location at the MB-SMF. The same coding to the content provider to the UE through the SI message is used for indicating the intended service area, as it is already in use for PWS [3].

Re-use the Warning Area Coordinates coding for MBS Broadcast service intended area

Signaling of the intended service area

Figure 5 - MBS broadcast session acquisition

The UE acquires necessary configurations for MCCH for MBS broadcast reception from SIB20 and SIB21.  After that, UE acquires configuration for MTCH from MCCH. RAN2 (RAN2#126 agreement) considers to include the service area information in SIB20, SIB21 or MBSBroadcastConfiguration.

For the NTN UE, it is beneficial in terms of energy consumption for RX to determine as soon as possible if any MBS broadcast service is available for it in the geographical area. Therefore, only decode the SIB before proceeding to the MCCH decoding. SIB20 is more relevant than SIB21 to transmit the geographical information for broadcast service since SIB21 is more related to frequency assignments and SIB20 for MCCH/MTCH configuration.

Use SIB20 to broadcast MBS broadcast service intended area

We propose the following for the intended service area signaling to the SIB20:

-- ASN1START -- TAG-SIB20-START SIB20-r17 ::= SEQUENCE { mcch-Config-r17 MCCH-Config-r17, cfr-ConfigMCCH-MTCH-r17 CFR-ConfigMCCH-MTCH-r17 OPTIONAL, -- Need S lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ cfr-ConfigMCCH-MTCH-RedCap-r18 CFR-ConfigMCCH-MTCH-r17 OPTIONAL, -- Need S mcch-ConfigRedCap-r18 MCCH-Config-r17 OPTIONAL -- Cond CFR-RedCap ]], [[ serviceAreaList-r19 ServiceAreaList-r19 OPTIONAL ]] } ServiceAreaList-r19 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF ServiceArea-r19 ServiceArea-r19 ::= SEQUENCE { mbs-SessionId-r17 TMGI-r17, warningAreaCoordinatesSegment OCTET STRING, geofencing-Preference-r19 ENUMERATED {intended, unintended} } -- TAG-SIB20-STOP -- ASN1STOP

Table 1 - SIB20 with proposed serviceAreaList

Where:

    - mbs-SessionID-r17 is the MBS Session ID associated to a MBS Session configured in MCCH

    - warningAreaCoordinatesSegment as defined for SIB8, coding defined in [4]

    - geofencing-Preference-r19 enables to tell that the service intended area is inside the coordinates or outside the coordinate

    - And maxNrofMBS-Session-r17 has already defined (=1024). 1024 is maybe too much for SIB size and regarding the NTN scenario. FFS to define a new maximum number of intended service area

Consider the SIB20 update presented in Table 1 to support MBS broadcast service intended area for Rel-19 NTN. FFS the maximum number of intended service area that can be broadcasted in the SIB

# 3	Conclusion

In the previous sections we made the following observations and proposals:

< MBS Broadcast session intended service area>

- The intended geographical service area for MBS broadcast could be larger than physical cell, smaller or astride multiple cells

The intended service area could correspond to a satellite physical beam in a cell and therefore best represented by a circle

The intended service area could correspond to a geographical region with borders and therefore best represented by a polygon

- < Geographical Area coding>

SIB8 for CMAS signaling uses the Warning Area Coordinated IE to encode geographical region

- Re-use the Warning Area Coordinates coding for MBS Broadcast service intended area

- < Signaling of the intended service area>

Use SIB20 to broadcast MBS broadcast service intended area

Consider the SIB20 update presented in Table 1 to support MBS broadcast service intended area for Rel-19 NTN. FFS the maximum number of intended service area that can be broadcasted in the SIB

# References

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN1 Vice-Chair, RAN#104, 2024-06, Shanghai, CN.

3GPP TS 23.247, “Architectural enhancements for 5G multicast-broadcast services”

3GPP TS 23.041, “Technical realization of Cell Broadcast Service (CBS)”

ATIS 0700041, “WEA 3.0: Device-Based Geo-Fencing”

SIB20 contains the information required to acquire the MCCH/MTCH configuration for MBS broadcast

SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI)

MBSBroadcastConfiguration: message contains the control information applicable for MBS broadcast services transmitted via broadcast MRB