3GPP TSG-RAN WG2 Meeting #127	R2-2406607

Maastricht, Netherlands, Aug. 19th~23rd , 2024

Agenda item:	7.5.3

Source:	Samsung

Title:	MAC Corrections for Rel-18 XR

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, RAN2 discussed the DRX_SFN_COUNTER initialization. However, the initialization during the HO procedure is unclear.

=> Remove the sentence “in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 [5]” from the procedure text. Adopt the TP for TS 38.321 for DRX_SFN_COUNTER initialization. FFS HO case

On DSR, we found a few corrections needed for DSR triggering and cancellation.

In this contribution, we will further address these issues.

DRX_SFN_COUNTER Initialization

In current specification, the DRX_SFN_COUNTER initialization is carried out as below:

2> if DRX is (re-)configured by RRC: 3> if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511): 4> set DRX_SFN_COUNTER to 1. 3> else: 4> set DRX_SFN_COUNTER to 0.

Such initialization behaviour is intended to make sure that both gNB and UE initialize DRX_SFN_COUNTER at the same time. During handover procedure, the RRC message containing DRX is transmitted by the source gNB, while the initialization should be carried out by target gNB and UE at the same time. Figure 1 illustrates the timeline for the RRCReconfiguration message transfer.

Fig. 1. Illustration of DRX_SFN_COUNTER initialization

As shown in the figure, the time point of Option 1 is aligned with the current specification, i.e., the initialization occurs when the RRC reconfiguration message is received. However, such time point does not work for handover case since the target gNB cannot know when the RRC reconfiguration message is sent to the UE.

To resolve this issue of option 1, option 2 can be considered. In particular, after success RACH procedure, the UE sends the RRCReconfigurationComplete message to the target gNB. The target gNB is aware of the access of the UE. Thus, both target gNB and UE can initialize DRX_SFN_COUNTER at the same time.

Proposal 1: RAN2 is kindly asked to agree that the initialization of DRX_SFN_COUNTER occurs when RRCReconfigurationComplete message is sent for HO case.

If Proposal 1 is agreed, UE will have different handlings on the initialization of DRX_SFN_COUNTER for different cases, which will increase the UE complexity. A better choice is to use a unified solution for all cases. To achieve this purpose, the time instance of transmitting RRCReconfigurationComplete message is the choice. Moreover, such time instance can cover other handover case, e.g., CHO, LTM.

Proposal 2: RAN2 is kindly asked to agree that the initialization of DRX_SFN_COUNTER occurs when RRC response message is sent by the UE for all cases.

DSR

## 3.1 Correction on DSR and SR timing

It can be noted in BSR section that MAC PDU assembly can happen at any point in time between uplink grant reception and actual transmission of the corresponding MAC PDU. Also, MAC specification includes a note that BSR and SR can be triggered after the assembly of a MAC PDU which contains a BSR MAC CE, but before the transmission of this MAC PDU. In addition, BSR and SR can be triggered during MAC PDU assembly.

However, this aspect is not clear for DSR, i.e., whether DSR and the SR for DSR can be triggered during or after the assembly of the MAC PDU containing a DSR MAC CE but before the transmission of the MAC PDU.

Proposal 3: RAN2 is kindly asked to consider clarifying that DSR and the SR for DSR can be triggered after the assembly of a MAC PDU which contains a DSR MAC CE, but before the transmission of this MAC PDU. In addition, DSR and SR for DSR can be triggered during MAC PDU assembly. Adopt TP 1.

## 3.2 Correction on DSR cancellation

During or after MAC PDU assembly (and before the MAC PDU transmission), there could be additional PDCP SDUs that become associated with a pending DSR, due to their remaining discardTimer values become less than remainingTimeThreshold. It is essential to clarify, the scope of the associated PDCP SDUs that should be considered in order to cancel a pending DSR. Specifically, as shown in Fig. 2, the amount of PDCP SDUs associated with a pending DSR A is A1 before MAC PDU assembly, while there are additional PDCP SDUs, whose remaining discardTimer values become less than remainingTimeThreshold, during or after MAC PDU assembly but before MAC PDU transmission. As a result, when the MAC PDU is transmitted, the total amount of PDCP SDUs associated with the DSR A becomes A2. It is unclear, based on the current spec that, whether A2 or A1 should be considered to cancel the pending DSR A.

Fig. 2. Illustration of the PDCP SDUs associated with a pending DSR.

From the feasibility point of view, for DSR cancellation, we should consider only the PDCP SDUs associated with the DSR before MAC PDU assembly (A1 in Fig. 2), since otherwise, the UE implementation becomes too complex to inclusively consider PDCP SDUs that become associated with the DSR during/after MAC PDU assembly and before MAC PDU transmission.

Proposal 4: RAN2 is kindly asked to agree that, for cancelling a pending DSR, only the PDCP SDUs associated with the DSR before MAC PDU assembly are reported in the DSR MAC CE or included in the MAC PDU. Adopt TP 2.

Conclusion

Proposal 1: RAN2 is kindly asked to agree that the initialization of DRX_SFN_COUNTER occurs when RRCReconfigurationComplete message is sent for HO case.

Proposal 2: RAN2 is kindly asked to agree that the initialization of DRX_SFN_COUNTER occurs when RRC response message is sent by the UE for all cases.

Proposal 3: RAN2 is kindly asked to consider clarifying that DSR and the SR for DSR can be triggered after the assembly of a MAC PDU which contains a DSR MAC CE, but before the transmission of this MAC PDU. In addition, DSR and SR for DSR can be triggered during MAC PDU assembly. Adopt TP 1.

Proposal 4: RAN2 is kindly asked to agree that, for cancelling a pending DSR, only the PDCP SDUs associated with the DSR before MAC PDU assembly are reported in the DSR MAC CE or included in the MAC PDU. Adopt TP 2.

References

- TP for DRX_SFN_COUNTER Initialization

### 38.321 ### 5.7 Discontinuous Reception (DRX) ... 1> if the drx-NonIntegerLongCycleStartOffset is configured: 2> increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0; 2> if DRX is (re-)configured by RRC: 3> if drx-TimeReferenceSFN is included in the RRC (re-)configuration and if the first PUSCH transmission which contains the response message for RRC (re-)configuration, as specified in TS38.331 [5], is transmitted during which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511): 4> set DRX_SFN_COUNTER to 1 in the same slot when the first PUSCH transmission which contains the response message for RRC (re-)configuration, as specified in TS38.331 [5], is transmitted. 3> else: 4> set DRX_SFN_COUNTER to 0 in the same slot when the first PUSCH transmission which contains the response message for RRC (re-)configuration, as specified in TS38.331 [5], is transmitted.

TP 1 for DSR

### 38.321 - 5.4.9 Delay status reporting ### …. NOTE 2: It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader. NOTE 3: MAC PDU assembly can happen at any point in time between uplink grant reception and actual transmission of the corresponding MAC PDU. DSR and SR can be triggered after the assembly of a MAC PDU which contains a DSR MAC CE, but before the transmission of this MAC PDU. In addition, DSR and SR can be triggered during MAC PDU assembly.

TP 2 for DSR

### 38.321 - 5.4.9 Delay status reporting ### … After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR before the MAC PDU assembly (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR before the MAC PDU assembly. ...