3GPP TSG-RAN WG2 Meeting #127	R2-2406609

Maastricht, Netherlands, Aug. 19th~23rd , 2024

Agenda item:		8.2.3

Source:		Samsung

Title:			Discussion on A-IoT paging

Document for:	Discussion & Decision

- 1 Introduction

The agreements that have been made related to A-IoT paging in the last RAN2 meeting is as follows.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Based on the agreements, in this contribution, we discuss further on A-IoT paging.

- 2 Discussion

Issue 1: CN information visibility for AIoT paging message generation

All AS procedures for AIoT are triggered by the AIoT paging message, while the AIoT paging message is triggered by the message from CN. So far, the AS procedures contain inventory only procedure, command only procedure, and inventory + command procedure. Moreover, after receiving the AIoT paging message, the device may carry out either contention based random access or contention-free random access. Thus, the AIoT reader should be able to determine which kind of procedures is triggered and which type of RA procedures is triggered based on the visible information from the CN:

- To determine the AS procedure, the Reader needs to be aware of the message type from the CN, e.g., inventory, command.

- For the RA procedure type, it is related to the number of devices being inventoried. In last meeting, RAN2 agreed that the AIoT paging message may contain: 1) a device ID to inventory a single device, 2) a group ID to inventory a group of device, or 3) no ID information to inventory all devices. Thus, the Reader needs to derive the ID information for the CN. Furthermore, when multiple devices are inventoried, the Reader may need to know the number of devices for inventory since this can help the resource allocation for the following-up contention based RA procedure. For example, if single device is inventoried, the contention-free RA can be triggered. While if multiple devices are inventoried, the Reader may configure the contention-based RA procedure. Moreover, if the number of inventoried devices is large, the Reader can allocate more resoruce for RA procedure.

Proposal 1-1: RAN2 is kindly asked to discuss the visible information from CN to trigger the AIoT paging message, e.g., ID(s) of devices or group ID, the number of devices, the message type information for inventory or command.

In addition, the last RAN plenary meeting reached the following conclusions:

Conclusion RAN1 to define “successfully received” for the purpose of evaluations of the latency for a single A-IoT device. It is in scope of the study to define inventory completion time for multiple devices.

The above conclusion indicates that the latency of completing the inventory is one of parameters to evaluate the performance of the inventory procedure. The latency requirement may be derived from the CN. Thus, the visible information from CN may also contain the performance requirement (e.g., latency) for the inventory only/command only/inventory+command procedure.

Proposal 1-2: RAN2 is kindly asked to discuss whether the visible information from CN contains the performance requirement (e.g., latency) for different procedures.

The visible information is from CN. However, how to receive those visible information needs further discussion. One possible way is to derive the information from the control plane signaling (e.g., NGAP message) so that the AMF needs to derive those visible information. Another way is to let the gNB interpret the message from the CN (e.g., interpret the message of AIoT APP layer).

Proposal 1-3: RAN2 is kindly asked to discuss how to derive the visible information from CN by the AIoT Reader.

Issue 2: information contained in the AIoT paging message

- Container for the message received from CN

The AIoT paging message can contain the message received from CN (e.g., NAS container), which may contain the criteria of determining the target A-IoT devices to be inventoried.

Proposal 2-1: RAN2 is kindly asked to agree that the container with the message received from CN can be included in AIoT paging message.

- ID information

As agreed in last RAN2 meeting, the AIoT paging message may include the ID information, which can be either ID of a single device or group ID of multiple devices.

Proposal 2-2: RAN2 is kindly asked to agree that the AIoT paging message can include ID information, i.e., either ID of a single device or group ID of multiple devices.

- Resource allocation information

We have agreed to support the contention-free procedure and contention-based RA procedures:

- Contention-free RA procedure

The AIoT device can perform D2R data transmission during dedicated time domain/access occasion. The AIoT paging message can indicate such dedicated time domain/access occasion via, e.g., the sequence number of time domain/access occasion (as Fig. 1 (a)), the number of time domain/access occasion since the AIoT paging message is received by AIoT device (as Fig. 1 (b)),  can be indicated via AIoT paging message.

Fig. 1 Indication of dedicated access occasion

- Contention-based RA procedure

The slotted-ALOHA-like random access can be considered as a baseline, where an A-IoT device will randomly select a time domain resource for very first D2R message transmission. The A-IoT reader can provide information assisting the determination of the time resource/random access occasion via AIoT paging message. One possible information is a parameter Q, which indicates 2Q time resource/random access occassions in total so that the A-IoT device can randomly select one resource/occasion among them for D2R transmission.

Proposal 2-3: RAN2 is kindly asked to discuss the resource allocation information in the AIoT paging message for two different RA procedures:

  - Contention-free RA: information indicating a dedicated time resource/random access occasion

  - Contention-based RA: a parameter Q indicating 2Q time resource/random access occassions in total

In addition, the frequency-domain random access resource information may be included if the FDM is applied for resource allocation.

Proposal 2-4: RAN2 is kindly asked to discuss whether the frequency-domain resource information can be included in the AIoT paging message to support the FDM based resource allocation.

Issue 3: transmission of AIoT paging message

The AIoT paging message is triggered based on the request from CN. The A-IoT reader may send AIoT paging messgae several times to trigger several inventory/random access rounds. On one hand, once the AIoT paging message is sent, it is possible that some A-IoT device(s) to be inventoried cannot receive it correctly. Thus, multiple AIoT paging messages can ensure that more devices can receive it, and the AIoT reader can inventory the A-IoT devices as much as possible. On the other hand, as discussed above, A-IoT reader should determine the suitable random access occasion configurations (e.g., time/frequency-domain resource) for the inventory procedure. This depends on the number of A-IoT devices to be inventoried. If the numbr is large, the A-IoT reader should assign more resources, while a few resources can be assigned when the number is small. With the increase of the number of successfully inventoried A-IoT devices, the number of A-IoT devices to be inventoried is reduced gradually. Thus, the A-IoT reader can use multiple AIoT paging messages to adjust the configurations for random access occasions. This can help the resource utilization efficiency.

Proposal 3-1: RAN2 is kindly asked to agree that the AIoT paging message can be sent by multiple times for one inventory request from CN.

Among the multiple AIoT paging messages, the Reader may change the information in each message. For example, as mentioned above, the Reader may change the resource configuration with the increase of the number of successfully inventoried A-IoT devices.

Proposal 3-2: RAN2 is kindly asked to discuss whether the information in AIoT paging message can be changed if it is sent by multiple times for one inventory request from CN.

In addition, after receiving inventory request from CN, the Reader can trigger the AIoT paging message transmission for several times. However, the Reader should be able to determine when to stop the inventory procedure (i.e., stop transmission of AIoT paging message). This may be determined by several aspects, e.g., whether the required inventoried devices are successfully inventoried, whether the time period used for inventory expires, whether the latency for inventory exceeds the configured value, etc.

Proposal 3-3: RAN2 is kindly asked to discuss how to determine that the transmission of AIoT paging message should be stopped.

Issue 4: AIoT paging message monitoring

As agreed in last meeting, the monitoring of AIoT paging message depends on the available energy. If energy is not enough, the A-IoT device is supposed to be powered down, thus intrinsically, A-IoT device can only receive R2D message opportunistically. Thus, to save energy, the discussion during last RAN plenary meeting indicates that RAN1 will discuss the power saving of AIoT device. Such power saving scheme determines the available period for the AIoT paging message monitoring. Thus, RAN2 can study AIoT paging monitoring after RAN1 discussion on the power saving mechanism.

Proposal 4: RAN2 is kindly asked to discuss AIoT paging message monitoring after RAN1 discussion on power saving.

- 3 Conclusion

Based on the discussion above, we have the following observations and proposals:

Proposal 1-1: RAN2 is kindly asked to discuss the visible information from CN to trigger the AIoT paging message, e.g., ID(s) of devices or group ID, the number of devices, the message type information for inventory or command.

Proposal 1-2: RAN2 is kindly asked to discuss whether the visible information from CN contains the performance requirement (e.g., latency) for different procedures.

Proposal 1-3: RAN2 is kindly asked to discuss how to derive the visible information from CN by the AIoT Reader.

Proposal 2-1: RAN2 is kindly asked to agree that the container with the message received from CN can be included in AIoT paging message.

Proposal 2-2: RAN2 is kindly asked to agree that the AIoT paging message can include ID information, i.e., either ID of a single device or group ID of multiple devices.

Proposal 2-3: RAN2 is kindly asked to discuss the resource allocation information in the AIoT paging message for two different RA procedures:

  - Contention-free RA: information indicating a dedicated time resource/random access occasion

  - Contention-based RA: a parameter Q indicating 2Q time resource/random access occassions in total

Proposal 2-4: RAN2 is kindly asked to discuss whether the frequency-domain resource information can be included in the AIoT paging message to support the FDM based resource allocation.

Proposal 3-1: RAN2 is kindly asked to agree that the AIoT paging message can be sent by multiple times for one inventory request from CN.

Proposal 3-2: RAN2 is kindly asked to discuss whether the information in AIoT paging message can be changed if it is sent by multiple times for one inventory request from CN.

Proposal 3-3: RAN2 is kindly asked to discuss how to determine that the transmission of AIoT paging message should be stopped.

Proposal 4: RAN2 is kindly asked to discuss AIoT paging message monitoring after RAN1 discussion on power saving.