3GPP TSG-RAN WG2 Meeting #127	R2-2406610

Maastricht, Netherlands, Aug. 19th~23rd , 2024

Agenda item:		8.2.5

Source:		Samsung

Title:			Discussion on A-IoT Topology 2

Document for:	Discussion & Decision

- 1 Introduction

As a part of SID, the topology 2 is one of scenarios for AIoT study. Since the beginning of this SI, RAN2 reached a high level agreement as

From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

In previous RAN2 meeting, the main discussions are focused on Topology 1. This meeting is the first meeting addressing the topology 2. In this contribution, we will discuss the main issues for such topology, which are not addressed in Topology 2.

- 2 Discussion

2.1. Protocol stack

The topology 2 is realized by conveying AIoT application layer traffic over the intermediate UE. The potential protocol stacks for topology 2 can be divided into two types, i.e., user plane based protocol stack and control plane protocol stack, as shown in Fig. 1.

Fig. 1 Protocol stack for topology 2

The two protocol stacks are differentiated in the following aspects:

- Aspect 1: NG/Uu interface

- User plane based protocol stack: the AIoT APP layer message is conveyed as the user plane traffic via the UPF. In particular, the AIoT APP layer message is transmitted as the PDU session from the UPF to the gNB (or from the gNB to the UPF), and as the DRB from the gNB to the intermediate UE (or from the intermediate UE to the gNB).

- Control plane based protocol stack: the AIoT APP layer message is conveyed as the control plane traffic via the AMF. In particular, the AIoT APP layer message is transmitted as a container in NGAP message from the AMF to the gNB (or from the gNB to AMF), and as the RRC message from the gNB to the intermediate UE (or from the intermediate UE to the gNB).

- Aspect 2: Derivation of visible information from AIoT APP layer message

When discussing topology 1, the following agreement was achieved:

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

It indicates that some information from the AIoT APP layer message may be visible to the Reader to perform the inventory/command procedure. Similarly, in topology 2, the intermediate UE may also need some visible information derived from the AIoT APP layer message. As shown in Fig. 1, the common part of both protocol stacks is that the intermediate node may derive some information from AIoT APP layer message. However, the method of deriving the visible information may be different:

- User plane based protocol stack: the intermediate node derives the visible information via

  - interpreting the AIoT APP layer message directly

- Control plane based protocol stack: the intermediate node derives the visible information via:

  - Option 1:RRC message, the interpretation of AIoT APP layer message is carried out by the AMF, and the AMF forwards the visible information to gNB. This option has Uu and NG interface impact

  - Option 2: interpreting the AIoT APP layer message directly.

- Aspect 3: handling of AIoT APP layer message at the intermediate node

- User plane based protocol stack: the intermediate node needs to differentiate the AIoT traffic from its own traffic when receiving packets from the gNB. One possible way is to use dedicated logical channel to convey the AIoT traffic.

- Control plane based protocol stack: the intermediate node can derive the AIoT APP layer message via the container contained in the RRC message directly.

The following table summarizes the comparison between two protocol stacks.

|  | User plane based protocol stack | Control plane based protocol stack |
| --- | --- | --- |
| NG interface | PDU session | Container in NGAP message |
| Uu interfac | DRB | SRB |
| Visible information derivation | Direct interpretation on AIoT APP layer message | RRC configuration or direct interpretation on AIoT APP layer message |
| Intermediate node handling | Use dedicated logical channel for AIoT traffic | Derive from the RRC message |

Proposal 1: RAN2 is kindly asked to discuss the protocol stack for topology 2, e.g., control plane based protocol stack, user plane based protocol stack.

2.2. Resource allocation

Different from topology 1, the communication between the reader and the device occurs between two UEs. To avoid the interference between the Uu interface and the AIoT Uu interface, gNB should take the responsibility for the resource allocation. One possible way is that the gNB allocates the resource for the communication between the Reader and the Device. Another way is that the gNB allocates the resource pool to the Reader, while the Reader determines the resource allocation between the Reader and the Device.

Proposal 2: RAN2 is kindly asked to discuss the resource allocation for topology 2 by considering the following two options:

- Option 1: resource allocation via gNB directly

- Option 2: resource allocation via the intermediate node based on the resource pool allocated by gNB.

- 3 Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 is kindly asked to discuss the protocol stack for topology 2, e.g., control plane based protocol stack, user plane based protocol stack.

Proposal 2: RAN2 is kindly asked to discuss the resource allocation for topology 2 by considering the following two options:

- Option 1: resource allocation via gNB directly

- Option 2: resource allocation via the intermediate node based on the resource pool allocated by gNB.