3GPP TSG-RAN WG2 Meeting #127	R2-2406611

Maastricht, Netherlands, Aug. 19th~23rd , 2024

Agenda item:	8.13.2

Source:	Samsung

Title:	Initial considerations on relay discovery and (re)selection

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

Introduction

In last RAN plenary meeting, new WI for NR sidelink multi-hop relay was approved with the following objectives:

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this contribution, we will share our initial thinking on the discovery and relay selection/re-selection.

Discussion

Scenarios

As indicated by WID, the prioritized scenario is remote UE -> first relay UE -> last relay UE -> gNB. In this scenario, three different types of link are involved, i.e., 1) link between remote UE and first relay UE, 2) link between first relay UE and last relay UE, and 3) link between last relay UE and gNB. In our understanding, the first two links are PC5 link, and third link is the Uu link. Moreover, the studies in Rel-19 should consider the enhancement for three links.

Proposal 1-1: RAN2 is kindly asked to discuss the enhancement for three link types, i.e., 1) link between remote UE and relay UE, 2) link between relay UEs, and 3) link between relay UE and gNB.

In addition, to facilitate the discussion, it is better to have a common terminology to three types of link. One possible example is: 1) remote-relay link, 2) relay-relay link, and 3) Uu link. In the following part of this contribution, we will use these terminologies.

Proposal 1-2: RAN2 is kindly asked to discuss the terminology for three types of links.

Relay UE discovery

In R19, the relay discovery should be performed over remote-relay link and relay-relay link. In our understanding, the legacy relay discovery can be reused for the remote-relay link.

Proposal 2-1: RAN2 is kindly asked to agree that the legacy discovery scheme is reused for the remote-relay link.

However, the discovery over relay-relay link deserves further discussions. Similar to legacy, the protocol stack for discovery between two relay UEs is shown as below, where the discovery message above PDCP layer is transmitted between two relay UEs

Proposal 2-2: RAN2 is kindly asked to agree the protocol stack of discovery between two relay UEs

Since the discovery may occur between two different types of links, the resultant issue is that how to identify the peer node in the discovery procedure is a relay UE or remote UE. This is related to the capability of each UE, i.e., whether an UE can be remote UE only, relay UE only, or both.

Proposal 2-3: RAN2 is kindly asked to discuss whether an UE is capable of acting as remote UE only, relay UE only, or both.

As legacy, two major issues for the discovery procedure are: 1) triggering of discovery message transmission and monitoring, and 2) resource allocation for discovery message transmission.

- For the first issue, the legacy scheme is that the relay UE transmits relay discovery messages based on configured maximum Uu RSRP threshold, minimum Uu RSRP threshold, or both. However, those thresholds are used to determine discovery message transmission towards remote UE. The question is whether the discovery message transmission towards another relay UE can reuse the same threshold.

- For the second issue, the legacy scheme is that the relay UE can use either the resource pool for NR sidelink communication or the resource pool for Relay discovery. However, regardless of which resource pool is used, the legacy scheme only considers the discovery message transmission between remote UE and relay UE. Thus, the question is whether the resource used for the discovery message transmission between two relay UEs is a separated resource pool.

Proposal 2-4: RAN2 is kindly asked to discuss the triggering condition and resource allocation for the discovery over relay-relay link.

R19 supports multiple hops between remote UE and the gNB. However, the total number of hops are limited. Thus, the remote UE/relay UE should not connect with its parent node when the total number of hops exceeds the supported number of hops. To avoid this, the discovery message can contain hop count related information.

Proposal 2-5: RAN2 is kindly asked to discuss whether the hop count information is included in the discovery message.

Relay UE selection/reselection

For the remote UE, the legacy evaluation criteria and the trigger condition for the relay UE selection/reselection are list in the following table:

|  | Relay selection | Relay reselection |
| --- | --- | --- |
| Measurement for evaluation | - SD-RSRP | - SL-RSRP (with data transmission from relay UE to remote UE) - SL-RSRP/SD-RSRP (no data transmission from relay UE to remote UE) |
| Trigger condition | - Uu signal quality < threshold; - Upper layer indication | - PC5 signal strength < threshold - Receive NotificationMessageSidelink from relay UE - Receive PC5-S link release message from relay UE - PC5 RLF - Upper layer indication |

In R19, remote UE can use the same method to perform the relay UE selection/reselection.

Proposal 3-1: RAN2 is kindly asked to agree that the legacy evaluation criteria and trigger condition for relay UE selection/reselection can be reused for remote UE.

The first relay UE may carry out relay UE selection/reselection to connect with the last relay UE. Thus, the issue is that whether the legacy evaluation criteria and triggering condition can be reused for the first relay UE. The intention is to let the relay UE select/reselect another relay UE. Since this is mainly related to the PC5 link quality, the legacy evaluation criteria is also applicable.

Proposal 3-2: RAN2 is kindly asked to agree that the legacy evaluation criteria and trigger condition for the relay UE selection/reselection can be reused at the relay UE side (i.e., relay UE selects/reselects another relay UE).

Except the existing triggering condition, the relay UE reselection at the remote UE/relay UE may be triggered by new conditions, e.g., the relay UE reselection, the PC5 RLF between two relay UEs. In addition, R19 has the restriction on the hop number. For example, as WID indicated, the prioritized case is that the remote UE is connected to gNB via two relay UEs. Later, the case can be extended to three relay UEs. Thus, the number of hops is restricted in R19, and the relay reselection at the remote UE/relay UE side may be triggered if the hop counts exceeds the supported number of hops.

Proposal 3-3: RAN2 is kindly asked to discuss new triggering condition for the relay UE reselection at the remote UE/relay UE side, e.g., relay UE reselection, PC5 RLF between two relay UEs, hop count exceeds the supported number of hops.

Conclusion

In this contribution, we share our initial view on the relay discovery and (re)selection, and have the following proposals:

Proposal 1-1: RAN2 is kindly asked to discuss the enhancement for three link types, i.e., 1) link between remote UE and relay UE, 2) link between relay UEs, and 3) link between relay UE and gNB.

Proposal 1-2: RAN2 is kindly asked to discuss the terminology for three types of links.

Proposal 2-1: RAN2 is kindly asked to agree that the legacy discovery scheme is reused for the remote-relay link.

Proposal 2-2: RAN2 is kindly asked to agree the protocol stack of discovery between two relay UEs

Proposal 2-3: RAN2 is kindly asked to discuss whether an UE is capable of acting as remote UE only, relay UE only, or both.

Proposal 2-4: RAN2 is kindly asked to discuss the triggering condition and resource allocation for the discovery over relay-relay link.

Proposal 2-5: RAN2 is kindly asked to discuss whether the hop count information is included in the discovery message.

Proposal 3-1: RAN2 is kindly asked to agree that the legacy evaluation criteria and trigger condition for relay UE selection/reselection can be reused for remote UE.

Proposal 3-2: RAN2 is kindly asked to agree that the legacy evaluation criteria and trigger condition for the relay UE selection/reselection can be reused at the relay UE side (i.e., relay UE selects/reselects another relay UE).

Proposal 3-3: RAN2 is kindly asked to discuss new triggering condition for the relay UE reselection at the remote UE/relay UE side, e.g., relay UE reselection, PC5 RLF between two relay UEs, hop count exceeds the supported number of hops.

References