3GPP TSG RAN WG2 Meeting #127               	R2-2406614

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item: 	8.2.2

Source:		Sony

Title:			Considerations on functionality aspects for Ambient IoT

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version of the SID for Ambient IoT in NR was approved including the following objectives [1].

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies. -

At RAN2#126 [2], the following agreements were made related to functionality aspects:

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

In this contribution, we discuss our view on some functionality related aspects.

# Discussions on Ambient IoT

## Functionality aspects

What functionality that is required for A-IoT devices are being discussed, and some agreements have been made on functionality that is not going to be supported, but some aspects need to be further discussed

Proposal 1: Below we propose a list for further discussion.

- BSR:

  - Even if no legacy BSR should be supported, is there any need for some information about amount of uplink data?

  - There might be mechanism similar to happy bit in UMTS.

- Duplication discard and sequence number:

  - Even if RLC like retransmission is not to be supported, how to handle if same message needs to be sent again?

  - One option is that the application layer retransmit the message.

- Segmentation:

  - For the case the A-IoT device cannot transmit all available data due to lack of energy and the need for harvesting, the option could be for the device to send segments of the data, and continue when enough harvesting has been done.

- Capabilities:

  - Is there a need and if so, a way to provide capabilities, e.g., device type (1,2a,2b?

- Repetition:

  - Instead of retransmission based on a NACK message, could the A-IoT device be configured to transmit the same message/data multiple times.

  - It seems RAN1 is considering block-size repetition

  - This may not be relevant for a low-power device.

- Visibility of inventory/command information at the AS layer

  - To what extent does the reader need to be aware of whether the trigger message is of inventory or command type?

  - As per SA2 Solution 3 [3], the proposal is “Whether the Reader needs to perform an Inventory before sending the command to the AIoT devices is a RAN2 decision” Though even if this would be upto RAN2, not clear if the information is needed on the AS layer between the Reader and the A-IoT device.

- Control plane/user plane related functionality

  - Is RRC like layer and functionality needed, or where to allocate relevant control functionality?

- CW configuration

  - How to handle the control and configuration of CW or excitation signal to be used by the A-IoT device when responding to the DL trigger message.

  - RAN1 is also discussing single-tone and using two unmodulated single-tones waveform, how to indicate to A-IoT device what to use?

  - Power control of CW node. A passive device (type 1 or 2a) will have little ability to power control its backscattered transmission and power control can be achieved by controlling the power of the CW

  - Turning on / off the CW node. When there are multiple AIoT devices in an area, a degree of spatial multiplexing can be achieved by activating different CW nodes in the coverage area. This can ease power control and capacity constraints in the system.

- AS layer above MAC or similar to Wifi. Furthermore, the more general question is then whether there is a need for any layer(s) above MAC and what functionality it should contain.

  - The AS layer could be a general purpose layer above PHY, similar to Wifi, 802.11 MAC layer and LLC layer

  - Legacy MAC layer is basically reused, and remaining needed functionality is placed in a new layer above MAC.

# Summary

In this contribution, we have discussed our views on aspects related to, random access, procedures.

Proposal 1: Below we propose a list for further discussion.

- BSR:

  - Even if no legacy BSR should be supported, is there any need for some information about amount of uplink data?

  - There might be mechanism similar to happy bit in UMTS.

- Duplication discard and sequence number:

  - Even if RLC like retransmission is not to be supported, how to handle if same message needs to be sent again?

  - One option is that the application layer retransmit the message.

- Segmentation:

  - For the case the A-IoT device cannot transmit all available data due to lack of energy and the need for harvesting, the option could be for the device to send segments of the data, and continue when enough harvesting has been done.

- Capabilities:

  - Is there a need and if so, a way to provide capabilities, e.g., device type (1,2a,2b?

- Repetition:

  - Instead of retransmission based on a NACK message, could the A-IoT device be configured to transmit the same message/data multiple times.

  - It seems RAN1 is considering block-size repetition

  - This may not be relevant for a low-power device.

- Visibility of inventory/command information at the AS layer

  - To what extent does the reader need to be aware of whether the trigger message is of inventory or command type?

  - As per SA2 Solution 3 [3], the proposal is “Whether the Reader needs to perform an Inventory before sending the command to the AIoT devices is a RAN2 decision” Though even if this would be upto RAN2, not clear if the information is needed on the AS layer between the Reader and the A-IoT device.

- Control plane/user plane related functionality

  - Is RRC like layer and functionality needed, or where to allocate relevant control functionality?

- CW configuration

  - How to handle the control and configuration of CW or excitation signal to be used by the A-IoT device when responding to the DL trigger message.

  - RAN1 is also discussing single-tone and using two unmodulated single-tones waveform, how to indicate to A-IoT device what to use?

  - Power control of CW node. A passive device (type 1 or 2a) will have little ability to power control its backscattered transmission and power control can be achieved by controlling the power of the CW

  - Turning on / off the CW node. When there are multiple AIoT devices in an area, a degree of spatial multiplexing can be achieved by activating different CW nodes in the coverage area. This can ease power control and capacity constraints in the system.

- AS layer above MAC or similar to Wifi. Furthermore, the more general question is then whether there is a need for any layer(s) above MAC and what functionality it should contain.

  - The AS layer could be a general purpose layer above PHY, similar to Wifi, 802.11 MAC layer and LLC layer

  - Legacy MAC layer is basically reused, and remaining needed functionality is placed in a new layer above MAC.

# References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- Minutes RAN2#126

- 23.700-13-v040