3GPP TSG RAN WG2 Meeting #127               	R2-24006615

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item: 	8.2.3

Source:		Sony

Title:			Considerations on paging for Ambient IoT

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version of the SID for Ambient IoT in NR was approved including the following objectives [1].

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies. -

At RAN2#125bis [6], the following agreements were made:

General aspects:

Agreements - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2. - RAN2 will support two use cases, “inventory” and “command”. The definition, detailed wording is FFS - Baseline procedure: Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS Step C: The device may perform the data communication with the reader as needed,: Details FFS - We will study the support of both “inventory” and “command” in the same procedure. - FFS if Initial Trigger Message can also include “command”. - RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input.

Control plane:

Agreements

- RRC connection management is not supported.  FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

- RRM L3 measurement reporting is not supported by Ambient IoT devices.

- RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

- Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

- RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

User plane:

Agreement

- SDAP is not supported for UP protocol stack.

- PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

- RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

- No HARQ and RLC AM

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

- RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

Paging:

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

Random Access:

Agreement

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

At RAN2#126 meeting [7], the following agreements were made:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In this contribution, we discuss our view on some RAN2 related aspects related to paging,procedure.

# Discussions on Ambient IoT

The Rel-18 RAN led study on Ambient IoT resulted in a Technical report [2], -outlining a set of different design targets, and requirements with a list of corresponding functionality.

## RAN design targets,

In [2], the following table summarizes RAN specific requirements and related functionalities.

Table 6.2-2: Required RAN functionality set #2: for supporting other requirements

| Requirement | Functionality |
| --- | --- |
| Device management | - RAN aspects of identification, activation/deactivation, and other management functionalities of Ambient IoT devices and other involved devices (e.g. readers) if applicable, and related signalling to/from the CN if any/needed |
| Security* | - Authentication (when needed), encryption, data integrity, authorization (when needed) |
| Mobility | - Mobility management (at least cell selection/re-selection -like function) for device C - Handling for Devices A and B |
| Interference management and coexistence | - Interference management/coordination scheme - Potential full duplex capability of BS/UE, including self-interference suppression, may be required for BS/UE to communicate with Device A and Device B, if carrier wave transmission and backscatter reception is performed simultaneously at least on the same band by the same BS/UE. - Coexistence with existing and adjacent network infrastructure, and possibility to reuse existing network deployments or use new network deployments. |
| CN connectivity | - RAN functionality for Ambient IoT to support CN (when present), with possibility of potential lightweight protocol stack architecture and simplified signaling procedures. |
| Compatibility among connectivity topologies | - From the perspective of the Ambient IoT device, strive for operation to be agnostic to RAN connectivity topologies. |

*NOTE:	This does not necessarily mean security has RAN impact, further study is needed.

Device management includes aspects related to e.g., identification. Identification may not be relevant for system information, but surely for Paging, Random access, as well as performing the security procedures.

Identification and registration are being discussed and specified in SA2 [3], and the whole registration and initial access procedure are handled by the Core Network (CN) functionality, and so RAN and RAN2 involvement may be limited.

These functionalities are likely to affect how RAN2 should design e.g., the triggering and random access procedures.

SA2 are concluding their study phase on Ambient IoT, related to amongst other A-IoT ID [7], and have not done any down selection or conclusion on any proposed solutions, and SA2 has sent LS to SA3 on a number of aspects related to identification and security.

see LS to SA3 [8]

It seems no agreements or conclusion have been made in any of the SA2 or SA3 groups.

Observation 1: No conclusions have been made neither in SA2 nor SA3 on how to handle security aspects for UE-id and identification.

Proposal 1: Wait for SA2 and SA3 progress related to usage of UE-identification and security.

## Paging (including content)

In legacy cellular systems, for the Network to reach the UE normally paging is used, for which a UE-id is needed. The UE-id is not only used for identification, but also to group the UEs in different wake-up groups for the system to page the UE without having to wake-up all Ues and for the Ues to save power, not having to listen for paging continuously.

In the case of Ambient IoT, the traffic assumptions have been defined as Device Terminated (DT), and Device Originated (DO). DO traffic only consider DO-DTT, Device Originated, Device Terminated Trigger, which means that the network wakes up the device but for expected or required uplink traffic. This could e.g., be sensor data that should be reported.

Type of trigger message:

Except for the initial registration procedure, two types of functionalities should be supported [3],

- Inventory

- Command

For example, when a service request is sent from e.g., Application Function to the Core Network (CN), the CN can trigger the reader(s) to make an inventory of a group of Ambient IoT devices. The reader then broadcasts a trigger message containing some sort of identification triggering all, a subset or group of devices or even a single device to respond.

Observation 2: The trigger message needs to contain information of what type of trigger message it is, (inventory or command).

Identification:

What devices that should be triggered should be contained in the trigger message, though the size of the trigger message may need further discussion, e.g., whether the list of devices should be similar to a paging record, whether the device id can be shorter, and whether a type of device or group id be used as an identifier.

In RAN TR 38.848 [2], the RAN design targets are defined, and for A-IoT devices, the maximum message size to be received or to be transmitted is approximately 1000 bits: This is the upper limit for number of bits to be used and can be a baseline for defining how many bits are preferred by RAN2 to be used for subgrouping.

At latest RAN2 meeting [6], it was agreed to further study multiple alternatives on how many devices that can be addressed in the trigger message.

•	a message containing an ID of a single A-IoT device.

•	a message containing a group ID that maps to multiple A-IoT devices.

•	a message that does not contain an ID, i.e., addressed for all devices that can receive the AioT message.

•	a message containing multiple IDs of A-IoT devices.  Need to confirm the need for this use case based on SA2 discussion.

For the case of addressing only a single A-IoT device, the size of the id could be larger…

For the case of addressing a group of A-IoT devices, the size of the id could be the same as if addressing only a single device, though it is intended for a group.

For the case of addressing multiple individual devices, the size of the id may have to be shorter to fit several different Id´s in one trigger message.

The simplest case is addressing all devices, in which case no Id would be needed.

Observation 3: The message needs to contain identification, and the size of the ID depends on the number of ID´s that is to be included in a single trigger message.

Proposal 2: Further discuss the size and maximum number of Device Ids in a single trigger message,

Configuration for UL transmission:

Furthermore, unless the Ambient IoT devices have received some previous configuration, or have been pre-configured, some information about resources to be used for responding to the trigger needs to be conveyed. This could also include what type of random access procedure to use like 4-step or 2-step random access.

Configuration of resources could be general and explicit in the case of Contention Based Random Access (CBRA), i.e., common resources will be given to all devices, e.g., via system information.

For any UE specific configuration, the configuration could be part of the first trigger message, or as a subsequent message/transmission, e.g., multiple command massages could be transmitted.

Proposal 3: Further discuss how to convey uplink configuration for performing Random Access like procedure.

For the case of Contention Free Random Access (CFRA), the device may have to be configured with resources to use. This could be done either by explicitly appending resource configuration in the trigger message, or by implicitly indicating some preconfigured resources to be used, (see also our related tdoc on Random access [10].

Proposal 4: Further study the alternatives of explicit and implicit resource configuration.

DRX and Subgrouping:

The devices with limited energy storage would have to carefully save energy and may benefit from having the possibility to not constantly have to be able to react to a trigger message. E.g., when the device is out of energy or is performing harvesting, the device is either not able to respond, or occupied with energy harvesting.

It was discussed and agreed in the last RAN2#125bis meeting to not support legacy paging, but not concluded on monitoring occasions for trigger message.

Some sort of DRX functionality could be relevant, and if so, the need for some sort of subgrouping based on the identity of the device, or potentially some other grouping mechanism, may also be relevant to avoid triggering all or any device as discussed in chapter above.

At least device types with energy storage labeled as “ii” under bullet A in the SID, (or device 2a/2b [4]) above can be considered for supporting DRX.

Proposal 5: For paging, further discuss the need for device identification, DRX functionality and subgrouping.

Multiple Readers scenario for Paging messages:

It has been agreed that RRC connection management and periodical system information (SIB and MIB) are not supported by AIoT devices. This may mean that a device is not connected to a specific cell. Hence, as discussed in [9], there are issues on multiple readers scenario for paging messages where a device is between multiple close-by Readers as shown on Figure 1. If we assume that Core Network (CN) initiates the paging message via gNB/Reader for the Device or a group of Devices, it is likely that paging messages may be sent to multiple close-by readers as CN does not know the exact location of a specific device. Note that there are cases where Device IDs may not be included, for example for the case of “Inventory” which is intended for everyone.

Therefore, RAN2 should discuss definition of the device behavior when the device receives more than one triggering message in a period from different readers. For example, paging ID included in the triggering message can indicate that the triggering message is coming from a different Reader (i.e., paging ID is reader specific).

Figure 1. A-IoT device locating in between Readers.

Proposal 6: For paging, RAN2 to discuss definition of the device behaviour when the device receives more than one triggering paging message in a period from different readers.

# Summary

In this contribution, we have discussed our views on aspects related to paging.

Observation 1: No conclusions have been made neither in SA2 nor SA3 on how to handle security aspects for UE-id and identification.

Proposal 1: Wait for SA2 and SA3 progress related to usage of UE-identification and security.

Observation 2: The trigger message needs to contain information of what type of trigger message it is, (inventory or command).

Observation 3: The message needs to contain identification, and the size of the ID depends on the number of ID´s that is to be included in a single trigger message.

Proposal 2: Further discuss the size and maximum number of Device Ids in a single trigger message,

Proposal 3: Further discuss how to convey uplink configuration for performing Random Access like procedure.

Proposal 4: Further study the alternatives of explicit and implicit resource configuration.

Proposal 5: For paging, further discuss the need for device identification, DRX functionality and subgrouping.

Proposal 6: For paging, RAN2 to discuss definition of the device behaviour when the device receives more than one triggering paging message in a period from different readers.

# References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- TR 38.848, Study on Ambient IoT (Internet of Things) in RAN (Release 18)

- TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)

- RAN1#116, Chair's notes RAN1#116 9.4 R19 Ambient IoT v07

- Minutes RAN2#126

- Minutes RAN2#127

- TR 23.700-13

- S2-2407231

- R2-2405603, “Discussion on initial trigger message (paging-like message) on Ambient IoT”, RAN2#126

- R2-2406616, Considerations on random access aspects for Ambient IoT