3GPP TSG RAN WG2 Meeting #127               	R2-2406616

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item: 	8.2.4

Source:		Sony

Title:			Considerations on random access aspects for Ambient IoT

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version of the SID for Ambient IoT in NR was approved including the following objectives [1].

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies. -

At RAN2#125bis [2], the following agreements were made:

Random Access:

Agreement

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

At RAN2#126 [3], the following agreements were made:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In this contribution, we discuss our view on some RAN2 related aspects related to random access procedure.

# Discussions on Ambient IoT

## Random access

Random access is used for a device to initiate a connection setup. The random access can either be initiated by the network, and then via paging, so called Mobile Terminated (MT), or by the device itself, Mobile Originated (MO).

In the RAN study the traffic assumptions have been defined as Device Terminated (DT), and Device Originated (DO). DO traffic only considers DO-DTT, Device Originated, Device Terminated Trigger, which means that the network wakes up the device but for expected or required uplink traffic. This could e.g., be sensor data that should be reported.

The type of trigger can be Inventory or Command as discussed under the paging agenda item.

Considering the low energy type of devices, the signalling should be kept to a minimum.

The 4-step RACH procedure includes a number of signalling steps and would require more energy and level of functionality in the device.

The 2-step RACH procedure is similar to the 4-step RACH, and content wise there is not much difference except for latency, which anyway is of less importance compared to energy.

Observation 1: Both 2-step and 4-step RACH procedure includes a fair amount of signalling for a low energy device.

In the case that the trigger is for a specific device, the reader could trigger the device with its Id, and the device could respond with a single message, like  a one-step like procedure. This could be possible if resources in which the device can respond in, is included in the trigger message. Due to the short distance between the reader and AIoT device no consideration needs to handle any timing advance.

Proposal 1: If the trigger is for a specific device, it should be possible for the reader to trigger the device to respond in one step, with no or low risk of collision.

In case multiple known devices are triggered in the same message, each device could be configured with individual resources to respond in in order to avoid any risk for collision. This is being discussed in the paging agenda item and would be relevant for contention free access.

Proposal 2: In case multiple known devices are triggered in the same message, each device could be configured with individual resources to respond in to avoid any risk for collision.

In order to allocate a proper number of resources, the content and size of the response needs to be determined. If the trigger message is of type “Inventory”, the response would contain the device Id and optionally “data”, and if the trigger message is of type “Command”, the response would contain “device Id” and “data”.

Proposal 3. The configuration of UL resources depends on the content/size of the response message.

Contention Free:

For the contention free alternative, it was agreed to further study both single and multiple device cases [3].

Basically, the procedure is a one step procedure, with contention free access for one or multiple resources which have been given device specific resources. By one step random access procedure it would mean that the device would transmit a single message in specific uplink resources, with no response or acknowledgement from the reader.

Proposal 4: For the contention free case, single or multiple devices transmit a single message in specific uplink resources, with no response or acknowledgement from the reader.

# Summary

In this contribution, we have discussed our views on aspects related to, random access, procedures.

Observation 1: Both 2-step and 4-step RACH procedure includes a fair amount of signalling for a low energy device.

Observation 2. If the device ID is not known, e.g., when doing inventory for initial registration, 4-step CBRA random access procedure may be needed, including the use of a temporary device Id.

Proposal 1: If the device-id is known, it should be possible for the reader to trigger the device to respond in one step, with no or low risk of collision.

Proposal 2: In case multiple known devices are triggered in the same message, each device could be configured with individual resources to respond in to avoid any risk for collision.

Proposal 3. The configuration of UL resources depends on the content of the response message.

Proposal 4: For the contention free case, single or multiple devices transmit a single message in specific uplink resources, with no response or acknowledgement from the reader.

# References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- Minutes RAN2#125bis

- Minutes RAN2#126