3GPP TSG RAN WG2 Meeting #127               	R2-2406619

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item: 	8.4.4

Source:		Sony

Title:			Considerations on LP-WUS/WUR in RRC Connected mode

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version for the WID for Low Power Wake-up Signal and Receiver was approved including the following objectives [1].

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline Specify necessary RRM requirements-

At RAN2#126 [3], the following agreements were made related to LP-WUS in RRC Connected mode:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In this contribution, we discuss our view on some RAN2 related aspects related to, LP-WUS and DCP and LP-WUS in relation to drx_onDuration, and UE capabilities

# Discussions on LP-WUS/WUR in RRC Connected mode

## LP-WUS related to drx_onDuration

In [2], it was discussed that there could be a benefit for power saving purpose for XR traffic to let the UE monitor LP-WUS instead of PDCCH monitoring while the drx_onDurationTimer is running, i.e., during C-DRX active time. The benefit would also be to handle unpredictable jitter, and hence reduce the uncertainty period of PDCCH monitoring. Though, since the Connected mode DRX periodicity is more statically configured, and that jitter potentially could also imply early or late arrival of packets, even outside PDCCH monitoring period, the potential wake-up signal should preferably not be related to the PDCCH monitoring period.

If the LP-WUS can be received during the PDCCH monitoring period, i.e., after the drx-onDuration timer is started, the location of the PDCCH monitoring period may depend on the transition time for the Main radio to get ready to receive on PDCCH after being triggered by the LP-WUR. Basically, it would be more beneficial to not start the drx-onDuration timer until the Main Radio is triggered by the LP-WUR, hence adjusting the PDCCH monitoring window according to the jitter, and thereby also being able to configure a short PDCCH monitoring period to receive the DL data.

Observation 1: It is preferable to start the drx-onDuration timer first after the Main Radio (MR) is triggered by LP-WUS/WUR.

It has been reported in TR-38.869 [2] that even a transition time 6 ms (for the case of light sleep), it may be too long for the XR jitter that has the range of +/- 4 ms.

Proposal 1: RAN2 to further discuss adjusting the start of the PDCCH monitoring period and the drx-onDurationTimer for the MR based on the trigger by LP-WUS/WUR.

## UE capabilities

At RAN1#117 meeting [4], it was agreed to support UE capability reporting according to the following;

Agreement

For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

- FFS: exact value(s) of the minimum time gap

- FFS: support of multiple minimum time gaps

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

Proposal 2: RAN2 to define capabilities according to RAN1 agreements.

# Summary

In this contribution, we have discussed our views on aspects related to.LP-WUS and DCP and LP-WUS in relation to drx_onDuration and UE capabilities

Observation 1: It is preferable to start the drx-onDuration timer first after the Main Radio (MR) is triggered by LP-WUS/WUR.

Proposal 1: RAN2 to further discuss adjusting the start of the PDCCH monitoring period and the drx-onDurationTimer for the MR based on the trigger by LP-WUS/WUR.

Proposal 2: RAN2 to define capabilities according to RAN1 agreements.

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TR 38.869 v18.0.0, Study on low-power wake up signal and receiver for NR

- Minutes RAN2#126

- Minutes RAN1#117