3GPP TSG-RAN WG2 Meeting #127  	                            R2-2406622

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.5.3

Source: 	Sony

Title:	Further study on Case 3 in on-demand SIB1

Document for:	Discussion

# Introduction

In RAN2#126 meeting, the agreements on on-demand SIB1 for IDLE/INACTIVE are as following:

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

In this contribution, we discussed some leftover issues in order to conclude the study on on-demand SIB1 for NES.

# Discussion

2.1 Whether/how to support Case 3 in on-demand SIB1

In RAN1#117 meeting, it was agreed that Case 2 (UE gets UL WUS configuration from Cell A, and send on-demand SIB1 request to and get reply from NES cell) is feasible and further study Case 3 (UE gets UL WUS configuration, send on-demand SIB1 request to and get reply from Cell A).

In Case 3, as the Cell A will respond to the UE’s request on NES cell’s on-demand SIB1, the NES cell can prolong its energy saving status until UE initiates RACH to the NES cell and this will maximize the energy saving performance of the NES cell. On the other hand, this may impose extra load and signalling overhead on Cell A, as it has to provide the on-demand SIB1 on behalf of some of the NES cells.

One challenge in Case 3 could be the mis-matched SIB1 information between NES cell and Cell A, especially in the case that the NES cell and Cell A are provided by different vendors. The delay between the information sharing between NES cell and Cell A may lead to the out-of-date NES SIB1 is provided in Cell A, and this may incur the RACH failure or access failure of the UE due to the update of NES cell SIB1 without a timely notification to Cell A.

Observation 1: Case 3 (UE gets UL WUS configuration, send on-demand SIB1 request to and get reply from Cell A) may maximise the energy saving performance of NES cell while on the other hand imposing extra load and signalling overhead on Cell A.

Observation 2: In Case 3, the mis-matched on-demand SIB1 between NES cell and Cell A may lead to the access failure when the updated on-demand SIB1 is not timely shared between NES cell and Cell A.

For a UE initially camped on a Cell A in Case 3, it can acquire the UL WUS configuration the same way as in Case 2. It may further get information regarding whether this Cell A can provide the on-demand SIB1 of certain NES cells and may acquire the on-demand SIB1 of those NES cells and store them until they are valid.

If the on-demand SIB1 of NES cell is provided as part of the on-demand SI of Cell A, it is envisaged that the legacy on-demand SI request procedure can be re-used. When the UE re-selects to one of those NES cells, it may initiate the RACH to NES cell while the stored on-demand SIB1 is still valid.

On the other hand, if the on-demand SIB1 of NES cell is provided as part of the SIB1 of Cell A, then the on-demand SIB1 request procedure discussed in Rel-19 NES should be used.

Proposal 1: The legacy on-demand SI request procedure is re-used for UE to acquire the on-demand SIB1 of NES cell from Cell A if it is provided as part of the on-demand SI of cell A. For a scenario where NES cell SIB1 is included in cell A SIB1, on-demand SIB1 request procedure is reused.

If the Case 3 is going to be supported, then the co-existence of Case 2 and Case 3 should be considered, where a Cell A can provide only the UL WUS configuration of certain NES cells while can provide UL WUS configuration as well as on-demand SIB1 of another NES cells. Then from a UE’s point of view, it should be notified to which cell it should send on-demand SIB1 request, where in Case 2 it should send request to NES cell and in Case 3, it should send request to Cell A.

Proposal 2: RAN2 to discuss if both case 2 and case 3 are to be supported.

# Conclusion

We have observations as following.

Observation 1: Case 3 (UE gets UL WUS configuration, send on-demand SIB1 request to and get reply from Cell A) may maximise the energy saving performance of NES cell while on the other hand imposing extra load and signalling overhead on Cell A.

Observation 2: In Case 3, the mis-matched on-demand SIB1 between NES cell and Cell A may lead to the access failure when the updated on-demand SIB1 is not timely shared between NES cell and Cell A.

We propose RAN2 to discuss following proposals:

Proposal 1: The legacy on-demand SI request procedure is re-used for UE to acquire the on-demand SIB1 of NES cell from Cell A if it is provided as part of the on-demand SI of cell A. For a scenario where NES cell SIB1 is included in cell A SIB1, on-demand SIB1 request procedure is reused.

Proposal 2: RAN2 to discuss if both case 2 and case 3 are to be supported.