3GPP TSG-RAN WG2 Meeting # 127                        	R2-2406624

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item: 	8.7.1.1

Source:		Sony

Title:	Views on LSs for SA2 and RAN3

Document for:	Discussion & Decision

# Introduction

RAN2 has received LSs from SA2 and RAN3. We provide our views on the questions addressed to RAN2.

# Discussions

S2-2407351:

SA2 has started to evaluate solutions for key issues for the study of XR and Media Services Ph2 (TR 23.700-70). SA2 thanks the LS replies for XRM Ph2 from RAN2, RAN3 and SA4 and SA2 would like to further coordinate on the following aspects: - Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension? - Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server? - Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

On question 1, SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

We think it is good to have this information and mainly for RAN3 to decide.

RAN2 can discuss if such information is needed or how to signal it for UL. RAN2 specified multiple PUSCH resources in a CG periodicity and to indicate unused resources in Rel-18. Also, burstArrivalTime and jitterRange were specified to indicate the expected arrival time of first packet of data burst and associated jitter range. But this information may not provide dynamic changes of the periodicity. If UE also provide similar information about dynamic changes of the periodicity to the gNB then gNB can use this information to manage resources and scheduling uplink accordingly.

Proposal 1: it is left for RAN3 to decide on dynamic changes of periodicity approach for DL. RAN2 to discuss whether such information will be useful for the gNB for scheduling UL.

On question 2, Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

The question is not clear whether it refers to UL or DL or both. This is very much similar to question 1 for both UL and DL that DL should be decided by RAN3 and UL should be discussed in RAN2 if such information would be useful for the gNB.

Proposal 2: it is left for RAN3 to decide on time to next burst for DL. RAN2 to discuss whether such information will be useful for the gNB for scheduling UL.

R3-243957

RAN3 is currently discussing XR DC signaling enhancements for various bearer types. In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured. Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

For an example scenario whereby:

MN indicates discard = deactivated

SN indicates discard =activated– SN is congested more than MN

If UE starts discarding based on an indication from SN, which is congested but MN is not congested, then we think there is no issue because application is able to sustain loss of these types of packets. Since packet is discarded in PDCP layer, MN may mistakenly consider UE radio conditions have gone bad due to lost packets. But we assume that there is a congestion indication information exchange between SN and MN so MN implementation may use this information along with PSI configuration for the DRB.

In other words, RAN3 should be able to address this issue without UE involvement and UE may discard in this case even if one of the receiving nodes is not congested. Also, network could configure UL data threshold for split traffic according to the congestion level. RAN2 can indicate that discard solution is on best-effort basis only.

Proposal 3: RAN2 does not think that UE involvement is required in this case and UE may discard packets if one of the receiving nodes is not congested.

# Summary

In this contribution, we have the following proposal for RAN2 to agree the following proposal:

Proposal 1: it is left for RAN3 to decide on dynamic changes of periodicity approach for DL. RAN2 to discuss whether such information will be useful for the gNB for scheduling UL.

Proposal 2: it is left for RAN3 to decide on time to next burst for DL. RAN2 to discuss whether such information will be useful for the gNB for scheduling UL.

Proposal 3: RAN2 does not think that UE involvement is required in this case and UE may discard packets if one of the receiving nodes is not congested.

# References

- RP-234057, “New WID: XR (eXtended Reality) for NR Phase 3,” RAN#102.

- TR 23.700-60, SA2, Study on XR (Extended Reality) and media services

Annex:

psiIdentification

Indicates whether the UE is able to identify PSI(s) for the QoS flow. This field shall only be set to true if pduSetIdentification is also set to true (or was set to true previously for the same QoS flow). If set to true, the UE is able to identify PSI(s) for the associated QoS flow, otherwise, the UE is not able to do so. Before receiving this indication, the network assumes the value is set to false.

In case of congestion, the gNB may use the PSI for PDU set discarding. For uplink, dedicated downlink signalling is used to request the UE to apply a shorter discard timer to low importance PDU Sets in PDCP.

NOTE 2:	How PDU Sets are identified as low importance is left up to UE implementation. When a PSI is available, it can be used according to the guidelines specified in TS 26.522 [58].

#### PSI-Based SDU Discard Activation/Deactivation MAC CE

The PSI-Based SDU Discard Activation/Deactivation MAC CE is identified by MAC subheader with an one-octet eLCID as specified in Table 6.2.1-1b.

It has a fixed size and consists of one octet defined as follows (Figure 6.1.3.73-1):

-	Di: This field indicates the activation/deactivation status of the PSI-based SDU discard of DRB i, where i is the ascending order of the DRB ID among the DRBs configured with discardTimerForLowImportance and with RLC entity(ies) associated with this MAC entity. The Di field set to 1 indicates that the PSI-based SDU discard shall be activated for DRB i. The Di field set to 0 indicates that the PSI-based SDU discard shall be deactivated for DRB i.

Figure 6.1.3.73-1: PSI-Based SDU Discard Activation/Deactivation MAC CE