3GPP TSG-RAN WG2 Meeting # 127                        	R2-2406625

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item: 	8.7.2

Source:		Sony

Title:	Need for MMSID and DRB mapping

Document for:	Discussion & Decision

# Introduction

A new WID was agreed in RANP_102 with following study objectives for multi-modality[1]:

The Rel-19 XR ph3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. - Note: Check in RAN#105 (check also other WG involvement if needed).

RAN2_125bis agreements for multi-modality are:

- For the purpose of study, RAN2 assumes that UE and gNB have some kind of multi-modal information. FFS what information is needed/useful, e.g. just multi-modal ID, association between the flow, synchronization requirement etc. - RAN2 will study both UL and DL directions based on the assumption of multi-modality association knowledge at RAN/UE - RAN2 will focus on analysing potential usage and benefits (e.g. in terms of capacity and power saving) of multi-modal association knowledge - Areas to study include: synchronization between the flows, FFS impact on QoS insurance and other areas - RAN2 assumes that traffic of different modals having different QoS requirements is mapped to different QoS flows - For different XR traffic flows belonging to the same Multi-modal service and having different QoS requirements, it should be possible to provide differentiated QoS handling over the air. RAN2 should study if that is possible with current mechanism or new ones are needed - Existing QoS flow to DRB mapping framework is used as a baseline, i.e. up to gNB how to map QoS flows to DRBs

- RAN2#126 agreements:

- Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. - Ask SA2 whether/what information could be provided to RAN - Clarify in the LS how RAN2 considers to use this information, e.g. coordinated handling of multi-modal flows. - Can consider while drafting the LS to indicate the potential benefits

LS was agreed to SA2 to look into multi-modality aspects in (R2-2405782). No reply LS was agreed by SA2 and RAN2_127 is the last meeting for RAN2 to conclude the study phase. In this contribution, we provide our views on the need of MMSID for multi-modality flows. We also think that RAN2 should explicitly agree the mapping between multi modal QoS flows and DRBs.

# Discussions

Multi-modality information at RAN:

It was agreed during RAN2_125bis that RAN2 assumes that UE and gNB have some kind of multi-modal information. FFS what information is needed/useful, e.g. just multi-modal ID, association between the flow, synchronization requirement etc.

Multi modal service ID (MMSID) is not available to RAN and eventually RAN cannot distinguish or coordinate between different QoS flows, being mapped to different DRBs. Also, when multi modal QoS flows are mapped to the same DRB then synchronisation requirements cannot be guaranteed since timing is dependent on when the data is scheduled, and the scheduler does not know the relation between the packets. Details are provided in our co-sourced tdoc [3] which was discussed during RAN2_126 and above agreements were made. If MMSID is available to RAN then for DL, gNB should be able to perform scheduling such that the synchronisation and the QoS coordination between DRBs associated with different IP flows is performed.

For UL, the UE is configured with logical channel priority (same priority for both DRBs in RRC configuration is possible today) such that multiple DRBs carrying multi modal flows are multiplexed to same MAC Transport Block and QoS and synchronisation requirements are met. These mappings and enhancements can be discussed later.

RAN or gNB could get this information about coordination between QoS flows and MMS ID either from AMF or from UE. We think that AMF providing this information is aligned to the design principle whereby QoS parameters are provided to the gNB by the core network and core network is a trustworthy node. However, this work should be done in RAN3 and SA2 during Rel-19. If this information is available from core network, then there is no need for UE to provide common session ID information to gNB.

Proposal 1: RAN2 assumes that multi modal service ID is received in RAN from the core network i.e. there is no need for UE to provide this information to the gNB.

Single vs multiple DRBs:

Agreements from RAN2_125bis on QoS management:

- RAN2 assumes that traffic of different modals having different QoS requirements is mapped to different QoS flows - For different XR traffic flows belonging to the same Multi-modal service and having different QoS requirements, it should be possible to provide differentiated QoS handling over the air. RAN2 should study if that is possible with current mechanism or new ones are needed - Existing QoS flow to DRB mapping framework is used as a baseline, i.e. up to gNB how to map QoS flows to DRBs

These three agreements are aligned to the current QoS design i.e. QoS flow to DRB mapping should be governed by QoS parameters and if multi modal flows have different QoS requirements then they could be mapped on to different DRBs. However, if QoS requirements are similar then these could be mapped to the same DRB. So, we think that both 1:1 and N:1 mapping between multi modal QoS flows and DRBs should be supported.

Proposal 2: Both 1:1 and N:1 mapping between multi modal QoS flows and DRBs is supported.

If above proposal is agreed, then there is one additional reason for the multi modal information to be available at RAN. This is due to the fact that there is a need for coordination between these multi modal flows if they are mapped to different DRBs.

# Summary

In this contribution, we have the following proposal for RAN2 to agree the following proposal:

Proposal 1: RAN2 assumes that multi modal service ID is received in RAN from the core network i.e. there is no need for UE to provide this information to the gNB.

Proposal 2: Both 1:1 and N:1 mapping between multi modal QoS flows and DRBs is supported.

# References

- RP-234057, “New WID: XR (eXtended Reality) for NR Phase 3,” RAN#102.

- TR 23.700-60, SA2, Study on XR (Extended Reality) and media services

- R2-2404549: RAN enhancements for Multi-Modality support, ZTE Corporation, Sanechips, China Telecom, Meta, Sony, China Unicom