3GPP TSG-RAN WG2 Meeting # 127 	                          R2-2406627

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item: 	8.7.5

Source:		Sony

Title:	RLC AM enhancements

Document for:	Discussion & Decision

# Introduction

RAN2_125bis agreements for RLC enhancements are:

We focus on RLC AM

RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR

RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

RAN2_126 agreements for timely retransmissions of RLC AM enhancements are:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

    - Autonomous retransmission (i.e., without status report) of PDUs based on some triggers (existing or new triggers can be considered)

    - Retransmission based on enhanced status report.

    - Retransmission based on enhanced polling.

- FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism.

Impact on capacity should be considered.

RAN2 focuses on the enhancements for UL traffic.

RAN2_126 agreements for avoiding unnecessary retransmissions RLC AM enhancements are:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

- Rx initiated approach.

- Tx initiated approach.

RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

For Tx initiated approach:

    - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs.

    - Tx side stops retransmit obsolete SDUs.

    - Rx side updates state variables according to the information from Tx side

For Rx initiated approach:

    - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

    - Tx side just processes the status report as in legacy.

    - FFS how Rx side determines that an SDU should be abandoned.

In this contribution, we provide our views on RLC AM enhancements more specifically how to to achieve timely retransmissions and how to avoid unnecessary retransmissions.

# How to achieve timely retransmissions for RLC AM

- In the last meeting, for achieving timely retransmissions on RLC layer for XR traffic (the delay critical RLC SDU), RAN2 short-listed few options for consideration as captured in the introduction. We discuss these options and compare them, for example from the system capacity impact point of view.

- Retransmission based on enhanced status report (i.e., Receiver initiated):

For RLC-AM in NR, UE performs the initial transmission of an RLC PDU. The UE triggers retransmission of an RLC PDU only if it receives an RLC status report from the gNB indicating the NACK for the RLC PDU. For RLC status reporting, the gNB can send a NACK status for an RLC PDU when detecting an SN gap between two consecutively received RLC PDUs. Such a procedure may cause a long delay for a retransmission of an RLC PDU, and it may not be suitable for an RLC PDU with a small packet delay budget (PDB). This is because, after such a long delay, the retransmission of the RLC PDU may arrive beyond the PDB of the PDU and, hence impacting the QoS.

Therefore, enhancing the RLC re-transmission for the operation of the RLC AM can be beneficial. To enhance the RLC re-transmission for an RLC PDU with small PDB, the delay gap between the initial transmission and the re-transmission of the RLC PDU can be reduced. One method is to allow the UE Polling is based on the sequence number (SN) rather than the number of PDUs. It means that the poll bit is always sent in a fixed SN (i.e., status PDU is sent periodically after certain number of SNs) as shown on Figure 1.

Figure 1. Status PDU is sent periodically after certain number of SNs.

The fixed SN is always known to both the transmitter and the receiver. Due to this, the receiver can proactively send a status report if it receives any PDU with a higher sequence number than the fixed polling SN. This is particularly useful in case the PDU containing a poll bit is received out of order or has been lost on the radio link and needs to be retransmitted. It allows the receiver to trigger a status report more quickly, and so any NACKed PDUs can be retransmitted more quickly, improving the overall performance as well as the processing benefit at the transmitter. In addition, the UE does not need to delay triggering of status report until certain conditions are met, i.e., the UE will send the status report immediately after certain number of SNs.

On the other hand, based on legacy RLC specification, the RLC Rx side may trigger the status report if there is a failure (i.e., detecting an out of order reception of an RLC SDU) after t-Reassembly timer has expired. Based on this, some companies proposed to set the t-Reassembly timer to smaller values to accelerate the status report, however, it is argued [3] that this will result pre-mature NACKs for some of the SDUs that are still being transmitted (or retransmitted by lower layers) or in other words, this will result increasing the number of false misdetections which can then increase the overheads across the network [4].

- Retransmission based on enhanced polling (i.e., Transmitter initiated):

Enhanced polling whereby the Tx side initiates to get some status report from the receiver side has also been discussed in the last meeting. Companies [7] highlighted that the current polling mechanism is only allowed under certain conditions. These conditions are “if PDU_WITHOUT_POLL >= pollPDU; or if BYTE_WITHOUT_POLL >= pollByte” and proposed to accelerate the polling mechanism for data with small delay budget (e.g., PDB/PSDB), for example by setting smaller values to the polling parameters: pollPDU, pollByte and t-PollRetransmit to trigger more frequent polling to the RLC receiver. In addition, Rx side parameters can also be set to smaller values as well such as t-Reassembly and t-StatusProhibit timers.

- Autonomous retransmission (i.e., Tx initiated without waiting Rx status report):

Another method is the autonomous retransmission (i.e., without waiting status report) of PDUs based on some triggers (existing or new triggers can be considered). The triggers/conditions for the autonomous retransmission can be, for example:

- For blind autonomous retransmission, triggered when the remaining time of high importance PDU is too short (i.e., below predefined threshold).

- For selective autonomous retransmission based on HARQ failure: Since Tx side knows the status of HARQ procedure (e.g., reaching max HARQ Retx), it can selectively do autonomous retransmissions. But it has been argued that for UL data transmissions there is no explicit HARQ ACK/NACK feedback in the downlink, and it takes time at the Tx side to know whether there was a failure (i.e., when NDI toggles for the corresponding HARQ process number). Moreover, it is unlikely RAN1 will define new explicit HARQ ACK/NACK feedback in the downlink due to substantial signalling overhead in the system as well as significant specification impact.

Table 1. Comparison of Options A, B and C described above:

| Options | Pros | Cons |
| --- | --- | --- |
| - Retransmission based on enhanced status report (i.e., Receiver initiated): | - Can accelerate the status report as it is periodic one-way communication (RxTx). - Receiver can periodically send the status report without Tx involvement. - No new signaling between Tx and Rx RLC entities. - No additional overhead in the network. | - Some specification impact on Rx side to define the configuration or triggering condition(s). |
| - Retransmission based on enhanced polling (i.e., Transmitter initiated): | - No new signaling/protocol between Tx and RX RLC entities. - No additional overhead in the network | - Two-way communications (TxRxTx), hence slower than option A and C. - Minor specification impacts such as adding smaller values to the polling parameters. |
| - Autonomous retransmission (i.e., Tx initiated without waiting Rx status report): | - Timely retransmissions, faster than options A and B - Selective Auto-Retx based on HARQ failure feedback has an advantage of deciding when to make retransmissions. | - Blind Auto-Retx wastes resources significantly, and it could lead to network congestion. - Selective Auto-Retx with HARQ failure feedback has significant impact on RAN1 specifications. |

Table 1 above shows comparison of three options for achieving timely retransmissions on RLC AM for XR traffic (i.e., per DRB), Option C (Selective Auto-Retx ) provides the fastest retransmissions, but it has a significant impact on RAN1 specifications, Option B is too slow as it needs two-way communications (TxRxTx), so it seems Option A is fast enough to provide immediate status report, and has advantage of reusing existing protocol between Tx and Rx entities. However, for Option A, there is small specification impact on the Receiver side to define the configuration or triggering condition(s).

Proposal 1: Periodic status report from the receiver based on fixed SN is introduced for RLC AM (i.e., status report is sent periodically after certain number of SNs).

# How to avoid unnecessary retransmissions for RLC AM

- The main issue is that RLC AM always performs retransmissions until max number of retransmissions is reached. However, if RLC PDUs are outdated after at least the first transmission then there is no need to do retransmissions as this causes resource wastage and possible congestion in the network. In the last meeting, for avoiding unnecessary retransmissions for RLC AM, RAN2 short-listed two approaches for consideration: Tx and Rx initiated approaches.

- For Tx initiated approach:

- The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs.

- Tx side stops retransmit obsolete SDUs.

- Rx side updates state variables according to the information from Tx side

- For this approach, if Tx side abandons some outdated SDUs due to elapsed delay budget (PDB), it must inform the receiver side the SN-gap (i.e., the SN of the discarded RLC PDUs) so that the receiver can update the state variables and timers accordingly. A proposal in [5] highlighted that in Rel-18 XR, RAN2 has specified PDCP SN gap reporting to reduce the reordering latency, hence similar reporting mechanism can be introduced to RLC AM to inform the receiver side. However, contribution in [6] stressed that this has significant impact on legacy UE implementation of RLC buffer management. Instead, the contribution proposed to limit the maximum number of retransmissions based on the ratio between packet delay budget and average round trip time of RLC transmissions, and as a result, the RLC transmitter would stop transmitting a PDU when its delay budget runs out. However, this further created another problem to RLF which needs to be solved as well.

Another reasonable option is that Tx side indicates ARQ/HARQ retransmissions are stopped after the current (re)transmission due to short remaining time (i.e., soon to be outdated) by employing signalling in MAC/PHY layer (e.g., UCI). Hence, afterwards, Rx side can send the status report to Tx side. Note that the UCI can be multiplexed with the retransmitted data (UCI+PUSCH).

- For Rx initiated approach:

  - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

  - Tx side just processes the status report as in legacy.

  - FFS how Rx side determines that an SDU should be abandoned.

- Regarding the Rx initiated approach, if the receiver knows whether PDUs under retransmissions are outdated (or soon to be outdated), it can inform the Tx side to stop transmitting outdated PDUs. Contribution in [7] proposed that Rx RLC entity does not send NACK (i.e., instead to send fake ACK) to the Tx RLC entity for the outdated data even if the data has not been received successfully. As a result, the Tx side stops retransmitting the outdated data. The triggering conditions could be a timer specified at the receiver. The timer values can be determined from QoS parameters (e.g., PDB/PSDB).

- Comparing these two approaches (Tx and Rx), the Rx-based approach seems simpler as it only modifies the receiver behaviour i.e., no spec changes for the signalling between the RLC peer entities as Tx side just processes the status report as in legacy. The underlying assumption/condition here is that RX side knows the information (or can work out) whether PDUs under retransmissions are outdated or not. However, if Rx side does not have the information, then Tx side can inform/indicate that RLC AM ARQ/HARQ retransmissions are stopped after the current (re)transmission where the indication is carried on MAC/PHY layer (e.g., UCI).

- Proposal 2: For avoiding unnecessary RLC AM retransmissions, we have two options:

- If RX side knows the information (or can work out) whether PDUs under retransmissions are outdated (or soon to be outdated), Rx initiated approach is introduced as it only modifies the receiver behaviour (i.e., no other changes), for example sending fake ACK for the outdated PDU to the Tx RLC entity.

- If RX side does not know the information (or cannot work out) whether PDUs under retransmissions are outdated (or soon to be outdated), Tx initiated approach is introduced where TX side informs/indicates that RLC AM ARQ/HARQ retransmissions are stopped after the current (re)transmission by employing signalling in MAC/PHY layer (e.g., UCI).

# Conclusion

In this contribution, we have provided our views on RLC AM enhancements more specifically how to to achieve timely retransmissions and how to avoid unnecessary retransmissions, and we have the following proposal for RAN2 to agree:

Proposal 1: Periodic status report from the receiver based on fixed SN is introduced for RLC AM (i.e., status report is sent periodically after certain number of SNs).

- Proposal 2: For avoiding unnecessary RLC AM retransmissions, we have two options:

- If RX side knows the information (or can work out) whether PDUs under retransmissions are outdated (or soon to be outdated), Rx initiated approach is introduced as it only modifies the receiver behaviour (i.e., no other changes), for example sending fake ACK for the outdated PDU to the Tx RLC entity.

- If RX side does not know the information (or cannot work out) whether PDUs under retransmissions are outdated (or soon to be outdated), Tx initiated approach is introduced where TX side informs/indicates that RLC AM ARQ/HARQ retransmissions are stopped after the current (re)transmission by employing signalling in MAC/PHY layer (e.g., UCI).

# References

- RP-234057, “New WID: XR (eXtended Reality) for NR Phase 3,” RAN#102.

- TR 23.700-60, SA2, Study on XR (Extended Reality) and media services

- R2-2404293, “RLC AM enhancements for XR”, Nokia, Nokia Shanghai Bell.

- R2-2404212, “Discussion on RLC AM Enhancements”, CANON Research Centre France.

- R2-2405493, “Discussion on RLC enhancements for XR”, Samsung.

- R2-2404177, “Discussion on RLC enhancements”, Qualcomm Incorporated.

- R2-2404652, “Views on RLC-AM Enhancements for Rel-19 XR”, Apple