3GPP TSG-RAN WG2 Meeting #127  	                            R2-2406628

Maastricht, Netherlands, Aug 19th – 23rd, 2024							      (Revision of R2-2404916)

Agenda Item:	8.8.4

Source: 	Sony

Title:	Broadcast service area signaling

Document for:	Discussion

# Introduction

In RAN2#126 meeting, we have agreements on support of broadcast service as following:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In this contribution, we discuss the broadcast service area signalling.

# Discussion

In Rel-18 NTN, the area of TN coverage is introduced to be broadcast to UE in SIB25. For each coverage area to be broadcast, there are an area ID, a reference location coordinate and a distance radius included.

It is still FFS regarding the geographical area formats to model the MBS service area, and there are two candidates:

Option 1: Circles (as in Rel-18 NTN for TN coverage).

Option 2: Geographical area information e.g., via polygon, to better approximate the intended shape of service area.

We think it is preferrable to re-use the Rel-18 NTN for TN coverage i.e. Option 1, and the reasons are listed in the following:

- Option 1 allows the re-use of legacy and save the specification work. Modelling the MBS service area as circle can work and the key is to broacast this information to indicate to the UE of the intended MBS service area within a NTN cell.

- The MBS service area change usually happens along the country border, due to the regulationary restrictions, and in this case, the polygon based model may not dramatically improve the approximation accuracy compared to circles. The polygon model may not always model the country border line in the more accurate way than the circle model, and at the same time it needs to include more coordinates information increasing the siganlling overhead.

Proposal 1: A new SIB is specified to include the MBS service area geographical information and model them in circles as in Rel-18 NTN TN coverage.

It was agreed that different satellite constellation types including earth moving cell with NGSO satellite will be included in the Rel-19 work. If a cell broadcast MBS service geographical area information, this information may change due to the movement of the satellite and it usually happens when the cell sweeps across the country border. On the other hand, a UE stays in one country only, it will know if it is allowed to access the MBS service or not. For the UE that is moving across the country border while the moving cell is sweeping across the country border, the UE may need to perform PLMN selection anyway. Therefore, we don’t see a strong motivation to discuss the impact due to the potentially frequent cell change.

Proposal 2: No need to address MBS service area geographical information broadcasting in country border areas.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: A new SIB is specified to include the MBS service area geographical information and model them in circles as in Rel-18 NTN TN coverage.

Proposal 2: No need to address MBS service area geographical information broadcasting in country border areas.