3GPP TSG-RAN WG2 Meeting #127  	                            R2-2406633

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.13.3

Source: 	Sony

Title:	Control plane procedure for multi-hop U2N relay

Document for:	Discussion

# Introduction

In RANP#104 meeting, a new WID on NR sidelink multi-hop relay in RP-241609 was agreed with following  objectives:

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

In this contribution, we discuss the initial consideration on control plane procedure in multi-hop U2N relay.

# Discussion

With the introduction of additional hops, the connection establishment procedure should be addressed. It is envisaged that the legacy RRC connection management for L2 U2N relay should be the baseline. For end to end connection management procedure with additional one or two hops, the legacy L2 U2N connection management procedure should be the baseline. While the connection establishment between intermediate nodes e.g., connection between first relay UE and last relay UE, part of the U2U connection establishment procedure can be (re)used. The details on the signalling procedure can be discussed later.

Proposal 1: The legacy L2 U2N connection management procedure is the baseline for multi-hop L2 U2N relay control procedure. U2U connection establishment procedure can be (re)used between first relay UE and last relay UE.

The additional hops may impose challenges on RLF management. Any RLF between the intermediate relay nodes may lead to the RRC connection re-establishment or relay re-selection on remote UE, therefore the RLF notification should be forwarded to the downstream relay node. For example, in additional one hop relay case, the Uu RLF notification between last relay and network should be forwarded to first relay and in additional two hop relay case, the RLF notification between last relay and second relay should be forwarded to first relay. The similar concept has been introduced in IAB and can be applied for multi-hop sidelink relay as well.

Proposal 2: Backhaul RLF notification in IAB should be introduced for multi-hop sidelink relay.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: The legacy L2 U2N connection management procedure is the baseline for multi-hop L2 U2N relay control procedure. U2U connection establishment procedure can be (re)used between first relay UE and last relay UE.

Proposal 2: Backhaul RLF notification in IAB should be introduced for multi-hop sidelink relay.