3GPP TSG-RAN WG2 Meeting #127                                    	R2-2406635

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.8.4

Source:	Qualcomm Incorporated

Title:	MBS broadcast service area information

Document for:	Discussion and Decision

# Introduction

In RAN#126, following agreements were made on MBS broadcast service.

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In this document, we provide details on information of MBS broadcast service area signaling and UE behavior.

# Discussion

Format of the MBS broadcast service area signaling

In the last meeting, it was also discussed whether beam level granularity can also be considered such as network implementation can handle the restriction of broadcast services in an area covered by a beam. The network may not transmit the MBS broadcast service in a beam which covers the area not intended for the broadcast service.

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

In our view, such solution alone is not practical as in most of the cases the beam size on earth surface would be large e.g., more than 50km. Then again such beam may cover large portion of area where service is intended and where service is not intended. Additionally specification change is needed to provide signaling information to UE in which beam the broadcast service is not provided as per WID objective.

In our view, such solution should be left to network implementation and from specification perspective we should try to define a simple solution that works in most deployment scenarios. Therefore, a geographical area information where broadcast service is intended can be provided and it is up to network not to transmit the broadcast service in a beam which is out of the provided geographical area.

a geographical area information where broadcast service is intended for can be provided and it is up to network not to transmit the broadcast service in a beam whose coverage is out of the provided geographical area.

Similar to TN coverage area list, the geographical area list can also be provided as a list of reference locations and corresponding cell radius.

Similar to TN coverage area list, the geographical area list can also be provided as list of reference locations with corresponding cell radius.

In addition to geographical area information, UE would also need association of geographical area and intended MBS broadcast service. Following the agreement from RAN2#126.

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In our view, UE can already map the MBS broadcast frequency to the associated MBS service through USD (User Service Description) or SIB21. It is to note that the UE needs to be aware of the frequency of the service it is interested in before it can receive the MBS broadcast service. The UE knows the interested frequency either from SIB21 or from USD. Therefore, it may be simple just to map the provided geographical area to one or more MBS broadcast service frequency.

The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

MBS FSAI (Frequency Selection Area Identity) is another way how the intended service area can be associated with MBS session. SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI. The UE can obtain the FSAI of the MBS service from USD. A frequency can cover multiple MBS service Areas and the MBS FSAI could be different for the different service area. In this case, the service area can be associated with the MBS session simply by FSAI.

Therefore, two simple ways to associate the intended service area with the MBS session are (1) listing frequency in each service area and (2) listing MBS FSAI in each service area. We think either option should work.

Each MBS broadcast service area in the list additionally includes one or more associated MBS broadcast frequencies or MBS FSAIs. Decide whether to include only MBS broadcast frequency or only MBS FSAIs.

Signaling of MBS broadcast service area

Similar to the list of TN coverage areas in SIB25, the list of MBS broadcast service areas can be broadcast. It may be possible to broadcast just a list of areas where the MBS broadcast service is not intended. For this the existing SIB25 can be extended, or a new SIB can be introduced. It is also possible to extend the existing SIB21 as it provides the information for service continuity such as neighbor frequency information.

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

... ,

[[

mbs-CoverageAreaInfoList-r19             MBS-CoverageAreaInfoList-r19       OPTIONAL,  -- Need R

]]

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-r17 ::= SEQUENCE {

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

MBS-CoverageAreaInfoList-r19 ::=   SEQUENCE (SIZE (1..maxMBS-AreaInfo-r19)) OF MBS-CoverageAreaInfo-r19

MBS-CoverageAreaInfo-r19 ::=       SEQUENCE {

mbs-FSAI-List-r19           MBS-FSAI-List-r17,

referenceLocation-r19       ReferenceLocation-r17,

distanceRadius-r19          INTEGER(0..65535)

}

-- TAG-SIB21-STOP

-- ASN1STOP

In our view, it may be simple just to extend the SIB25 or SIB21 and there is no need to add a new SIB. Since SIB21 already provides the association of FSAI and frequencies, it may be better we extend the SIB21 to provide list of MBS geographical service areas and make its association with MBS frequency/FSAI simpler as shown in text proposal above.

Extend existing SIB21 for broadcasting the list of MBS broadcast service areas.

UE behaviour

Figure 1 Example of intended and not intended MBS broadcast service areas

NTN cell coverage can be very large and MBS broadcast via such cell means the MBS broadcast service is available in the whole area of the cell coverage. For some cases the intended service area is expected to be smaller than the coverage of a cell, for example, the beam covers multiple territories or countries, some enhancements may be needed to make UEs aware of the intended service area.

As enhancements, the objective in WID is following:

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

We want to note that the WID objective does not mandate to specify any UE behaviour.

NR NTN WID objective only mandates to specify SIB signaling and not to specify the UE behaviour.

In RAN2#126 it was also agreed that there is no procedure define to prevent the reception of the content of the service outside of the intended service area. This would mean if any UE moved out of the intended service area, it may not stop receiving the content. This could also depend on user’s preference.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

However, we could define what is UE’s expected action when moving in or out of the intended service area i.e., how a UE is supposed to act upon receiving such information. For example, what are the actions for UE1 and UE2 in figure 1. A UE may move in and out of the intended service area without interruption on the MBS broadcast service reception. It is just about UE prioritizing the frequency of interest when in the intended service area.

In TS 38.304, following behavior is specified for selection MBS frequency, however, it is to note that such behavior is recommendation and not the mandated behavior.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled: 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20; 2) Either: - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.346 [20], or - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service. NOTE 0g: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the service the UE is interested in. If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition

Similarly, if we look at the TS 38.331, we also see some recommendation for UE’s expected behavior for different cases such as leaving the cell or stopping of MBS broadcast service.

### 5.9.3 Broadcast MRB configuration #### 5.9.3.1 General The broadcast MRB configuration procedure is used by the UE to configure PDCP, RLC, MAC and the physical layer upon starting and/or stopping to receive a broadcast MRB transmitted on MTCH, or upon modification of a configuration of a broadcast MRB received by the UE. The procedure applies to MBS capable UEs that are interested to receive or that are receiving an MBS broadcast service that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMTCH or searchSpaceMCCH. NOTE: How to perform a modification of a broadcast MRB which is already configured in the UE is left to UE implementation. #### 5.9.3.2 Initiation The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in. The UE applies the broadcast MRB release procedure to stop receiving a session of an MBS broadcast service. The procedure may be initiated e.g. upon stop of the MBS session, upon leaving the cell broadcasting the MBS service the UE is interested in, upon losing interest in the MBS service, when capability limitations start inhibiting reception of the concerned service.

In our view it is also ok to add the case where the UE initiates the broadcast MRB configuration procedure upon entering or leaving the intended service area as long as we follow the existing text in TS 38.331, i.e., considering it as optional. Therefore, the following FFS from RAN2#126 can be confirmed with “May”.

Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)

Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

In TS 38.331 section 5.9.3.2, “upon entering intended service area” is added as additional condition where the UE may initiate the broadcast MRB establishment procedure.

In TS 38.331 section 5.9.3.2, “upon leaving intended service area” is added as additional condition where the UE may initiate the broadcast MRB release procedure.

Similarly, the additional criteria based on location can be added in TS 38.304 for the MBS broadcast capable UE that is receiving or interested to receive an MBS broadcast service(s) to prioritize MBS service frequency during cell reselection evaluation process. For example, when USD provides multiple frequencies for the service the UE is interested in, currently it is up to UE to select one of the frequencies. Now with association of service geographical area information, it is easier for UE to decide which frequency to select.

In TS 38.304 for cell reselection evaluation, UE may prioritize the frequency if the UE is in the geographical Area associated with the frequency and UE is interested in or receiving the MBS service in that frequency.

# Conclusion

Following proposals are made.

Observation 1.	a geographical area information where broadcast service is intended for can be provided and it is up to network not to transmit the broadcast service in a beam whose coverage is out of the provided geographical area.

Proposal 1	Similar to TN coverage area list, the geographical area list can also be provided as list of reference locations with corresponding cell radius.

Observation 2.	The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

Proposal 2	Each MBS broadcast service area in the list additionally includes one or more associated MBS broadcast frequencies or MBS FSAIs. Decide whether to include only MBS broadcast frequency or only MBS FSAIs.

Proposal 3	Extend existing SIB21 for broadcasting the list of MBS broadcast service areas.

Observation 3.	NR NTN WID objective only mandates to specify SIB signaling and not to specify the UE behaviour.

Proposal 4	In TS 38.331 section 5.9.3.2, “upon entering intended service area” is added as additional condition where the UE may initiate the broadcast MRB establishment procedure.

Proposal 5	In TS 38.331 section 5.9.3.2, “upon leaving intended service area” is added as additional condition where the UE may initiate the broadcast MRB release procedure.

Proposal 6	In TS 38.304 for cell reselection evaluation, UE may prioritize the frequency if the UE is in the geographical Area associated with the frequency and UE is interested in or receiving the MBS service in that frequency.