3GPP TSG-RAN WG2 Meeting #127                                    	R2-2406636

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.8.5

Source:	Qualcomm Incorporated

Title:	Discussion on regenerative payload

Document for:	Discussion and Decision

# Introduction

Following agreements were made in the RAN2#126 meeting.

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

In this document, we further discuss the clarifications needed for the support of regenerative architecture.

# Discussion

RACH-less handover

In RAN2#126, it was discussed whether to RACH-less handover can be supported without optimization.

RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

In our view, the inter-gNB RACH-less handover is supported in NTN and it should work without any optimization when gNB is on board satellite given there is coordination between source and target gNBs, in this case, coordination between source and target satellite gNBs. It should be up to network implementation how/when the RACH-less resource should be considered valid for a given UE in the target cell taking into account the satellite switch time.

However, satellite switch time for HO is not specific uncertainty of regenerative architecture. Satellite switch time is needed for RACH-less HO in the transparent scenario as well. In both transparent and regenerative architectures, the RACH-less time-based CHO seems to be useful solution.

- RAN2 assume the inter-gNB RACH-less handover works without any optimization when gNB is on board satellite.

RRC Reestablishment and RRC_INACTIVE state

It was discussed whether any optimization needed for RRC reestablishment procedure or for the support of RRC_INACTIVE state in case of gNB onboard satellite, as source gNB will disappear and target may not have retrieved UE’s context. We think how target can retrieve UE’s context and be prepared beforehand should be out of RAN2 scope.

But specific to RAN2, we do not foresee any specific impact to UE. As legacy procedure, the target gNB can send RRC setup message if the target does not have UE’s context. RAN2 can revisit the issue if there is any impact foreseen due to RAN3 work on this matter.

- RAN2 assume the RRC reestablishment and RRC_INACTIVE state works without any optimization when gNB is on board satellite.

Common TA/Kmac

Following was captured in RAN2#126.

RAN2 assumes that the typical setting for common TA and Kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

In our view, it is up to network implementation how to set the values of common TA and Kmac. The gNB on board may be able to set the common TA as a fixed offset and Kmac as another fixed offset to RTT length calculated by the UE. If the common TA and Kmac are non-zero values, then the UE’s calculated full UE to gNB RTT would not be the actual RTT, which is between UE and satellite. In this case, it is up to network how to handle the UL and DL scheduling considering proper values of cell specific and UE specific Koffset. However, this does not impact the UE’s existing behavior.

- Capture that values of common TA and Kmac can be set to zero in regenerative architecture.

Early contention resolution

Figure 1 RA-SDT procedure in regenerative payload architecture

As shown in figure 1, the gNB on-board may want to immediately send the early contention resolution MAC CE (i.e., without RRC message) while waiting for the response from the ground network. The purpose of doing this is to avoid the possibility of expiry of contention resolution timer while the UE is waiting for Msg4. In this case, it certainly is not power efficient for UE to continuously monitor PDCCH unnecessarily after sending the HARQ feedback of the contention resolution MAC CE (i.e., after UE is able to resolve the contention successfully). Similar to delaying the start contention resolution timer, the PDCCH for monitoring further RRC message can be delayed after sending HARQ feedback of the contention resolution MAC CE.

Even if there is retransmission of contention resolution MAC CE, the UE does not need to receive it as it already decoded it successfully and ACK of it is just sent. Then the next task is that gNB knows about it (i.e., UE successfully resolved the contention) and gNB sends the further DL data to UE, which would take one more RTT time.

- After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message.

Support of satellite switch with resync

In RAN2#126, following was captured.

Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary

Figure 2 Mirroring of the satellites

As shown in figure 2, it could be possible by network implementation given ideal ISL backhaul. In this case, the old satellite will transfer all data (including gNB context, all UE’s context etc.) to the new satellite serving the same area, i.e., new satellite is the mirror of the old satellite. This can be done by proprietary ISL transport layer. This may take some data transfer time same as in HARD satellite switch with resync. Gateway can handle the satellite routing to new mirror satellite by a proprietary method. No impact to RAN or core network is foreseen due to this unless otherwise RAN3 identifies any work.

The soft switch may not be possible due to switch time where the t-ServiceStart needs to be after the t-Service. However, hard satellite switch can be possible with very minor specification change. In hard satellite switch case, it is better to support t-ServiceStart. The only change needed is to clarify that the t-ServiceStart occurs after t-Service, i.e., position gap (t-Service – t-ServiceStart) is possible for hard satellite switch with resync in the regenerative architecture.

- From RAN2 perspective, whether to indicate the support of satellite switch with resync for regenerative payload is left to network implementation. Clarify the +ve gap of satellite switch i.e., t-ServiceStart can occur after t-Service.

# Conclusion

Following proposals are made.

Proposal 1	RAN2 assume the inter-gNB RACH-less handover works without any optimization when gNB is on board satellite.

Proposal 2	RAN2 assume the RRC reestablishment and RRC_INACTIVE state works without any optimization when gNB is on board satellite.

Proposal 3	Capture that values of common TA and Kmac can be set to zero in regenerative architecture.

Proposal 4	After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message.

Proposal 5	From RAN2 perspective, whether to indicate the support of satellite switch with resync for regenerative payload is left to network implementation. Clarify the +ve gap of satellite switch i.e., t-ServiceStart can occur after t-Service.