3GPP TSG-RAN WG2 Meeting #127                                    	R2-2406637

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.8.6

Source:	Qualcomm Incorporated

Title:	Idle mode mobility from LTE to NR NTN

Document for:	Discussion and Decision

# Introduction

In RAN2#126, following agreements were made.

Agreements:

- For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope.

- For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE.

- SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID.

4.	To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters:

-	Satellite ephemeris information

-	TA common information

-	k-Mac

-	epoch time

-	validity duration

-	ntn-PolarizationDL (FFS if mandatory or optional)

5.	The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information.

6.	The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

7.	RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur):

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

8.	Introduce the clarification in the field description of measTimingConfig (configured via SIB24 in TS 36.331) that it is configured based on the assumption that the gNB-UE propagation delay equals to 0 ms, and UE can adjust the offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID.

Working Assumption:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

In this document, we discuss possible signaling options.

# Discussion

Signaling options

The neighbor satellite information should include at least ephemeris, epoch time, common TA parameters, Kmac. The reference point for the epoch time should be provided in serving cell’s downlink timing. The list of NR frequencies for each satellite should also be provided and optionally the PCI information can also be provided. These are provided as neighbor cell information in SIB19 in NR NTN and NR TN.

The SIB24 contains information relevant for inter-RAT cell re-selection (i.e. information about NR frequencies and NR neighbouring cells relevant for cell re-selection) and it is already agreed that this can be extended to include satellite ID to associate the NR frequency to a satellite.

EUTRA SIB33 includes neighbor satellite information but it is currently used by EUTRA TN to provide EUTRA satellite information. It would be good not to impact the structure of SIB33 which is used by eMTC UEs in IoT NTN.

However, it was clarified in the last meeting that this specific enhancement is mainly for full LTE UEs. Therefore, it may be simple just to define separate neighbour satellite information list to provide the NR satellite information in SIB33. The reason is that extension is anyway needed to include ntn-PolarizationDL. In addition, the satellite list size of maxSat-r17 (= 4) may not be sufficient and larger list can be introduced. However, it is to note that network could still reuse the existing neighSatelliteInfoList-r18 if no more than 4 satellites information is needed and polarization information does not need to be provided. Text proposal is shown below.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

... ,

[[

neighSatelliteInfoList-r19	NeighSatelliteInfoList-r19			OPTIONAL,	-- Need OR

]]

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

NeighSatelliteInfoList-r19 ::=	SEQUENCE (SIZE(1..maxSat-r19)) OF NeighSatelliteInfo-r19

NeighSatelliteInfo-r19 ::=	SEQUENCE {

satelliteId-r19				SatelliteId-r18,

ephemerisInfo-r19				CHOICE {

stateVectors-r19				EphemerisStateVectors-r17,

orbitalParameters-r19			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r19		SEQUENCE {

nta-Common-r19					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r19			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r19	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r19					SEQUENCE {

startSFN-r19					INTEGER (0..1023),

startSubFrame-r19				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r19						INTEGER (1..512)				OPTIONAL,	-- Need OP

ntn-PolarizationDL-r19          ENUMERATED {rhcp,lhcp,linear}   OPTIONAL,  -- Need R

ntn-PolarizationUL-r19          ENUMERATED {rhcp,lhcp,linear}   OPTIONAL   -- Need R

}

maxSat-r19					INTEGER ::= 8	-- Maximum number of satellites

-- ASN1STOP

- Define new neighbor satellite list neighSatelliteInfoList-r19 with larger size maxSat-r19 (=8) in SIB33.

Multiple SMTC

RAN2 made following working assumption in RAN2#125bis meeting.

- We don’t introduce multiple SMTCs in LTE

In our view, there can be multiple satellites operating in the same carrier frequency. For each satellite a different SMTC offset needs to be provided. Therefore, the working assumption can be confirmed by ensuring that multiple satellites associated with the same frequency can be provided with different SMTC offsets.

- Confirm that multiple satellites associated with the same frequency can have different SMTC offsets.

# Conclusion

Following proposals are made.

Proposal 1	Define new neighbor satellite list neighSatelliteInfoList-r19 with larger size maxSat-r19 (=8) in SIB33.

Proposal 2	Confirm that multiple satellites associated with the same frequency can have different SMTC offsets.