3GPP TSG-RAN WG2 Meeting #127                                    	R2-2406638

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.8.2

Source:	Qualcomm Incorporated

Title:	Discussion on cell DTX

Document for:	Discussion and Decision

# Introduction

In RAN#126, following agreements were made on DL coverage enhancements.

Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

Ask RAN1 to keep us informed on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values.

More in general, ask RAN1 whether/how the solution they are investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive

Also remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status will have to relate to beams visible to the UE (e.g. SSB beams)

Also ask RAN1 whether they are also working on UL beam hopping (and whether this is separate from DL beam hopping)

RAN2 assumes that both EFC and EMC are supported (no need to send this in the LS to RAN1)

In this document, we cell DTX aspect for NTN.

# Discussion

Due to power budget limitation at satellite, all beams may not be possible to be active at the same time. It is also possible that which beam needs to be turned off at a time t1 may depend on what is the traffic load around or before time t1. As discussed in [1], TDM pattern is an effective way to enable flexible power sharing among satellite beams and it allows the options of using multiple patterns.

Figure 1 Cell DTX for different UEs.

In our view, the major concern for the power budget is the periodic broadcast of SSB and SIB1, which consumes most of the beam power budget for UEs in RRC_IDLE and RRC_INACTIVE. RAN2 is waiting feedback from RAN1 but it may be better RAN2 to assume the support of the case where NTN cell DTX is complete cell off, i.e., all broadcast signals are off.

RAN2 assume that new NTN cell DTX with ON and OFF pattern of the beam is supported.

As mentioned earlier, a single semi-static cell DTX pattern may not be efficient. A dynamic cell DTX pattern based on traffic load of the beams can be more efficient. For this, dynamic DCI based cell DTX activation/deactivation or multiple cell DTX patterns can be considered.

Rel-18 NES supports a single DTX pattern per serving cell, however, it already supports DCI-based activation and deactivation of the DTX pattern. This mechanism can also be extended for NTN scenario. Dynamically one beam may be deactivated with cell DTX or activated with a new cell DTX pattern depending on traffic load and power sharing management. We can consider the DCI-based activation/deactivation command for both cases when the cell configures UE with multiple DTX pattern or just a single DTX pattern.

RAN2 assumes dynamic activation/deactivation of new NTN cell DTX patterns based on beam/cell traffic load is supported. Existing DCI-based cell DTX activation/deactivation indication can be baseline.

RAN2 is waiting reply LS from RAN1 to be clear on cell DTX solutions. Once it is clear whether cell DTX is supported in which RRC states or applies to all RRC states, RAN2 should consider how to handle the legacy UEs and neighbor cell measurements. For UEs in RRC_IDLE, delay in initial access would need to be considered. Cell DTX pattern can also affect the neighbor cell measurements specially in RRC_IDLE. One simple option is to bar legacy UEs in a cell operating in cell DTX mode.

RAN2 wait reply LS from RAN1 to further discuss the impacts of cell DTX on UE measurements and initial access.

# Conclusion

Following proposals are made.

Proposal 1	RAN2 assume that new NTN cell DTX with ON and OFF pattern of the beam is supported.

Proposal 2	RAN2 assumes dynamic activation/deactivation of new NTN cell DTX patterns based on beam/cell traffic load is supported. Existing DCI-based cell DTX activation/deactivation indication can be baseline.

Proposal 3	RAN2 wait reply LS from RAN1 to further discuss the impacts of cell DTX on UE measurements and initial access.

# References

[1] R1-2403211, “Downlink coverage enhancement for NR NTN”, Changsha, China, April 15th – 19th, 2024.