3GPP TSG-RAN WG2 Meeting #127                                    	R2-2406639

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.9.2

Source:	Qualcomm Incorporated

Title:	Support of S&F mode operation

Document for:	Discussion and Decision

- Introduction

In RAN2#126 meeting, following agreements were made.

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

SA2 has also made some recommendation to start the work item with only MME on board architecture.

In this document, we provide the details for solution to be addressed by RAN2.

- Discussion

As recommended by SA2 study item in [1], followings are the architectures to be supported.

- S&F solution applies to only EPC

- Only interface between UE and satellite will be standardized, no 3GPP solution to the interface between satellite and ground station.

Architecture option#1:

- MME functionality is split into two parts: MME-onboard (the MME part which is onboard the satellite) and MME-ground (the MME part which is on the ground network) with an interface between the parts of the MME being out of scope of 3GPP.

- HSS is on the ground

Architecture option#2:

- The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc.

- The implementation of the proxies and the interface between them is out of 3GPP scope.

As per RAN WID, it should be clear whether RAN2 can work on “eNB only onboard satellite” solution. Since SA2 is mostly working on the case where MME is onboard satellite, we think RAN2 should exclude the “eNB only onboard satellite” solution.

RAN2 does not consider the case of “eNB only onboard satellite” unless it is supported by SA2.

Since the MME is on board satellite, most of the assistance information for selecting correct satellite and wait time to access the satellite in S&F operating mode is expected to be provide to the UEs via NAS message. We think barring mechanisms in S&F cells, and cell reselections/measurement procedures are in the scope of RAN2 to work.

Considering SA2 study item recommendation, works related to barring mechanisms in S&F cells, and the cell reselections/measurement procedures are in the scope of RAN2.

S&F indication and cell barring

It is debatable whether a new cell barring specific to S&F mode operation is needed or just S&F mode indication broadcast in SIB would be sufficient.

SA2 have identified the following normative impact in [1].

2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

It is possible that the UE may have to be rejected in the first attach or TAU attempt by the MME on-board satellite with a cause, for example, “S&F mode operation” and ask UE to retry with wait timer. This mechanism needs new signaling and may not work for the legacy UEs as legacy UEs may end up retrying the attach or TAU procedure wasting power. Therefore, we think it is simple to bar the legacy NTN UEs from accessing the cell operating in S&F mode, i.e., operating when feeder link is not available.

A cell operating in S&F mode, i.e., operating when feeder link is not available should bar the legacy UEs from accessing the cell.

Table 1: Use of S&F mode indication

| Legacy NTN barring (cellBarred-NTN-r17) | New S&F indication | UE action |
| --- | --- | --- |
| present | Not present | - Legacy NTN cell - S&F mode operation not supported. |
| barred | Present | - New cell supporting S&F mode but currently the cell has NO feeder link connection. - Only S&F supporting UE can access the cell |
| notBarred | Present | - New cell supporting S&F mode but currently the cell has feeder link available. - Any NTN capable UE can access the cell. - This option may also depend on SA2 design if S&F mode satellite can serve legacy UEs. |

RAN2 has already agreed that S&F indication can be provided by SIB. We think such indication alone can be sufficient for the operation. The absence of this indication should be understood as normal NTN cell. When S&F mode indication is present, the UE should understand that the cell supports in S&F mode operation. In this case, there is no need to introduce S&F mode specific barring mechanism as shown in Table. If the S&F mode cell wants to bar all NTN UEs, then it can set the cellBarred-NTN-r17 to “notBarred” and remove the S&F indication. When the S&F mode cell allows access to only UEs supporting S&F mode operation, then it can set the cellBarred-NTN-r17 to “barred” and include the S&F indication.

No S&F mode specific barring mechanism is introduced, i.e., 1 bit S&F mode indication is sufficient to indicate that UEs to access NTN cell in S&F mode are not barred.

When feeder link is not available, the S&F mode supporting cell should include the S&F mode indication and set the cellBarred-NTN-r17 to “Barred”.

When feeder link becomes available, it may be possible for the network to also support legacy NTN UEs and want to set the legacy NTN cellBarred-NTN-r17 to “notBarred” as shown in Table 1. However, it is not clear yet how S&F mode works for legacy UEs. For this option, we think SA2 confirmation is needed.

For the case when feeder link is available, wait SA2 progress to confirm whether the S&F mode supporting cell can include the S&F mode indication and set the cellBarred-NTN-r17 to “notBarred”.

Updating S&F mode indication

Whenever the satellite establishes feeder link connection (feeder link becomes available), it is also possible that a cell may want to remove the S&F indication (for example, switch the S&F mode to transparent architecture mode). In this case, the UEs supporting S&F mode operation may also support normal mode operation. If cellBarred-NTN-r17 is changed, the existing procedure should follow to update the SIB1.

Whenever the satellite loses feeder link connection, cell supporting S&F mode should make sure that S&F indication is included in SIB. In this case, the cell can release all UEs that do not support S&F mode operation in RRC_CONNECTED to IDLE mode. We think it is very important to avoid paging notification to eMTC and NB-IoT UEs. The existing cell stop time mechanism can be used to inform the legacy UEs that they need to find other cells. This can be useful for both cases, i.e., when feeder link will be lost or when feeder link will be available.

For new UEs, a new indication (probably in SIB19) can be introduced to indicate that the cell stop time is associated with the update of S&F mode operation, i.e., at cell stop time, the cell loses feeder link and operates in S&F mode. This can be further discussed.

Repurpose cell stop time broadcast in SIB for both EMC and EFC if update of the S&F mode operation is needed based on the availability of the feeder link.

Cell reselection

Existing frequency Priorities and cell reselection offsets can be used for a UE to perform measurements for cell reselection. However, the UE could prioritize to select a cell operating in normal mode over a cell operating in S&F mode. This is specially for NB-IoT case where no frequency priorities are considered. This can be used in case equal frequency priority or in case UE meets cell selection criteria for both S&F and normal mode cells. When waiting in S&F mode cell or when camping on S&F mode, it can be left to UE whether it periodically searches for cells operating in normal mode.

When in RRC_IDLE in S&F mode cells, it can be left to UE to periodically search for better and normal mode cell (i.e., cells with feeder link connection available).

When in RRC_IDLE and waiting to connect to a S&F mode satellite, it can be left to UE to prioritize the search for better and normal mode cell (i.e., cells with feeder link connection available).

- Conclusion

Following observation and proposals are made:

Proposal 1	RAN2 does not consider the case of “eNB only onboard satellite” unless it is supported by SA2.

Observation 1.	Considering SA2 study item recommendation, works related to barring mechanisms in S&F cells, and the cell reselections/measurement procedures are in the scope of RAN2.

Proposal 2	A cell operating in S&F mode, i.e., operating when feeder link is not available should bar the legacy UEs from accessing the cell.

Proposal 3	No S&F mode specific barring mechanism is introduced, i.e., 1 bit S&F mode indication is sufficient to indicate that UEs to access NTN cell in S&F mode are not barred.

Proposal 4	When feeder link is not available, the S&F mode supporting cell should include the S&F mode indication and set the cellBarred-NTN-r17 to “Barred”.

Proposal 5	For the case when feeder link is available, wait SA2 progress to confirm whether the S&F mode supporting cell can include the S&F mode indication and set the cellBarred-NTN-r17 to “notBarred”.

Proposal 6	Repurpose cell stop time broadcast in SIB for both EMC and EFC if update of the S&F mode operation is needed based on the availability of the feeder link.

Proposal 7	When in RRC_IDLE in S&F mode cells, it can be left to UE to periodically search for better and normal mode cell (i.e., cells with feeder link connection available).

Proposal 8	When in RRC_IDLE and waiting to connect to a S&F mode satellite, it can be left to UE to prioritize the search for better and normal mode cell (i.e., cells with feeder link connection available).

- References

[1] TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3