3GPP TSG-RAN WG2 Meeting #127                                    	R2-2406640

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.9.3

Source:	Qualcomm Incorporated

Title:	Discussion on EDT enhancements

Document for:	Discussion and Decision

- Introduction

In RAN2#126, some initial agreements were made.

Agreements:

- Contention Resolution Diversity Slotted Aloha (CRDSA): RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

- RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources).

- If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3. Check this with RAN1 and RAN4.

In this document, we provide descriptions for the solutions on first objective and second objective of the WID [1].

- Discussion

## 1. First objective

The first objective of the EDT enhancement is mainly enhancement of the random-access procedure as existing preamble transmission and RAR reception is not efficient, i.e., collision is detected by UE only after reception of Msg4 and restarting whole RACH procedure is too costly in NTN due to very long RTT. The example of enhancement includes Msg3 transmission without msg1/ Random Access Response (RAR). This enhancement is even more relevant in NTN, since the UEs are GNSS capable (i.e., they know their own location before accessing the NTN cell), and they know the satellite’s location (from the ephemeris information broadcasted in SIB). As a result, these NTN UEs have a valid timing advance from these two quantities (UE’s location and satellite’s location), and a PRACH preamble transmission is not strictly required.

Currently EDT is a four-step random access procedure, i.e., EDT specific Msg1, RAR, RRCEarlyDataRequest message (Msg3) and RRCEarlyDataComplete message (Msg4). In our understanding the objective is to reduce the EDT procedure from 4 steps to 2 steps, i.e., UE directly transmitting RRCEarlyDataRequest message (Msg3) and receiving RRCEarlyDataComplete message (Msg4). And this is without Msg1. i.e., preamble, therefore, it is not 2 step RACH procedure.

Figure 1 An example of EDT without Msg1 and RAR

As shown in figure 1, this can be interpreted as combination of EDT and PUR. However, PUR configuration is UE specific and will not work once a satellite moves out and a new satellite comes in the service. Therefore, it could be considered as RACH-less EDT and a common PUSCH resource can be provided via SIB. Similar to configuring UEs to randomly selecting a PRACH resource (or preamble) based on coverage enhancement (CE) level to reduce the collision rate, a resource of PUSCH resources can be considered where a UE can select a random resource based on current CE level.

For the first objective, the enhancement of Msg3 (RRC connection request/UL data) transmission over contention-based common PUSCH resource is considered.

Similar to PRACH resource pool, a pool of contention-based common PUSCH resources per CE level is considered.

Initiating procedure

Currently EDT and PUR have different sections for initiating procedure. However, same as EDT, the contention-based common PUSCH resource needs to be cell specific and needs to be provided per CE level. Similar to EDT, multiple TBS concept of EDT can also be considered as UEs may have different size of small UL data transmissions. It is also simple to consider the UL data transmission to be considered as EDT session from higher layer perspective to minimize the impact to other working groups such as SA2 and CT1.

Though this new procedure can have a separate section, the UL data transmission initiating procedure can be same as the EDT initiating procedure. Currently EDT initiating procedure does not allow mo-signaling and mo-VoiceCall, we think same as in NR, the mo-signaling should be allowed, i.e., UE can also send TAU update message via RACH-less EDT. The EDT procedure can be initiated for a voice call and network can bring the UE in RRC_CONNECTED. Therefore, the mo-VoiceCall can also be allowed.

The RACH-less EDT establishment cause can be mo-Data or mo-Signalling or mo-ExceptionData or delayTolerantAccess or mo-VoiceCall.

RNTI and contention resolution

In legacy EDT (with Msg1 and RAR), after transmitting the preamble, the UE monitors RA-RNTI to receive the RAR. Since there will be no RAR, hence there will be no such RA-RNTI. However, in line with a similar mechanism as RA-RNTI, a new RNTI would be needed for UEs to monitor the response of the Msg3 transmission over the contention-based common PUSCH resource. RAN1 is also discussing OCC mechanism which can also be considered as orthogonal resource. We can also consider a new RNTI to be derived from the time/frequency/OCC resource of the contention-based common PUSCH resource occasion where Msg3 transmission occurs.

A new RNTI to monitor response is derived from the resource of the contention-based common PUSCH occasion where Msg3 transmission occurs.

Since the resource will be common, there will be chance that one or more UEs select the same PUSCH resource and collision occurs. The collision can still be handled in the same existing way, i.e., by the use of contention resolution timer and contention resolution MAC CE in Msg4.

For the first objective, confirm that, for Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

## 2. Diversity Slotted Aloha (DSA)

Current random-access procedures in 3GPP can be seen as a form of slotted Aloha: if two users choose the same preamble (or NPUSCH resource, for the objective under discussion), at most one of the UE’s Msg3 (PUSCH) will be successful and, in most cases, both UE’s Msg3 will fail the random-access attempt due to a collision. If we want to design a system targeting a small probability of collision, the random-access resources need to be dimensioned with a very large margin - for example, for a probability of collision of 1%, the random-access resources need to be approximately 100x larger than the expected arrival rate. Equivalently, for a probability of collision of 1%, 99% of the resources will remain unused.

Modern satellite communication systems incorporate improved random-access techniques that allow for a more efficient utilization of the random-access resources while keeping a low probability of collisions/errors. For instance, DVB-RCS2 incorporates Contention Resolution Diversity Slotted Aloha (CRDSA) [2, Section 7.2.5.2.2], and S-band Mobile Interactive Multimedia (S-MIM) incorporates Spread Spectrum Aloha (SSA) [3]. These techniques outperform basic slotted Aloha, e.g., in terms of supporting a larger traffic arrival rate at a given collision probability, as shown in [4-6]. This directly translates to improved system capacity (in terms of the number of UEs that can simultaneously be supported in the uplink over a given set of resources). In NTN—especially NB-IoT over NTN, where spectrum (frequency resources) is scarce and expensive, such techniques are pivotal for an efficient design (and specification) of “RACH-less EDT” (Msg3 transmission without msg1).

Observation 1: Enhanced random-access techniques such as CRDSA would better utilize available uplink resources and improve uplink capacity, vis-à-vis basic slotted ALOHA-based access techniques.

How DSA and CRDSA work

Here, we depict an example operation of RACH-less EDT employing DSA/CRDSA for random access, for NB-IoT over NTN.

Each RACH-less EDT occasion has R EDT resources (in frequency) for UEs to choose from, to transmit. A UE chooses one out of these R EDT resources at random to transmit in, on each occasion. For NB-IoT case, up to 48 subcarriers with a subcarrier spacing of 3.75𝑘𝐻𝑧 can be used for RACH-less EDT transmission (total carrier BW: 180𝑘𝐻𝑧). We assume single subcarrier mode of uplink transmission.

For DSA/CRDSA, N such RACH-less EDT occasions in time (described above) are grouped together to form 1 “RACH-less EDT occasion group”. This is depicted with an example in Figure 2. A UE needs to choose 𝑘 (≤𝑁) out of the 𝑁 occasions in the group to transmit in. In each of the chosen occasions, the UE chooses 1 among the 𝑅 resources to transmit in. The choice of frequency resource for each occasion is independent of the choice for other occasions. The UE then transmits identical data in all the chosen (resource, occasion) pairs.

For DSA (depicted in Figure 3), transmissions in a non-colliding frequency resource and EDT occasion are successful. A UE with at least 1 successful Tx is successful. In Figure 3, successful UEs (employing the N=3, k=2 scheme, in the running example depicted in Figure 2) are colour coded in green. The successful UEs in Figure 3 are: B, C, D, E, H, I, K, L.

For CRDSA (depicted in Figure 4), interference from successful UE from previous steps are cancelled. Newly successful UEs in the same running example—UEs F and J, after a single step of interference cancelation—are colour coded in yellow, in Figure 4. To visualize the benefit from interference cancellation, e.g., for UE F, note that in EDT Occasion 2, and Frequency Resource R0, a successful prior decoding of UE C (e.g., from EDT Occasion 3, R4) would allow for UE C’s interference to be cancelled, and thus a successful decoding of UE F would ensue (which would not have been possible with DSA alone).

Figure 2: Example with 𝑁=3 EDT occasions comprising a “EDT occasion group”. The burst size is 𝑘=2, i.e., each UE must choose 2 out of the 3 occasions in the group and choose an EDT resource in frequency, for each of these, independently randomly. 𝑅=8 subcarriers in frequency are part of each EDT occasion, in this example.

Figure 3: UEs decodable with DSA, color coded in green.

Figure 4: Additional UEs decodable with CRDSA, color coded in yellow, after one round of interference cancellation.

Simulation assumptions, results and gain

To evaluate the performance of DSA and CRDSA and compare it with that of a baseline implementation of RACH-less EDT (which would use simple slotted ALOHA for random access), we use simulation assumptions (including some “derived” parameters) as depicted in Table 1.

Table 1: Simulation assumptions for performance evaluation of DSA and CRDSA vs baseline RACH-less EDT.

| Simulation Parameters |  |
| --- | --- |
| Scenario | 3GPP Set 1 GSO (from 3GPP TR 38.821) |
| Channel for LLS | AWGN |
| Uplink link budget SNR (BW = 3.75 kHz; UE EIRP = 23 dBm) | 2.8 dB |
| TBS | 144 bits |
| Subcarrier Spacing | 3.75kHz |
| Number of resources in frequency (R) | 48 |
| Duration of EDT occasion (derived from link-level simulations, as the minimum transmission duration to achieve a PHY BLER of 1%) | 64 ms |
| UE traffic model | Poisson arrival process |
| Retransmission | No Retransmission |
| DSA and CRDSA specific parameters |  |
| (N,k) | (2,2) or (3,3) |
| Niter (only for CRDSA) (corresponds to Niter-1 rounds of interference cancellation) | 2 or 4 |

We assume that the UE traffic model is Poisson distributed with arrival rate λ/s i.e. the average number of UEs/s attempting transmission. We can then obtain the average MAC load as λ*TBS.

We obtain the packet error rate using Monte Carlo Simulation. For each “RACH-less EDT occasion group” we sample the number of UEs attempting transmission from a Poisson distribution with parameter Nλ/TEDT  where TEDT is the duration of the EDT occasion in seconds (0.064s here). For each UE we choose k occasions within the group to transmit in and a data resource in each chosen occasion independently randomly. We assume that any transmission is successfully decoded if and only if it does not collide with any other transmission whose interference has not been cancelled. For CRDSA, we assume that the base station is aware of the choice of occasions and resources of a UE once at least one copy of transmission from that UE is successfully decoded. For CRDSA, we assume that once a transmission from a UE is successfully decoded, interference from all copies transmitted by the UE can be completely cancelled. We compute the packet error rate as the ratio of number of unsuccessful UEs to the total number of UEs.

In Figure 5, we demonstrate the performance (in terms of achievable system-wide MAC load (in kbps) vs probability of packet failures) for a baseline strategy (S-ALOHA), for different values of N,k for DSA, as well as for CRDSA with an Niter=2 or 4, i.e., with 1 and 3 rounds of successive interference cancellation respectively.

Figure 5: Performance evaluation of baseline RACH-less EDT (S-ALOHA), DSA and CRDSA.

At acceptable packet error rates (typically ~10-2 for HARQ-less GEO operation), we can observe significant gains (as given in Table 2) in system throughput (which translates to capacity, in terms of number of UEs that can be supported on a NB-IoT carrier) for DSA and CRDSA, w.r.t. baseline S-ALOHA.

| Gain with respect to S-ALOHA | DSA | CRDSA |  |
| --- | --- | --- | --- |
|  |  | Niter=2 | Niter=4 |
| N = k = 2 | ~4x | ~13x | ~18.9x |
| N = k = 3 | ~6.5x | ~22.7x | ~42x |

Table 2: Throughput gains in supportable MAC loads (at packet error rate <10-2) for DSA and CRDSA vs baseline S-ALOHA.

Based on the results presented in this section, and the demonstrable gains achievable from DSA and CRDSA with minimal specification impact, these enhanced random-access techniques should be considered an integral component of the design and specification for “Msg3 transmission without msg1”. Therefore, RAN2 should consider introducing some enhancements to the random-access procedure such that 3GPP technology remains competitive with state-of-the-art satellite standards.

For the first objective, Support enhanced random access techniques, such as Contention Resolution Diversity Slotted Aloha (CRDSA), for improved random-access performance of msg3 transmission without NPRACH.

## 3. Second objective

The second objective of the EDT enhancement is to reduce the congestion/overhead for transmission of Msg4 to several UEs. For example, many UEs may initiate EDT and network may exhaust the PDCCH or PDSCH resources while scheduling Msg4 (which comprises a PDCCH and at least one PDSCH) to each individual UE. There may be cases where network may not have any downlink response data and network would need to schedule Msg4 just including RRCEarlyDataComplete message (without DL NAS PDU) and contention resolution MAC CE of multiple UEs.

Instead of sending individual Msg4’s in such instances, a “multiplexed Msg4” to several UEs may be considered. An example procedure where multiplexed transmissions to several UEs using a single PDSCH are performed exists in case of Random-Access response. As shown in figure 2, network can schedule multiple RARs in a single MAC PDU.

Figure 6 TS 36.321 Figure 6.1.5-4: Example of MAC PDU consisting of a MAC header and MAC RARs

In this case there could be several possible options for the enhancements, some examples being:

- Reusing a PUR-like L1 ACK procedure

  - The L1 ACK may also be used to resolve contention between UEs

  - The L1 ACK can move UE back to RRC_IDLE

- Reusing concept of RAR-like multiplexing (e.g., of multiple users RAPIDs in a single RAR)

  - A single PDU can multiplex contention resolution MAC CEs (or Msg4) of more than one UE.

- Others such as existing Multi-TB scheduling for Msg4 could also be studied for multi-user scenario.

The second objective can be on the top of existing 4 steps EDT procedure as shown in figure 3, or applicable for the case of RACH-less EDT as described for the first objective.

For the second objective, a PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

For the second objective, multicast Msg4 (multi-user Msg4 multiplexing) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

Figure 7 Msg4 enhancement for EDT

- Conclusion

Following proposals are made:

Proposal 1	For the first objective, the enhancement of Msg3 (RRC connection request/UL data) transmission over contention-based common PUSCH resource is considered.

Proposal 2	Similar to PRACH resource pool, a pool of contention-based common PUSCH resources per CE level is considered.

Proposal 3	The RACH-less EDT establishment cause can be mo-Data or mo-Signalling or mo-ExceptionData or delayTolerantAccess or mo-VoiceCall.

Proposal 4	A new RNTI to monitor response is derived from the resource of the contention-based common PUSCH occasion where Msg3 transmission occurs.

Proposal 5	For the first objective, confirm that, for Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

Proposal 6	For the first objective, Support enhanced random access techniques, such as Contention Resolution Diversity Slotted Aloha (CRDSA), for improved random-access performance of msg3 transmission without NPRACH.

Proposal 7	For the second objective, a PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

Proposal 8	For the second objective, multicast Msg4 (multi-user Msg4 multiplexing) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

- References

[1] RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

[2] ETSI EN 301 545-2, “Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 2: Lower Layers for Satellite standard”

[3] ETSI TS 102 721-3, “Satellite Earth Stations and Systems (SES); Air Interface for S-band Mobile Interactive Multimedia (S-MIM); Part 3: Physical Layer Specification, Return Link Asynchronous Access”

[4] S. Scalise, C. P. Niebla, R. De Gaudenzi, O. Del Rio Herrero, D. Finocchiaro and A. Arcidiacono, "S-MIM: a novel radio interface for efficient messaging services over satellite," in IEEE Communications Magazine, vol. 51, no. 3, pp. 119-125, March 2013

[5] E. Casini, R. De Gaudenzi and O. Del Rio Herrero, "Contention Resolution Diversity Slotted ALOHA (CRDSA): An Enhanced Random Access Schemefor Satellite Access Packet Networks," in IEEE Transactions on Wireless Communications, vol. 6, no. 4, pp. 1408-1419, April 2007

[6] De Gaudenzi, R., Del Rio Herrero, O., Gallinaro, G., Cioni, S., and Arapoglou, P.-D. (2018) Random access schemes for satellite networks, from VSAT to M2M: a survey. Int. J. Satell. Commun. Network., 36: 66–107.