3GPP TSG-RAN WG2 Meeting #127                                    	R2-2406641

Maastricht, Netherlands, 19th - 23rd August 2024

Agenda item:	7.7.2

Source:	Qualcomm Incorporated

Title:	Discussion on soft satellite switch with re-sync

Document for:	Discussion and Decision

# Introduction

In RAN2#126, RAN2 sent LS to RAN1 and RAN4 asking feedback on changing the reference point for SSB time offset to gNB.

In this document, we further discuss the response of the LS from RAN1 in [1] and RAN4 in [2]. In addition, we provide some clarification on SSB position change for soft satellite switch.

# Discussion

In RAN2#126, RAN2 asked following question.

In RAN2#125bis meeting, RAN2 is considering adopting the gNB as the reference point of ssb-TimeOffset. RAN2 would like to check with RAN4 and RAN1 whether this would be acceptable. To RAN4, RAN1: ACTION: RAN2 respectfully asks RAN4 and RAN1 to take the above into consideration and come back if any issues are determined.

Both RAN1 and RAN4 provided response that there is no issue in the current text keeping the reference point of ssb-TimeOffset at ULSRP.

However, if we move the reference point of ssb-TimeOffset to gNB, then there is some doubt on the feasibility of the solution due to the amount of blind search for the SSB of the target satellite.

RAN1 responded following in [1].

- If adopting gNB as the reference point of ssb-TimeOffset, then there is no common understanding in RAN1 on whether the signalling of ssb-TimeOffset can help the UE reduce the amount of blind search for the SSB of the target satellite. - This implies RAN1 has no common understanding on whether it is feasible to set gNB as the reference point of ssb-TimeOffset. - If adopting the UL time synchronization reference point as the reference point of ssb-TimeOffset, then the amount of blind search for the SSB of the target satellite depends on the granularity of the ssb-TimeOffset. - This implies there is no technical issue from RAN1 perspective by setting UL time synchronization reference point as the reference point of ssb-TimeOffset.

Since there is no issue, we suggest to keep the existing text, i.e., keeping the reference point of ssb-TimeOffset at ULSRP and add clarification if the current text is not clear enough. In our view, it is ok to clarify the current text such that the source satellite and target satellite may have its own different ULSRP. Text proposal is shown below.

SatSwitchWithReSync field descriptions

ssb-TimeOffset Indicates the time offset between the SSB from source and target satellite at the uplink time synchronization reference point between a SSB burst of the target cell at its uplink time synchronization reference point and the SSB burst of the source cell at its uplink time synchronization reference point. It is given in number of subframes.

- Based on LS response, keep the reference point of ssb-TimeOffset at ULSRP and clarify the field description of ssb-TimeOffset that the source satellite and target satellite may have its own different ULSRP as below:

- Indicates the time offset between the SSB from source and target satellite at the uplink time synchronization reference point between a SSB burst of the target cell at its uplink time synchronization reference point and the SSB burst of the source cell at its uplink time synchronization reference point. It is given in number of subframes.

In RAN2#126, it was clarified that when the UE completes satellite switch in HARD satellite switch case, the SSB location of the cell is not changed.

RAN2#126 agreement:

RAN2 understands that in hard switch (t-serviceStart absent), cell timing/SSB location is unchanged (at gNB)

However, it should be clear that cell timing/SSB location will change in case of soft satellite switch, i.e., when t-serviceStart is present. This also applies to the UEs supporting only HARD satellite switch, i.e., ignoring t-serviceStart broadcast in SIB. After the switch is complete, the network is supposed to update the configuration by sending RRC reconfiguration message. However, timers such as HARQ RTT timer may be running. According to current specification, the duration of a timer is not updated until it is stopped or expires and when the soft satellite switch is complete, the timer may still be running.

TS 38.321 3 Definitions, symbols and abbreviations 3.1 Definitions NOTE 1: A timer is running once it is started, until it is stopped or until it expires; otherwise it is not running. A timer can be started if it is not running or restarted if it is running. A Timer is always started or restarted from its initial value. The duration of a timer is not updated until it is stopped or expires (e.g. due to BWP switching). When the MAC entity applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

There could be confusion whether to adjust the expiry time of the running timer after soft satellite switch, for example, if the timer is to be expired at subframe X in the old satellite, then the timer will expire at subframe Y in the new satellite, especially when the value of ssb-TimeOffset is large. We think issue of UL/DL scheduling in new satellite due to HARQ RTT timer should be handled by proper network implementation and no further change is foreseen. We could consider updating NOTE 1 in section 3.1 by adding another example as “due to soft satellite switch with resynchronization” in addition to “due to BWP switching”.

- Confirm that there is no impact to running timers (e.g., HARQ RTT timer) due to cell timing change after soft satellite switch with resync. Consider updating NOTE 1 in section 3.1 by adding another example as “due to soft satellite switch with resynchronization” in addition to “due to BWP switching”.

# Conclusion

Following proposals are made.

Proposal 1	Based on LS response, keep the reference point of ssb-TimeOffset at ULSRP and clarify the field description of ssb-TimeOffset that the source satellite and target satellite may have its own different ULSRP as below:

Indicates the time offset between the SSB from source and target satellite at the uplink time synchronization reference point between a SSB burst of the target cell at its uplink time synchronization reference point and the SSB burst of the source cell at its uplink time synchronization reference point. It is given in number of subframes.

Proposal 2	Confirm that there is no impact to running timers (e.g., HARQ RTT timer) due to cell timing change after soft satellite switch with resync. Consider updating NOTE 1 in section 3.1 by adding another example as “due to soft satellite switch with resynchronization” in addition to “due to BWP switching”.

# References

[1] R2-2406215, Reply LS on reference point for SSB-TimeOffset, Source RAN1

[2] R2-2406229, Reply LS on Reference Point for SSB-TimeOffset, Source RAN4