3GPP TSG-RAN WG2 Meeting #127		     R2-2406643

Maastricht , NL: 19th August – 23rd August 2024

Source:	Qualcomm Incorporated

Title:	On LCM for UE-sided models

Agenda Item:	8.1.2.2 LCM for UE-sided model  for Beam Management use case

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, at RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved in [3].

This paper discusses the “LCM of UE-sided models” considering recent RAN1/RAN2 agreements.

# LCM of UE-sided Models

## 2.1 LCM for Beam management Use case

The rapporteur summarized the Post RAN2#126 email discussions [POST126][032][AIML PHY] LCM (Intel, Samsung) [4] in the following proposals:

NOTE 1: In this email discussion and summaries, rapporteur assumes NW-side additional condition as associated ID (which is referred by majority of companies), and other inference configuration (e.g. CSI-RS resource configuration, etc) is considered separately from NW-side additional condition, i.e. it is not considered as part of NW-side additional condition in below proposals. It is up to RAN1 about the details of NW-side additional condition. Proposal 1: Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. Proposal 2: “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS the content of configuration. Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE. Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc Proposal 5: Step 5: 1) Network configures full inference configuration to UE after applicable functionality reporting, if configuration of supported functionality is not provided in Step 3 (i.e. full inference configuration is provided in Step 5). 2) If configuration of supported functionality is agreed to be provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

### 2.1.1	Network Side Additional Conditions

In the RAN1#116-bis meeting [5], RAN1 has the following agreements for achieving consistency between model training and inference

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

In RAN1#116bis meeting [5], in the beam management agenda item, RAN1 further had the following agreement

Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded.

We agree that RAN1 is discussing two options for beam management use case, first based on associated ID, and another based on performance monitoring. However, we believe that achieving constancy based on performance monitoring is not feasible, because of the following reasons

- Extra burden at the UE for monitoring before the functionality is activated for inference

  - Note that UE has to monitor the performance of a significantly large number of models to determine the best model for current network-side additional conditions.

  - Monitoring-based model selection needs to be performed every time network side additional condition changes. Furthermore, the network may required to signal to the UE when network-side additional conditions change.

- Will always be a hit/fail method

  - UE cannot determine with certainty the consistency, as without the information about the network-side additional condition, the UE may not be able to download the relevant models

    - Note that UE may only have limited memory to store the models. It may require frequent download/discard of the model to support monitoring-based consistency.

- UE may need to monitor the performance for a significantly large amount of time before UE can determine a suitable model. This will result in a significant delay in determining applicable functionality at the UE.

Therefore, the signaling of associated IDs for determining applicable functionalities should be mandatorily supported by the network.

Observation 1: Achieving constancy based on performance monitoring is not feasible, because of the following reasons

- Extra burden at the UE for monitoring before the functionality is activated for inference

  - Note that UE has to monitor the performance of a significantly large number of models to determine the best model for current network-side additional conditions.

  - Monitoring-based model selection needs to be performed every time network side additional condition changes. Furthermore, the network may required to signal to the UE when network-side additional conditions change.

- Will always be a hit/fail method

  - UE cannot determine with certainty the consistency, as without the information about the network-side additional condition, the UE may not be able to download the relevant models

    - Note that UE may only have limited memory to store the models. It may require frequent download/discard of the model to support monitoring-based consistency.

- UE may need to monitor the performance for a significantly large amount of time before UE can determine a suitable model. This will result in a significant delay in determining applicable functionality at the UE.

Proposal 1: For beam management use case, signaling of associated IDs for determining applicable functionalities should be mandatorily supported by the network.

#### 2.1.1.1	Uniqueness of the associate ID

In the RAN1#117 meeting [10], RAN1 had the following working assumption

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study).

Observation 2: RAN1 assumed that NW-side additional conditions with the same associated ID are consistent at least within a cell.

While consistency within a cell is useful for UE-side for model development and runtime LCM (e.g., model selection/switching/activation/deactivation), it is not scalable. If the associated ID at different cells represents different network-side additional conditions, then it will be hard for the UE side to generalize the model across multiple cells. This poses challenges not only on UE-side model development but also on runtime LCM. Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions. Therefore, the uniqueness of associated IDs extended to the PLMN. How the uniqueness of the associated IDs within a PLMN is achieved can be left up to OAM implementation.

Observation 3: The uniqueness of associated ID within a single cell is not scalable, as it will create challenges for the UE side to generalize models across multiple cells.

Observation 4: The lack of uniqueness of the associated ID poses challenges not only on UE-side model development but also on runtime LCM.

Observation 5: Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions.

Proposal 2: NW-side additional conditions with the same associated ID are consistent within a PLMN.

Proposal 3: How the uniqueness of the associated IDs within a PLMN is achieved can be left up to OAM implementation.

#### 2.1.1.2	Reporting of applicable functionalities for neighboring cells

Consider the below scenarios for AI/ML inference configuration during the handover:

Fig. 2.1.1.2-a Configuration for applicability functionality reporting is provided for candidate/target/neighboring cells by the source cells.

Fig. 2.1.1.2-b Configuration for applicable functionalities reporting is provided by the target cell after the successful handover

The above two methods can be used for retrieving applicable functionalities

- The source cell configures UE to provide applicable functionalities for the target / candidate / neighboring cell(s). The applicable functionality information can be provided to the target / candidate cell(s) during handover preparation. The target/candidate cell(s) provides inference configuration together with the handover command.

- Target cell configures UE to provide applicable functionality for it after successful handover completion. Inference configuration can be provided together with applicable functionality configuration or after applicable functionalities are reported (after a successful handover).

Observation 6: The following two methods can be used for retrieving applicable functionalities

- The source cell configures UE to provide applicable functionalities for the target / candidate / neighboring cell(s). The applicable functionality information can be provided to the target / candidate cell(s) during handover preparation.

- Target cell configures UE to provide applicable functionality for it after successful handover completion.

By supporting the scenario, where the source cell can configure UE to provide applicable functionalities for the target / candidate / neighboring cell(s), inference configuration latency can be reduced significantly. Therefore, RAN2 is requested to discuss if UAI can be used for reporting the applicability functionality for target/candidate/neighboring cells.

Observation 7: In procedures where the source cell can configure UE to provide applicable functionalities for the target / candidate / neighboring cell(s), inference configuration latency can be reduced significantly.

Proposal 4: RAN2 is requested to discuss if UAI can be used for reporting the applicable functionalities for target/candidate/neighboring cells.

### 2.1.2	Functionalities Configuration, Applicability, and Activation at the UE

the Post RAN2#126 email discussions [POST126][032][AIML PHY] LCM (Intel, Samsung) [4], it is FFS If the configuration of supported functionality is provided in Step 3. In this section, we discuss how by providing the configuration for supported functionalities the need for frequent inference configuration can be reduced. Let us consider the two configuration methods

- Only the applicable functionalities are configured by the network at the UE.

- Supported functionalities are configured by the network at the UE.

#### 2.1.2.1	Inference configuration for applicable functionalities

In the non-AI/Ml-based feature/feature groups, the configuration and activation of a feature/feature group are performed simultaneously. In the legacy procedure, UE declares RLF if UE cannot comply with the provided configuration, which results in radio link failures. However, as seen in the figure 2.1.2.1 below, the network may need to provide UE with new/updated inference configurations every time applicable functionalities change. Please note that applicability functionality can change due to

- Change in network side additional condition,

- UE-side additional condition, or

- Model availability.

Therefore, changes in applicable functionalities may change quite frequently. Thus, the network may need to provide new/updated inference configurations quite frequently.

Fig. 2.1.2.1 The change in network-side additional conditions, UE-side additional conditions, or model availability will result in a change in applicable functionalities. This will necessitate frequent new/updated inference configurations from the network.

Observation 8: Applicability functionality can change due to

- Change in network side additional condition,

- UE-side additional condition, or

- Model availability.

Observation 9: When the inference configuration is provided based on applicable functionalities information, the network may need to provide UE with new/updated inference configurations quite frequently as applicable functionalities can change frequently.

#### 2.1.2.2	Inference configuration for supported functionalities

By allowing UE to be configured with supported functionalities that are not yet applicable (see Fig. 2.1.2.2), the need for frequent inference configuration can be reduced. However, note that a configured functionality, not applicable yet, cannot be activated at the UE. For example, the network cannot activate a configured functionality if the model is unavailable at the UE, and others. Therefore, a configured functionality is activated at the UE, once it becomes applicable.

Fig. 2.1.2.2 By allowing UE to be configured with supported functionalities, the need for frequent configuration can be reduced. However, the network should not activate a functionality that is not applicable.

Moreover, note that once the initial inference configuration is provided, the UE may be updated with the current associated ID (or, network-side additional condition), using RRC /  MAC CE. Upon such indication, if the relevant functionality is applicable at the UE, it can apply the relevant configured functionalities and indicate to the network using UAI / MAC CE. Alternatively, if the relevant functionality is not applicable it can indicate network that the relevant functionalities for the associated ID are not applicable using UAI / MAC CE. UE can further update the network when the relevant functionalities for the associated ID become applicable using UAI / MAC CE.

Observation 10: By allowing UE to be configured with supported functionalities that are not yet applicable (see Fig. 2.1.2.2), the need for frequent inference configuration can be reduced.

Proposal 5: For LCM for UE-sided model for Beam Management use case, even if functionality(es) is not applicable they can be configured at the UE.

Proposal 6: The applicable functionality is initially considered deactivated when the inference configuration is provided based on supported functionalities information.

Proposal 7: A configured functionality can only be activated when it becomes applicable. FFS on required procedures/signaling.

### 2.1.3   Life Cycle Management and Signaling

In the RAN2#125bis meeting, RAN2 agreed the following

Agreements:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

In this section, we discuss procedures for supporting

- “Network decision, network initiated” AI/ML Management,

- “UE autonomous, decision reported to the network”, AI/ML Management, and

- “Network decision, UE-initiated” (i.e. proactive approach)” AI/ML Management.

#### 2.1.3.1	“Network decision, network initiated” AI/ML Management

Note that to facilitate “Network decision, network initiated” AI/ML Management, the network can configure UE

- With data collection to report required information elements for supporting monitoring at the network, or

- With KPIs evaluated at the UE that can be used at the network for AI/ML management decisions.

Proposal 8: To facilitate “Network decision, network initiated” AI/ML Management, the network can configure UE with

- Data collection to report required information elements for supporting monitoring at the network, or

- KPIs evaluated at the UE that can be used at the network for AI/ML management decisions.

Note that the network’s AI/ML management decision can be based on several factors, e.g., network issues, performance issues, etc. The AI/ML management decision at the network can be due to

- Performance issue,

- Change in additional condition (or network configuration) at the network, or

- Temporary network overload/failure to support AI/ML-based feature/feature groups.

Observation 11: The network’s AI/ML management decision can be based on several factors, e.g., network issues, performance issues, etc. The AI/ML management decision at the network can be due to

- Performance issue,

- Change in additional condition (or network configuration) at the network, or

- Temporary network overload/failure to support AI/ML-based feature/feature groups.

To facilitate appropriate action (for example, model retraining, etc.), the network should signal the cause of management instruction.

Proposal 9: To facilitate appropriate action (for example, model retraining, etc.), the network should signal the cause of AI/ML management together with the AI/ML management signal.

#### 2.1.3.2	“UE autonomous, decision reported to the network”, AI/ML Management

The AI/ML Management decisions at the UE can be due to

- Performance issue, or

- Change in applicable functionalities, due to changes in

  - Model availability at the UE, or

  - Additional conditions at the UE or network.

Observation 12: The AI/ML management decisions at the UE can be due to

- Performance issue, or

- Change in applicable functionalities, due to changes in

  - Model availability at the UE, or

  - Additional conditions at the UE or network.

Proposal 10: To facilitate “UE autonomous, decision reported to the network”, AI/ML Management, the network provides assistance information (e.g., network side additional condition) reporting from the network to the UE.

Proposal 11: UE reports the management decision (without cause) to the network upon taking the AI/ML management decisions.

#### 2.1.3.3	“Network decision, UE-initiated” (i.e. proactive approach)” AI/ML Management

In some cases, when the AI/ML functionalities were deactivated, or fallback to legacy procedures based on “Network decision, network initiated” AI/ML Management, or “UE autonomous, decision reported to the network”, AI/ML Management, the UE can request network to activate a functionality. In our understanding, as the functionality is already configured, the UE can evaluate the applicable functionalities and request activation of the application functionality.

Proposal 12: When the AI/ML functionalities were deactivated, or fallback to legacy procedures based on “Network decision, network-initiated” AI/ML Management, or “UE autonomous, decision reported to the network”, AI/ML Management,

The UE can evaluate the applicable functionalities and request activation of the application functionality

## 2.2 LCM for Positioning Enhancement Use case

### 2.2.1	Functionality Identification

At RAN2#126, the following agreements were made:

RAN2#126: The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. Wait for RAN1 for associate ID discussion.

As part of the email discussion [POST126][032][AI/ML PHY] [4], the following "functionality types" were proposed:

-	Supported functionalities refer to functionalities that UE can indicate by using UE capability signalling.

-	Applicable functionalities refer to functionalities that the UE is ready to apply for model inference.

-	Activated functionalities refer to functionalities already activated and performing inference.

In the following sub-sections, we discuss the applicability of these proposed "functionality types" to the positioning use case which then also leads to the proposed handling of "dynamic capabilities" in the agreement above.

### 2.2.2	Supported Functionality

For positioning, the UE "supported functionality" (capability) is defined within the LPP protocol and transferred between the target and the server using LPP transport (which may be NAS control plane, or IP user plane transport [6]).

The exchange of capabilities between a target and a server may be initiated by a request or sent as "unsolicited" information. If a request is used, the server sends an LPP Request Capabilities message to the target device with a request for individual/specific capability information. The target sends an LPP Provide Capabilities message.

At RAN2#125bis, the following was agreed:

RAN2#125bis: Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message. Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality.

Therefore, the "proactive" and "reactive" terminology used in the agreement above correspond to the existing LPP "unsolicited" and "solicited" transactions, respectively.

Observation 13: The terminology of "solicited" and "unsolicited" capability/assistance data/measurement transfer is used/supported in LPP since Rel-9 and can corresponds to the "reactive" and "proactive" reporting, respectively, by arranging the individual LPP transactions into appropriate procedures (see also Figure 2.2.3 and discussion below).

The LPP positioning capabilities contain the following main information [7]:

- Positioning methods supported (e.g., DL-TDOA, DL-AoD, Multi-RTT, etc.).

- Positioning modes per method supported (UE-based, UE-assisted, standalone).

- Auxiliary positioning measurements and information per method supported (e.g., RSRP, multipath measurements, LOS/NLOS measurement, etc.).

- Assistance data details per method supported (e.g., DL-PRS, position calculation assistance, etc.).

The above capabilities may be enhanced with "AI/ML functionality" (e.g., AI/ML enhanced RSTD, etc. measurements or AI/ML enhanced location estimate). Therefore, the agreement from RAN2#126 listed in section 2.2.1 above can be slightly modified:

Proposal 13: The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML functionality.

### 2.2.3	Applicable Functionality

Whether a "supported functionality" is also "applicable" for an LPP session depends on whether the UE available/trained model is applicable under the current network conditions. Although, the details of these network conditions may depend on RAN1 progress, it appears obvious that these network conditions comprise (at least) all aspects affecting position measurements and position calculation. These aspects have already been standardized as assistance data which include (for the NR positioning methods) [7], [8]:

-   DL-PRS configuration information (IE NR-DL-PRS-AssistanceData)

-   Available on-demand DL-PRS configurations (IE NR-On-Demand-DL-PRS-Configurations)

-   Position calculation assistance comprising:

-    TRP/ARP location information (IE NR-TRP-LocationInfo)

-    Boresight direction of DL-PRS Resources (IE NR-DL-PRS-BeamInfo)

-    Synchronization information between TRPs (IE NR-RTD-Info)

-    DL-PRS beam-power information (IE NR-TRP-BeamAntennaInfo)

-    Expected LOS/NLOS at a-priori UE location (IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance)

-    Timing Error Groups for the DL-PRS Resources (IE NR-DL-PRS-TRP-TEG-Info)

-    Integrity information

-	PRU measurement and location information (IE NR-PRU-DL-Info)

The above assistance data can also be available from System Information (posSIB).

Support of the above assistance data depends on the UE supported positioning method and mode and are part of the UE capabilities [7].

Observation 14: The "Network Conditions" for positioning comprise (at least) the already standardized LPP Assistance Data required for positioning measurements and/or position calculation. UE supported Assistance Data are already part of the UE capabilities.

Dependent on the positioning method and UE model/implementation, a change of some of the above information may mean that a UE available/trained model is not applicable under these changed network conditions (i.e., discrepancy between model training and inference). The UE would realize this once the Assistance Data applicable for the LPP session have been received. Therefore, an (unsolicited) LPP Provide Capabilities message may be needed to update/inform an LMF on the applicable functionality during an LPP session (see also further down below).

Providing updated positioning capabilities to an LMF is an existing LPP feature. Once an LPP session has been instigated by an LMF (i.e., a Correlation ID is available [6]), a UE can at any time send an appropriate UL LPP message to the LMF (i.e., LPP Provide Capabilities message in this case) [7], [8]. Examples of UE capability changes include (see e.g., [9]):

-	Radio configuration dependency: Positioning capabilities based on current/active radio configuration are not static (e.g., the srs-PosResourceConfigCA-BandList in TS 37.355 [7] is provided for the current configured CA band combination).

-	Power Savings: A (e.g., IoT) UE whose battery level is low may switch off positioning support in order to conserve battery power for more important tasks such as communicating with an external server or may report lower processing capabilities.

-	Processing Resources Constraints: The available processing resources (processors, memory, etc.) may be shared between "communication" and "positioning operations".  If the "communication operation" requires increased processing resources, the resources allocated to the "positioning operation" may temporarily be reduced.

-	Operation as a U2N Remote UE: A UE may transmit a ProvideCapabilities message when it starts or stops operation as a U2N Remote UE during an LPP session [7].

Further, as agreed by RAN2 [9]:

"RAN2 would like to point out that the above are examples and there may be other situations in which the UE positioning capability may change."

Therefore, the "FFS" in the agreement from RAN2#126 listed in section 2.2.1 above:

"FFS how to handle dynamic capabilities"

is already part of LPP (since Rel-9, indeed).

Observation 15: A change in UE positioning capabilities can happen at any time during an LPP session. A UE can provide its updated capabilities via unsolicited LPP Provide Capabilities message.

Proposal 14: Regarding the "FFS" on how to handle dynamic capabilities in the agreement from RAN2#126, no additional specification changes are needed. I.e., "dynamic capabilities" can be handled as already specified in LPP via unsolicited Provide Capability messages.

As mentioned above, whether a UE reported capability ("supported functionality") is also "applicable" for an LPP session depends on whether the network conditions are consistent with the trained/available model. RAN1 may also define "network additional conditions" for the positioning use cases (if needed in addition to the already specified "network conditions" summarized above (Observation 10)). It has been proposed that any "Network-side Additional Conditions" are identified via an "associated ID" (see agreement from RAN2#128 listed in section 2.2.1 above). However, this appears unnecessary for the positioning use case and rather sub-optimal, because of the following reasons.

(1)	Assistance Data are always needed for (NR) positioning. At minimum, the UE needs to know the available DL-PRS configuration to perform DL-PRS measurements. For position calculation, additional assistance data like TRP/ARP locations, DL-PRS beam information, synchronization information, etc. are required, dependent on the positioning method. Therefore, the NW provides these assistance data to a target anyhow, either via dedicated LPP signalling or via System Information (posSIB). Thus, an "implicit" identification via an "associated ID" seems not needed. Additional assistance data ("network-side additional conditions") can be easily added to the existing framework, if needed (as it has been done in the past from Release to Release).

(2)	Any "associated ID" for the "Network-side Additional Conditions" would imply a "hard-decision" and thus, suboptimal. For example, if the synchronization information in the network changes, the "associated ID" would have to change to indicate a change in "Network-side Additional Conditions". Such a change may imply that there is a discrepancy between model training and inference. However, this may not necessarily be the case. E.g., some implementations/methods may be rather robust for changes in NW synchronization, or may tolerate a certain level of change, etc. A simple "associated ID" change indication does not provide any information to what degree the information has changed, and therefore, corresponds to a "hard decision" not to use a trained model for inference in the given scenario. Providing the (normal) assistance data as "Network-side Additional Conditions" leaves the decision on whether a trained model is still suitable for inference in a given situation up to implementation. E.g., a UE would not only recognize any condition change between training and inference, but also which parameter has changed and to what degree it has changed and could decide (based on model implementation) whether such a change has any impact on inference in the current scenario.

In addition, assuming the "network side conditions" are the already defined positioning assistance data (per Observation 10), any "network side additional conditions" can be easily added to the existing assistance data framework. Although, we note from the agreement from RAN2#126:

"Wait for RAN1 for associate ID discussion"

we propose based on the discussion above:

Proposal 15: RAN2 assumes that any "Network-side (Additional) Conditions" for the positioning use case are the LPP Assistance Data elements, possibly with some enhancements, if needed. An "Associated ID" to identify "Network-side (Additional) Conditions" is not needed.

The above discussion can be summarized in the message sequence example shown in Fig. 2.2.3 below. Note, the illustrated procedure is existing LPP functionality. I.e., a non-AI/ML based positioning session typically will/can be executed as shown. LPP procedures are not required to occur in any fixed order, in order to provide greater flexibility in positioning [7],[8]. Thus, a UE may provide capabilities or request assistance data at any time in order to comply with a previous request for location measurements from the LMF, etc.

Based on Fig. 2.2.3 and existing LPP specification we propose:

Proposal 16: There is no need to explicitly distinguish between "supported functionality" and "applicable functionality" in a e.g., LPP Provide Capabilities message. The server always considers the UE reported capabilities as "applicable" (as currently defined in LPP). However, the "applicable functionality" (i.e., LPP capabilities) may change during an LPP session.

Fig. 2.2.3: Generic LPP positioning procedure.

1.	The server may send a request for the positioning capabilities of the target (e.g., if the stored capabilities were not provided by the AMF). The message may include a request for the AI/ML supported functionality of the target.

2.	The target transfers its positioning capabilities to the server, including the supported "AI/ML functionality".

NOTE 1: 	At this point, the UE has not received a location request, and may not have received assistance data (e.g., via posSIB). Therefore, the reported capabilities would indicate the UE "supported functionality".

3.	The server selects a positioning method or methods based on the UE and network capabilities and transfers assistance data to the target.

NOTE 2:	The Assistance Data ("NW side (additional) conditions") are assumed to be the currently defined assistance data for the individual positioning methods, possibly with enhancements if needed (RAN1).

NOTE 3:	The server may know whether the UE "supported functionality" provided at step 2 is also "applicable" for this session, e.g., because the assistance data and associated attributes/quality for the UE "supported functionality" have not changed in the network recently.

4.	The target may infer from the received assistance data at step 3 that the "supported functionality" provided at step 2 is not "applicable" for this session (e.g., some network conditions (assistance data) may be incompatible with the available/trained model) and provides its updated capabilities to the server ("applicable functionality").

NOTE 4:	At this stage, the reported capabilities would indicate updated/changed "supported functionality", which correspond to the "applicable functionality" in the given situation (i.e., based on the assistance data/network conditions from step 3). This could also mean that AI/ML based positioning is not available/possible in the given situation/network configuration.

5.	The server may transfer additional and/or updated assistance data to the target matching the updated capabilities (i.e., "applicable functionality") reported at step 4.

NOTE 5:	This may imply a change of selected positioning method in some cases (e.g., change from AI/ML enhanced DL-AoD to AI/ML enhanced DL-TDOA, etc.), or fallback to non-AI/ML based positioning.

6.	The server may send a request for location information to the target for the updated capabilities ("applicable functionality") provided at step 2 or step 4.

7.	The target determines the requested location information.

8.	In response to step 6, the target transfers location information to the server.

### 2.2.4	Activated Functionality

According to the proposals in [4], "activated functionality refers to functionalities already activated and performing inference". For positioning, there seems no use case/need for such a definition. According to Fig. 2.2.3 above, there is no difference in UE/server behaviour when a LPP Provide Capabilities message provides "supported"/"applicable" or "activated" functionality. A UE may have multiple LPP location sessions ongoing (i.e., already performing inference for another location request) and a LMF may also be aware of multiple location requests for a UE. The handling of concurrent location requests is up to UE and LMF implementation [6].

For example, if a UE has functionalities already activated and performing inference when a LPP Request Capabilities message at Step 1 in Fig. 2.2.3 arrives, the UE will provide its 'supported' = 'applicable' = 'activated' functionality at Step 2. The UE and LMF would not behave differently in this case. A UE may receive a location request from an LMF after the LMF receives a location request from an AMF [6], [8]. If there is already an ongoing location session with the same UE ("activated functionality"), and this location request fulfils the requirements for the new location request, the LMF normally does not send a second concurrent location request but uses the UE results of the first (ongoing = "activated") request to fill both location requests from the AMF. Therefore, there seems no need/use for an "activated functionality" reporting. Functionality is/will be activated when a UE receives a LPP Request Location Information message. A LMF will be aware of "activated functionality" once it sent a LPP Request Location Information to a UE.

Proposal 17: There is no need to report "activated functionality" to an LMF e.g., in a LPP Provide Capabilities message.

Proposal 18: AI/ML positioning functionality is activated in a UE when an applicable/valid LPP Request Location Information message has been received and may be deactivated in the UE once the LPP Provide Location Information message has been returned to the server.

# Conclusion

LCM for Beam management Use case:

Observation 1: Achieving constancy based on performance monitoring is not feasible, because of the following reasons

- Extra burden at the UE for monitoring before the functionality is activated for inference

  - Note that UE has to monitor the performance of a significantly large number of models to determine the best model for current network-side additional conditions.

  - Monitoring-based model selection needs to be performed every time network side additional condition changes. Furthermore, the network may required to signal to the UE when network-side additional conditions change.

- Will always be a hit/fail method

  - UE cannot determine with certainty the consistency, as without the information about the network-side additional condition, the UE may not be able to download the relevant models

    - Note that UE may only have limited memory to store the models. It may require frequent download/discard of the model to support monitoring-based consistency.

- UE may need to monitor the performance for a significantly large amount of time before UE can determine a suitable model. This will result in a significant delay in determining applicable functionality at the UE.

Proposal 1:  For beam management use case, signaling of associated IDs for determining applicable functionalities should be mandatorily supported by the network.

Observation 2: RAN1 assumed that NW-side additional conditions with the same associated ID are consistent at least within a cell.

Observation 3: The uniqueness of associated ID within a single cell is not scalable, as it will create challenges for the UE side to generalize models across multiple cells.

Observation 4: The lack of uniqueness of the associated ID poses challenges not only on UE-side model development but also on runtime LCM.

Observation 5: Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions.

Proposal 2: NW-side additional conditions with the same associated ID are consistent within a PLMN.

Proposal 3: How the uniqueness of the associated IDs within a PLMN is achieved can be left up to OAM implementation.

Observation 6: The following two methods can be used for retrieving applicable functionalities

- The source cell configures UE to provide applicable functionalities for the target / candidate / neighboring cell(s). The applicable functionality information can be provided to the target / candidate cell(s) during handover preparation.

- Target cell configures UE to provide applicable functionality for it after successful handover completion.

Observation 7: In procedures where the source cell can configure UE to provide applicable functionalities for the target / candidate / neighboring cell(s), inference configuration latency can be reduced significantly.

Proposal 4: RAN2 is requested to discuss if UAI can be used for reporting the applicable functionalities for target/candidate/neighboring cells.

Observation 8: Applicability functionality can change due to

- Change in network side additional condition,

- UE-side additional condition, or

- Model availability.

Observation 9: When the inference configuration is provided based on applicable functionalities information, the network may need to provide UE with new/updated inference configurations quite frequently as applicable functionalities can change frequently.

Observation 10: By allowing UE to be configured with supported functionalities that are not yet applicable (see Fig. 2.1.2.2), the need for frequent inference configuration can be reduced.

Proposal 5: For LCM for UE-sided model for Beam Management use case, even if functionality(es) is not applicable they can be configured at the UE.

Proposal 6: The applicable functionality is initially considered deactivated when the inference configuration is provided based on supported functionalities information.

Proposal 7: A configured functionality can only be activated when it becomes applicable. FFS on required procedures/signaling.

Proposal 8: To facilitate “Network decision, network initiated” AI/ML Management, the network can configure UE with

- Data collection to report required information elements for supporting monitoring at the network, or

- KPIs evaluated at the UE that can be used at the network for AI/ML management decisions.

Observation 11: The network’s AI/ML management decision can be based on several factors, e.g., network issues, performance issues, etc. The AI/ML management decision at the network can be due to

- Performance issue,

- Change in additional condition (or network configuration) at the network, or

- Temporary network overload/failure to support AI/ML-based feature/feature groups.

Observation 12: The AI/ML management decisions at the UE can be due to

- Performance issue, or

- Change in applicable functionalities, due to changes in

  - Model availability at the UE, or

  - Additional conditions at the UE or network.

Proposal 9: To facilitate appropriate action (for example, model retraining, etc.), the network should signal the cause of AI/ML management together with the AI/ML management signal.

Proposal 10: To facilitate “UE autonomous, decision reported to the network”, AI/ML Management, the network provides assistance information (e.g., network side additional condition) reporting from the network to the UE.

Proposal 11: UE reports the management decision (without cause) to the network upon taking the AI/ML management decisions.

Proposal 12: When the AI/ML functionalities were deactivated, or fallback to legacy procedures based on “Network decision, network-initiated” AI/ML Management, or “UE autonomous, decision reported to the network”, AI/ML Management,

- The UE can evaluate the applicable functionalities and request activation of the application functionality.

LCM for Positioning Enhancement Use case:

Observation 13: The terminology of "solicited" and "unsolicited" capability/assistance data/measurement transfer is used/supported in LPP since Rel-9 and can corresponds to the "reactive" and "proactive" reporting, respectively, by arranging the individual LPP transactions into appropriate procedures (see also Figure 2.2.3 and discussion below).

Proposal 13: The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML functionality.

Observation 14: The "Network Conditions" for positioning comprise (at least) the already standardized LPP Assistance Data required for positioning measurements and/or position calculation. UE supported Assistance Data are already part of the UE capabilities.

Observation 15: A change in UE positioning capabilities can happen at any time during an LPP session. A UE can provide its updated capabilities via unsolicited LPP Provide Capabilities message.

Proposal 14: Regarding the "FFS" on how to handle dynamic capabilities in the agreement from RAN2#126, no additional specification changes are needed. I.e., "dynamic capabilities" can be handled as already specified in LPP via unsolicited Provide Capability messages.

Proposal 15: RAN2 assumes that any "Network-side (Additional) Conditions" for the positioning use case are the LPP Assistance Data elements, possibly with some enhancements, if needed. An "Associated ID" to identify "Network-side (Additional) Conditions" is not needed.

Proposal 16: There is no need to explicitly distinguish between "supported functionality" and "applicable functionality" in a e.g., LPP Provide Capabilities message. The server always considers the UE reported capabilities as "applicable" (as currently defined in LPP). However, the "applicable functionality" (i.e., LPP capabilities) may change during an LPP session.

Proposal 17: There is no need to report "activated functionality" to an LMF e.g., in a LPP Provide Capabilities message.

Proposal 18: AI/ML positioning functionality is activated in a UE when an applicable/valid LPP Request Location Information message has been received and may be deactivated in the UE once the LPP Provide Location Information message has been returned to the server.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- "Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)", Intel Corporation, Samsung.

- RAN1#116bis meeting report

- 3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

- 3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

- 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

- R2-2108960, "Response LS on storage of UE Positioning Capabilities", RAN2#115.

- RAN1#117 meeting report