3GPP TSG-RAN WG2 Meeting #127		     R2-2406644

Maastricht , NL: 19th August – 23rd August 2024

Source:	Qualcomm Incorporated

Title:	On Network Side Data Collection

Agenda Item:	8.1.3 NW Side Data Collection

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, at RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved in [3].

This contribution paper discusses the data collection model training, inference, and monitoring for the network side models, for beam management and positioning enhancements use cases.

# Data Collection for Network Side Models

## 2.1   Data Collection for Network-side Model for the Beam Management

In the RAN2#126 meeting [4], RAN2 made the following agreements for the network-side model training for the beam management use case:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting

During the RAN2#126 meeting [4], the events for data collection for network-side model training were discussed briefly, however, no agreements were made on the events definitions for data collection/reporting. Furthermore, RAN2 agreed to use immediate MDT for OAM-centric data collection. In this paper, we will discuss

- Event definitions for data collection for network-side model training, and

- Issues associated with logging UE measurement.

### 2.1.1	Event definition for data collection for model training

Events / filtering mechanism for data collection and reporting are an effective tool for collecting meaningful data while reducing the power and memory consumption at the UE. Therefore, we believe that events-triggered data collection / filtering mechanism for network-side model training can be supported for beam management use cases. However, we also believe that events / filtering mechanism for the data collection / filtering mechanism for the network side model training should be defined such that it reduces the power and memory consumption at the UE without creating biases in the data distribution

- Reduction of redundant data collection and reporting (including logging, if supported) at the UE,

- Collect meaningful data for model training, without creating biases.

Observation 1: Events / filtering mechanism for data collection and reporting are effective tools for collecting meaningful data while reducing the power and memory consumption at the UE.

Proposal 1: Events / filtering mechanism for the data collection / filtering mechanism for the network side model training should be defined such that it reduces the power and memory consumption at the UE without creating biases in the data distribution

- Reduction of redundant data collection and reporting (including logging, if supported) at the UE,

- Collect meaningful data for model training, without creating biases.

### 2.1.2	Logging and reporting of training data

In the RAN2#126 meeting [4], RAN2 agreed that “For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.” In this section, we ask for RAN2 clarification on several issues when logging is supported at the UE.

Observation 2: For the beam management use case, RAN2 agreed that logging of L1-RSRP measurement for gNB-centric / OAM-centric data collection for NW-side model training is an optional feature for the UE.

#### 2.1.2.1	SRB used for reporting logged training data

Note that L3 and immediate MDT measurements are reported using SRB1 / SRB3. In the legacy L3 and immediate MDT measurement collection and reporting, data logging is not supported. Therefore, using SRB1 / SRB3 for reporting L3 and immediate MDT measurements is okay, as the report size is quite small. However, when logging is supported (or size of single sample of training data even if logging is not supported expected to be large) for training data collection/reporting, reporting the training data over SRB1 / SRB3 is not desired, as it can cause head-of-line blockage to important message/measurement reporting. Therefore, SRB4 / SRB5 / a new SRB should be used for reporting training data.

Observation 3: In the legacy L3 / immediate MDT measurement collection and reporting, data logging is not supported.

Observation 4: When logging is supported (or size of single sample of training data even if logging is not supported expected to be large) for training data collection/reporting, reporting the logged training data over SRB 1 is not desired, as it can cause head-of-line blockage to important message/measurement reporting.

Proposal 2: Reporting of the training data should be performed using SRB4 / SRB5 / a new SRB, irrespective of whether the logging is supported or not.

#### 2.1.2.2 	Leakage of collected training data

This section discusses some concerns associated with the leakage of the training data from one PLMN to another and one infra-vendor to another infra-vendor.

##### 2.1.2.2.1  From one PLMN to another PLMN

In the legacy immediate MDT measurement collection and reporting, data logging is not supported. This implies that serving gNB configures UE with measurement/reporting configuration based on the configuration received from the OAM, and receives the measurements which is then forwarded to TCE. During the handover / Reestablishment / Resume / Reconnection, the immediate MDT configuration is forwarded from the source gNB / AMF to the target gNB if they belong MDT PLMN List configured by the OAM. While the reporting of measurement collected is immediate UE is not required to check PLMN before reporting. UE is not aware of the MDT PLMN List.

Let us consider the following scenario (see Fig. 2.1.2.1) where a cell gNB1 (belonging to vendor 1 and PLMN 1) configures UE with training data collection (measurement and reporting configuration), but cannot retrieve the logged training data because of the following reasons

- The gNB1 was overloaded and couldn’t retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by gNB1.

Fig. 2.1.2.1.1: OAM-centric data collection, whether the logged training data can be reported to other PLMN or other-infra-vendor cells if they belong to the same/different PLMN.

We prefer RAN2 to clarify the UE behavior when UE has unretrieved logged training data during handover, RRCReestablishment, or RRC state transitions. RAN2 is further requested to clarify if there are any concerns associated with the leakage of collected training data from one PLMN to another PLMN (also one infra-vendor to another infra-vendor for OAM-centric data collection) if UE reports the collected training data target cell / Reestablishment / Reconnection cell / Resume cell if it belongs to the different PLMN.

##### 2.1.2.2.2  From one infra-vendor to another infra-vendor

In the legacy L3 measurement collection and reporting, data logging is not supported. This implies that serving gNB configures UE with measurement/reporting configuration, and receives the measurements. During the handover, the measurement/reporting configuration is handled by the following methods

- UE maintains the configuration if configured with need code M

- UE releases the configuration if configured with need code R

- Serving/target cell can explicitly release the configuration by sending a reconfiguration message

Let us consider the following scenario (see Fig. 2.1.2.1) where a single PLMN deploys the cells/gNBs from two different infra-vendors. Let us assume that gNB1 from vendor 1 configures the data collection at the UE, but cannot retrieve the logged training data because of the following reasons

- The gNB1 was overloaded and couldn’t retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by gNB1.

Fig. 2.1.2.1.2: gNB-centric data collection, whether the logged training data can be reported to other infra-vendor cells if they belong to the same/different PLMN.

We prefer RAN2 to clarify the UE behavior when UE has unretrieved logged training data during handover, RRCReestablishment, or RRC state transitions. RAN2 is further requested to clarify if there are any concerns associated with the leakage of collected training data from one infra-vendor to another infra-vendor if UE reports the collected training data target cell / Reestablishment / Reconnection cell / Resume cell if it belongs to the same/different PLMN. Alternatively, UE can provided with the AreaConfig. (e.g., list of cells/gNBs) where UE can report the logged training data.

In our understanding, the following can be the possible solutions:

- UE releases the training data configuration (measurement /reporting config.) and unretrived training data upon handover / RLF / RRC state transition.

- UE can report the unretrived training data to any cell within a list of PLMNs. FFS How UE determines in which PLMN UE can report unretrieved logged training data.

- UE can report the unretrived training data to any cell if they belong to the same PLMN.

- UE can report the unretrived training data to cells configured. Configuring gNB should provide the list of cells/gNBs where the unretrieved logged training data should be reported.

Observation 5: In the following scenarios, the configuring gNB may not be able to retrieve the logged training data

- The configuring gNB was overloaded and could not retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by the configuring gNB1.

Proposal 3: For OAM-centric data collection, RAN2 is requested to clarify if there are any concerns associated with the leakage of collected training data when it is reported from

- Configuring PLMN (or MDT PLMN List) to another PLMN. FFS How does UE know which PLMNs UE can report unretrieved logged training data

- From one infra-vendor (configuring) to another infra-vendor

Proposal 4: For gNB-centric data collection, RAN2 is requested to clarify if there are any concerns associated with the leakage of collected training data when it is reported from

- From one infra-vendor (configuring) to another infra-vendor.

Proposal 5: RAN2 is requested to clarify the UE behavior when UE has unretrieved logged training data during handover, RRCReestablishment, or RRC state transitions, among the following

- UE releases the training data configuration (measurement /reporting config.) and unretrived training data upon handover / RLF / RRC state transition.

- UE can report the unretrived training data to any cell within a list of PLMNs. FFS How UE determines in which PLMN UE can report unretrieved logged training data.

- UE can report the unretrived training data to any cell if they belong to the same PLMN.

- UE can report the unretrived training data to cells configured. Configuring gNB should provide the list of cells/gNBs where the unretrieved logged training data should be reported.

## 2.2   Data Collection for Network-side Model for Positioning Enhancements

### 2.2.1  Network-based positioning/NG-RAN node assisted (Case 3a/b)

For Case 3a and 3b there may be at least two aspects with potential RAN2 specification impacts:

- UE SRS configuration/transmission, and

- "ground truth" label generation for model training and label-based performance monitoring.

For both, Case 3a and 3b the UE requirement for positioning is only to transmit SRS, which is an existing functionality and should not require additional RAN2 specification changes.

Potential RAN2 impacts are related to the label generation ("ground truth") required for both, model training and (label-based) performance monitoring. For model training and label-based model performance monitoring some "ground truth" labels are required e.g., to compare model output with 'truth'. This boils down to having knowledge of the actual UE/PRU position estimate available at the training/monitoring entity. In addition, TRP/ARP location information may be needed, e.g., to determine 'true' UL rTOA or gNB Rx-Tx Time differences, etc.

Location Services (LCS) allows an LCS Client to obtain the location of one or more target UEs [7] using a variety of positioning methods [8]. Location information for one or multiple target UEs ("bulk location request" [7]) may be requested by and reported to an LCS client or an AF within or external to a PLMN, or a control plane NF within a PLMN.

For PRUs, the PRU location would be "automatically" available at an LMF e.g., during PRU registration [7].

These UE/PRU LCS procedures also ensure that user privacy is preserved via various means [7]. Therefore, for ground-truth label generation, existing LCS and UE positioning procedures/methods can be used. This allows determining a desired UE location with any UE and NW supported method(s) (incl. RAT-dependent and RAT-independent methods, as well as network-based methods) and using PRUs with known location (e.g., surveyed location).

Proposal 6: For Positioning Enhancements Case 3a/3b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [7] and UE positioning [8] procedures are used to obtain the UE/PRU location estimate by an LMF.

If UE/PRU location information is needed in the NG-RAN (e.g., for model training or label-based monitoring in Case 3a), a LMF could provide this information to the NG-RAN, e.g., upon NG-RAN request. However, NG-RAN as LCS Client/AF is currently not supported, primarily due to privacy and security issues; e.g. ([8], clause 5.2):

"The case that the NG-RAN node functions as an LCS client is not supported in this version of the specification."

RAN3 studied NG RAN acting as LCS client before, but without conclusion [9]. Therefore, it can be left to RAN3 (and possibly SA2/SA3) to investigate if/how UE or PRU ground-truth labels can be available at NG-RAN.

Proposal 7: For Positioning Enhancements Case 3a, it is up to RAN3 (and possibly SA2/SA3) to decide whether UE/PRU ground-truth information (labels) can be available in the NG-RAN.

### 2.2.2  UE-assisted positioning with LMF-side model (Case 2b)

For case 2b, the UE reports PRS-based measurements to an LMF as it is the case with e.g., DL-TDOA, DL-AoD, or Multi-RTT positioning. The UE is not, and does not need to be, aware of how an LMF processes the measurements (e.g., using conventional positioning, AI/ML positioning, or a combination (hybrid) of various positioning approaches, etc.). Therefore, the UE behaviour is not different when reporting measurements for e.g., position calculation, LMF model inference, performance monitoring, or data collection for LMF-side model training. As concluded by SA2, the trigger for data collection and for model training in an LMF is up to implementation [10].

As also discussed in section 2.3.1 above, for obtaining ground truth information existing LCS/UE positioning procedures can be used. For example, for data collection, the network can instigate a MT-LR to collect desired measurements from a UE/PRU, or to obtain the position of a UE/PRU using any of the UE/PRU supported positioning methods, etc.

Therefore, potential RAN2 specification impacts for Case 2b are enhancements to LPP to support any new RAN1 defined measurement information (if needed), for both, model training and validation, and model deployment and inference.

Proposal 8: For LMF-side data collection (Positioning Enhancements Case 2b), existing LCS procedures like NI-LR, MT-LR and/or deferred MT-LR are used.

Similar to the discussion in section 2.3.1:

Proposal 9: For Positioning Enhancements Case 2b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [7] and UE positioning [8] procedures are used to obtain the UE/PRU location estimate by an LMF.

# Conclusion

Data collection and reporting for beam management use case:

Observation 1: Events / filtering mechanism for data collection and reporting are effective tools for collecting meaningful data while reducing the power and memory consumption at the UE.

Proposal 1: Events / filtering mechanism for the data collection / filtering mechanism for the network side model training should be defined such that it reduces the power and memory consumption at the UE without creating biases in the data distribution

- Reduction of redundant data collection and reporting (including logging, if supported) at the UE,

- Collect meaningful data for model training, without creating biases.

Observation 2: For the beam management use case, RAN2 agreed that logging of L1-RSRP measurement for gNB-centric / OAM-centric data collection for NW-side model training is an optional feature for the UE.

Observation 3: In the legacy L3 / immediate MDT measurement collection and reporting, data logging is not supported.

Observation 4: When logging is supported (or size of single sample of training data even if logging is not supported expected to be large) for training data collection/reporting, reporting the logged training data over SRB 1 is not desired, as it can cause head-of-line blockage to important message/measurement reporting.

Proposal 2: Reporting of the training data should be performed using SRB4 / SRB5 / a new SRB, irrespective of whether the logging is supported or not.

Observation 5: In the following scenarios, the configuring gNB may not be able to retrieve the logged training data

- The configuring gNB was overloaded and could not retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by the configuring gNB1.

Proposal 3: For OAM-centric data collection, RAN2 is requested to clarify if there are any concerns associated with the leakage of collected training data when it is reported from

- Configuring PLMN (or MDT PLMN List) to another PLMN. FFS How does UE know which PLMNs UE can report unretrieved logged training data

- From one infra-vendor (configuring) to another infra-vendor

Proposal 4: For gNB-centric data collection, RAN2 is requested to clarify if there are any concerns associated with the leakage of collected training data when it is reported from

- From one infra-vendor (configuring) to another infra-vendor.

Proposal 5: RAN2 is requested to clarify the UE behavior when UE has unretrieved logged training data during handover, RRCReestablishment, or RRC state transitions, among the following

- UE releases the training data configuration (measurement /reporting config.) and unretrived training data upon handover / RLF / RRC state transition.

- UE can report the unretrived training data to any cell within a list of PLMNs. FFS How UE determines in which PLMN UE can report unretrieved logged training data.

- UE can report the unretrived training data to any cell if they belong to the same PLMN.

- UE can report the unretrived training data to cells configured. Configuring gNB should provide the list of cells/gNBs where the unretrieved logged training data should be reported.

Data collection and reporting for positioning enhancements use case:

Proposal 6: For Positioning Enhancements Case 3a/3b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [7] and UE positioning [8] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 7: For Positioning Enhancements Case 3a, it is up to RAN3 (and possibly SA2/SA3) to decide whether UE/PRU ground-truth information (labels) can be available in the NG-RAN.

Proposal 8: For LMF-side data collection (Positioning Enhancements Case 2b), existing LCS procedures like NI-LR, MT-LR and/or deferred MT-LR are used.

Proposal 9: For Positioning Enhancements Case 2b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [7] and UE positioning [8] procedures are used to obtain the UE/PRU location estimate by an LMF.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- Meeting Report, “RAN2#125bis meeting report”

- Meeting Report, “RAN1#116bis meeting report”

- Meeting Report, “RAN1#116 meeting report”

- 3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

- 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

- 3GPP TR 38.856: "Study on local NR positioning in NG-RAN".

- S2-2407188, "Interim conclusions for KI#1", SA2#163.