3GPP TSG-RAN2 Meeting #127	R2-2406651

Maastricht, Netherlands, August 19-23, 2024

Agenda Item:	8.2.3

Study Item:	FS_Ambient_IoT_Solutions

Source: 	Qualcomm Incorporated

Title: 	Paging for Ambient IoT

Document for:	Discussion/Decision

# Introduction

RAN#103 approved SI for Ambient IoT in [1].

RAN2#125bis discussed on paging aspects of AIoT and agreed the following:

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

RAN2#125bis discussed on stage-2/general aspects of AIOT and agreed the following:

Agreements - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2. - RAN2 will support two use cases, “inventory” and “command”. The definition, detailed wording is FFS - Baseline procedure: Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS Step C: The device may perform the data communication with the reader as needed,: Details FFS - We will study the support of both “inventory” and “command” in the same procedure. - FFS if Initial Trigger Message can also include “command”. - RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input.

RAN2#126 agreed the following:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

This document provides more details about paging aspects for Ambient IoT, which are applicable to both topologies and all device types.

# Discussion

## AIoT inventory and/or command towards the AIoT device

In both topologies, inventory refers to determining the identity of all or a subset of AIoT devices within the range of Reader(s). The AIoT application sends the inventory request optionally including the information such as a list of Reader IDs or area information to limit the inventory to specific group of Readers, the filter criteria to limit the inventory to certain AIoT devices matching the criteria. This is being discussed in SA2.

SA2 is discussing the details on the command protocol as well as the procedures for Ambient IoT Application Function to trigger AIoT inventory and AIoT command.

Each Reader that received the inventory request should trigger the inventory procedure to discover or trigger AIoT devices to perform AIoT access procedure. In inventory procedure, Reader attempts to discover all or a subset of AIoT devices in the range of Readers. In that sense, before the Reader performs the inventory, the Reader has no information on how many AIoT devices or which specific AIoT devices are or maybe nearby the Reader.

For AIoT command, the AIoT application function or AIoT Controller can indicate the last known reader to initiate AIoT command towards the AIoT device(s). However, the last known reader may not be able to reach the AIoT device(s). In such case, the AIoT AF or AIoT Controller may perform escalation techniques which are outside of RAN2 discussion scope. However, it should be noted that this should not mean a AIoT reader creates a cell.

RAN2 assumes the AIoT Application Function or AIoT Controller indicates the last known reader to initiate AIoT inventory or command towards the AIoT device(s).

## Contents of initial trigger message (aka AIoT paging)

In the inventory or command procedure, the trigger message may optionally include the filter criteria sent by the upper layer to identify the AIoT devices that are supposed to respond to the inventory request or command message. The Reader forwards the filter criteria in the inventory signaling to the AIoT devices. AIoT device is required to check the conditions whether the criteria are satisfied or not before initiating the AIoT access.

For the AIoT access, the AIoT device should check the filter criteria provided by Reader before performing the AIoT access. Only if the criteria are satisfied, the AIoT devices are allowed to perform the AIoT access to the Reader. What information are included in the filter criteria can be up to SA2 for further study.

Reader may include filter criteria (to identify the AIoT devices that are supposed to respond) in the Initial Trigger message (aka AIoT paging) sent towards the AIoT device(s). It is up to SA2 to define the details of the filter criteria.

RAN2 previously agreed that the AIoT paging message may or may not contain any device ID, and in the latter case, it is considered to be addressed for all AIoT devices within the coverage of the reader that can receive the AIoT message. If filter criteria is introduced, RAN2 should amend the previous agreement also considering the (absence of) filter criteria.

If neither any filter criteria nor any device ID is included in the initial trigger message, the message is considered as addressed for all devices that can receive the AIoT message.

In the past RAN2 meetings, the AIoT ‘4-step’ or 2-step random access was preliminary discussed. Since AIoT access is triggered by Reader, the Reader is responsible for providing the access resource considering the different service types, i.e., inventory request or command message. Reader may have preference on which type of AIoT access to be triggered to respond the service request.

For example, if Reader would like to only issue the ‘Write’ command to a small number of AIoT devices, a 2-step AIoT access can be selected for AIoT devices to feed back the simple confirmation after the command execution. In another example, Reader may indicate AIoT devices to perform so called ‘4-step’ AIoT access for the case that a large number of AIoT devices perform AIoT access by using the contention-based resource. Therefore, Reader should be able to indicate the type of AIoT access to AIoT devices in the trigger message.

Reader may indicate the type of AIoT access (e.g., 2-step or 4-step) to AIoT devices in the initial trigger message (aka AIoT paging).

RAN2#126 discussed the so-called ‘Msg4’. RAN2 agreed to not use the term ‘msg4’ for further discussion of the AIoT random access. Instead, it could be called subsequent R2D message after D2R transmission. Such message may be a failure indication for Msg3 transmission or could be the acknowledgement like the Msg4 of NR random access.

While some scenarios would need the subsequent R2D message (e.g. for acknowledging the reception of the AIoT device ID and/or AIoT upper layer data sent in the previous D2R message or for triggering the next round of AIoT re-access or AIoT data communication), in other scenarios there would be no such subsequent R2D message (e.g. Reader requesting information from temperature sensors and no intention to ACK/NACK successful reception).

It is beneficial for AIoT device to have correct expectation on potential future steps after transmitting a D2R communication in response to the initial trigger/AIoT paging. To facilitate the AIoT device actions, the initial trigger message can indicate to the AIoT device whether a subsequent R2D message would be present.

The initial trigger message indicates the presence and information carried in the subsequent R2D message.

  - On FFS if Initial Trigger Message can also include “command”

As shown in section 1, there is an FFS from RAN2#125bis.

5	We will study the support of both “inventory” and “command” in the same procedure.

6	FFS if Initial Trigger Message can also include “command”.

In our view, there may be valid use cases where the initial trigger message also includes command. E.g. suppose the AIoT application has some idea about the AIoT devices in a given area (e.g. based on deployment information within a warehouse) but does not have most up to date inventory information. In such case, the application may want to initiate a procedure which involves inventorying all the devices that satisfy certain criteria (e.g. all devices which are temperature sensors and belonging to booth # X). One might see the benefit of having ability to send a single message in the R2D link to indicate to the sensors (satisfying the criteria) to reply with their temperature values in the D2R.

However, there may be some challenges to this. Firstly, it depends on how many devices would satisfy the criteria. It would be difficult if the reader does not have a good idea without prior inventory. However, in some use cases based on deployment information, the reader might already know how many devices may possibly respond to the initial trigger. Depending on the number of AIoT devices that need to respond, the reader can estimate the resources needed for the D2R link, e.g. whether the devices have to go through contention based random access procedure or whether they can be provided with contention free resources. In case of contention-based RA, the amount of potential collisions also depends on the potential number of devices that get triggered.

Therefore, in our view, it is ok to support both ‘inventory’ and ‘command’ in the same ‘request’ from the application to the Reader. In addition, in the air interface between the reader and the device, for certain use cases, using a single message for both ‘initial trigger’ and ‘command’ can also be supported.

As previously agreed, supporting both ‘inventory’ and ‘command’ based on a single request from application can be achieved as an inventory followed by command back-to-back as part of a single procedure.

Using a single message to initiate both initial trigger and command can add complexity if the device(s) were not already inventoried and may bring some challenges (e.g. how to provide resources for BL depends on potential number of devices that gets triggered). However, for some use cases it can be beneficial.

RAN2 will study solutions where Initial Trigger Message from the Reader also includes “command”.

  - Awareness of Upper layer request at the Reader

RAN2 agreed as a first step of the baseline procedure, the reader sends the Initial Trigger Message indicating device(s) that need to respond. Such trigger would be based on the service request from the upper layer and details were left as FFS.

One of the discussion points is whether the details of such upper layer request will be visible to the Reader, or is it transparently passed to the AIoT device(s) by the Reader without any knowledge of the content.

Obviously, the two extreme viewpoints would be to have everything transparent to Reader vs having everything visible to the Reader. Both approaches have their pros and cons as summarized in the following table.

|  | Pros | Cons |
| --- | --- | --- |
| Command/Request Content fully visible to Reader | Reader can do more optimized resource management based on each command/request | Reader may be more complex to handle different type of commands separately More optimized procedures depending on different type of command/request may need to be studied/defined in specification End to end security between the device and AIoT Application may not be possible as the Reader would need to understand the content. |
| Command/Request transparent to the Reader | Specification may be simple End to end security between device and AIoT Application can be possible | Reader may not know whether a specific request needs a response from device(s), and expected size of the response. Then resource over-provisioning may be needed. |

Between the above two extremes, RAN2 should explore whether there is some middle ground. For example, the Reader may be made aware of some information, e.g.:

- Request type is known to reader (e.g. inventory vs command).

- Whether acknowledgment is expected for a device is known to the reader.

- Expected response size or type is known to the reader.

- The device information such as Device ID (if provided by upper layer) is visible to Reader at least for the command procedure.

- The information that may impact the cast type determination for trigger message and resource indication is visible to the Reader.

It should be possible to maintain the end-to-end “transparent” nature of the upper layer requests (i.e. the AIoT Application can send the Reader a request message to send to AIoT device(s) without modifying it) while also making the Reader aware of needed information to enable more efficient operation of R2D and D2R radio link.

It is possible to have end-to-end requests between the AIoT Application and the AIoT Device(s) while also making Reader aware of required information to facilitate the AIoT Paging by the Reader.

RAN2 discuss on the information that the Reader needs to be made aware about the upper layer request from AIOT Application.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	SA2 is discussing the details on the command protocol as well as the procedures for Ambient IoT Application Function to trigger AIoT inventory and AIoT command.

Observation 2.	As previously agreed, supporting both ‘inventory’ and ‘command’ based on a single request from application can be achieved as an inventory followed by command back-to-back as part of a single procedure.

Observation 3.	Using a single message to initiate both initial trigger and command can add complexity if the device(s) were not already inventoried and may bring some challenges (e.g. how to provide resources for BL depends on potential number of devices that gets trigered). However, for some use cases it can be beneficial.

Observation 4.	It is possible to have end-to-end requests between the AIoT Application and the AIoT Device(s) while also making Reader aware of required information to facilitate the AIoT Paging by the Reader.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN2 assumes the AIoT Application Function or AIoT Controller indicates the last known reader to initiate AIoT inventory or command towards the AIoT device(s).

Proposal 2:	Reader may include filter criteria (to identify the AIoT devices that are supposed to respond) in the Initial Trigger message (aka AIoT paging) sent towards the AIoT device(s). It is up to SA2 to define the details of the filter criteria.

Proposal 3:	If neither any filter criteria nor any device ID is included in the initial trigger message, the message is considered as addressed for all devices that can receive the AIoT message.

Proposal 4:	Reader may indicate the type of AIoT access (e.g., 2-step or 4-step) to AIoT devices in the initial trigger message (aka AIoT paging).

Proposal 5:	The initial trigger message indicates the presence and information carried in the subsequent R2D message.

Proposal 6:	RAN2 will study solutions where Initial Trigger Message from the Reader also includes “command”.

Proposal 7:	RAN2 discuss on the information that the Reader needs to be made aware about the upper layer request from AIOT Application.

# References

[1] RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, March 2024

[2] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18