3GPP TSG-RAN WG2 Meeting #127	 R2-2406653

Maastricht, NL, August 19-23, 2024

Source:	KDDI Corporation

Title:	Discussion on Ondemand-SIB1 for RAR

Tdoc Type:	Discussion

Agenda Item:	8.5.3	On-demand SIB1

Document for:  Discussion

Release:       Rel-19

1	Introduction

In RAN2#126[1], on-demand SIB1 was discussed and below agreements were made.

Agreements on on-demand SIB1: RAR monitoring for MSG1 based OD-SIB1 REQ 13. RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

In this contribution, we further discuss on-demand SIB1 for RAR.

2	Discussion

It has been agreed that the UE will receive RAR in response to the preamble transmission in the Msg1-based on-demand SIB1 procedure as the baseline. However, the specific sequence of RAR has not yet been discussed. For example, the following points are assumed to need consideration in the discussion.

- The PDCCH configuration information necessary for the UE to receive RAR

- Whether the UE should expect to receive SIB1 or RAR first

- Error handling in case of RAR reception failure

2.1 PDCCH configuration for RAR

UE receives DCI format 1_0 with CRC scrambled by RA-RNTI, which includes the scheduling information for RAR on PDSCH, in the Type1-PDCCH Common Search Space configured by the ra-searchspace of PDCCH ConfigCommon[2]. As a means of notifying the configuration information of the Type1-PDCCH Common Search Space, it is possible to notify through the WUS configuration received before sending the WUS or through the MIB. In the case of the MIB, it would be necessary to define new configuration information for the Type 1 PDCCH Common Search Space. However, it is possible to utilize the existing standards by including it in the WUS configuration notified through SIB.

From Section 10.1 of TS 38.213: a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

Proposal1: RAN2 agree that WUS configuration includes PDCCH configuration to receive Type-1 PDCCH Common Search Space for DCI format 1_0 with CRC scrambled by RA-RNTI, RAR scheduling on PDSCH.

2.2 SIB1 and RAR reception sequence

After sending the WUS, the UE should expect to receive both SIB1 and RAR, and it is necessary to discuss whether NES Cell should send SIB1 or RAR first to UE. We think it seems reasonable to consider the case where the RAR is sent first, followed by the reception of SIB1.

For example, by sending the RAR before the SIB1, the NES Cell can include the information about the resource blocks necessary for SIB1 reception (CORESET#0), which can be considered as an option. Traditionally, it is necessary to discuss whether to include the pdcch-configSIB1 information contained in the MIB, which is required for SIB1 reception. However, expecting the UE to receive the SIB1 after receiving the RAR has its advantages.

Figure1. Ondemand-SIB1 with RAR

Proposal2: RAN2 agree that UE starts receiving SIB1 after receiving RAR from the NES Cell. How to acquire CORESET#0 from NES_Cell is FFS.

2.3 Error Handling for RAR Reception Failure

It is necessary to discuss the error handling procedures if UE fails to receive RAR within the raResponseWindow. Scenarios where RAR cannot be received include instances where the NES cell does not receive the RACH preamble, or where the NES cell transmits RAR but UE fails to receive it. In these cases, UE is unable to determine whether the steps required for Ondemand SIB1 reception have been successfully completed. If UE does not retransmit the WUS despite failing to receive RAR, it will be unable to determine when to abort WUS procedure to NES cell. Therefore, if UE fails to receive RAR, retransmitting WUS allows WUS procedure to continue seamlessly.

Additionally, by introducing a maximum retransmission count for the WUS retransmission process, it is possible to reduce excessive resource consumption caused by the UE transmitting RACH preamble without limitation.

Proposal3: RAN2 agree that UE will retransmit RACH preamble when UE fails to receive the RAR from NES Cell within the raResponseWindow.

Proposal4: RAN2 agree to introduce maximum retransmission numbers of WUS signals (RACH preambles).

Furthermore, in RACH preamble retransmissions, the transmission power is increased according to the powerRampingStep with each retransmission in legacy RACH preamble transmission procedure. In Ondemand-SIB1 scenario, it is also conceivable that the RACH preamble transmission might fail due to insufficient Uplink transmission power at UE. Therefore, increasing WUS signal transmission power according to the powerRampingStep can help address the issue of insufficient transmission power.

Proposal5: RAN2 agree to introduce the use of powerRampingStep to increase the uplink transmission power for WUS signal retransmissions.

3	Conclusion

We make the following observations and proposals:

Proposal1: RAN2 agree that WUS configuration includes PDCCH configuration to receive Type 1 PDCCH Common Search Space for DCI format 1_0 with CRC scrambled by RA-RNTI, RAR scheduling on PDSCH.

Proposal2: RAN2 agree that UE starts receiving SIB1 after receiving RAR from the NES Cell. How to acquire CORESET#0 from NES_Cell is FFS.

Proposal3: RAN2 agree that UE will retransmit RACH preamble when UE fails to receive the RAR from NES Cell within the raResponseWindow.

Proposal4: RAN2 agree to introduce maximum retransmission numbers of WUS signals (RACH preambles).

Proposal5: RAN2 agree to introduce the use of powerRampingStep to increase the uplink transmission power for RACH preamble retransmissions.

3	　Reference

[1]　RAN2#126 chairnote

[2]　3GPP TS 38.213 V18.1.0