3GPP TSG-RAN WG2#127	R2-2406654

Maastricht, Netherlands, 19-23 August 2024

Agenda Item:	8.2.2

Source: 	MediaTek Inc., Samsung, vivo, ZTE Corporation, Sanechips, Qualcomm Incorporated

Title:  	Ambient IoT transport block size and MAC layer segmentation

Document for:	Discussion, decision

# 1	Introduction

This document gives a brief analysis of ambient IoT transport block size in the context of a potential need for segmentation in the MAC layer.

# 2	Discussion

The PHY layer’s ability to carry data is of course in RAN1 remit.  From RAN2 perspective, there is no particular reason to choose one block size over another, beyond the need to achieve a “reasonable” minimum size for data transport (which RAN1 should target anyway), and it does not seem productive to discuss the question of transport block size in RAN2.  Thus we propose that RAN2 leave the decision to RAN1, with the expectation that a TB will be provided from MAC to fit the capacity of PHY.

Proposal 1: Indicate to RAN1 that RAN2 can define MAC PDU sizes to correspond to the capacity of the PHY layer, and RAN2 will accordingly follow RAN1 on the definition of the TB size.

It can be discussed if it is useful to provide some indication to RAN1 of the size of identifiers that RAN2 consider necessary, since the largest identifiers (e.g., upper-layer persistent identifiers) would impose a lower bound on the acceptable TB size in certain situations.  However, it seems safe to expect that RAN1 will design the largest TB sizes that meet link criteria, and RAN2 do not so far have a definitive view of the upper-layer identifier sizes.  Thus, no proposal is made in this respect, and we assume that companies can coordinate offline between RAN1, RAN2, and SA2 in relation to identifier sizes.

However, the maximum upper-layer data block (equiv. MAC SDU) size may need to be checked with SA2.  In section 5.5 of [1], it is indicated that a maximum “message” size of ~1000 bits is a design target, “based on the maximum application layer packet size”.  The point of reference appears to be [3], which lists a variety of use cases (sections 6.3-6.5) with maximum or “typical” message sizes of 1000 bits or less.  We see two issues with adapting between the upper-layer message size and the lower-layer TB size:

- Some use cases in [3] quote only “typical” maximum values, and it is not clear if these should be understood as true maximum sizes or if there could be occasional “jumbo” blocks that exceed them.  In the latter case, the MAC could see an upper-layer block exceeding the target 1000-bit threshold.

- It is not clear if the maximum TB size would be always available for transmission.  If the offered TB size depends on radio conditions (e.g., to allow lower-rate communication with greater coverage, or to cope with obstructions/interference/degradation that would impair communication at higher data rates), or if the device may not have enough energy reserves to transmit/receive for the amount of time required by the maximum size, then the MAC may need to segment upper-layer messages of less than 1000 bits to fit into small TBs.

To address point 1, we suggest inquiring of SA2 about whether upper layers are ever expected to provide the MAC with a data block above 1000 bits in size.

Proposal 2: Ask SA2 what data block sizes are expected to be delivered from upper layers to AIoT AS layers.

For point 2, some guidance is needed from RAN1.  It may be sufficient to ask RAN1 to clarify, along with the selected TB size(s), the conditions under which different sizes may be used.

Proposal 3: Inquire of RAN1 regarding the conditions under which different TB sizes may be used in both the D2R and R2D directions.

When answers are available from RAN1 and SA2, RAN2 should be able to infer whether and when MAC segmentation is needed.  We understand that the implications are as shown in table 1.  The analysis in the table is provided for information; no action is proposed now, but the responses from SA2 and RAN1 will indicate what the MAC layer needs to do.

| SA2 outcomes RAN1 outcomes | Upper layer size <= 1000 bits always, not segmented | Upper layer size can exceed 1000 bits, not segmented | Upper layer segments to <= max TB size |
| --- | --- | --- | --- |
| Max TB size >= 1000 bits, always available | No MAC segmentation needed | MAC segmentation needed unless maximum upper layer size aligns with TB size | No MAC segmentation needed |
| Max TB size >= 1000 bits, but availability depends on radio/power conditions | MAC segmentation needed | MAC segmentation needed | MAC segmentation or real-time negotiation between PHY and upper layers |
| Max TB size < 1000 bits, always available | MAC segmentation needed unless maximum upper layer size aligns with TB size | MAC segmentation needed | No MAC segmentation needed |
| Max TB size < 1000 bits, availability depends on radio/power conditions | MAC segmentation needed | MAC segmentation needed | MAC segmentation or real-time negotiation between PHY and upper layers |

Table 1: RAN1 and SA2 implications for MAC segmentation

# 3	Conclusion

This document promulgated the following proposals:

Proposal 1: Indicate to RAN1 that RAN2 can define MAC PDU sizes to correspond to the capacity of the PHY layer, and RAN2 will accordingly follow RAN1 on the definition of the TB size.

Proposal 2: Ask SA2 what data block sizes are expected to be delivered from upper layers to AIoT AS layers.

Proposal 3: Inquire of RAN1 regarding the conditions under which different TB sizes may be used in both the D2R and R2D directions.

A draft LS is appended.

# 4	References

[1] TR 38.848
[2] TR 23.700-13 (v0.4.0)
[3] TS 22.369

# 5	Annex: Draft LS to RAN1/SA2

Title:	LS on data block sizes for Ambient IoT

Response to:

Release:	Release 19

Work Item:	FS_Ambient_IoT_solutions

Source:	RAN2

To:	RAN1, SA2

Cc:

Contact Person:

#### Name: Nathan Tenny

#### E-mail Address:	nathan.tenny@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 have discussed the role of the MAC in handling upper-layer data blocks (MAC SDUs) and processing them into transport blocks (MAC PDUs).  It is RAN2 understanding that this processing may require segmentation as a MAC function, depending on the upper-layer block sizes supported by SA2 and the transport block sizes supported by RAN1.

RAN2 wish to inform RAN1 that RAN2 can define MAC PDU sizes to correspond to the capacity of the physical layer.  Accordingly, RAN2 intend to follow RAN1 on the values used for transport block sizes.  If RAN1 define multiple transport block sizes, RAN2 will need to understand the maximum and minimum TB sizes, in both D2R and R2D directions, and the conditions under which each size can be used over the AIoT interface.

Depending on the relationship between the supportable TB sizes and the size of a data block provided to the AS by upper layers (i.e., the maximum MAC SDU size), RAN2 may need to specify segmentation functionality as part of the AIoT MAC.  RAN2 understand from TR 38.848 and TS 22.369 that a maximum “typical” message size of ~1000 bits is expected, but it is not clear to RAN2 what the true maximum size is, nor if upper layers would perform some segmentation to adapt large messages to the TB size.  Accordingly, RAN2 would like to understand from SA2 the maximum data block size that is expected to be provided from upper layers to AS layers, in both D2R and R2D directions.

RAN2 would like to observe that there may be benefit to aligning the minimum TB size across different device types, to allow a consistent design of the first messages sent in each direction during paging/access procedures.

2. Actions:

To RAN1:

RAN2 respectfully request that RAN1 take this information into account, and feed back when it is possible to indicate what maximum and minimum TB sizes are expected to be supportable in PHY, in both D2R and R2D directions, and, in case there are multiple TB sizes, the conditions under which TBs of different sizes can be transmitted.

To SA2:

RAN2 respectfully ask SA2 to feed back on the expected maximum data block size delivered from upper layers to the AIoT AS layers, in both D2R and R2D directions..

3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #127bis 	14-18 October 2024	Hefei, CN

TSG-RAN WG2 Meeting #128	18-22 November 2024	Orlando, FL, US

RAN1 are considering data rates including 5 Kbps and 1 Kbps, which would imply that transmitting a 1000-bit TB would require 200 ms or 1 s respectively.  It is plausible that the device might not have the energy reserves to transmit/receive for a full second.