3GPP TSG-RAN WG2#127	R2-2406655

Maastricht, Netherlands, 19-23 August 2024

Agenda Item:	8.2.3

Source: 	MediaTek Inc.

Title:  	Procedures and signalling for ambient IoT paging

Document for:	Discussion, decision

# 1	Introduction

This document comprises a short discussion of paging for ambient IoT.

# 2	Discussion

## 2.1	Current state of paging

Based on agreements up to RAN2#126 (see [1]), AIoT paging has the basic functionality of (1) identifying one or more devices that should respond, and (2) conveying a description of radio resources to carry a response.  There is no concept so far of a “paging cycle” or a “tracking area”; the latter is somewhat outside RAN2 scope, but from RAN2 perspective there is no expectation that the device supports any kind of tracking area update functionality that affects AS paging behaviour.  (There could be upper-layer behaviour that helps the controller determine which readers should page the device, etc., but any such functionality should be transparent to the access stratum.)

We do not see the paging functionality as bound specifically to either the “inventory” or “command” use case.  From the reader perspective, it has an instruction from the controller that may involve collecting respondent identities (inventory) and/or delivering a transmission of data (command), and either of these operations requires bringing the device into contact with the reader.  Accordingly, this document will discuss the paging procedure as agnostic to inventory/command.

## 2.2	Data in paging message

In principle, and if both the transport block size and security considerations permit, it would be possible to embed some R2D data (which could be provided by upper layers, a command from the MAC layer, etc.) in the paging message itself.  If the R2D data block to be delivered is small (which should be a common case—e.g., a read command with only a target address and size), embedding it in paging would reduce the number of messages and so conserve energy, and it should be possible without much impact on the number of devices that can be addressed in a single paging message (limited by the R2D TB size).

We assume that most or all data transmissions would require an acknowledgement our response at some layer, so the reader and/or the application function can know the data were received (e.g., in case of a write command) or receive the requested application data (read command), but data-in-paging followed by an acknowledgement/response using the access procedure involves less overhead and less time than sending multiple messages in each direction, as shown in figure 1.

Figure 1: R2D data and ack, with and without data-in-paging

(The figure assumes contention; contention-free access would remove two steps from each flow.  Potential D2R data responsive to the R2D data, like a response to a read command, is not shown but could also occur in steps 2c/4.)

Note that it is not clear whether a “Msg3” D2R transmission is needed in case of paging for R2D data delivery (right side of the figure).  The device would not be expected to have anything to transmit, but the reader should know that contention resolution succeeded—e.g., Msg2 was not lost over the air—before transmitting the R2D data.  An alternative approach could be to piggyback the data on Msg2, but then the ack and/or Msg3 would become essential to confirm that the reader successfully reached the right device.  These aspects can be revisited when more progress has been made on the random access procedure.

Of course, if the data do not require an acknowledgement, the difference is even greater: If the data can be embedded in the paging message, the paging message could request no response at all, and the flow would collapse to a single arrow (paging+data).  Without data-in-paging, the device would still need to go through the access procedure to establish communication and allow transmission of the R2D data (the flow would proceed through step 3 of the right side of figure 1).  We are unsure if this is a valid case; in general, most data should be acknowledged so that the transmitting side knows if stored data can be discarded, the next block of data can be sent, etc.  Note that this analysis applies whenever the device needs to send a D2R transmission in response to the R2D data, irrespective of which layer processes the response as an ack.

On balance, we see including R2D data in the paging message as a useful way of reducing congestion on the AIoT interface.  The latency benefits are secondary, since AIoT data normally should not be very time-sensitive.  It seems premature to agree to a mechanism before the transport block size is determined, however, so we propose only to study the possibility at this time.

Proposal 1: RAN2 studies the possibility of including R2D data (from upper layers and/or MAC control information) in the paging message, contingent upon the transport block size(s) being able to support it.

In keeping with the concept of being agnostic to the inventory and command cases, proposal 1 can apply whenever the reader has data to deliver—e.g., if the controller has requested a command or a combined inventory/command operation, but also if the reader needs to originate MAC communication with the device for some reason of its own (such as maintaining timing, updating or invalidating an identifier, etc.—whether such use cases exist will depend on further discussion).  We do consider that data-in-paging would be a natural behaviour for handling a combined inventory/command request from the controller, as shown in figure 2.

The figure can be lightly modified to handle inventory-only and command-only cases, without changing the flow of the paging procedure on the AIoT interface (except for the presence/absence of the command itself in step 2 and the responsive ack/data in step 5—but the messages in each step are the same for all cases).

Figure 2: Example of data-in-paging for combined inventory/command operation

(The handling of the acknowledgement in figure 2, particularly in step 6, depends on the design of communication between the controller and the reader, outside RAN2 scope.  If the controller needs to know the result of the MAC layer’s transmission of the command, the ack would need to be indicated in step 6.)

## 2.3	Response to paging

The current agreements somewhat assume that paging always requires a response.  This is clearly appropriate for the basic paging use cases: The reader needs to inventory one or more devices, retrieve data from a device, or establish communication for D2R and/or R2D data exchange.  However, there may be cases where a certain device should not respond to a paging message, e.g.:

- The paging message addresses a group of devices, and the reader repeats the message for the benefit of devices that did not receive the first transmission.  In this case, devices that did receive the first transmission and respond successfully generally should not also respond to subsequent transmissions (they could be inventoried twice, receive duplicate commands/data, and so on).

- The paging message retrieves data (read command) from one or more devices, the reader is interested only in responses from devices that have (new) data to transmit, and a particular device has no data; such a device should not clutter the D2R link with a vacuous response.

- The data included in the paging message do not require acknowledgement, as discussed above (this may or may not be a valid case).

We therefore suggest that there should be some facility for indicating which of the addressed device(s) should actually respond to a paging message.  Multiple ways of achieving this effect can be imagined (cause codes, explicit flags, a transaction identifier, etc.), but the principle of controlling the responses seems valid.

Proposal 2: RAN2 studies mechanisms for indicating whether and which of the addressed devices should respond to a paging message, with finer granularity than the device identifier in the paging record.

## 2.4	Message structure

It would be obviously premature to define a detailed paging message structure, but it seems clear that certain information is needed in the message:

- Message type/PDU type (something to distinguish paging from other transmissions), unless RAN1 provide differentiation in PHY (e.g., with a P-RNTI-like mechanism)

- One or more “paging record” structures containing:

  - Device identification (including possible group identifiers, all-devices reserved values, etc.)

  - Length of the record, if multiple paging records are possible (if there are multiple records in the message, devices not addressed by this record should be able to skip to the next one)

  - Control of response (any causes/flags/transaction values that would affect whether the device responds, as discussed above)

  - Response radio resources/configuration if needed

  - Optional data block (in accordance with proposal 1 above)

Whether multiple paging records in a single message makes sense depends significantly on the TB size, for which RAN2 need input from RAN1 (this issue is discussed further in [2]).  It seems possible to outline the general direction of study without committing to the number of paging records included in a message.

Proposal 3: The paging message contains a type identifier (e.g., PDU type; to be revisited if RAN1 provide differentiation for the paging message in the PHY layer) and one or more paging records.  The number of possible paging records depends on the transport block size and the size of the items in the paging record.

Proposal 4: A paging record contains at least a device identification, fields to control whether addressed device(s) will respond, and optional radio resources for the response(s).

Proposal 5: If multiple paging records can be carried in one paging message, the paging record includes a record length field.

# 3	Conclusion

This document promulgated the following proposals:

Proposal 1: RAN2 studies the possibility of including R2D data (from upper layers and/or MAC control information) in the paging message, contingent upon the transport block size(s) being able to support it.

Proposal 2: RAN2 studies mechanisms for indicating whether and which devices should respond to a paging message.

Proposal 3: The paging message contains a type identifier (e.g., PDU type; to be revisited if RAN1 provide differentiation for the paging message in the PHY layer) and one or more paging records.  The number of possible paging records depends on the transport block size and the size of the items in the paging record.

Proposal 4: A paging record contains at least a device identification, fields to control whether addressed device(s) will respond, and optional radio resources for the response(s).

Proposal 5: If multiple paging records can be carried in one paging message, the paging record includes a record length field.

# 4	References

[1] Chair’s notes, RAN2#126
[2]	R2-2406654: “Ambient IoT transport block size and MAC layer segmentation”, MediaTek Inc. / Samsung / vivo / ZTE Corporation / Sanechips / Qualcomm Incorporated, RAN2#127