3GPP TSG RAN WG2 #127			R2-2406659

Maastricht, Netherlands, August 19th – August 23rd, 2024

Source: 	Fujitsu

Agena item:	8.5.3

Title:	Discussion on on-demand SIB1 transmission for network energy savings

Document for:	Discussion and decision

Background

In RAN2#126 meeting, RAN2 made some agreements as follows [1].

Agreements on on-demand SIB1:

Triggering condition for UL-WUS

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

UE behavior in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell

RAR monitoring for UL-WUS

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline

RACH failure

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

On the other hand, there are some proposals that RAN2 couldn’t agree in the last meeting, so they will be discussed at the next meeting.

Case 3 scenario in RAN1

RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

UL-WUS configuration

After the UE camps on NES cell, UE can acquire WUS configuration from the NES cell.

SIB1 validity

SIB1 validity is supported, i.e. if a UE has a valid version of SIB1 stored, UE does not initiate the on-demand SIB1.

In this contribution, we would like to discuss and provide our views on the on-demand SIB1 operation from RAN2 perspective.

Discussion

Scenario clarification

Case 3 in RAN1

In RAN1#117 meeting, RAN1 had the following agreements on the case of on-demand SIB1[2]:

Agreement For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Case 1, Case 2 and Case 3 discussed in RAN1 are shown in the following table:

Table.1 - On-demand SIB1 cases in RAN1

| RAN1 cases | Scenarios | Descriptions |
| --- | --- | --- |
| Case 1 |  | UE obtains the UL WUS configuration from NES Cell. UE transmits UL WUS to NES Cell. UE receives on-demand SIB1 from NES Cell. |
| Case 2 |  | UE obtains the UL WUS configuration from Cell A. UE transmits UL WUS to NES Cell. UE receives on-demand SIB1 from NES Cell. |
| Case 3 |  | UE obtains the UL WUS configuration from Cell A. UE transmits UL WUS to Cell A. UE receives on-demand SIB1 from Cell A. |

In Case 3, on the network energy saving gain, the NES cell does not need to detect UL-WUS transmitted by the UEs or transmit on-demand SIB1, thus the network energy of the NES cell can be saved compared with Case 2. On the other hand, Cell A needs to receive UL-WUS from the UE and transmit the requested NES cell’s on-demand SIB1 to the UE, which consumes radio resource and power of Cell A. According to RAN1 simulation, the energy saving gain in Case 3 compared with Case 2 is about 2% which is slightly high.

On the specification impact in RAN2, if the on-demand SIB1 of the NES cell is transmitted in an on-demand SI message of Cell A, the specification impact is relatively low because the legacy on-demand SI request procedure in the current RRC specification with some small enhancements can be used for the on-demand SIB1 request procedure, e.g. by introducing a new SIB including the NES cell’s on-demand SIB1. The new SIB can be transmitted in the SI-window of the associated SI message in Cell A, which is also aligned with RAN1 agreement that a time window is used to transmit the NES cell’s on-demand SIB1.

From our perspective, Case 3 has low complexity and the specification impact is not so big.

Observation 1: Case 3 defined in RAN1 is feasible from RAN2 point of view.

Proposal 1: How to support Case 3 can be discussed later if Case 3 is supported in RAN1.

Triggering condition for UL-WUS

SI change case

In RAN2#126 meeting, there are following agreements:

SIB1 acquisition upon SIB change notification in NES cell 15. Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

The NES UE needs to reacquire on-demand SIB1 when the UE receives SIB change notification from NES cell. However, it is not clear how the UE can obtain the SIB1 from the NES cell after the reception of SIB change notification.

In the current SI change procedure, a modification period is used, i.e. updated SI message is broadcasted in the next modification period following the one where SI change indication is transmitted, as show in the following figure.

Figure.1 - SI modification period

One option is that the UE should transmit the UL-WUS to request the updated on-demand SIB1 and then acquire the on-demand SIB1 in the time window. In this option, a new trigger for UL-WUS transmission may be needed. The new trigger can be e.g., the SI change indication is received. In order to request the on-demand SIB1 from the NES cell, the UE can use the stored UL-WUS configuration transmitted from Cell A. Meanwhile, if the UE cannot have a valid UL-WUS configuration corresponding to the NES cell, the UE simply considers the NES cell is not a candidate cell for camping and would reselect another cell.

Another option is that the NES cell should transmit the updated on-demand SIB1 proactively from the next modification period and the UE just tries to acquire the updated on-demand SIB1 in the next modification period following the legacy behavior. Therefore, in either option, the NES cell does not need to transmit WUS configuration to the UE in case of SIB1 change.

Observation 2: When the UE camps on the NES cell, it is possible to acquire SIB1 from the NES cell without WUS configuration transmitted from NES cell.

Proposal 2: For SIB1 change, NES cell does not need to provide UL-WUS configuration for on-demand SIB1.

However, RAN1 has not discussed how the on-demand SIB1 is transmitted, such as the on-demand SIB1 transmission occasions and how long the on-demand SIB1 should be transmitted. These details should be further discussed based on the RAN1 progress.

Proposal 3: RAN2 to discuss how the UE acquires the updated on-demand SIB1 in the NES cell. The following options can be considered and details will be discussed in a normative phase.

- Option 1: The UE transmits the UL WUS using the stored WUS configuration to request the updated on-demand SIB1 and then acquire the on-demand SIB1.

- Option 2: The NES cell transmits the updated on-demand SIB1 proactively in the next modification period.

Other cell selection cases

It was agreed to use legacy intra-F/inter-F cell re-selection procedure as baseline for UL-WUS triggering condition in the last meeting. On the other hand, RAN2 has not mentioned Initial cell selection to NES cell. In the case of supporting the initial selection, the UE shall receive UL-WUS configuration from NES cell. In order to allow the UE to Initial cell selection to NES cell, NES cell need to periodically broadcast UL-WUS configuration, which would reduce the NES gain. Besides that, for a case of recovery from out of coverage (to the NES cell), UL-WUS configuration in Cell A may be changed during the UE is out of coverage, but the UE can't know that therefore it is natural not to support this procedure.

Proposal 4: RAN2 to not support Initial cell selection and recovery from out of coverage for the on-demand SIB1 procedure.

UL-WUS transmission

Msg3 based SIB1 request

For on-demand SIB1 acquisition, it was agreed to reuse Msg1 based SI request in the previous meeting, whereas Msg3 based SI request was kept as FFS. In the current SI request procedure, if dedicated RA resources are not configured for SI request, Msg3 based procedure is triggered to request the required SI message(s). Similar to the legacy procedure, Msg3 based on-demand SIB1 request is beneficial for the cases where the RA resources are not reserved due to preamble shortage especially in Case 3 in RAN1. Meanwhile, FFS on whether to just enhance Msg3 based SI request.

Proposal 5: Msg3 based SIB1 request is triggered if dedicated RA resources for SIB1 request is not configured. Details will be discussed in a normative phase.

Random Access Response acquisition

RAN2 agreed that RAN2 assumes the UE is expected to receive the RAR for responding to the preamble transmission for Msg1-based on-demand SIB1 procedure as the baseline. According to this agreement, the UE will not be able to acquire SIB1 unless the UE successfully receives a RAR corresponding to the transmitted preamble. If the UE couldn’t receive RAR during RA procedure and the number of UL-WUS retransmission is larger than preambleTransMax, this UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. The NES cell has to transmit RAR for each preamble transmitted from the UEs, so that the UEs will be able to acquire SIB1. It may increase the number of RARs and the number of UL-WUS retransmissions. To reduce the signalling during SIB1 request procedure, the UE should check whether SIB1 is already transmitted from NES cell as the triggering condition and stopping condition for UL-WUS procedure.

Proposal 6: The UE should check whether SIB1 is transmitted from NES cell as the triggering condition and stopping condition for UL-WUS procedure.

On-demand SIB1

On-demand SIB1 validity

In RAN2#126 meeting, it is discussed whether SIB1 validity is supported but no conclusion is made. In our understanding, SIB1 validity is useful, e.g., in case that the UE has a valid version of the on-demand SIB1 of an NES cell and reselects the NES cell again, or in case that the UE is camping on an NES cell and it has acquired the on-demand SIB1 of the NES cell some time before. The validity of on-demand SIB1 can reduce the UE efforts on UL-WUS transmission and SIB1 acquisition, and thus it may save radio resources and power of UE and NW.

Observation 3: Introducing on-demand SIB1 validity can reduce the number of UL-WUS triggering and on-demand SIB1 transmission.

In the current TS38.331 section 5.2.2.2, there are following descriptions on SIB validity[3].

#### 5.2.2.2 SIB validity and need to (re)-acquire SIB ##### 5.2.2.2.1 SIB validity The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request (e.g., a positioning request) from upper layers; and whenever the UE does not have a valid version of a stored SIB or posSIB or a valid version of a requested SIB. When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList for non-NPN-only cells or the first NPN identity (SNPN identity in case of SNPN, or PNI-NPN identity in case of PNI-NPN) in the NPN-IdentityInfoList for NPN-only cells, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. If the UE stores the acquired posSIB, then the UE shall store the associated areaScope, if present, the cellIdentity, the systemInformationAreaID, if present, the valueTag, if provided in assistanceDataSIB-Element, and the expirationTime if provided in assistanceDataSIB-Element. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication. The valueTag and expirationTime for posSIB is optionally provided in assistanceDataSIB-Element, as specified in TS 37.355 [49]. (omitted) NOTE: The storage and management of the stored SIBs in addition to the SIBs valid for the current serving cell is left to UE implementation. The UE shall: 1> delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

As highlighted in yellow above, the UE shall delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid. This means that the UE shall delete the stored version of on-demand SIB1 of the NES cell after 3 hours. In the online discussions last meeting, some companies have concern on the 3-hour validity of on-demand SIB1 and think that the network does not know when 3 hours will expire for each UE. From our perspective, when the 3-hour in a UE expires, as the UE has no valid version of the on-demand SIB1, the UE shall acquire on-demand SIB1 by transmitting UL-WUS and then the network can transmit the on-demand SIB1 of the NES cell.

Proposal 7: RAN2 to introduce a validity for on-demand SIB1.

From the descriptions above highlighted in blue, the UE needs to acquire SIB1 upon cell reselection. Because currently there is no value tag for SIB1, the UE cannot check the validity from the on-demand SIB1. Therefore, it always requires initiating on-demand SIB1 procedure. However, from our perspective, introducing a validity for on-demand SIB1 is useful to reduce UL-WUS triggering. It can be ensured, for example, by an indication in UL-WUS configuration via Cell A. On the other hand, the contents of UL-WUS configuration should be discussed in a normative phase. Thus, RAN2 should discuss whether/how to check the validity of on-demand SIB1 in a normative phase.

Proposal 8: RAN2 to discuss whether/how the UE can check a validity of the stored version of on-demand SIB1 during cell re-selection to the NES cell.

RAN2 summary on SI phase

Although there are some open issues for on-demand SIB1 operation, but it can be resolved in a normative phase. From our perspective, RAN2 impacts are reasonable, and RAN2 can conclude that on-demand SIB1 operation is feasible from RAN2 point of view.

Proposal 9: RAN2 to consider the on-demand SIB1 operation is feasible from RAN2 perspective.

Conclusion

In this contribution, the following proposals and observations were made:

Observation 1: Case 3 defined in RAN1 is feasible from RAN2 point of view.

Proposal 1: How to support Case 3 can be discussed later if Case 3 is supported in RAN1.

Observation 2: When the UE camps on the NES cell, it is possible to acquire SIB1 from the NES cell without WUS configuration transmitted from NES cell.

Proposal 2: For SIB1 change, NES cell does not need to provide UL-WUS configuration for on-demand SIB1.

Proposal 3: RAN2 to discuss how the UE acquires the updated on-demand SIB1 in the NES cell. The following options can be considered and details will be discussed in a normative phase.

- Option 1: The UE transmits the UL WUS using the stored WUS configuration to request the updated on-demand SIB1 and then acquire the on-demand SIB1.

- Option 2: The NES cell transmits the updated on-demand SIB1 proactively in the next modification period.

Proposal 4: RAN2 to not support Initial cell selection and recovery from out of coverage for the on-demand SIB1 procedure.

Proposal 5: Msg3 based SIB1 request is triggered if dedicated RA resources for SIB1 request is not configured. Details will be discussed in a normative phase.

Proposal 6: The UE should check whether SIB1 is transmitted from NES cell as the triggering condition and stopping condition for UL-WUS procedure.

Observation 3: Introducing on-demand SIB1 validity can reduce the number of UL-WUS triggering and on-demand SIB1 transmission.

Proposal 7: RAN2 to introduce a validity for on-demand SIB1.

Proposal 8: RAN2 to discuss whether/how the UE can check a validity of the stored version of on-demand SIB1 during cell re-selection to the NES cell.

Proposal 9: RAN2 to consider the on-demand SIB1 operation is feasible from RAN2 perspective.

References

- R2-2405701, Report from session on V2X/SL, R19 NES and MOB

- RAN1#117 Chair notes

- 3GPP TS38.331 V18.2.0