3GPP TSG-RAN WG2 Meeting #127														     R2-2406663

Maastricht, NL,  August 19 – 23, 2024

Agenda item:	8.3.4.1

Source:	Apple

Title:	On RLF prediction

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our views on various pending RLF prediction issues and open questions.

# 2   	Discussion

### 2.1 RLF prediction window

In the last RAN2#126 meeting there was a somewhat confusing discussion about the RLF prediction window, which unfortunately resulted in the agreement (provided below) that needs to be revisited, clarified and fixed.

RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case. FFS on expected RLF time and indirect case.

First off, the language of the agreement uses the terms “time window”, “time instance” and “RLF time”. These presumably refer to the same thing, but the usage of three different definitions is rather confusing. It seems obvious that predicting RLF at an exact point in time (e.g. “476ms from now”) may be technically possible, but is not very useful. After all, it doesn’t matter whether the RLF is expected to occur exactly “476ms” into the future or “477ms” – the corrective action would be the same in both cases, not to mention that attempting to predict an RLF at a precise point in time with e.g. ms resolution would give very low accuracy. Therefore, it seems obvious (but apparently worth clarifying) that we are interested in a “time window” rather than a “point in time”.

Observation 1: Predicting RLF occurrence in a time window is much more useful than predicting RLF occurrence at a point in time.

This is illustrated in the following simple diagram. The time period “RLF window” indicates that RLF is expected to occur with a certain probability in that time window.

And so, quite naturally, our proposal is to use the term “time window”, identified by “start time” and “end time”. The only question in this regard is whether the start time should be anything rather than the time of the prediction. In our view, at this point of the study, a simpler approach of predicting RLF in a time window starting from the time of the prediction is sufficient, as illustrated by the diagram below. The main reason is that running simulations and machine learning training not only with various end times, but also with various start times, would increase the evolution effort without any benefits (as far as the study is concerned).

Observation 2: running simulations and machine learning training not only with various end times, but also with various start times, would increase the evolution effort without any benefit for the study.

Note: this simpler RLF prediction window is proposed for the purpose of the study. It does not mean that in the normative work we should not consider the definition with start and end times, as it is actually more useful in deployments.

Proposal 1: adopt the following (revised) RLF prediction definition: “RLF prediction result is the RLF probability within a time window starting from the time of the prediction till an agreed/reported time in the future.”

With regards to the end of the RLF window, we can take a similar approach as we have done for measurement prediction window – agree on a set of values (see below) and revise them when initial results are available.

The next, equally confusing, part of the discussion we had at RAN2#126 is related to a potential difference of the RLF prediction window definition for directed and indirect RLF prediction. First, we hope it is agreeable that the desire should be to have a common definition for both direct and indirect, as indeed what kind of machine learning prediction approach to take should be left to each company’s discretion and the reporting should use the same definition and the same format.

Observation 3: it is desirable have the same RLF prediction window definition for direct and indirect RLF prediction.

Direct vs. indirect approaches have been defined as follows:

Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR). Study Direct: Directly RLF prediction by AI/ML models.

With direct approach, the usage of the RLF window term is rather obvious:

- Model inputs: SINR (and or other measurements and inputs)

- Model training label/output: whether RLF occurs within time window XX ms, with confidence level

The indirect approach is indeed more complicated, but it can be made to work as follows:

- Model inputs: SINR (and or other measurements and inputs)

- Model training label/output: series of SINR measurements within time window XX ms, with confidence level for each measurement

The algorithm would then need to inspect all the predicted SINR measurements in the prediction window, find in-sync and out-of-sync occurrences, detect whether RLF is expected to occur and estimate the confidence level based on the estimated confidence level of the SINR measurement predications which trigger in-sync and out-of-sync occurrences. This is of course somewhat complicated (which is why we prefer the direct approach), but possible if companies insist on using the indirect approach. Therefore, the same definition can be used for direct and indirect.

Observation 4: RLF prediction window definition can also be made to work with indirect RLF prediction.

We would like to further note that the complexity for the indirect approach is not because of a particular RLF prediction window definition but rather inherent in the approach itself. The direct RLF prediction approach is simpler, regardless of the RLF prediction window definition we may adopt.

So we propose the following revised definition: “RLF prediction result is the RLF probability within a time window starting from the time of the prediction till an agreed/reported time in the future.

Proposal 2: remove FFS on “direct vs. indirect” in RLF prediction result agreement.

### 2.1 Metrics/KPIs

RAN2 have essentially agreed already on the most important metric: RLF probability. Furthermore, based on the discussions, it is also clear that the prediction window is also needed. It would perhaps be beneficial to formalize these agreements as follows:

Proposal 3: use “RLF prediction accuracy” as the metric for the RLF prediction study.

The next question is how to go about the prediction window. Our proposal would be to agree on a set of reasonable window lengths, reasonable in the sense they are long enough to be useful but still within the limits of what appears to be feasible for ML algorithms. Naturally, this initial set of values can be revisited once companies submit the initial set of results.

Proposal 4: consider the following prediction window values: 400ms, 800ms, 2s, 3s, 4s, 5s.

We don’t believe additional (beyond RLF prediction accuracy and prediction window) metrics are needed, but we are aware there are proposals to also consider various system level metrics. In TR 36.839, the following RLF HO performance metrics were discussed:

- Definition 2: The RLF performance metric is defined as: the average number of RLF occurrences per UE per second. RLF performance in states 1 and 2 are logged separately.

- Definition 7: The total number of handover failures per UE per second is defined as the total number of handover failures averaged over the total travel time of all the simulated UEs.

- Definition 8: The total number of successful handovers per UE per second is defined a s the total number of successful handovers averaged over the total travel time of all the simulated UEs.

These definitions were important in the context of the Release-11 HetNet mobility study, but are of no particular importance for the current study which focuses on AI/ML prediction. To reiterate, the focus of the current study is to evaluate whether and with what accuracy ML can predict RLF, and how that prediction can be used – while not changing the basic mobility principles.

There are of course other KPIs which are relevant to the performance of AI/ML prediction algorithms which can be considered, specifically:

- Ping-pong HO prediction

- Too early HO prediction

- Too late HO prediction

- HO to wrong cell prediction

- Time of stay prediction

At this stage we are not sure these additional metrics are useful. In our view, it would be quite sufficient to establish whether AI/ML can predict basic RLF. Furthermore, we will need to study different AI/ML algorithms and architectures (which are many) to determine what works best. And finally, we will need to study how the prediction results can be used Therefore, our proposal is to only consider any additional KPIs after the basic efficiency of AI/ML prediction for RLF has been established, provided there is sufficient justification to consider system-level RLF-related KPIs.

Proposal 5: consider additional KPIs (beyond RLF prediction accuracy and prediction window) once it has been established that AI/ML is capable of predicting RLF with sufficiently high accuracy and sufficiently long prediction window.

Having said that, companies of course should be allowed to report additional metrics and information of relevance, such as whether they used model inputs other than SNR, direct vs. indirect, etc.

### 2.2 Data set and simulation assumptions

As previously agreed, we shall use both simulated/synthesized data sets and field data. We’ve seen companies reporting difficulties in producing sufficient number of RLFs in simulations. This just reinforces the point that for this study to be useful, real/field data must be used.

Observation 5: for RLF, using real/field data is even more important than for RRM measurement prediction.

We’ve also seen proposals to artificially tweak the simulation assumptions to increase the likelihood of RLFs. This is problematic for two reasons:

- First, there are obvious benefits in having as much commonality between the simulation assumptions for measurement prediction and RLF prediction

- Second, we wonder whether simulation results obtained in an environment specifically tweaked to produce certain events are even meaningful for the ML study of predicting these events

Therefore, we should reduce the difference between RRM prediction and RLF prediction simulations assumptions to a minimum. In fact, RAN2 have already agreed on this (“The assumption is that we will reuse RRM simulation assumptions (where possible)”), so it is just a reminder.

Observation 6: RLF prediction simulation assumptions should be as close as possible to the RRM simulation assumptions.

Naturally, one important difference between the RRM and the RLF simulations is that we need SNR measurements instead of RSRP. Other than that, any additional changes should be carefully weighted and generally avoided. We are aware of some proposals to change the assumptions for RLF, therefore below we provide some considerations regarding those proposals.

With regards to boundary processing options, there were doubts that options 2 (bouncing circle) and option 3 (terminated) are suitable for producing SNR. We don’t believe this to be the case, as in both options the simulation may include additional cells (beyond the bouncing circle and beyond the cell boundary for terminating UE mobility) to generate interference and make SNR measurements meaningful. Therefore, as in RRM simulation, we can keep all three boundary processing options and leave the choice for companies’ discretion. If needed, a clarification along the lines of what we provided above can be added.

Observation 7: all 3 boundary processing options can be used for RLF prediction (with some minor adjustments to enable SNR measurements) and the choice should be left for companies’ discretion.

Another assumption parameter which we think we should revisit is the UE speed. For RRM measurement prediction, we’ve agreed to use the following: 30,60,90,120 Km/h. We believe (and this is true for RRM prediction as well, actually) that it is important to consider pedestrian speeds. The reason is that the majority of users are actually pedestrians. The rationale for excluding pedestrian speeds (which was never actually articulated) is perhaps the thinking that if we can achieve sufficiently good accuracy with e.g. 30 km/h speeds than for sure the algorithm will work for pedestrian speeds. While this is true, the prediction window will be different. While longer prediction window for RRM measurements (beyond a reasonable limit of e.g. few hundreds of ms) doesn’t necessarily improve performance (in whatever metric we want to optimize), that is not the case with RLF prediction. The reason is that RLF mitigation may take time, so the solution for RLF prediction within e.g. 200ms or 1s may be very different.

Proposal 6: consider also pedestrian speed, e.g. 5km/h, for RLF prediction.

### 2.3 RRC Parameters

RAN2 have agreed on the following RLF definition for the purpose of the study:

The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

In TS 38.331, RLF is declared upon T310 expiry. T310 is started up receiving N310 consecutive "out-of-sync" indications from the lower layers and stopped upon receiving N311 consecutive "in-sync" indications.  Furthermore, in TS 38.133 out-of-sync and in-sync are defined either in terms of SNR as follows:

The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate (BLERout). The threshold Qin is defined as the level at which the downlink radio link quality can be received with significantly higher reliability than at Qout and shall correspond to the in-sync block error rate (BLERin).

Therefore, RAN2 needs to agree on T310, N310, N311, Qout , and Qin.

Observation 8: RAN2 needs to agree on T310, N310, N311, Qout , and Qin.

Here TS 36.839 is a useful baseline and while we can of course debate whether and how to adjust these parameters to the current study, by the end of the day the exact values don’t matter much – what matters is to align on common values. Therefore, our proposal is to adopt the following values:

| Items | Description |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| T310 | 1s |
| N310 | 1 |
| N311 | 1 |

Proposal 7: to adopt the following values for the purpose of RLF evaluation: Qout: -8 dB, Qin: -6 dB, T310: 1s, N310: 1, N311: 1.

# 3	Conclusions and Proposals

Observation 1: Predicting RLF occurrence in a time window is much more useful than predicting RLF occurrence at a point in time.

Observation 2: running simulations and machine learning training not only with various end times, but also with various start times, would increase the evolution effort without any benefit for the study.

Observation 3: it is desirable have the same RLF prediction window definition for direct and indirect RLF prediction.

Observation 4: RLF prediction window definition can also be made to work with indirect RLF prediction.

Observation 5: for RLF, using real/field data is even more important than for RRM measurement prediction.

Observation 6: RLF prediction simulation assumptions should be as close as possible to the RRM simulation assumptions.

Observation 7: all 3 boundary processing options can be used for RLF prediction (with some minor adjustments to enable SNR measurements) and the choice should be left for companies’ discretion.

Observation 8: RAN2 needs to agree on T310, N310, N311, Qout , and Qin.

Proposal 1: adopt the following (revised) RLF prediction definition: “RLF prediction result is the RLF probability within a time window starting from the time of the prediction till an agreed/reported time in the future.”

Proposal 2: remove FFS on “direct vs. indirect” in RLF prediction result agreement.

Proposal 3: use “RLF prediction accuracy” as the metric for the RLF prediction study.

Proposal 4: consider the following prediction window values: 400ms, 800ms, 2s, 3s, 4s, 5s.

Proposal 5: consider additional KPIs (beyond RLF prediction accuracy and prediction window) once it has been established that AI/ML is capable of predicting RLF with sufficiently high accuracy and sufficiently long prediction window.

Proposal 6: consider also pedestrian speed, e.g. 5km/h, for RLF prediction.

Proposal 7: to adopt the following values for the purpose of RLF evaluation: Qout: -8 dB, Qin: -6 dB, T310: 1s, N310: 1, N311: 1.

# 4	References

[1] 		RAN#126 meeting minutes

[2]		TS 36.839 Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks

[3]		TS 38.331 NR; Radio Resource Control (RRC); Protocol specification

[4]		TS 38.133 NR; Requirements for support of radio resource management