3GPP TSG-RAN WG2 Meeting #127														     R2-2406664

Maastricht, NL, August 19 – 23, 2024

Agenda item:	8.3.2.1

Source:	Apple

Title:	RRM measurement prediction results for case B

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide initial RRM measurement prediction results for case B, as per the agreements in the previous meeting. Even though a substantial number of cases is covered in this submission, our main intention is not so much to provide the results for all the cases, but rather help focus the study.

With this in mind, this contribution provides prediction results for a number of selected cases which should help us scope the study, for example by answering the questions such as: what are the reasonable measurement reduction rates we can hope to achieve with reasonably high accuracy, what models (or at least model types/categories) we should focus on, etc.

# 2   	Discussion

### 2.1 Methodology

Besides providing the measurement prediction results for inclusion in the TR (which can be found in Appendix below), the intention of this paper is to attempt to draw preliminary conclusions to focus the study.

The results provided in the contribution, even though limited to a single scenario of ‘FR1-to-FR1 intra-frequency temporal domain case B (intra-cell)’ contain numerous predictions with a large number of varying parameters (models, measurement reduction rates, etc).

In order to be able to draw some (admittedly preliminary) conclusions, in the next sections we used the following methodology: in order to study a certain parameter (e.g. a model) we fixed all the other parameters (e.g. speed, measurement reduction rates) to show the results isolating the effect of that single parameter.

Using this methodology the following issues were evaluated:

- Which models provide better performance

- What is an acceptable MRR (measurement reduction rate) besides 50% (which was tentatively agreed)

### 2.2 Model Evaluation

In this clause we describe the models evaluated in this contribution, specifically:

- Exponential Moving Average (EMA) with alpha=0.9

- Autoregressive Integrated Moving Average (AR), ARIMA(n,0,0) and ARIMA(n,1,0)

- XGBoost: eXtreme Gradient Boosting with number of estimators t=10, max depth d=7

- Feedforward NN (multi-layer perceptron) with n inputs (n being the number of measurements in the observation window)

- LSTM (GRU variant): 	Long Short-Term Memory simplified as a Gated Recurrent Unit model (GRU)  (with n inputs, n being the number of measurements in the observation window)

Additionally, for some models we considered two variants in terms of model output:

- RSRP, and

- RSRP difference compared to the RSRP at the previous measurement instance (measured or predicted), which is then converted to RSRP.

The latter variants are shown with “Diff” prefix in the Figure 1 below and referred to as “differenced models” in the text.

Note that the first 2 models are not technically ML models, however as we show below they provide good performance at much smaller cost (in terms for memory and complexity) compared to “true ML” models such as Feedforward NN and LSTM and therefore deserve serious consideration.

The goal of this exercise was to evaluate the relative performance of these models. To do so, we “fixed” some of the parameters (for the purpose of this particular evaluation) as follows:

- Speed: 30 km/h

- Measurement Reduction Rate: 50%

The results are shown in Figure 1 below, where the x axis (titled “number of lags”) shows different observation windows. One “lag” equals 200ms in the figure, e.g. lag=2 means 400ms observation window (2 observations).

Figure 1: Prediction models comparison (in terms of accuracy)

One must be cautions when drawing conclusions based on this somewhat limited data (covering a single scenario with fixed parameters such as speed and measurement reduction rate), however one conclusion is evident: simple models, including simple ML models such as decision trees and even simpler non-ML models such as ARIMA do perform well, compared to much more complex “deep learning” models such as LSTM.

So far we’ve compared the models in terms of their performances (i.e. prediction accuracy). The next equally important comparison metric is complexity, which is shown in Table 1 below:

| Model | Number of parameters | Model size | FLOPs |
| --- | --- | --- | --- |
| EMA | 1 | 8 bytes | 3 |
| ARIMA(n,0,0) | n + 2 n=4: 6 | (n + 2) * 8 Bytes n=4: 48 Bytes | 2n+1 |
| ARIMA(n,1,0) | N + 3 n=4: 7 | (n + 3) * 8 Bytes N=4: 56 Bytes | 2*n+3 |
| XGBoost | 63600 | 0.25 MB | 800 |
| Feedforward NN | 64*n+65 for n=4: 321 | (64*n + 65)*8 For n=4: 2.5 KB | ~128n+192 |
| LSTM (GRU) | 384*n + 298112 For n=6: 3 MB | 384*n + 298112 For n=4: 3 MB | ~1.5k*n+200k |

Table 1: Prediction model comparison (in terms of complexity)

Notice that we used straightforward complexity numbers for ML models without assuming any particular optimization that can happen during implementation. The idea is to have a rough estimate of the complexity that can be used to compare between models

Note that for some models, specifically Feedforward ARIMA, NN and LSTM, complexity depends on the observation window. In this contribution we evaluated observation windows up to 800 ms (lag=4) where we still observed performance improvement, so we used n=4 in complexity calculation for these models.

Observation 1: observation window of 800 ms provides good results; larger window sizes do not necessarily increase accuracy.

Naturally, non-ML models such as EMA and ARIMA are by far the simplest in terms of both models size and computational complexity. Decision tree models (e.g. XGBoost) and feedforward NN models are somewhat more demanding, and complex models such as LSTM as the most demanding.

The most important conclusion is that simple ML models (such as decision trees) and even simpler non-ML models (such as ARIMA) provide good performance for the goal of measurement reduction rate. While it is premature to jump to the conclusions that any particular model (such as Diff-AR in this particular case) outperforms all others (such conclusion would require more results for other scenarios and speeds), the trend is rather clear – simple models, including non-ML models, can provide adequate performance for RRM measurement prediction.

Observation 2: simple models, including non-ML models, provide good prediction accuracy at low complexity.

We therefore believe that simple models, including non-ML models, should be an important part of this study.

### 2.3 MRR Evaluation

One of the important goals of this study is to establish what measurement reduction rates (MRR) are achievable, while maintaining good accuracy. In the previous meeting, 50% MRR was tentatively agreed with the understanding different values need to be studied and reported and further studied.

The goal of this particular exercise was to evaluate different measurement reduction rates. To do so, we “fixed” some of the parameters (for the purpose of this particular evaluation) as follows:

- Speed: 30 km/h

- Observation window: 5 lags (1000 ms)

Figure 2: Measurement reduction rates (different models)

Notes:

- MRR = 2 is 50% measurement reduction rate

- MRR = 3 is 67% measurement reduction rate

- MRR = 4 is 75% measurement reduction rate

- MRR = 5 is 80% measurement reduction rate

Naturally, performance decreases with higher reduction rates and the rate of decrease seem to be similar to all the models evaluated. It is difficult to draw conclusions of what MMRs provide acceptable performance as it is not clear what is an acceptable error (MAE). This is discussed in further detail in section 2.5.

Observation 3: Performance (prediction accuracy) decreases with higher reduction rates (rate of decrease similar to all the models evaluated).

Another approach to study MRR is to consider a single model and to evaluate MRRs for different observation windows (lags). To do so, we “fixed” some of the parameters (for the purpose of this particular evaluation) as follows:

- Speed: 30 km/h

- Model: ARIMA (n,0,0), n is the number of observation lags

The results are shown in Figure 3 below.

Figure 3: Measurement reduction rates (different observation windows) for ARIMA(n,0,0)

Once again, it is hard to see at this stage what’s the optimal observation window, as we are yet to establish what’s an acceptable error (MAE). However, some preliminary trends can be observed:

- For higher measurement reduction rates, the effect of higher lags (longer observation window) is more significant.

- For low measurement reduction rates (e.g, 50%), the effect is less important.

However, it is observed that when Diff-AR is used, longer observations may not always be better for different MRR, as the variation in MAE is quite small across different observation windows.

- Speed: 30 km/h

- Model: ARIMA (n,1,0)

Figure 4: Figure 3: Measurement reduction rates (different observation windows) for ARIMA(n,1,0)

Observing these results, it is hard to make conclusions about the optimum size of the prediction window. It is likely that the window size depends on a particular model used.

Observation 4: the optimum observation window size likely depends on a model used.

### 2.4 Preliminary conclusions and proposals to facilitate further study

In this section we provide some preliminary conclusions based on the results obtained so far and make some proposals intended to further facilitate the study.

While this study is for AI/ML, one important observation which is already clear based on the results at hand is that at least for some tasks/scenarios (e.g. the scenario of measurement prediction presented in this paper), simple models, including non-ML models such as ARIMA and XGBoost, provide good results at low complexity. For example, as we’ve shown above, ARIMA(n,1,0) outperforms Feedforward NN and LSTM (at least in the scenarios considered so far).

While there are no explicit agreements limiting the models companies can use, considering the results we present here we think it would be good to encourage companies to provide more results with simple models. Therefore, we propose to “formally” include simple models, including non-ML models, in the study.

Proposal 1: the study should also include simple models, including non-ML models.

So far, we’ve agreed to use prediction accuracy (MAE) as the performance metric. This is a good metric which is easy to calculate and to compare results from different companies and for different scenarios. The question though is, what should be considered a good/acceptable accuracy?

One option, which was suggested before, is to consider “system level KPIs” such as handover failure rate. To clarify, we still believe that MAE is a good metric which we should continue using. The purpose of using system level KPIs would be to establish an acceptable MAE.

Proposal 2: to discuss what is considered an acceptable error (MAE) and to consider using (a limited number of) system level KPIs such as handover failure rate.

The results discussed in this contribution are provided in the submission template in the Appendix. We propose to include them in the TR.

Proposal 3: include in the TR prediction results provided in the Appendix.

### 2.5 Simulation Assumptions

In this contribution we of course followed all the simulation assumptions previously agreed in RAN2. Some of them have been left for company choice and in this clause, we elaborate on these.

For UE trajectory modelling, RAN2 agreed to allow 3 options:

- Option 1: Linear trajectory model with random direction change

- Option 2: Linear trajectory model with random and smooth direction change

- Option 3: Random direction straight-line trajectories

We’ve decided to use option 1, the rationale being not only simulation simplicity but also the fact that it is indeed somewhat more challenging for prediction, as opposed to option 2 with smooth direction change. Overall, option 1 appears to be a good trade-off for the study and therefore it was used in our simulations.

For UE trajectory boundary processing, RAN2 agreed to allow 3 options:

- Option 1: wrap-around

- Option 2: bouncing circle

- Option 3: terminated

We’ve decided to use option 3.

Other assumptions are shown in Table 2 below:

| Parameter | FR1 |
| --- | --- |
| Number of Tx beams | 1 |
| Number of Rx beams | 1 |
| LOS Soft | No |
| Inter-frequency correlation assumption in general | N/A |
| Inter-frequency shadow fading correction | N/A |
| Spatial Consistency | A |

Table 2: simulation assumptions

Since in this paper we focus on the temporal domain intra-frequency scenarios, the inter-frequency correlation is not applicable.

# 3	Observations and Proposals

Observation 1: observation window of 800 ms provides good results; larger window sizes do not necessarily increase accuracy.

Observation 2: simple models, including non-ML models, provide good prediction accuracy at low complexity.

Observation 3: Performance (prediction accuracy) decreases with higher reduction rates (rate of decrease similar to all the models evaluated).

Observation 4: the optimum observation window size likely depends on a model used.

Proposal 1: the study should also include simple models, including non-ML models.

Proposal 2: to discuss what is considered an acceptable error (MAE) and to consider using (a limited number of) system level KPIs such as handover failure rate.

Proposal 3: include in the TR prediction results provided in the Appendix.

# 4	References

[1] 		RAN#126 meeting minutes

# Appendix – Prediction Results (for inclusion in the TR)

The results below are provided for FR1-to-FR1 intra-frequency temporal domain case B (intra-cell).

| Report parameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | 1 | 1 | 1 | 1 | 1 | 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | 3 | 3 | 3 | 3 | 3 | 3 |
|  | UE speed (30,60,90,120 Km/h) | 30 | 30 | 30 | 30 | 30 | 30 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | n/a | n/a | n/a | n/a | n/a | n/a |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | n/a | n/a | n/a | n/a | n/a | n/a |
|  | Whether LOSsoft is modeled or not | no | no | no | no | no | no |
|  | spatial consistency option (A or B) | A | A | A | A | A | A |
|  | Number of TX beams | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Number of RX beams | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Measurement reduction rate(50%,…Note2) | 50% | 50% | 50% | 50% | 50% | 50% |
|  | Observation window(Note3) | 800ms | 800ms | 800ms | 800ms | 800ms | 800ms |
|  | Prediction window (Note3) |  |  |  |  |  |  |
|  | Any other parameters (Note 4) |  |  |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 560K | 560K | 560K | 560K | 560K | 560K |
|  | Testing | 140K | 140K | 140K | 140K | 140K | 140K |
| AI/ML model input/output | Model input (Note 5) | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP |
|  | Model output(Note 6) | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | EMA 0.9 | ARIMA (3,1,0) | XGboost | Diff-XGBoost | Diff-FNN (n=3) | LSTM(GRU) (n=3) |
|  | Model complexity in a number of parameters(M) | 1 | 7 | 63KB | 63KB | 257 | 3M |
|  | Model complexity in model size (e.g. Mbyte) | 8 bytes | 56 Bytes | 0.25M | 0.25M | 2.1 KB | 2.4 MB |
|  | Computational complexity [FLOPs] | 3 | 9 | 800 | 800 | 576 | 200K |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.534 | 0.486 | 0.560 | 0.505 | 0.515 | 0.517 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) |  |  |  |  |  |  |

| Report parameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | 1 | 1 | 1 | 1 | 1 | 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | 3 | 3 | 3 | 3 | 3 | 3 |
|  | UE speed (30,60,90,120 Km/h) | 30 | 30 | 30 | 30 | 30 | 30 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | n/a | n/a | n/a | n/a | n/a | n/a |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | n/a | n/a | n/a | n/a | n/a | n/a |
|  | Whether LOSsoft is modeled or not | no | no | no | no | no | no |
|  | spatial consistency option (A or B) | A | A | A | A | A | A |
|  | Number of TX beams | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Number of RX beams | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Measurement reduction rate(50%,…Note2) | 50% | 67% | 50% | 67% | 50% | 67% |
|  | Observation window(Note3) | 800ms | 800ms | 800ms | 800ms | 800ms | 800ms |
|  | Prediction window (Note3) |  |  |  |  |  |  |
|  | Any other parameters (Note 4) |  |  |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 560K | 560K | 560K | 560K | 560K | 560K |
|  | Testing | 140K | 140K | 140K | 140K | 140K | 140K |
| AI/ML model input/output | Model input (Note 5) | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP |
|  | Model output(Note 6) | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ARIMA (3,1,0) | ARIMA (3,1,0) | XGboost | Diff-XGBoost | Diff-FNN (n=3) | LSTM(GRU) (n=3) |
|  | Model complexity in a number of parameters(M) | 7 | 7 | 63KB | 63KB | 257 | 3M |
|  | Model complexity in model size (e.g. Mbyte) | 56 bytes | 56 Bytes | 0.25M | 0.25M | 2.1 KB | 2.4 MB |
|  | Computational complexity [FLOPs] | 9 | 9 | 800 | 800 | 576 | 200K |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.492 | 0.677 | 0.524 | 0.746 | 0.524 | 0.764 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) |  |  |  |  |  |  |

| Report parameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | 1 | 1 | 1 | 1 | 1 | 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | 3 | 3 | 3 | 3 | 3 | 3 |
|  | UE speed (30,60,90,120 Km/h) | 30 | 30 | 30 | 30 | 30 | 30 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | n/a | n/a | n/a | n/a | n/a | n/a |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | n/a | n/a | n/a | n/a | n/a | n/a |
|  | Whether LOSsoft is modeled or not | no | no | no | no | no | no |
|  | spatial consistency option (A or B) | A | A | A | A | A | A |
|  | Number of TX beams | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Number of RX beams | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Measurement reduction rate(50%,…Note2) | 75% | 80% | 75% | 80% | 75% | 80% |
|  | Observation window(Note3) | 800ms | 800ms | 800ms | 800ms | 800ms | 800ms |
|  | Prediction window (Note3) |  |  |  |  |  |  |
|  | Any other parameters (Note 4) |  |  |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 560K | 560K | 560K | 560K | 560K | 560K |
|  | Testing | 140K | 140K | 140K | 140K | 140K | 140K |
| AI/ML model input/output | Model input (Note 5) | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP |
|  | Model output(Note 6) | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP | L3 cell RSRP |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ARIMA (3,1,0) | ARIMA (3,1,0) | XGboost | Diff-XGBoost | Diff-FNN (n=3) | LSTM(GRU) (n=3) |
|  | Model complexity in a number of parameters(M) | 7 | 7 | 63KB | 63KB | 257 | 3M |
|  | Model complexity in model size (e.g. Mbyte) | 56 bytes | 56 Bytes | 0.25M | 0.25M | 2.1 KB | 2.4 MB |
|  | Computational complexity [FLOPs] | 9 | 9 | 800 | 800 | 576 | 200K |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.838 | 0.963 | 0.968 | 1.1 | 1.0 | 1.152 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) |  |  |  |  |  |  |