3GPP TSG-RAN WG2 Meeting #127														     R2-2406665

Maastricht, NL, August 19 – 23, 2024

Agenda item:	8.3.2.1

Source:	Apple

Title:	Field data for RRM measurement prediction

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide initial results for RRM measurement prediction using field data.

The intention is not (yet) to provide results for inclusion in the TR, this will follow in the next meeting, but rather to:

- Demonstrate the usefulness of using field data in ML training

- Revise certain assumptions to better reflect the reality in the field

# 2   	Discussion

### 2.1 	Dataset description

It must be understood that, while using field data provides significant advantages in ML, compared to synthetic/simulated data, it comes with certain limitations. In particular, one cannot expect that the field data would perfectly match the simulation assumptions agreed in RAN2, as unlike fully a controlled simulation environment there is no such level of control when collecting field data. This, however, is also an advantage as such data better reflects the reality and provides better insights into how ML algorithms might perform in real deployment. Furthermore, field data is obviously sensitive (e.g. for reasons of user privacy) and should be anonymized before usage and therefore there are limitations to the amount of information we can provide for such data.

For this evaluation we use field data of ~21.7M L3-layer RSRP measurement samples which consists of measurements from 1008 cells and 66K UEs. We extract 9.95M L3-RSRP measurement samples with ~160ms measurement period for performance evaluation. The important note here is that in real deployments measurements are not always performed at unform intervals, which is something that ML time series analysis requires. So certain data preprocessing is needed to bring the data closer to the form suitable for time series ML.

One immediate difference we observe when inspecting this data (in comparison to simulations) is UE mobility patterns. While RAN2 agreed to use the following UE speeds (30,60,90,120 km/h), in field data the mobility patterns are quite different. In particular, most UEs appear to be pedestrians and some are stationary, which doesn’t match the agreed UE speeds for simulation.

Observation 1: the majority of UEs in the field are pedestrians and/or stationary, therefore the assumption of evaluating UE speeds (30,60,90,120 km/h) does not reflect the reality.

We believe that the initial agreement of UE speed assumptions (30,60,90,120 km/h) was a bit premature. We also note that it doesn’t match the agreements in the study documented in TR 36.839 [2], where the following UE speeds were used: 3 km/h, 30km/h, 60km/h, 120km/h. Therefore, we propose to include in the study pedestrian speeds.

Proposal 1: to also consider pedestrian speeds (e.g. 3 km/h) and possibly stationary UEs in the study.

If adding another speed value is considered an issue due to increased evaluation load, we believe that e.g. 60 km/h can be safely excluded. At any rate, we think that evaluation pedestrian speeds of e.g. 3 km/h is more useful than e.g. 60 km/h.

Additionally, to demonstrate the advantages of using field data, we provide some results based on simulations which are in accordance with the RAN2 assumptions. With regards to the assumptions which have been left for companies’ choice, we’ve used the following:

Table 1: simulation assumptions

| Parameter |  |
| --- | --- |
| UE trajectory model | 1 |
| UE trajectory boundary processing | 1 |
| Speed | 30 kmh |
| Number of Tx beams | 64 |
| Number of Rx beams | 1 |
| LOS Soft | No |
| Inter-frequency correlation assumption in general | N/A |
| Inter-frequency shadow fading correction | N/A |
| Spatial Consistency | A |
| Number of samples | ~700K |

For simplicity and because the intention is, as stated before, not yet to provide data for inclusion in the TR but rather demonstrate ML performed on field data, we studied the FR1 intra-cell prediction case A.

### 2.1	Model description

We consider 4 prediction models:

- ARIMA(p,d,q),

- XGBoost,

- Transformer and

- Multi-layer Perceptron (MLP).

The rationale for choosing these specific models was to consider models with different complexity, starting from simple non-ML model, to simple ML non-deep learning model and finally deep learning models such as Transformer.

We split data into 80% training and 20% validation/test data for AIML model training and performance evaluations.

Table 2 below summarizes the models’ descriptions, detailed parameters and their complexity.

Table 2: Model description of prediction algorithms used in performance evaluation

| Model | Configurations | # of parameters (n) | Model storage complexity | Model computational complexity |
| --- | --- | --- | --- | --- |
| Sample & hold | Use the last measurement results as the predicted RSRP | N/A | N/A | N/A |
| Diff-AR | Diff-AR = ARIMA(1,1,0), differenced first-order autoregressive model Measurement window size w=10 Ŷt = μ + Yt-1 + ϕ1 (Yt-1 - Yt-2) where Yt is L3-RSRP measurement at time index t | 2 | 8Bytes | 4 FLOPs |
| XGBoost (XGB) | Input window size, w = 10 Number of estimators, t= 100 Max depth, d = 7 | # of nodes*(feature index + threshold + child node indexes) + # of leaves = ~63K (calculated) | 126KB | 900 FLOPs O(td) |
| Transformer | Input window size, w = 10 Model dimension, d = 32 Number of heads, h = 2 Number of layers, l = 2 | O(l*h*w^2*d) | 1.1 MB | 50432 FLOPs l * O(h*w*d^2 + w^2 *d) |
| Multi-layer perceptron (MLP) | 2 layer FCN with input size w =10, hidden layer size h = 64 and output o=1 | O(w*h+h) | 1408Bytes | 704 FLOPs |

### 2.2 	Prediction results

For the purpose of this preliminary analysis, we evaluated the “FR1 to FR1 intra-frequency temporal domain case A” case. It was selected for simplicity to be used for illustration of the usage of field data. Even though it is currently marked as low priority, and regardless of whether its priority remains low, the preliminary conclusions provided below would also be applicable to other cases (of higher priority), such as case B.

Below we provide performance evaluation results for 3GPP compliant simulated data in Figure 1 and Table 3.

Figure 1:Training and testing of prediction models over 3GPP compliant simulation data. Transformer, multi-layer perceptron (MLP), and XGBoost (XGB) are trained over 80% of the data and evaluated over the remaining 20% of the data (test data). Diff-AR and Sample&Hold are evaluated over the same test data.

Table 3: 3GPP compliant simulation data Avg. L3-cell RSRP difference in dB

| Model Description\Forecast Horizon(ms) | 80 | 160 | 320 | 480 | 640 |
| --- | --- | --- | --- | --- | --- |
| Transformer | 1.00 | 1.46 | 2.25 | 3.02 | 3.78 |
| Multi-layer perceptron (MLP) | 1.10 | 1.58 | 2.31 | 3.06 | 3.85 |
| XGBoost (XGB) | 1.07 | 1.53 | 2.32 | 3.07 | 3.81 |
| Sample & hold | 1.11 | 1.64 | 2.54 | 3.42 | 4.22 |
| Diff-AR | 1.10 | 1.61 | 2.49 | 3.36 | 4.18 |

We further provide performance evaluation results for NR field dataset in Figure 2 and Table 4.

Figure 2: Training and testing of prediction models over NR field dataset. Transformer, multi-layer perceptron (MLP), and XGBoost (XGB) are trained over 80% of the data and evaluated over the remaining 20% of the data (test data). Diff-AR and Sample&Hold are evaluated over the same test data.

Table 4: NR field dataset avg. L3-cell RSRP difference in dB

| Model Description\Forecast Horizon(ms) | 160 | 320 | 480 | 640 |
| --- | --- | --- | --- | --- |
| Transformer | 0.56 | 0.87 | 0.97 | 1.08 |
| Multi-layer perceptron (MLP) | 0.59 | 0.87 | 1.03 | 1.09 |
| XGBoost (XGB) | 0.56 | 0.87 | 1.08 | 1.21 |
| Sample & hold | 0.54 | 0.85 | 1.07 | 1.22 |
| Diff-AR | 0.54 | 0.86 | 1.10 | 1.26 |

One observation we can make based on the results shown above in Figures 1 and 2 is that models trained on field data provide better accuracy than models trained on simulated data.

Observation 2: models trained on field data provide better accuracy than models trained on simulated data.

A potential explanation is that, as mentioned above, while in simulations the minimum agreed UE speed is 30 km/h, in the field data most UEs are pedestrians or stationary. Having said that, the conclusion is not final in the sense that there maybe additional factors at play explaining the difference. Regardless, this does show that:

- Using ML for RSRP prediction in the field is promising, and

- There are indeed differences between models trained on field and simulated data

One might argue that low mobility field data is of less interest for the purpose of this study, however we don’t think this is the case.

While for the purpose of the study goal 2 (handover performance improvements) we should indeed be interested in more extreme cases (e.g. high mobility, cell edge cases, etc), this is actually not the case for the study goal 1 (reduction of measurement overhead) where it makes more sense to address the most common use case rather than the most extreme ones.

Observation 3: for study goal 1 (reduction of measurement overhead) we should be interested in the most common scenarios, not the most extreme ones.

Proposal 2: companies are encouraged to consider field data in their evaluations.

Another observation we can make is that the initially agreed prediction window for FR1 of 200ms is too conservative.

Observation 4: prediction window of longer than (previously agreed) 200ms (for FR1) may provide adequate results

The results above show that longer prediction windows (at least in FR1) should also be considered. The exact recommended value for prediction window is hard to establish, as we are yet to agree what should be considered an acceptable error (we elaborate further on this issue in [2]), however extending it at least to 400ms seems reasomable for the purpose of this study. This is of course assuming that FR1 temporal prediction case A (where this value is applicable to) remains part of the study.

Proposal 3: to consider prediction windows of at least up to 400ms in FR1 (ideally 600ms).

Note: the proposal 3 above is made based on temporal prediction case A in FR1, which is currently marked as low priority. Having said that the conclusion that it is worthwhile to consider longer prediction windows is likely to apply to other cases (e.g. FR2 where 400ms was selected), as both prediction windows for FR1 and FR2 were initially selected rather arbitrary and this analysis shows that companies’ initial assumptions perhaps were too conservatives.

Yet another important observation is that simple models, such as XGBoost and even ARIMA, do provide adequate results which are not far away from more complex models (such as Transformer and MLP) at least in some cases (i.e. relatively low prediction windows). Since these models are significantly simpler, and even short prediction windows may be of interest (less so in case A but definitely in case B), and so we think simpler models should be considered and indeed be “formally” part of this study.

Observation 5: simple models, such as XGBoost and even ARIMA, provide good performance (at least in some scenarios).

The observation above is also supported by the results we provide in [3], which are based on simulated data for high priority case “FR1 to FR1 intra-frequency temporal domain case B”.

Proposal 4: the study should also include simple models, including non-ML models.

# 3	Proposals

Observation 1: the majority of UEs in the field are pedestrians and/or stationary, therefore the assumption of evaluating UE speeds (30,60,90,120 km/h) does not reflect the reality.

Observation 2: models trained on field data provide better accuracy than models trained on simulated data.

Observation 3: for study goal 1 (reduction of measurement overhead) we should be interested in the most common scenarios, not the most extreme ones.

Observation 4: prediction window of longer than (previously agreed) 200ms (for FR1) may provide adequate results

Observation 5: simple models, such as XGBoost and even ARIMA, provide good performance (at least in some scenarios).

Proposal 1: to also consider pedestrian speeds (e.g. 3 km/h) and possibly stationary UEs in the study.

Proposal 2: companies are encouraged to consider field data in their evaluations.

Proposal 3: to consider prediction windows of at least up to 400ms in FR1 (ideally 600ms).

Proposal 4: the study should also include simple models, including non-ML models.

# 4	References

[1]		RAN2#126 meeting minutes

[2]		TR 36.839 Mobility enhancements in heterogeneous networks (Release 11)

[3]		R2-2406664, RRM measurement prediction results for case B, Apple