3GPP TSG-RAN WG2 Meeting #127														     R2-2406666

Maastricht , NL,  August 19 – 23, 2024									  Revision of R2-2404628

Agenda item:	8.2.2

Source:	Apple

Title:	Functional Aspects of A-IoT devices

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#125bis[3] and RAN2#126 [7], the following function aspects on A-IoT have been agreed:

RAN2#125bis agreements: 1 RRC connection management is not supported. FFS how the resource configuration is provided to the device (if needed based on RAN1 progress) 2 RRM L3 measurement reporting is not supported by Ambient IoT devices. 3 RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding. 4 Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals. 5 RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC 6. SDAP is not supported for UP protocol stack. 7. PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 discussion) and any other really needed functionalities. 8. RLC layer is not needed. FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size). RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed. 9. No HARQ and RLC AM 10 FFS about the level of visibility required by the reader and what information is necessary for AS layer operations. 11 RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2 RAN2#126 agreements: 1. Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue. 2 legacy NR BSR/SR is not needed for A-IoT communication. 3 FFS whether further indication of device message size/status is needed 4 AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed)

In this contribution we provide our views on the outstanding stage-2 issues and functional aspects of Ambient IoT design.

# 2   	Discussion

### 2.1	A-IoT device types

RAN1 consider 3 types of A-IoT devices:

- Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

While the details of this device categorization do not directly impact RAN2 discussion, the fact there are different A-IoT devices with different capabilities does! Therefore, it makes sense to also consider different device capabilities in terms of their baseband (i.e. RAN2-related) capabilities.

Observation 1 	RAN1 agreed to consider different A-IoT device types and therefore it makes sense to consider different device types in terms of their baseband/RAN2-related capabilities.

One way to approach this problem is to start from the RAN1 device type definitions and assumptions and make our way up from there to discuss their baseband related capabilities. This approach is likely to be difficult, though, as it is not clear how we could agree on those. One point to keep in mind though is that if we consider various RFID EPC Class 1 Gen 2 devices in the market (see our contribution submitted to RAN2#125bis in [4]), their capabilities vary rather radically: from read-only devices to devices with writable memory, from sticker form factor to cards, etc. Needless to say, those devices also vary radically in terms of cost.

Observation 2 	Even RFID EPC Class 1 Gen 2 devices come in different types in terms of their baseband capabilities, form factors and cost.

Finally, SA2 have recently agreed on Solution #33 in TR 23.700-13 [6] where two Ambient IoT device types are considered:

- Type A AIoT device that may not need to address privacy requirements and may also use a fixed permanent

- device identifier;

- Type B AIoT device that would use a temporary identifier, to address privacy concerns.

Observation 3 	SA2 have recently agreed to Solution #33 introducing two device types (type A and type B).

This discussion eventually boils down to questions of cost and complexity, which incidentally would affect very much other important topics under discussion, such as security. It is rather obvious that RAN2 has two conflicting goals for the A-IoT design:

- A-IoT device is extremely cost-sensitive, which suggests the simplest possible design.

- There are already RFID EPC C1G2 (and many other) low-cost devices in the market. In order to compete with them, 3GPP solution must provide differentiation – this suggests a somewhat more advanced design with features not currently supported by the devices on the market.

One possible way out of this conundrum is to consider (as RAN1 did!) at least two A-IoT device types: low-end A-IoT device and high-end A-IoT device. Naturally, there can be more than two types, but we think it is sufficient and is definitely a good starting point. Not only would this align with RAN1 design approach, but would also provide a way to resolve many of the conflicting requirements and opinions on the design. For example:

- the low-end A-IoT device type many not need to address privacy requirements and may also use permanent device identifier

- the high-end A-IoT device would use a temporary identifier, to address privacy concerns

This is just one example of such device categorization provided here for illustrative purposes only. All we are proposing at the moment is:

Proposal 1	RAN2 to consider two A-IoT device types (e.g. low-end and high-end) in terms of their baseband/RAN2-related capabilities.

If this is agreeable, eventually we may need to discuss how the RAN1 device types “map” to the RAN2 A-IoT device types, but this issue can be addressed later.

### 2.2	Inventory and Command

In RAN2#125bis we’ve agreed to introduce the terminology of “inventory” and “command” use cases. In particular, the following two agreements of relevance have been made:

- RAN2 will support two use cases, “inventory” and “command”.  The definition, detailed wording is FFS

- We will study the support of both “inventory” and “command” in the same procedure.

It should be noted that as of now, the terms “inventory” and “command” haven’t actually been defined – neither in RAN2, nor in SA2.

Observation 4 	The terms “inventory” and “command” haven’t actually been defined – neither in RAN2, nor in SA2.

We think the difficulty in discussing the concepts of “inventory” and especially “command” in 3GPP is because 3GPP follows a very different design pattern and working procedures compared to EPC, where the terms inventory and command originate from. The figure 1 below illustrates the differences.

EPC G2C1 device

3GPP A-IoT device

Tag-identification layer

Application

A-IoT NAS

A-IoT PHY

A-IoT MAC

PHY

Figure 1. EPC Protocol Stack Example

In 3GPP, we always follow a somewhat strict layer separate model, in which RAN2 defines radio related protocol layer(s) (e.g. A-IoT MAC in this case and maybe even a dedicated control plane layer), SA2/CT1 define non-access stratum protocol layer(s) (which may or may not be needed for A-IoT) and the application layer is usually outside of the 3GPP scope.

On the other hand, EPC G2C1 specification (where the terms “inventory” and “command” originate from) doesn’t follow such protocol layer separation – in that specification, there is only one protocol layer (referred to as “tag-identification layer”) which encompasses the functionality of 3GPP A-IoT MAC, other 3GPP protocols (if we end up defining additional protocol layers for A-IoT) and also the application layer (which is unlikely to be defined in 3GPP).

Observation 5 	Unlike 3GPP, EPC G2C1 specification doesn’t have protocol layer separation – everything from MAC to the application is defined in a single protocol referred to as tag-identification layer.

It is now perhaps obvious why the discussion of “inventory” and more so “command” is difficult in RAN2. As a side note, in EPC G2C1 spec inventory is a command, so separating them into two seemingly different groups further increases confusion.

Observation 6 	In EPC G2C1 spec Inventory is A Command.

To illustrate the core issue here, let’s take a look at the Access Command, as it is defined in EPC G2C1:

“Access. Communicating with an identified Tag. The Interrogator may perform a core operation such as reading, writing, locking, or killing the Tag; a security-related operation such as authenticating the Tag; or a file-related operation such as opening a particular file in the Tag’s User memory. Access comprises multiple commands. An Interrogator may only access a uniquely identified Tag.”

“The access command set comprises Req_RN, Read, Write, Lock, Kill, Access, BlockWrite, BlockErase, Block- Permalock, Authenticate, ReadBuffer, SecureComm, AuthComm, KeyUpdate, Untraceable, FileOpen, FileList, FilePrivilege, FileSetup, and TagPrivilege.”

As one can see, the Access Commands supports operations such as Read, Write, BlockWrite, FileList, etc which would perhaps belong to the application layer in the 3GPP protocol stack model.

Observation 7 	EPC G2C1 Access Commands supports operations such as Read, Write, BlockWrite, FileList, etc which would perhaps belong to the application layer in the 3GPP protocol stack model.

Therefore, in order to proceed the discussion of the protocol stack design, it is important that everybody in RAN2 are on the same page of what protocol stack model we are discussion. Hence the proposal.

Proposal 2 	RAN2 to confirm that in the A-IoT study we stick to the layer separation as we always do in 3GPP, i.e. A-IoT MAC only carries radio related information and application information is in the upper layers outside of the RAN2 scope.

We would like to acknowledge that there can be indeed technical reasons why EPC adopted the single protocol stack model. The obvious one id overhead reduction, which in turn results in cost reduction. Having said that, if we deviate from the usual 3GPP protocol stack and work separation which can be summarized as follows:

- RAN2 define radio/AS protocols,

- SA2/CT1 define non-AS protocols, and

- Application protocols are defined outside of 3GPP,

This would require quite a bit of coordination between RAN2, SA2 and CT1 which is likely to delay the study.

### 2.3 Multiplexing in A-IoT MAC PDUs

RAN2 has agreed that “Multiple AIoT logical channels” for upper layer data are not supported.” But it is still regarded as an open modeling issue whether multiplexing is still allowed in MAC layer. This is because MAC PDU design may or may not support different types of “upper layer” traffic. Although the upper layer traffic may be diverse with multiple different use cases (inventory, command, etc) to support,  we think some abstraction from MAC layer perspective can be beneficial to clarify this design aspect.

For DL traffic, we can simply classify the R2D traffic as follows :

- R2D data that should trigger response

  - R2D data triggering response with CBRA

  - R2D data triggering response w/o CBRA (e.g., CFRA)

- R2D data (no ACK/response needed)

Strictly speaking, there are at most only three above message types to be potentially supported in A-IoT MAC layer design for DL traffic. Moreover, for each “destination ID” (device ID, group ID, etc), there should be only one valid type of R2D data at one time. For example, there is no such a case that an A-IoT reader sends “ R2D data triggering response” and “R2D data (no response needed)” simultaneously to the same device, because aggregating those two upper-layer R2D data shall be merged by CN as a single “super-command” in need of device response, and such “aggregation” operation is transparent to the reader and A-IoT air interface, and should be out of RAN2 scope.

Therefore, we think the multiplexing issue in DL is equivalent to determine whether we allow a MAC PDU that can contain different R2D data towards different A-IoT destinations. For example, if a R2D MAC PDU should contain multiple “Paging records”, with each of which intends to reach a different set of devices. If yes, then multiplexing is need in A-IoT DL. Otherwise, then multiplexing is not needed. Thus, RAN2 need first discuss whether multiple R2D data to different destinations are allowed to be included in one R2D transmission or not.

Proposal 3	Multiplexing support in DL MAC PDU depends on whether multiple R2D data to different destinations are allowed to be included in one R2D transmission or not.

For UL traffic, we can also have three D2R traffic defined as the following:

- CBRA Msg 1

- CBRA Msg 3

- Other D2R response for DO-DTT (included response transmitted via CFRA)

Note that if CBRA Msg1 contains random ID, then it is supposed to be terminated in the reader and not reach the CN. But for all the other cases, the upper layer D2R data (including device ID) encapsulated in D2R transmissions from a certain A-IoT device will be terminated at CN. Logically, the A-IoT UL MAC PDU can be very simple, it will be always one of the following two cases:

- Msg1 (Random ID)

- Device ID and/or any upper layer “D2R response” data

Logically, the above two cannot occur at the same time in one A-IoT device. Therefore, there is no multiplexing need in UL MAC PDU.

Proposal 4	Not support Multiplexing in UL MAC PDU.

### 2.4 Segmentation and Assembly

RAN2 has agreed that RLC layer is not needed in A-IoT [1]. But whether segmentation and assembly are to be supported still needs further discussion.

To analyze this issue, we can divide this into DL and UL case respectively.

R2D Traffic:

For the DL traffic, the data for command use case is basically the A-IoT command(s). Literally speaking, “command”, as suggested by the word itself, implies a much narrower scope than a more general term such as “information”. The transmission of a DL command results a certain action taken by the device, instead of reception of a generic information message. Even for the so-called “write” command, this is surely not for a generic information download but very tiny write-in information (e.g. <=16bit) due to the limited availability and high-cost of “write” in non-volatile memory of A-IoT devices.

This being said, there may be a valid use case for large information transfer/data download, e.g. for firmware update. It is unlikely that all potential firmware sizes will fit in a single DL packet. One way to support this, though, without increasing A-IoT complexity is to assume that segmentation (and reassembly) are performed at the application layer. This is a reasonable approach as such cases of large information transfer, while need to be supported in some cases, are not expected to be frequent.

Observation 8: 	Special cases of large information transfer in DL can be handled by application layer segmentation and reassembly.

Thus, we think it is safe to assume DL command message is quite small, and even if multiplexing multiple DL commands in the same TB are supported. There is no need to introduce segmentation, because the reader does not benefit by sending a “partial” command to the device earlier and it is more straight forward to use another grant to send remaining DL commands if there is no enough room in TB.

D2R Traffic:

For the UL traffic, the data size estimate is relatively harder. Strictly speaking, the only must-have information for the inventory use case is the “Device ID”, which is likely no more than 128-bit size. Nonetheless, we cannot rule out if any additional “information” the device may be asked to provide to the application server in D2R transmission (DO traffic) when it is triggered by some DL command or other DTT triggers. Given that the very low-cost, low-complexity nature of A-IoT device, the device may not be able to collect and store a lot of data. So, it is hard to imagine that the DO traffic would be more than a few hundreds of bits, other than the Device ID itself.

However, as a cautionary step to exactly determine the potential entropy of “device-originated information”, RAN2 would better to get more clear confirmation/input from upper layer design requirements. If SA2 or SA1 can provide an upper bound of DO traffic in different use cases, then we can compare it with PHY layer’s UL transmission capacity of A-IOT device and determine the necessity of segmentation and reassembly of DL traffic. Also, if RAN1 can provide some information about possible TB size limits, it would be very helpful for us to make a device on this aspect.

And even if SA2/SA1 reply suggests that UL segmentation is needed due to potentially large data size, given its implication to device complexity, it may need to only be supported by high-end A-IoT devices (e.g.. depending on device capability).

Proposal 5	For R2D traffic, RAN2 assumes that the DL A-IoT Data PDU can be always accommodated in the minimum-size TB supported in PRDCH channel, thereby segmentation and reassembly for DL traffic not supported.

Proposal 6	For the necessity of segmentation of D2R traffic, RAN2 wait for other WGs progress first before making a decision.

### 2.5 Ambient IoT Protocol Stack

For Ambient IoT user plane support, only PHY layer and MAC layer are needed. As agreed in RAN2#125bis meeting [1] that:

RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

For Control Plane design, there could be a dedicated A-IoT CP protocol (e.g. for paging and initial access) on top of MAC layer, but it can also be implemented as MAC Control PDUs. Thus, the presence of “AIoT CP Protocol” is not really needed. For the sake of time efficiency in Rel-19 study, we propose to remove this layer and RAN2 can focus on MAC layer design only for both UP and CP altogether.

Proposal 7	no need for a dedicated AS control plane protocol (i.e. control plane functions are part of the MAC protocol).

On top of data transmission protocol in MAC layer, there could be an upper layer designated as “AIoT layer” (e.g., AIoT-NAS for CP, FFS UP) to communicate with Core Network for both UP and CP, which is to be studied by SA2 and CT1, but out of RAN2 scope, as those data contents may be transparent to the gNB reader.

We think RAN can consider the protocol stack in Figure 2 as a baseline. Of course, depending on the inputs from other 3GPP WGs (RAN1, SA2 or SA3), certain changes would still be considered.

Figure 2. Protocol Stack for A-IoT Topology 1

Proposal 8	RAN2 consider use the above protocol stack as baseline for Ambient IoT Topology 1.

# 3	Conclusions and Proposals

We have the following observations:

Observation 1 	RAN1 agreed to consider different A-IoT device types and therefore it makes sense to consider different device types in terms of their baseband/RAN2-related capabilities.

Observation 2 	Even RFID EPC Class 1 Gen 2 devices come in different types in terms of their baseband capabilities, form factors and cost.

Observation 3 	SA2 have recently agreed to Solution #33 introducing two device types (type A and type B).

Observation 4 	The terms “inventory” and “command” haven’t actually been defined – neither in RAN2, nor in SA2.

Observation 5 	Unlike 3GPP, EPC G2C1 specification doesn’t have protocol layer separation – everything from MAC to the application is defined in a single protocol referred to as tag-identification layer.

Observation 6 	In EPC G2C1 spec Inventory is A Command.

Observation 7 	EPC G2C1 Access Commands supports operations such as Read, Write, BlockWrite, FileList, etc which would perhaps belong to the application layer in the 3GPP protocol stack model.

Observation 8 	Special cases of large information transfer in DL can be handled by application layer segmentation and reassembly.

Then, we have the following proposals:

Proposal 1	RAN2 to consider two A-IoT device types (e.g. low-end and high-end) in terms of their baseband/RAN2-related capabilities.

Proposal 2 	RAN2 to confirm that in the A-IoT study we stick to the layer separation as we always do in 3GPP, i.e. A-IoT MAC only carries radio related information and application information is in the upper layers outside of the RAN2 scope.

Proposal 3	Multiplexing support in DL MAC PDU depends on whether multiple R2D data to different destinations are allowed to be included in one R2D transmission or not.

Proposal 4	Not support Multiplexing in UL MAC PDU.

Proposal 5	For R2D traffic, RAN2 assumes that the DL A-IoT Data PDU can be always accommodated in the minimum-size TB supported in PRDCH channel, thereby segmentation and reassembly for DL traffic not supported.

Proposal 6	For the necessity of segmentation of D2R traffic, RAN2 wait for other WGs progress first before making a decision.

Proposal 7	no need for a dedicated AS control plane protocol (i.e. control plane functions are part of the MAC protocol).

Proposal 8	RAN2 consider use the above protocol stack as baseline for Ambient IoT Topology 1.

# 4	References

[1] 		RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2]		TR 38.848, Study on Ambient IoT (Internet of Things) in RAN

[3] 		RAN2#125bis meeting minutes

[4]		R2-2402896, A-IoT device hardware capabilities, Apple

[5]		EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard

[6]		TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things

[7] 		RAN2#126 meeting minutes