3GPP TSG-RAN WG2 Meeting #127														     R2-2406667

Maastricht , NL,  August 19 – 23, 2024

Agenda item:	8.2.5

Source:	Apple

Title:	Discussion on Topology 2 for A-IoT

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our views on the Ambient IoT UE reader (i.e. in Topology 2) functionality, interfaces and overall design.

# 2   	Discussion

So far, RAN2 have focused on the A-IoT air interface design, which is understandable as it is the biggest and the most important part of the overall A-IoT design.

However, we must note that there are ongoing discussions in RAN3 and most recently in RANP, where issues having direct RAN2 impact (and also those which should be led by RAN2) have been brought up. Therefore, it is important that RAN2 discuss and make at least some high-level agreements on the topics related to the A-IoT UE Reader design, which are in RAN2 responsibility. One such example is the interface used to control an A-IoT UE Reader by the A-IoT gNB.

## 2.1 Background

In RAN#104, the issue of A-IoT architecture for topology 2 was brought up in [1]. Unfortunately, the issue wasn’t resolved in RAN and therefore needs to be discussed in the WGs.

Previously, in RAN3#124, the following (rather confusing) architecture was agreed (with a note that whether it applies to topology 2 hasn’t been agreed)

Editor’s Note 9: whether the logical system architecture above is applied to topology 2 to be discussed in next meeting.

In RAN3#125, the group couldn’t reach an agreement on the architecture for topology 2. As mentioned above, the issue was brought up in RAN, but wasn’t resolved there either. While the architecture discussions can happen in RAN3, what is important is that in RAN2 there is an agreement on the radio interfaces that need to be studied – in particular, to support topology 2 with UE Reader.

## 2.2 A-IoT Interfaces

As mentioned, in this paper we focus on interfaces (especially the interfaces A-IoT UE Reader needs to support). Therefore, the diagrams below are intended for this purpose and should be viewed as an illustration which is useful to discuss the interfaces.

In Topology 1, there are only two interfaces:

- A-IoT radio interface between A-IoT device and A-IoT gNB Reader (which is being discussed in RAN2), and

- XX interface between A-IoT gNB Reader and A-IoT CN (which should be discussed in RAN3)

This is illustrated by the Figure 1 below.

Figure 1: Radio and network interfaces for Topology 1

In Topology 2, there are three interfaces:

- A-IoT radio interface between A-IoT device and A-IoT gNB Reader (which is being discussed in RAN2), and

- A radio interface between A-IoT UE Reader and A-IoT gNB, and

- XX interface between A-IoT gNB and A-IoT CN (which should be discussed in RAN3)

This is illustrated by the Figure 2 below.

Figure 2: Radio and network interfaces for Topology 2

While it might be obvious that the interfaces between a UE (A-IoT UE Reader in this case) and a gNB (A-IoT gNB in this case) is the Uu interface, in the light of the recent RAN3/RANP discussions we think it would be beneficial to make this a formal agreement, hence the proposal. This of course doesn’t imply there would be no enhancements to support A-IoT UE Reader functionality.

Proposal 1: The interface between A-IoT gNB and A-IoT UE Reader (i.e. topology 2) is Uu (with enhancements as needed).

In the figures 1 and 2 above, we use the following terminology:

- A-IoT Device: a device, e.g. “a tag” supporting A-IoT radio interface

- A-IoT radio interface: the new radio interface between an A-IoT device and an A-IoT Reader

- A-IoT UE Reader: a UE implementing the A-IoT radio interface and acting as a Reader

- A-IoT gNB Reader: a gNB implementing the A-IoT radio interface and acting as a Reader

- A-IoT gNB: a gNB capable of controlling an A-IoT UE Reader

- A-IoT CN: a CN capable of communicating with A-IoT gNB and A-IoT gNB Reader

- XX interface: the interface between A-IoT CN and A-IoT gNB/A-IoT gNB Reader (name and definition up to RAN3)

Note: the definition and usage of the XX interface is up to RAN3 and SA2, we can use a temporary term e.g. XX for now and revise when/if needed based on RAN3/SA2 decisions. RAN3 and/or SA2 may also decide if the protocol(s) for XX interface for A-IoT gNB (in Topology 2) and A-IoT gNB reader (in Toplogy 1) should be the same or designed differently.

Sone of these terms have defined, some are perhaps rather self-explanatory, but nevertheless it would be good to capture the definitions and the diagrams for topologies 1 and 2 in the TR.

Proposal 2: capture in the TR the figures for topologies 1 and 2 and the definitions for:

- A-IoT Device: a device, e.g. “a tag” supporting A-IoT radio interface

- A-IoT radio interface: the new radio interface between an A-IoT device and an A-IoT Reader

- A-IoT UE Reader: a UE implementing the A-IoT radio interface and acting as a Reader

- A-IoT gNB Reader: a gNB implementing the A-IoT radio interface and acting as a Reader

- A-IoT gNB: a gNB capable of controlling an A-IoT UE Reader

- A-IoT CN: a CN capable of communicating with A-IoT gNB and A-IoT gNB Reader

- XX interface: the interface between A-IoT CN and A-IoT gNB/A-IoT gNB Reader (name, definition and details up to RAN3/SA2)

The next, slightly less obvious but hopefully still simple, question is which control and user plane protocols are used on the Uu interface between A-IoT UE Reader and A-IoT gNB. Considering the amount of work facing us to study the A-IoT radio interface (between A-IoT device and Reader), we certainly hope RAN2 can quickly agree to re-use the existing protocols (i.e. RRC for control and PDCP/RLC/MAC for user plane), in short the existing control and user plane protocol stacks on this interface.

Proposal 3: the Uu interface between A-IoT gNB and A-IoT UE Reader uses the legacy control and user plane protocol stacks RRC/PDCP/RLC/MAC (with enhancements where needed).

Since we are having this discussion anyway, it might be useful to expand a bit on the functionality that needs to be supported on this interface. This is not an exhaustive discussion, as at this point in time in the study it would be sufficient to identify the main functions and the actual protocol design can be done in the WI phase.

While the final details will be eventually provided by RAN1, it is safe to assume that an A-IoT UE Reader, being a UE, would not be able to transmit in licensed spectrum without being controlled by the network. Therefore, it is reasonable to assume and can be agreed already at this stage, that the A-IoT gNB is responsible for controlling A-IoT radio resources (utilized by A-IoT UE Reader). .

Proposal 4: A-IoT gNB is responsible for controlling A-IoT radio resources (utilized by A-IoT UE Reader).

Since we’ve introduced the A-IoT UE Reader concept, we think it would be good to discuss briefly what kind of UE that would be (beyond the short definition provided above). Considering that while A-IoT applications are potentially manyfold, most if not all of them are hardly something each and every typical use would perform on their handset. That is, it appears more likely that special handset will be developed to act as A-IoT UE Readers.

Observation 1: A-IoT UE Reader is likely to be a special UE designed, developed and deployed for this purpose.

Which leads as to the following proposal, that A-IoT UE Reader functionality is an optional capability.

Proposal 5: A-IoT UE Reader functionality is an optional capability.

Finally, we would like to address the issue of A-IoT Inventory and Command and how they relate to the A-IoT gNB and UE Reader functionalities in Topology 2, respectively. First off, we acknowledge that the details of the A-IoT protocol in general, and how Inventory and Command are implemented in that protocol (and whether Command is standardized at all) are for SA/CT WGs to decide. For example, UE reader may communicate A-IoT traffic with CN functions via either control plane (e.g., NAS container) or user plane (e.g., PDU session), but this should be discussed in SA2.  From the RAN2 perspective, we are not sure that Command or Inventory should be even visible to RAN (but we are willing to discuss the pros and cons).

Observation 2: the details of the protocol for Inventory and Command are up to SA/CT WGs.

## 2.3 Reflections on RAN3 discussions which have RAN2 impact

In this section we would like to highlight some ongoing discussions in RAN3, which in our view have RAN2 implications and in some cases should not be discussed in RAN3 at all.

Here we refer to the following figure from [3], which wasn’t agreed in RAN3, but was discussed:

As one can see, the figure from the RAN3 A-IoT architecture moderator’s proposal (not agreed) seems to attempt to define an “A-IoT enabled UE” architecture of sorts, which somehow would have “common reader function”, which would be a common function to a UE and a gNB, which has some rather far fetching implementation implications, all of which are outside of scope of 3GPP.

Observation 3: TPs brought up in RAN3 attempt to define internal UE architecture, which is always an implementation choice and is never standardized.

Hence the proposals:

Proposal 6: A-IoT UE “internal architecture” (whether it is a regular UE or an A-IoT UE reader) should not be discussed in RAN3, but rather in RAN2.

Proposal 7: As a matter of fact, “internal UE architecture” should not be discussed in 3GPP at all as it is not standardized and always left for UE implementation.

## 2.4 Uu RRC Enhancements for UE reader

The RRC protocol would be enhanced to support UE reader control in Topology 2 to provide control/configuration for UE reader operation. One thing is to consider SIB indication of the support of this function by A-IoT-enable gNB. This will help mobile UE choose a suitable cell for the reader operation, as we can’t assume all the gNBs will be upgraded to support A-IoT.

Proposal 8: to consider as part of RRC enhancements SIB indication of UE reader support by gNB.

Then to help NW select a proper UE reader to reach the targeted A-IoT device(s), we think it could be beneficial for UE reader to report some assistance information to NW (e.g., location, battery status). This can be either done via RRC or NAS, but RAN2 should study whether this can be supported via the enhancement of UAI (UEAssistanceInformation).

Proposal 9: to study UAI message enhancements to allow UE reader to report assistance information for A-IoT operation.

There are certain radio aspects of radio configuration such as bandwidth, frequency resources, time resources which would need to be configured by the UE reader’s serving cell. Therefore, it is necessary to add new IEs for A-IoT operations in RRCReconfiguration message.

Proposal 10: to study RRCReconfiguration message enhancements for UE reader control by the network (in Topology 2).

Finally, it would be interesting to discuss whether a UE reader can operate A-IoT air interface in IDLE/INACTIVE RRC state. NW may need to control UE reader directly in RRC_CONNENCTED state, but such a control may not need to be very dynamic and can be somehow semi-static. Since there is very light traffic for A-IoT communication, some IDLE/INACTIVE UE may still support A-IoT operation as a UE reader. For example, SDT operation may be considered to be a match for A-IoT UE reader. At the very least, UE reader behaviour in IDLE and INACTIVE must be specified.

Proposal 11: to discuss whether/how UE reader in IDLE/INACTIVE can be supported.

## 2.5 Uu MAC Enhancements for UE reader

NW (A-IOT gNB) may allocate R2D and/or D2R resources for the UE reader operation in Topology 2. Besides using RRC signalling to configure A-IoT grants, L1/L2 indication of those A_IoT resources in Uu interface are also possible and should be considered.  From the UE reader perspective, BSR-like procedure may be needed for solicit A-IoT grants. This is because the A-IoT traffic initiated by a CN A-IoT function may be transparent to gNB and serving gNB is not aware of actual traffic situation in the UE reader, in general. This is similar to the SL BSR design where the grants of another radio interface are solicited by a BSR procedure.

Proposal 12: Define new BSR MAC CEs in Uu to support A-IoT reporting buffered R2D/D2R data.

It is also very likely that some fine-tuning of UE reader operation regarding certain radio parameters are needed. For example, Tx power control  in A-IoT air interface can be done via a DL MAC CE in Uu interface to regulate the potential interference issue for gNB’s Uu operation . Also, if a UE reader is also acting as a CW node, then some ON/OFF switch of CW generation may be considered. We think this has to be first discussed in RAN1, then RAN2 can decide how to support with DL MAC CE(s) based on RAN1 agreements.

Proposal 13: RAN2 to study the support of DL MAC CE enhancements based on RAN1 agreements.

# 3	Conclusions and Proposals

Proposal 1: the interface between A-IoT gNB and A-IoT UE Reader (i.e. topology 2) is Uu (with enhancements as needed).

Proposal 2: capture in the TR the figures for topologies 1 and 2 and the definitions for:

- A-IoT Device: a device, e.g. “a tag” supporting A-IoT radio interface

- A-IoT radio interface: the new radio interface between an A-IoT device and an A-IoT Reader

- A-IoT UE Reader: a UE implementing the A-IoT radio interface and acting as a Reader

- A-IoT gNB Reader: a gNB implementing the A-IoT radio interface and acting as a Reader

- A-IoT gNB: a gNB capable of controlling an A-IoT UE Reader

- A-IoT CN: a CN capable of communicating with A-IoT gNB and A-IoT gNB Reader

- XX interface: the interface between A-IoT CN and A-IoT gNB/A-IoT gNB Reader (name and definition up to RAN3)

Proposal 3: the Uu interface between A-IoT gNB and A-IoT UE Reader uses the legacy control and user plane protocol stacks RRC/PDCP/RLC/MAC (with enhancements where needed).

Proposal 4: A-IoT gNB is responsible for controlling A-IoT radio resources (utilized by A-IoT UE Reader).

Observation 1: A-IoT UE Reader is likely to be a special UE designed, developed and deployed for this purpose.

Proposal 5: A-IoT UE Reader functionality is an optional capability.

Observation 2: the details of the protocol for Inventory and Command are up to SA/CT WGs.

Observation 3: TPs brought up in RAN3 attempt to define internal UE architecture, which is always an implementation choice and is never standardized.

Proposal 6: A-IoT UE “internal architecture” (whether it is a regular UE or an A-IoT UE reader) should not be discussed in RAN3, but rather in RAN2.

Proposal 7: As a matter of fact, “internal UE architecture” should not be discussed in 3GPP at all as it is not standardized and always left for UE implementation.

Proposal 8: to consider as part of RRC enhancements SIB indication of UE reader support by gNB.

Proposal 9: to study UAI message enhancements to allow UE reader to report assistance information for A-IoT operation.

Proposal 10: to study RRCReconfiguration message enhancements for UE reader control by the network (in Topology 2).

Proposal 11: to discuss whether/how UE reader in IDLE/INACTIVE can be supported.

Proposal 12: Define new BSR MAC CEs in Uu to support A-IoT reporting buffered R2D/D2R data.

Proposal 13: RAN2 to study the support of DL MAC CE enhancements based on RAN1 agreements.

# 4	References

[1] 			RP-241270, Views on Ambient IoT architecture for Topology 2, Qualcomm, Vivo, Apple,

Vodafone, MediaTek

[2]			R3-243807, [TP for TR 38.769] CB:#AIoT1_Architecture], Ericsson (Moderator), Huawei