# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406668
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	7.3.2

Source:	Apple

Title:	Remaining issue on SFN timing of Cell DTX/DRX

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

R2-2404945	Discussion on the unaligned SFN for Cell DTXDRX in MAC	CATT	discussion	Rel-18	Netw_Energy_NR-Core

Proposal: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DTX/DRX duration.

-	LG doesn’t think we need that change as each serving cell would look for it’s own time, as DTX is executed per serving cell.  Apple doesn’t think this is even in scope.

-	Nokia wonders if the UE knows the SFN of the SCells.  LG thinks that the UE knows the SFN of the serving cell.   Nokia explains that the SFN is in the MIB.   LG and Lenovo explains that the UE has to read the MIB for synchornization purposes.  Samsung thinks that there is no requirement to read the SFN in the MIB

-	Xiaomi aks if we can do CELL DTX/DRX with SSBless

=>	Noted

In RAN2#126 [1], it was discussed the timing of Cell DTX / DRX in case of the unaligned SFN. But there was no consensus due to different views. The related Chair Note can be found below:

In this contribution, we further discuss this issue and its potential specification impacts.

# 2 Discussion

Since there were confusion / misunderstanding on the NOTE in TS 38.321 [2], we first provide some background information in Section 2.1 and then discuss solutions in Section 2.2.

## 2.1 Background information

### 2.1.1 Synchronized CA

In LTE and NR Rel-15, only synchronized CA is supported, where frame timing and SFN are aligned across all serving cells within one cell group.

According to TS 38.321 [2], SFN is required for UE DRX, SPS and CG, and related parts are copied below:

DRX

1>	if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

SPS

For configured downlink assignments, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

CG

For configured uplink grants, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

where CURRENT_symbol=(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

Please note that it was not specified which serving cell’s SFN is used. There is no ambiguity in synchronized CA because all carriers have the same SFN (i.e. the UE is allowed to use SFN of any serving cell for DRX, SPS and CG).

Observation 1: In LTE and NR Rel-15, only synchronized CA is supported. Therefore, although SFN is required for UE DRX, SPS and CG, TS 38.321 didn’t specify which serving cell’s SFN is used because the SFN of all serving cells are aligned (i.e. the UE is allowed to use SFN of any serving cell for DRX, SPS and CG).

### 2.1.2 Asynchronized CA

Asynchronized CA was introduced in Rel-16 WI of DCCA enhancement. The intention is to support inter-band CA which are deployed by different operators. As illustrated in Figure.1, two bands (A and B) co-existed with different operators. Operator A and Operator B, who have adopted different starting points, respectively, which requires to adopt different starting points for the two bands. It has blocked some operator from deploying LTE CA between these two bands and may continue to block deploying NR CA between these two bands.

MNO A

MNO B

Sta

rtingpoint A

Sta

rtingpoint

B

Figure.1 Illustration of async CA

To resolve this issue, Rel-16 WI of DCCA enhancement introduced asynchronized CA, where the misalignment is limited to ±76800Ts and explicit RRC signaling was introduced to indicate the slot offset.

Observation 2: To support inter-band CA deployed by different operators, asynchronized CA was introduced in Rel-16 WI of DCCA enhancement. The misalignment across serving cells is limited to ±76800Ts and explicit RRC signaling was introduced to indicate the slot offset.

As the SFN timing of different serving cells may not be same in asynchronized CA, it was necessary to clarify which cell’s SFN is used for DRX, CG and SPS. In RAN2#108, it was agreed that DRX calculation is based on the SFN of primary cell in a cell group with the justification that the DRX configuration is per cell group configuration [3]. Without this clarification, the UE and the network may have different understanding on the DRX ON/OFF period.

However, different from DRX, it was agreed that SPS and CG calculation is based on the SFN of the concerned serving cell in RAN2#109-e [4]. This is because using SFN of PCell may cause some ambiguity when the numerology of PCell and SCell are different [5], as illustrated in Figure. 2.

The related agreements were copied below for reference.

- RAN2#108

Under async CA, clarify that the UE uses SFN of primary cell (i.e. PCell or PSCell) within the same cell group for DRX on-duration determination, i.e. no change of rel-15 legacy UE behaviour.

- RAN2#109-e

Revert the RAN2#108 agreements on CG/SPS in Async CA as:
-    Under async CA, the UE uses SFN of concerned serving cell for the calculation of HARQ Process ID in SPS/CG (no change of rel-15 legacy UE behaviour).   
-     Under async CA, the UE uses SFN of concerned serving cell for the calculation of downlink/uplink assignment occurrences of SPS/CG (no change of rel-15 legacy UE behaviour).

Capture the clarifications on DRX, CG and SPS in Async CA in TS 38.321.

Figure.2 Illustration of ambiguity if SFN of PCell is used for CG and SPS

Observation 3: It was agreed to use different serving cell’s SFN for DRX and SPS/CG in asynchronized CA:

- UE C-DRX: based on the SFN of primary cell in a cell group.

- SPS and CG: based on the SFN of the concerned serving cell.

Please note that the above different SFN handling for DRX and SPS/CG doesn’t bring extra requirement for both NW and UE because SPS and CG are not impacted by UE C-DRX.

Observation 4: Different SFN handling for DRX and SPS/CG doesn’t bring extra requirement for both NW and UE because SPS and CG are not impacted by UE C-DRX.

## 2.2 Impacts of asynchronized CA to Cell DTX/DRX

First, please note that the SFN timing of Cell DTX/DRX had not been discussed in Rel-18 yet. Our observation is that most companies seemed to assume that timing of different serving cells were aligned in Cell DTX/DRX. That is why the below RAN2#123b agreement specified the following requirements of timing alignment.

Agreements (123 bis)

Cell DTX/DRX configuration is provided per Serving Cell with the following restrictions:

- 	 A maximum of two cell DTX/DRX patterns can be configured per MAC entity

- 	 The two configured patterns are aligned,

o	   The start and slot offset are common for the two patterns.

o	   one periodicity is an integer multiple of the other.

Meanwhile, please note that asynchronized CA is introduced to handle multi-operator case but Rel-18 NES never discussed multi-operator energy saving. Thus, it seems that asynchronized CA is not in scope of Rel-18 WI of NES.

Observation 5: The SFN timing of Cell DTX/DRX had not been discussed in Rel-18 yet. Furthermore, asynchronized CA is introduced to handle multi-operator case but Rel-18 NES never discussed multi-operator energy saving. Thus, it seems that asynchronized CA is not in scope of Rel-18 WI of NES.

Then, we would like to highlight below differences from pervious SFN timing discussion in asynchronized CA:

- Different from UE C-DRX, SPS and CG are impacted by Cell DTX/DRX.

      - If NW and UE are not aligned with SFN timing, the consequence is that maybe NW think one SPS occasion is within active duration of Cell DTX, but the UE regards that it is outside the active time and will not monitor the SPS.

      - To avoid misalignment between NW and UE, it is better that Cell DTX/DRX uses the same SFN timing as CG/SPS. Otherwise, it requires NW to take the timing difference into consideration when configuring CG/SPS.

- Cell DTX/DRX is specified to be always configured with UE C-DRX.

      - Please note that retransmission in Cell DTX/DRX relies on retransmission timer which is based on UE C-DRX timing. Thus, it is better that Cell DTX/DRX uses the same SFN timing as UE C-DRX. Otherwise, it requires NW to take the timing difference into consideration when configuring UE CDRX.

- If two Cell DTX/DRX patterns of one cell group are configured, they are specified to be aligned in starting time.

      - To ensure alignment, it is better that Cell DTX/DRX uses the same SFN timing (i.e., primary cell). Otherwise, it requires the UE to apply timing difference for each serving cell when it activates Cell DTX/DRX. Note that it can’t rely on NW to configure carrier timing offset of Cell DTX/DRX to the UE because Cell DTX/DRX is cell specific, but the timing offset is UE specific.

Observation 6: The SFN issue of Cell DTX/DRX is different from pervious SFN timing discussion of asynchronized CA in below aspects:

- Different from UE C-DRX, SPS and CG are impacted by Cell DTX/DRX.

- Cell DTX/DRX is specified to be always configured with UE C-DRX.

- If two Cell DTX/DRX patterns of one cell group are configured, they are specified to be aligned in starting time.

Based on above analysis, we think there are below 3 alternatives:

      - Alt-1: Cell DTX/DRX is not configured together with asynchronized CA in Rel-18.

      - Alt-2: Cell DTX/DRX uses the SFN of primary cell (i.e., same as UE CDRX), and NW takes the timing difference into consideration when configuring SPS/CG.

      - Alt-3: Cell DTX/DRX uses the SFN of the concerned cell (i.e., same as SPS/CG), and the UE is specified to apply timing difference for each serving cell when it activates Cell DTX/DRX.

We provide our comparison among these 3 alternatives in Table.1.

|  | Pros | Cons | Potential spec changes |
| --- | --- | --- | --- |
| Alt-1 | - Simplest NW and UE operation (i.e., the same timing can be used in Cell DTX/DRX, UE CDRX and CG/SPS). - No extra spec work. | Cell DTX/DRX + Asynchronized CA is not supported in Rel-18. | N/A (an agreement in Chair Note is sufficient) |
| Alt-2 | Aligned with SFN timing of UE CDRX. Issue 2) and 3) are avoided. | A new NW requirement needs to be introduced (i.e. NW takes the timing difference into consideration when configuring SPS/CG). | Capture a Note in TS 38.321, e.g. “NOTE x: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DTX/DRX duration. gNB takes the timing difference into consideration when configuring SPS/CG”. |
| Alt-3 | Aligned with SFN timing of SPS/CG. Issue 1) is avoided. | A new UE behavior needs to be introduced (i.e. apply timing difference for each serving cell when it activates Cell DTX/DRX). | - Introduce normative text in TS 38.321 to specify the UE to apply timing difference for each serving cell when it activates Cell DTX/DRX. - Capture a Note in TS 38.321, e.g. “NOTE x: In case of unaligned SFN across carriers in a cell group, the SFN of the concerned serving cell is used to calculate the Cell DTX/DRX duration.”. |

Table.1 Comparison among 3 alternatives on SFN timing of Cell DTX/DRX

Among them, our considerations can be found below:

- We don’t prefer Alt-3 because normative spec changes should be avoided at this stage. Meanwhile, we tend to think issue 2) is more serious and it may lead to some other corner case issues when there is timing difference between Cell DTX/DRX and UE C-DRX.

- Between Alt-1 and Alt-2, we slightly prefer Alt-1 because it is not clear whether asynchronized CA is in scope of Rel-18 NES and it seems that no clear requirement from MNO for this feature to work with Cell DTX/DRX. But Alt-2 is also acceptable to us if majority prefer.

Proposal 1: RAN2 down-select between two alternative way-forward on SFN timing issue of Cell DTX/DRX:

      - Alt-1: RAN2 confirm asynchronized CA is not in Rel-18 scope, and thereby Cell DTX/DRX is not configured together with asynchronized CA in Rel-18.

      - Alt-2: Cell DTX/DRX uses the SFN of primary cell (i.e., same as UE CDRX), and NW takes the timing difference into consideration when configuring SPS/CG whose SFN is from the concerned serving cell.

# 3 Conclusion

In this contribution, we discuss remaining issue SFN timing of Cell DTX/DRX. Our observations can be found below.

Observation 1: In LTE and NR Rel-15, only synchronized CA is supported. Therefore, although SFN is required for UE DRX, SPS and CG, TS 38.321 didn’t specify which serving cell’s SFN is used because the SFN of all serving cells are aligned (i.e. the UE is allowed to use SFN of any serving cell for DRX, SPS and CG).

Observation 2: To support inter-band CA deployed by different operators, asynchronized CA was introduced in Rel-16 WI of DCCA enhancement. The misalignment across serving cells is limited to ±76800Ts and explicit RRC signaling was introduced to indicate the slot offset.

Observation 3: It was agreed to use different serving cell’s SFN for DRX and SPS/CG in asynchronized CA:

- UE C-DRX: based on the SFN of primary cell in a cell group.

- SPS and CG: based on the SFN of the concerned serving cell.

Observation 4: Different SFN handling for DRX and SPS/CG doesn’t bring extra requirement for both NW and UE because SPS and CG are not impacted by UE C-DRX.

Observation 5: The SFN timing of Cell DTX/DRX had not been discussed in Rel-18 yet. Furthermore, asynchronized CA is introduced to handle multi-operator case but Rel-18 NES never discussed multi-operator energy saving. Thus, supporting asynchronized CA is not in scope of Rel-18 WI of NES.

Observation 6: The SFN issue of Cell DTX/DRX is different from pervious SFN timing discussion of asynchronized CA in below aspects:

- Different from UE C-DRX, SPS and CG are impacted by Cell DTX/DRX.

- Cell DTX/DRX is specified to be always configured with UE C-DRX.

- If two Cell DTX/DRX patterns of one cell group are configured, they are specified to be aligned in starting time.

Based on the observation, the proposals are:

Proposal 1: RAN2 down-select between two alternative way-forward on SFN timing issue of Cell DTX/DRX:

      - Alt-1: RAN2 confirm asynchronized CA is not in Rel-18 scope, and thereby Cell DTX/DRX is not configured together with asynchronized CA in Rel-18.

      - Alt-2: Cell DTX/DRX uses the SFN of primary cell (i.e., same as UE CDRX), and NW takes the timing difference into consideration when configuring SPS/CG whose SFN is from the concerned serving cell.

# 4 References

[1] RAN2#126, Chair Notes

[2] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

[3] RAN2#108, Chair Notes

[4] RAN2#109-e, Chair Notes

[5] R2-2002230	, Summary report: [AT109e][049][DCCA] - Remaining issues of Async CA, Qualcomm.