# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406669
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.5.2

Source:	Apple

Title:	Discussion on RAN2 work of on-demand SSB for SCell

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The Rel-19 WID of Network Energy Savings (WID RP-240170) was agreed in RAN#102 [1], the WI objective on on-demand SSB (OD-SSB) SCell operation is copied below:

OD-SSB had not been discussed in RAN2 yet due to time limitation. However, please note that RAN1#117 made below agreements on signaling to indicate OD-SSB transmission and measurement [2], where agreements with RAN2 impacts are highlighted:

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Based on these agreements, I think RAN2 should at least start discussion on below two topics:

- MAC-CE format design for trigger signaling

- L3 measurement based on OD-SSB

In this contribution, we share our view on its RAN2 work based on latest RAN1 progress.

# 2 Discussion

## 2.1 Use cases of on-demand SSB

RAN1#116 [3] made below agreement:

Agreement (RAN1#116) For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission

RAN1#116b [4] further updated above agreement, but the above 4 scenarios are still on the table:

Agreement (RAN1#116b) For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

RAN1#117 didn’t further make agreements on use cases of OD-SSB [2]. Based on these RAN1 agreements, we understand below 3 use cases are still on the table for further down-selection, which are illustrated in Figure. 1. And their clarification can be found below:

- Scenario 2: For triggering L3 measurement before SCell is activated between T0 and T1.

  - On-demand SSB is intended for the NW to determine which SCell(s) to activate.

    - The NW triggers the UE to perform L3 measurements based on on-demand SSB towards SCell(s) which are configured but not activated.

- Scenario 2A/2B: For SCell activation/deactivation between T1 and T2.

  - On-demand SSB is intended to allow the UE to perform AGC and L1/L3 measurement for SCell activation.

    - As specified in TS 38.133 [5], T2 (i.e. when SCell is activated) is the time that the UE performs AGC, L1/L3 measurements and successfully report valid CSI to gNB.

- Scenario 3B: For SSB-less operation after SCell is activated after T2.

  - On-demand SSB is intended to allow the UE to perform L3 measurements and time / frequency tracking to maintain intra-band or inter-band SSB-less CA operation if OD-SSB transmission is terminated in T3.

    - Please note that both intra-band SSB-less CA and inter-band SSB-less CA have restricted scenarios. Thus, this use case is intended to extend scenarios for SSB-less CA.

      - Intra-band SSB-less CA: only support PCell and SCell co-located case.

      - Inter-band SSB-less CA: only support PCell and SCell co-located in FR1 case.

Figure.1 Illustration of 3 use cases of on-demand SSB on the table

Observation 1: According to RAN1#116 agreement, below 3 use cases of on-demand SSB were identified for further down-selection:

- Scenario 2: For triggering L3 measurement before SCell is activated between T0 and T1.

- Scenario 2A/3A: For SCell activation/deactivation between T1 and T2.

- Scenario 3B: For SSB-less operation after SCell is activated and OD-SSB transmission is terminated (i.e., L3 measurement and time/frequency tracking to maintain SSB-less CA).

RAN1 had agreed to support Scenario 2 and Scenario 2A, but whether to support Scenario 3A/3B is still FFS. According to above analysis, we think whether to support scenario 3A/3B is in RAN1 / RAN4 expertise. One reasonable way is that RAN1 make conclusion with RAN4 input, but RAN2 don’t need to involve in this discussion. Thus, we propose:

Proposal 1: RAN2 wait RAN1 to conclude whether Scenario 3A/3B of on-demand SSB is supported. Meanwhile, RAN2 start discussion on specification impacts of on-demand SSB based on Scenario 2 and 2A.

## 2.2 MAC-CE design to trigger on-demand SSB transmission

RAN1#117 agreed to support MAC-CE based signaling to trigger on-demand SSB transmission [2]. The related agreements are copied below:

Agreement (RAN2#117) - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement (RAN2#117) - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

Since RRC signaling still has some FFSs in above agreement, we think RAN2 can wait further RAN1 input before implementing RRC signaling. Meanwhile, we think these agreements are sufficient for RAN2 to start discussion on MAC-CE design.

First, according to the highlighted information agreed to be included in RRC, we think that the configurations related to OD-SSB transmission pattern doesn’t need to be included in MAC-CE, i.e., it is sufficient to only include 1bit indication of activation / deactivation of OD-SSB transmission on the corresponding SCell in MAC-CE.

Proposal 2: Following RAN1 agreement, configuration on OD-SSB transmission pattern (e.g. frequency, SSB position and transmission periodicity) doesn’t need to be included in MAC-CE, i.e., it is sufficient to only include 1bit indication of activation / deactivation of OD-SSB transmission on the corresponding SCell in MAC-CE.

Next, we discuss whether new MAC-CE needs to be introduced for OD-SSB activation / deactivation. As indicated in our Proposal 1, RAN2 need to consider both Scenario 2 and Scenario 2A:

- Scenario 2: For triggering L3 measurement before SCell is activated between T0 and T1.

- Scenario 2A: For SCell activation/deactivation in T1.

It is obviously that a new MAC-CE needs to be introduced at least to support Scenario 2 because OD-SSB is transmitted before the SCell activation command and thereby the legacy SCell activation/deactivation MAC CE can’t be reused.

Observation 2: A new MAC-CE needs to be introduced at least to support Scenario 2 because OD-SSB is transmitted before SCell activation command and thereby the legacy SCell activation/deactivation MAC CE can’t be reused.

Then, we think the controversial issue is how to support Scenario 2A with this new MAC-CE. Basically, we identify the following 2 alternatives of design choices, whose design example with up to 7 SCells is illustrated in Figure. 2:

Figure.2 Design example of new OD-SSB MAC-CE with up to 7 SCells

- Alt-1: The new MAC-CE is fixed size only to indicate OD-SSB transmission, and NW send both new MAC-CE and legacy SCell activation MAC-CE for Scenario 2A.

  - The Bi field is set to 1 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be activated. The Bi field is set to 0 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be deactivated.

- Alt-2: The new MAC-CE is variable size to indicate either only OD-SSB transmission (scenario 2), or joint OD-SSB transmission and SCell activation / deactivation (scenario 2A).

  - The first Oct (i.e. Oct1) is mandatory present and the meaning of Bi field is same as Alt-1.

  - The second Oct (i.e. Oct2) is optional present. The Ci field is set to 1 to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to 0 to indicate that the SCell with SCellIndex i shall be deactivated (i.e. same as legacy SCell activation / deactivation MAC-CE).

We think both alternatives can work, and a comparison between them is provided in Table. 1.

|  | Pros | Cons | Extra specification works |
| --- | --- | --- | --- |
| Alt-1 | Lower payload size | It may bring UE behavior ambiguity in Scenario 2A if NW sends legacy SCell MAC-CE and new MAC-CE in different MAC PDUs. For example: - UE behavior when legacy SCell A/D MAC-CE is lost or missed. - UE behavior when legacy SCell A/D MAC-CE is received too early / late, compared with the reception time of new MAC-CE. | New RAN4 requirement on SCell activation latency *Note: although new MAC-CE and legacy SCell A/D MAC-CE can be multiplexed in the same MAC PDU, RAN4 normally specify requirement for worst case (i.e. two MAC-CE are sent in different MAC PDU). |
| Alt-2 | UE behavior in Scenario 2A is clearer because NW always provides SCell activation / deactivation and OD-SSB indications in the same MAC CE. | Higher payload size | Specify UE behavior if it receives both new MAC-CE and legacy SCell A/D MAC-CE. |

Table.1 Comparison between two alternatives of new MAC-CE design

We think both alternatives can work, and don’t have strong preference between them. Thus, we propose RAN2 to down select one:

Proposal 3: RAN2 down select between the following two alternatives of new OD-SSB MAC-CE design:

- Alt-1: The new MAC-CE is fixed size only to indicate OD-SSB transmission, and NW send both new MAC-CE and legacy SCell activation MAC-CE for Scenario 2A.

  - The Bi field is set to 1 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be activated. The Bi field is set to 0 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be deactivated.

- Alt-2: The new MAC-CE is variable size to indicate either only OD-SSB transmission (scenario 2), or joint OD-SSB transmission and SCell activation / deactivation (scenario 2A).

  - The first Oct (i.e. Oct1) is mandatory present and the meaning of Bi field is same as Alt-1.

  - The second Oct (i.e. Oct2) is optional present. The Ci field is set to 1 to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to 0 to indicate that the SCell with SCellIndex i shall be deactivated (i.e. same as legacy SCell activation / deactivation MAC-CE).

As analyzed in Table.1, we think RAN4 needs to specify a new requirement of SCell activation latency if Alt-1 is adopted. Thus, RAN2 need to inform RAN4 on the issues.

Proposal 4: If Alt-1 of new OD-SSB MAC-CE design is adopted, RAN2 send LS to inform RAN4 on the ambiguity of UE behavior in Scenario 2A when new MAC-CE and legacy SCell A/D MAC-CE are sent in different MAC PDUs (e.g., if legacy SCell A/D MAC-CE is missed or received too early / late).

If Alt-2 is adopted, we think RAN2 need to specify the UE behavior if both new MAC-CE and legacy SCell A/D MAC-CE are received. Please note that similar UE behavior was already specified in Rel-17 enhanced SCell A/D MAC-CE with TRS and Rel-18 enhanced SP CSI reporting A/D MAC CE. The related specifications in TS 38.321 [6] can be found below:

#### 6.1.3.10	SCell Activation/Deactivation MAC CEs

…omit unrelated part...

NOTE:	If UE receives the SCell Activation/Deactivation MAC CE for an SCell configured with TRS for fast activation of the SCell, such TRS is not used for the corresponding SCell.

#### 6.1.3.16	SP CSI reporting on PUCCH Activation/Deactivation MAC CE

…omit unrelated part...

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within ltm-CSI-ReportConfigToAddModList if L field is set to 1, or csi-ReportConfigToAddModList if L field is set to 0, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated. If the Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigToAddModList, the Si field is set to 0 to additionally indicate that all sub-configurations within csi-ReportSubConfigToAddModList shall be deactivated;

-	R: Reserved bit, set to 0.

Observation 3: If Alt-2 is adopted, RAN2 need to specify the UE behavior if both new MAC-CE and legacy SCell A/D MAC-CE are received. Similar UE behavior was already specified in Rel-17 enhanced SCell A/D MAC-CE with TRS and Rel-18 enhanced SP CSI reporting A/D MAC CE.

Similar to UE behavior specified in Rel-17 and Rel-18 discussion, we identify below two options:

- Option 1: For a UE supports OD-SSB operation and has been configured a SCell with OD-SSB, if it receives legacy SCell activation / deactivation MAC-CE, it only activates / deactivates SCell correspondingly and keeps the status of OD-SSB transmission in the SCell.

  - For example, if OD-SSB is not activated before reception of SCell activation MAC-CE, the UE regards it is still not activated after reception of the legacy MAC-CE.

- Option 2: For a UE supports OD-SSB operation and has been configured a SCell with OD-SSB, if it receives legacy SCell activation / deactivation MAC-CE, it activates / deactivates SCell correspondingly and deactivates OD-SSB transmission in these SCells.

We prefer Option 1 because we understand that SCell activation / deactivation can be decoupled with OD-SSB transmission status in RAN1 discussion, which is aligned with Option 1. On the contrary, Option 2 implies that SCell A/D is coupled with OD-SSB transmission status, which is not flexible. Thus, we propose:

Proposal 5: If Alt-2 of new OD-SSB MAC-CE design is adopted, RAN2 specify the UE behavior if it has been configured a SCell with OD-SSB but receives legacy SCell activation / deactivation MAC-CE, the UE only activates / deactivates SCell correspondingly and keeps the status of OD-SSB transmission in the SCell.

Finally, please note that legacy SCell A/D MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells, according to TS 38.321 [6].

Observation 4: Legacy SCell A/D MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells, according to TS 38.321.

Similarly, the new OD-SSB MAC-CE should also support the two formats, irrespective of which alternative being adopted. An example of long format indicating up to 31 SCells are illustrated in Figure.3.

Proposal 6: Irrespective of which design alternative being adopted, the new OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Figure.3 Design example of new OD-SSB MAC-CE with up to 31 SCells

## 2.3 UL signalling to trigger on-demand SSB transmission

Agreement (RAN2#116)

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement (RAN2#116b)

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

RAN1#116 [3] and RAN1#116b [4] made below related agreement:

Thus, RAN1 didn’t conclude whether to support on-demand SSB SCell operation triggered by UE. We think whether to support it depends on the use case of on-demand SSB. For example, if Scenario 3B (i.e., for SSB-less operation after on-demand SSB is terminated) is finally precluded, we think it is unnecessary to introduce UL signaling as legacy SCell activation / deactivation procedure. In this case, it is left to the NW implementation to determine when to activate / deactivate one SCell.

Observation 5: Whether to support on-demand SSB triggered by UE depends on the use case of on-demand SSB. Specifically, if Scenario 3B (i.e., for SSB-less operation after on-demand SSB is terminated) is precluded, it is unnecessary to introduce UL signaling because trigger timing can be left to NW implementation as legacy.

Due to such dependency, RAN2 can wait RAN1 on whether to introduce on-demand SSB SCell operation triggered by UE and related signaling.

Proposal 7: RAN2 wait RAN1 on whether to introduce on-demand SSB SCell operation triggered by UE and related signaling.

## 2.4 L3 measurement based on on-demand SSB

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

In RAN1#116b [4], it was agreed to support L1 and/or L3 measurement based on on-demand SSB.

And in RAN1#117 [2], it was further agreed some details of L1 measurement based on on-demand SSB.

Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Please note that RAN1 also discussed L3 measurement in RAN1#117 [2], but majority preferred to leave it to RAN2. Thus, only agreement on L1 measurement was made.

As usual, L3 measurement is RAN2 expertise and RAN2 should lead the discussion. In details, L3 measurement generally includes RRM and RLM. Our view on their RAN2 impacts can be found in the following sub-sections.

### 2.4.1 RRM based on on-demand SSB

In legacy RRM based on SSB, the NW need to configure UEs SMTC and SSB_ToMeasure in MeasObjectiveNR, as highlighted below:

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

omit unrelated part

}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

omit unrelated part

}

And according to Section 5.5.2.10 of TS 38.331 [7], the UE shall not consider SSB outside the SMTC occasion for RRM measurements:

#### 5.5.2.10	Reference signal measurement timing configuration

…omit unrelated part...

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], clause 9.3.8).

It means that the UE only performs L3 measurements based on SSB within the SMTC window. Then, as on-demand SSB may use a periodicity different from always-on SSB and/or termination of its transmission, which may lead to change of SMTC (e.g. due to change of periodicity) and SSB_ToMeasure (e.g. due to termination of transmission). Meanwhile, because RAN1 introduced MAC-CE to indicate UE on-demand SSB transmission (and DCI is FFS), the legacy RRC reconfiguration of SMTC and/or SSB_ToMeasure may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing RRM.

Observation 6: In legacy RRM, SMTC and SSB_ToMeasure need to be configured in MeasObjectiveNR. And the UE only preforms L3 measurements based on SSB within the SMTC window. Thus, legacy RRC reconfiguration may not adapt to the new OD-SSB MAC-CE which may lead to failure of L3 measurement based on OD-SSB.

An example is illustrated in Figure.4, where the UE is configured with one SMTC whose window covers the always-on SSB (yellow in Figure.4). However, when OD-SSB is triggered after T1, the configured SMTC window can’t cover OD-SSB (green in Figure.4). Thus, although the UE may know when OD-SSB starts transmission via activation MAC-CE, it can’t perform L3 measurements towards OD-SSB because OD-SSB is not covered within its configured SMTC. Since OD-SSB may use different periodicity from always-on SSB and SMTC is periodical window configured per frequency (rather than per cell), we think the issue is not a corner case. Furthermore, we don’t think NW implementation can well address it without causing negative impacts to legacy UE. For example, NW may configure a SMTC window with short periodicity to cover both always-on SSB and OD-SSB, but it will lead to UE extra power consumption, especially for the legacy UEs. To resolve the issue, one possible solution is to introduce another SMTC which can be used when OD-SSB is triggered via MAC-CE.

Observation 7: If NW configures a SMTC window with short periodicity to cover both always-on SSB and OD-SSB, it will lead to UE extra power consumption, especially for the legacy UEs.

Figure.4 Illustration of issue scenario: one single SMTC can only cover always-on SSB but can’t cover OD-SSB. So, UE can’t perform L3 measurements towards OD-SSB, although it knows when OD-SSB is triggered.

Thus, we propose RAN2 to study how to address this issue after RAN1 conclude the transmission pattern and signaling of on-demand SSB.

Proposal 8: RAN2 study how the UE adapts L3 RRM configuration (e.g. SMTC and SSB_ToMeasure) to perform L3 measurement based on OD-SSB which is indicated by the new OD-SSB MAC-CE.

### 2.4.2 RLM based on on-demand SSB

As RLM is only performed in PCell and/or PSCell, we don’t think any spec change is needed for on-demand SSB.

Proposal 9: As RLM is only performed in PCell and/or PSCell, no spec change is foreseen to support RLM based on on-demand SSB.

# 3 Conclusion

In this contribution, we share our view on RAN2 work of on-demand SSB for SCell. Our observations are:

Observation 1: According to RAN1#116 agreement, below 3 use cases of on-demand SSB were identified for further down-selection:

- Scenario 2: For triggering L3 measurement before SCell is activated between T0 and T1.

- Scenario 2A/3A: For SCell activation/deactivation between T1 and T2.

- Scenario 3B: For SSB-less operation after SCell is activated and OD-SSB transmission is terminated (i.e., L3 measurement and time/frequency tracking to maintain SSB-less CA).

Observation 2: A new MAC-CE needs to be introduced at least to support Scenario 2 because OD-SSB is transmitted before SCell activation command and thereby the legacy SCell activation/deactivation MAC CE can’t be reused.

Observation 3: If Alt-2 is adopted, RAN2 need to specify the UE behavior if both new MAC-CE and legacy SCell A/D MAC-CE are received. Similar UE behavior was already specified in Rel-17 enhanced SCell A/D MAC-CE with TRS and Rel-18 enhanced SP CSI reporting A/D MAC CE.

Observation 4: Legacy SCell A/D MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells, according to TS 38.321.

Observation 5: Whether to support on-demand SSB triggered by UE depends on the use case of on-demand SSB. Specifically, if Scenario 3B (i.e., for SSB-less operation after on-demand SSB is terminated) is precluded, it is unnecessary to introduce UL signaling because trigger timing can be left to NW implementation as legacy.

Observation 6: In legacy RRM, SMTC and SSB_ToMeasure need to be configured in MeasObjectiveNR. And the UE only preforms L3 measurements based on SSB within the SMTC window. Thus, legacy RRC reconfiguration may not adapt to the new OD-SSB MAC-CE which may lead to failure of L3 measurement based on OD-SSB.

Observation 7: If NW configures a SMTC window with short periodicity to cover both always-on SSB and OD-SSB, it will lead to UE extra power consumption, especially for the legacy UEs.

Based on above observations, our proposals are:

Use cases of on-demand SSB

Proposal 1: RAN2 wait RAN1 to conclude whether Scenario 3A/3B of on-demand SSB is supported. Meanwhile, RAN2 start discussion on specification impacts of on-demand SSB based on Scenario 2 and 2A.

MAC-CE format design

Proposal 2: Following RAN1 agreement, configuration on OD-SSB transmission pattern (e.g. frequency, SSB position and transmission periodicity) doesn’t need to be included in MAC-CE, i.e., it is sufficient to only include 1bit indication of activation / deactivation of OD-SSB transmission on the corresponding SCell in MAC-CE.

Proposal 3: RAN2 down select between the following two alternatives of new OD-SSB MAC-CE design:

- Alt-1: The new MAC-CE is fixed size only to indicate OD-SSB transmission, and NW send both new MAC-CE and legacy SCell activation MAC-CE for Scenario 2A.

  - The Bi field is set to 1 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be activated. The Bi field is set to 0 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be deactivated.

- Alt-2: The new MAC-CE is variable size to indicate either only OD-SSB transmission (scenario 2), or joint OD-SSB transmission and SCell activation / deactivation (scenario 2A).

  - The first Oct (i.e. Oct1) is mandatory present and the meaning of Bi field is same as Alt-1.

  - The second Oct (i.e. Oct2) is optional present. The Ci field is set to 1 to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to 0 to indicate that the SCell with SCellIndex i shall be deactivated (i.e. same as legacy SCell activation / deactivation MAC-CE).

Proposal 4: If Alt-1 of new OD-SSB MAC-CE design is adopted, RAN2 send LS to inform RAN4 on the ambiguity of UE behavior in Scenario 2A when new MAC-CE and legacy SCell A/D MAC-CE are sent in different MAC PDUs (e.g., if legacy SCell A/D MAC-CE is missed or received too early / late).

Proposal 5: If Alt-2 of new OD-SSB MAC-CE design is adopted, RAN2 specify the UE behavior if it has been configured a SCell with OD-SSB but receives legacy SCell activation / deactivation MAC-CE, the UE only activates / deactivates SCell correspondingly and keeps the status of OD-SSB transmission in the SCell.

Proposal 6: Irrespective of which design alternative being adopted, the new OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

UL signaling

Proposal 7: RAN2 wait RAN1 on whether to introduce on-demand SSB SCell operation triggered by UE and related signaling.

Potential RAN2 work on L3 measurement of on-demand SSB

Proposal 8: RAN2 study how the UE adapts L3 RRM configuration (e.g. SMTC and SSB_ToMeasure) to perform L3 measurement based on OD-SSB which is indicated by the new OD-SSB MAC-CE.

Proposal 9: As RLM is only performed in PCell and/or PSCell, no spec change is foreseen to support RLM based on on-demand SSB.

# 4 References

[1] RP-240170, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN1#117, Chair Notes.

[3] RAN1#116, Chair Notes.

[4] RAN1#116b, Chair Notes.

[5] TS 38.133-v18.4.0, Requirements for support of radio resource management, 2024-1.

[6] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

[7] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.