# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406670
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.5.3

Source:	Apple, BT

Title:	Finalize study of on-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3]

  - Triggering method by uplink wake-up-signal using an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

    - Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Checkpoint for normative work in RAN#105

The Rel-19 WID of Network Energy Savings (WID RP-240170) was agreed in RAN#102 [1], the study objective on on-demand SIB1 (OD-SIB1) for UEs in IDLE/INACTIVE state is copied below:

OD-SIB1 was discussed in RAN2#125b [2] and RAN2#126 [3] with a bunch of RAN2 agreements. We think that this study is already in good shape, and thereby RAN2#127 only need to address the following postponed issues which may impact conclusion of this study before Sept. RAN plenary.

- Whether support case 3

- OD-SIB1 validity mechanism

- Whether need WUS configuration acquisition from NES cell

- Impacts to RRC_CONNECTED UEs

- Other issues

In this contribution, we share our view on above essential issues from RAN2 perspective. And based on our analysis, we further discuss conclusion of OD-SIB1 from RAN2 perspective.

# 2 Discussion

## 2.1 Whether support case 3

This is one issue discussed in RAN2#126 [3], but postponed due to divergence of company view:

Proposal 1 (modified): RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

- Postponed.

[Fraunhofer]: Ok with the proposal. In addition, we need to consider how to reduce the signalling overhead. [Nokia, Ericsson]: Case 3 requires quite close coordination between two cells in order to guarantee the latest SIB1. [Ericsson]: Once the UE gets SIB1 from cell A and camps on NES cell, if the UE requires OD-SIB1 again for the camped NES cell, will the UE back to cell A and request it again? [Samsung]: Once camped on the NES cell, the SIB1 will be maintained by SIB change notification, so the scenario Ericsson mentioned will not happen.

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

The issue of frequent inter-node signaling exchange was discussed in RAN2#126 [3]. We think it can be left to RAN3 for further evaluation. From RAN2 perspective, we think that Case 3) has below two issues:

Issue 1: it may lead to significant specification changes on NR SIB

According to TS 38.331 [4], SIB1 or a SI message is limited to 2976 bit.

NOTE 2:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

As we know, the payload size of SIB1 is increasing very quickly in each release. Meanwhile, it was agreed that one cell A may cover multiple NES cells in RAN2#126 [3]. Thus, it is questioned how multiple NES cells’ SIB1s can be included in one single new SIBx of Cell A. In our understanding, it may require the following two specification changes:

- Allow variable number of new SIBs to include multiple NES cells’ SIB1.

- Introduce a mapping configuration between NES cell and index of new SIB(s) of Cell A

One example to implement above specification changes can be illustrated in Figure.1.

Figure.1. Design example to support Case 3) in Cell A’s SIB (assuming Cell A covers N NES cells)

We tend to think that the above two specification changes are significant change of NR SIB design. Especially, spec change 1) is not aligned with current NR SIB structure where each SIB type has its usage clearly specified in TS 38.331 [4].

Observation 1: The payload size of SIB1 is increasing very quickly in each release. Meanwhile, it was agreed that one cell A may cover multiple NES cells. Thus, it is questioned how multiple NES cells’ SIB1s can be included in one single new SIBx of Cell A, whose maximum size is limited to 2976 bit.

Observation 2: If we introduce variable number of new SIBs to include multiple NES cells’ SIB1, it is not aligned with current NR SIB structure where each SIB type has its usage clearly specified in TS 38.331.

Issue 2: it is not clear how Case 3) can co-exist with Case 2)

Both Case 2) and Case 3) rely on SIB of Cell A to provide NES’s access information. Then, it is not clear whether/how Case 3) co-exist with Case 2). It will be weird if two new SIBx are introduced to cover UL WUS configuration of NES cell in Case 2) and full SIB1 of NES cell in Case 3), respectively.

Observation 3: It is not clear whether/how Case 3) co-exist with Case 2). It will be weird if different SIBx are introduced to cover UL WUS configuration of NES cell in Case 2) and full SIB1 of NES cell in Case 3), respectively.

Due to above two issues, if we want to support Case 3) on top of Case 2), we think maybe a variant of Case 3) can be considered by including multiple NES cells’ SIB1 in RRC release message.

Proposal 1: RAN2 consider a variant of Case 3), i.e., Cell A provides NES cells’ full SIB1 in its RRC release message.

## 2.2 OD-SIB1 validity mechanism

This is also one issue discussed in RAN2#126 [3], but postponed due to divergence of company view:

Proposal 8: SIB1 validity is supported, i.e. if a UE has a valid version of SIB1 stored, UE does not initiate the on-demand SIB1.

- Postponed.

[Samsung]: Question is how it really works. If it operates based on timer only, the NW may not be able to update the SIB1 (e.g. NW does not know when a UE acquires SIB1, so the NW does not know when 3 hours will end). If timer + value tag, where is the value tag sent? [Huawei]: Intention is to agree with the principle and to discuss details later. [Intel]: Ok with the proposal, but we need further discussion on many FFS. However, we don’t want to include value tag of SIB1 into MIB. [Samsung]: Understands the acquired SIB1 is valid until the reception of SIB change notification. Don’t understand why it is not enough and we attempt to change the UE behaviour.

According to TS 38.331 [4], legacy SIB validity mechanism includes the following 3 approaches:

- 5bit valueTag for other SIBs except SIB1 are included in SIB1 (i.e., SIB1 SI-SchedulingInfo SchedulingInfoSIB-MappingSIB-TypeInfovalueTag).

- Paging short message for system information modification, as illustrated in below table from TS 38.212 [5].

- The acquired SIB is regarded as invalid if stored for more than 3 hours, according to TS 38.331.

Then, on validity mechanism of OD-SIB1, at least above approach 2) and 3) can be reused.

Observation 4: On validity mechanism of OD-SIB1, at least legacy approach via paging short message for system information modification and time-based solution (FFS whether still 3 hour) can be reused.

Above approach 1) can’t be directly reused because SIB1 is on-demand transmitted in NES cell. One possible solution is to include valueTag of OD-SIB 1 in MIB of NES cell. One may argue that there is no spare bit in MIB to include valueTag of SIB1. However, please note that legacy MIB has 8bit field pdcch-ConfigSIB1 which indicates CORESET#0 and SearchSpare#0 for UE to identify SIB1 transmission. When SIB1 is not broadcasted in NES cell, this 8bit IE is useless, and thereby can be reused to include valueTag of SIB1. We think the details of this solution can be discussed in normative phase.

Observation 5: Legacy MIB has 8bit field pdcch-ConfigSIB1 which indicates CORESET#0 and SearchSpare#0 for UE to identify SIB1 transmission. When SIB1 is not broadcasted in NES cell, this 8bit IE is useless, and thereby can be reused to include valueTag of SIB1.

Based on above analysis, we think that SIB1 validity mechanism is supported, i.e., the UE can know when the acquired OD-SIB1 is invalid. Its details can be discussed in normative phase (e.g. whether to include valueTag of OD-SIB 1 in MIB of NES cell).

Proposal 2: SIB1 validity mechanism is supported, i.e., the UE can know when the acquired OD-SIB1 is invalid. Its details can be discussed in normative phase.

## Whether need WUS configuration acquisition from NES cell

This is also one issue discussed in RAN2#126 [3], but postponed:

Proposal 9b (modified): After the UE camps on NES cell, UE can acquire WUS configuration from the NES cell in a new SIB via OD-SIB peocedure.

- Postponed.

[Session chair]: Understand the scenario is the UE reselected from cell A to NES cell. While UE stays in the NES cell, the UE may need to re-acquire SIB1. Then the question is how the UE gets WUS configuration? E.g. performs cell reselection to cell A and reacquires WUS configuration or reacquire from NES cell itself? [Rakuten]: If NW tries to transmit SIB1 always to guarantee 3 hours invalidity, how NW can have energy saving? [Nokia]: Understand NES cell does not transmit SIB1, then how its SIB1 can be transmitted to cover this concern?

As agreed in RAN2#125b [2], UL WUS configuration includes at least RACH configuration which is used for the UE to request OD-SIB1.

Agreements on on-demand SIB1:

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least RACH configuration.

Please note that all existing RACH configurations for IDLE / INACTIVE UEs are included in SIB1, as illustrated in Figure.2.

Figure.2. Illustration of existing RACH configurations for IDLE / INACTIVE UEs

Then, we think it is straight forward that NES cell’s own UL WUS configuration (at least RACH resource) is also included in its SIB1. Correspondingly, after the UE acquires OD-SIB1, it can obtain the latest / updated RACH configuration for UL WUS. With this approach, it is unnecessary to include UL WUS configuration of other NES cells in NES cell’s new SIB.

Proposal 3: NES cell doesn’t need to include UL WUS configuration of other NES cells in its new SIB. Instead, it includes its own UL WUS configuration in its SIB1 as one RACH partition, and the UE can obtain the updated / latest RACH configuration of UL WUS after it acquires OD-SIB1.

## Impacts to RRC_CONNECTED UEs

During offline discussion, there were some concerns about whether OD-SIB1 will impact RRC_CONNECTED UEs, including legacy UEs in RRC_CONNECTED state. As it impacts legacy UEs, we tend to think it is an essential issue to address.

Our understanding is that OD-SIB1 will not impact both legacy and Rel-19 RRC_CONNECTED UEs. We can discuss CONNECTED operations one by one:

L3 Measurements

According to section 5.5.1 of TS 38.331 [4], if the UE is configured, the UE shall perform L3 measurements towards serving and neighbour cells, irrespective of whether SIB1 being broadcasted in these cells. Thus, even legacy RRC_CONNECTED UEs performs L3 measurements towards NES cells with OD-SIB1.

From Section 5.5.1 of TS 38.331: The measurement procedures distinguish the following types of cells: 1. The NR serving cell(s) – these are the SpCell and one or more SCells. 2. Listed cells – these are cells listed within the measurement object(s). 3. Detected cells – these are cells that are not listed within the measurement object(s) but are detected by the UE on the SSB frequency(ies) and subcarrier spacing(s) indicated by the measurement object(s). For NR measurement object(s), the UE measures and reports on the serving cell(s)/serving Relay UE (for L2 U2N Remote UE), listed cells and/or detected cells. For inter-RAT measurements object(s) of E-UTRA, the UE measures and reports on listed cells and detected cells and, for RSSI and channel occupancy measurements, the UE measures and reports on the configured resources on the indicated frequency. For inter-RAT measurements object(s) of UTRA-FDD, the UE measures and reports on listed cells. For CLI measurement object(s), the UE measures and reports on configured measurement resources (i.e. SRS resources and/or CLI-RSSI resources). For L2 U2N Relay object(s), the UE measures and reports on the serving NR cell(s), as well as the discovered L2 U2N Relay UEs.

CA/DC

According to section 5.2.1 of TS 38.331 [4], there is no requirement for SCell(s) in CA/DC to broadcast SIB1. Thus, even legacy RRC_CONNECTED UEs can be configured with NES cells with OD-SIB1 as its PSCell/SCells in CA/DC.

From Section 5.5.1 of TS 38.331: - For PSCell and SCells, the network provides the required SI by dedicated signalling, i.e. within an RRCReconfiguration message. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, the network releases and adds the concerned SCell. For PSCell, the required SI can only be changed with Reconfiguration with Sync.

Handover

We also think the legacy UEs in RRC_CONNECTED state can be handed over to a NES cell with OD-SIB1. According to TS 38.331 [4], the SIB1 of target cell can be included in HO command (i.e. RRCReconfigurationWithSync), and the DL sync operation during handover only relies on SSB/MIB for target cell. Thus, OD-SIB1 operation doesn’t impact handover.

Based on above analysis, we can conclude that OD-SIB1 doesn’t impact UEs in RRC_CONNECTED state.

Proposal 4: OD-SIB1 doesn’t impact both legacy and Rel-19 RRC_CONNECTED UEs (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

## Other issues

We noticed the following 2 issues were also discussed in RAN2#126 [3]:

Issue 5: How to avoid the legacy UE camping at the cell A attempting measurement/cell reselection to the NES cell?

Proposal 5 (modified): How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

- Agreed.

[OPPO]: Clarified that intention is to set different cell reselection priority to legacy UE and R19 NES UE. [Xiaomi]: First we need to understand whether NES cell dedicated carrier is realistic scenario or not. [Huawei]: Agree with Xiaomi. Understand NES cell is per cell turn-on and off. [OPPO]: There is no restriction for NES cell dedicated carrier. [Apple]: Even with mixed carrier case, assume the number of macro cell is small on that carrier, so still to deprioritize the carrier makes a sense. [Samsung]: For the NES cell dedicated carrier, why not the existing UE dedicated cell reselection priority work? For the mixed carrier case, we can utilize the black cell list for the legacy UE. [Vivo]: Similar discussion happened for the dedicated cell reselection priority in R18, it was not pursued. [Rakuten]: If NES cell turns on, question is whether we consider it normal cell for legacy UE or not. [Session chair]: We’ve just agreed NW need to bar the legacy UE. [OPPO]: Regardless of barring, the intention is to deprioritize the NES carrier, which doesn’t mean the legacy UE will never attempt that carrier. [Nokia]: Clarified intention is not aligned with P5 itself. Think P5 itself is ok to companies.

We think this issue has already been concluded in study item phase (i.e. some barring / prioritization mechanism can be introduced). The detailed barring / prioritization solution should be discussed in normative phase.

Issue 6: Detailed trigger conditions of OD-SIB1 and specification changes on cell reselection

Proposal 9: In addition to the already agreed conditions (i.e. UE doesn’t have a valid SIB1, the cell doesn’t broadcast SIB1 and supports on-demand SIB1) introduce a new RSRP triggering condition for the NES UE to be allowed to send WUS to trigger on-demand SIB1. FFS what is the triggering condition and whether the triggering condition is a new threshold.

[Samsung, Xiaomi, OPPO, NEC, Lenovo]: Think the existing cell reselection criterion is enough. [Nokia]: May be useful, but reluctant to agree with them now. We can discuss it in stage 3. [ZTE]: Support the proposal. It is good for NW is allowed to configure separate threshold than cell reselection criterion threshold. [LG]: Current cell reselection criterion is not absolute threshold, it’s relative to the serving cell’s radio condition. Support the proposal. [IDC]: If we have new threshold and the threshold is not met, anyway the existing cell reselection criterion should be used to trigger WUS transmission. [Vivo]: With the new threshold, it will make UE behaviour more complicated. We need to consider many combination cases, e.g. new threshold is {met, not met} + existing cell reselection criterion is {met, not met}. [Samsung]: Rather discussing whether new threshold is required or not, it’s better to discuss whether we’ll allow earlier or later WUS transmission triggering (than cell reselection criterion). The UE may achieve it w/o new threshold. [Apple]: Seems each company has different understanding on the use of the threshold. It’s good to see overall companies’ understanding on how the threshold is used or works.

We think the below agreements are sufficient to conclude study item. And the detailed trigger conditions of OD-SIB1 and specification changes on cell reselection should be discussed in normative phase.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

Thus, we propose:

Proposal 5: RAN2 discuss below issues in normative phase:

- Detailed solutions of how to avoid the legacy UE camping at the cell A attempting measurement/cell reselection to the NES cell.

- Detailed trigger conditions of OD-SIB1 and specification changes on cell reselection

## Study conclusion of OD-SIB1

Based on above discussion, we think that RAN2 has made a good progress of on-demand SIB1. Based on the bunch of agreements made in RAN2, we think on-demand SIB1 is feasible at least under Case 2) and it should transit into normative phase. Thus, we propose that RAN2 make below conclusion:

Proposal 6: RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1 (i.e., Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell).

# 3 Conclusion

In this contribution, we further discuss on-demand SIB1 for IDLE/INACTIVE UEs. Our observations are:

Observation 1: The payload size of SIB1 is increasing very quickly in each release. Meanwhile, it was agreed that one cell A may cover multiple NES cells. Thus, it is questioned how multiple NES cells’ SIB1s can be included in one single new SIBx of Cell A, whose maximum size is limited to 2976 bit.

Observation 2: If we introduce variable number of new SIBs to include multiple NES cells’ SIB1, it is not aligned with current NR SIB structure where each SIB type has its usage clearly specified in TS 38.331.

Observation 3: Case 3) leads to frequent inter-node signaling exchange and close coordination to guarantee the latest SIB1.

Observation 4: On validity mechanism of OD-SIB1, at least legacy approach via paging short message for system information modification and time-based solution (FFS whether still 3 hour) can be reused.

Observation 5: Legacy MIB has 8bit field pdcch-ConfigSIB1 which indicates CORESET#0 and SearchSpare#0 for UE to identify SIB1 transmission. When SIB1 is not broadcasted in NES cell, this 8bit IE is useless, and thereby can be reused to include valueTag of SIB1.

Based on observations, our proposals are:

Proposal 1: RAN2 consider a variant of Case 3), i.e., Cell A provides NES cells’ full SIB1 in its RRC release message.

Proposal 2: SIB1 validity mechanism is supported, i.e., the UE can know when the acquired OD-SIB1 is invalid. Its details can be discussed in normative phase.

Proposal 3: NES cell doesn’t need to include UL WUS configuration of other NES cells in its new SIB. Instead, it includes its own UL WUS configuration in its SIB1 as one RACH partition, and the UE can obtain the updated / latest RACH configuration of UL WUS after it acquires OD-SIB1.

Proposal 4: OD-SIB1 doesn’t impact both legacy and Rel-19 RRC_CONNECTED UEs (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

Proposal 5: RAN2 discuss below issues in normative phase:

- Detailed solutions of how to avoid the legacy UE camping at the cell A attempting measurement/cell reselection to the NES cell.

- Detailed trigger conditions of OD-SIB1 and specification changes on cell reselection

Proposal 6: RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1 (i.e., Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell).

# 4 References

[1] RP-240170, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#125b, Chair Notes.

[3] RAN2#126, Chair Notes.

[4] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[5] TS 38.212-v18.0.0, Multiplexing and channel coding, 2023-12.