# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406671
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.5.4

Source:	Apple

Title:	Further discussion on common signal transmission adaptation

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial

    - This study is to be done in 2Q’2024 only

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Checkpoint for normative work in RAN#105

The Rel-19 WID of Network Energy Savings (WID RP-240170) was agreed in RAN#102 [1], the WI objective on adaptation of common signal transmission is copied below:

In RAN2#125b [2], only paging adaptation was discussed, and below agreement was made. SSB adaptation and RACH adaptation were not discussed due to time limitation.

Agreements on paging adaptation:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this contribution, we further discuss RAN2 work on adaptation of common signal transmission.

# 2 Discussion

## 2.1 Adaptation of paging occasions

### 2.1.1 Legacy UE compatibility

We first discuss below legacy UE compatibility issue identified in RAN2#125b [2]:

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

Option 1 means MNOs need to deploy Rel-19 NES cells which only support Rel-19 UEs capable of paging adaptation. We don’t think it makes sense especially at late release of NR because it may lead to coverage holes and it goes against the intention of introducing NES.

Observation 1: It doesn’t make sense to deploy Rel-19 NES cells which only support Rel-19 UEs capable of paging adaptation especially at late release of NR because it may lead to coverage holes and it goes against the intention of introducing NES.

Thus, we propose RAN2 to focus on Option 2.

Proposal 1: RAN2 don’t introduce new legacy UE barring mechanism for paging adaptation, i.e., Rel-19 NES cells provide separate paging resources for Rel-19 UEs capable of paging adaptation and legacy UEs.

### 2.1.2 Solution details

In this subsection, we further discuss paging adaptation solutions identified by below RAN2#125b agreement [2].

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

First, according to our Proposal 1 on legacy UE impacts, we think both options need to introduce an additional paging configuration only for Rel-19 UEs. And RAN2 need to discuss their legacy UE impacts. Please note that the legacy UE impacts may be different for each option. For example, Rel-19 UEs and legacy UEs may see different number of POs in the same PF in option b). Thus, legacy UE impacts should be evaluated for the two options respectively.

Proposal 2: For both identified paging adaptation options, introduce an additional paging configuration for Rel-19 UEs different from PCCH-Config for legacy UEs, and RAN2 discuss their legacy UE impacts respectively.

Then, we think RAN2 need to refine the details of the two identified solutions before analyzing their Pros and Cons. In our understanding, option b) is clear enough but option a) needs further discussion because detailed mechanism on how to bundle paging frames is not clear. For option b), we think there are the following two alternatives:

Alt-1: via subgroup ID

As illustrated in Figure.1, two legacy UEs use one set of paging configuration with sparse PF value (i.e. N=T/16) while Rel-19 NES UEs use another set of paging configuration with denser PF value (i.e. N=T/8). With legacy PF formula, the two legacy UEs are assigned to PF in RF#0 and RF#15. If these 4 Rel-19 UEs still use the legacy PF formula, they will be assigned to PF in RF#0, RF#7, RF#15, and RF#23. It will make the NW can’t sleep in the whole paging cycle. Instead, if Rel-19 UEs use below new PF formula with UE_ID replaced with SubGroup_ID introduced in Rel-17:

(SFN + PF_offset) mod T = (T div N)*(UE_ID SubGroup_ID mod N)

they will be assigned to PF in RF#0 and RF#7. Thus, the NW can sleep after RF#15. By flexibly setting group_ID, NW could ensure that all these UEs are mapped to particular PFs instead of spreading out all over the paging cycle.

Alt-2: via reducing time interval of two consecutive PFs

As illustrated in Figure.2, another alternative is to reduce time interval of two consecutive PFs. In legacy PF calculation formula, the time interval of two consecutive PFs is (T div N). Thus, we could concentrate PFs by reducing this interval. For example, the time interval can be reduced to 1 via removing the term (T div N) as illustrated below:

(SFN + PF_offset) mod T = (T div N)*(UE_ID Mod N)

Figure.1 Alt-1 of Option a): map UEs to concentrated PFs via subgroup ID

Figure.2 Alt-2 of Option a): map UEs to concentrated PFs via reducing time interval of two consecutive PFs

Thus, we propose to refine option a) with detailed mechanisms for further study (e.g., comparison with option b).

Proposal 3: On paging adaptation Option a), RAN2 capture the following two detailed mechanisms of bundling paging frames for further study:

- Alt-1: via subgroup ID (SFN + PF_offset) mod T = (T div N)*(UE_ID SubGroup_ID mod N)

- Alt-2: via reducing interval of consecutive PFs (SFN + PF_offset) mod T = (T div N)*(UE_ID Mod N)

### 2.1.3 Comparison between option a) and b)

With details of Option a) illustrated in Proposal 3, RAN2 can start discussion on comparison between Option a) and Option b). We provide our initial analysis in Table.1.

|  | Pros | Cons | Potential cross-WG impact |
| --- | --- | --- | --- |
| Option a): bundle PF | - More NES gain and more flexible to NW. - Extra UE power saving gain because NW may provide TRS right before the bundled PFs. | - Higher spec impacts (esp. formula change in TS 38.304). - It may result in RACH collision because the UEs are paged during a confined short period. | RAN1: support PEI with bundled PF. |
| Option b): extend value ranges of N, and increase Ns | Minor RAN2 spec impacts (i.e., extend candidate values of N and Ns in TS 38.331). | - Less flexible than option a). - Higher PO density (when Ns>4) may impact scheduling of other traffic, esp. SI update or PWS notifications. - It may also lead to RACH collision when Ns>4. | RAN1: support PEI when Ns>4* *Note: in legacy PEI, one PEI can map to 1,2,4 POs in one PF or 8 POs in two PFs. |

Table.1 Initial comparison between option a) and option b) of paging adaptation

We propose RAN2 to further study Pros and Cons of these two options with Table.1 as starting point.

Proposal 4: RAN2 further compare between Option a) and Option b) of paging adaptation with Table.1 as starting point.

|  | Pros | Cons | Potential cross-WG impact |
| --- | --- | --- | --- |
| Option a): bundle PF | - More NES gain and more flexible to NW. - Extra UE power saving gain because NW may provide TRS right before the bundled PFs. | - Higher spec impacts (esp. formula change in TS 38.304). - It may result in RACH collision because the UEs are paged during a confined short period. | RAN1: support PEI with bundled PF. |
| Option b): extend value ranges of N, and increase Ns | Minor RAN2 spec impacts (i.e., extend candidate values of N and Ns in TS 38.331). | - Less flexible than option a). - Higher PO density (when Ns>4) may impact scheduling of other traffic, esp. SI update or PWS notifications. - It may also lead to RACH collision when Ns>4. | RAN1: support PEI when Ns>4* *Note: in legacy PEI, one PEI can map to 1,2,4 POs in one PF or 8 POs in two PFs. |

Table.1 Initial comparison between option a) and option b) of paging adaptation

## 2.2 Adaptation of PRACH in time domain

RAN1#116, RAN1#116b and RAN1#117 had discussed RACH adaptation [3][4][5], and a lot of agreements were made. The complete list of agreements can be found in Appendix 1. And we understand that RAN1 progress can be highlighted as below:

- Introduce adaptation mechanism between legacy PRACH resources and one new additional PRACH resources for NES-capable UE in all RRC states.

  - The adaptation signaling (e.g. L1 signaling or higher layer signaling) is still under RAN1 discussion.

- The additional PRACH resources are provided via a prach-ConfigurationIndex and a separate SSB-RO mapping is performed for the additional PRACH resources.

  - It is still under discussion whether to allow any overlapping between legacy PRACH resources and the additional PRACH.

  - It is still under discussion whether prach-ConfigurationIndex for the additional PRACH resources is same as the one for the legacy resource.

Based on latest RAN1 progress, we share our view on potential RAN2 work in the followed sub-sections.

### 2.2.1 Legacy UE compatibility

First, we discuss the legacy compatibility issue. Because RAN1 agreements have considered legacy UE impacts, our understanding is that RAN2 don’t need to introduce new legacy UE barring mechanism for RACH adaptation. Meanwhile, same as paging adaptation, we don’t think it makes sense to deploy Rel-19 NES cells which only support Rel-19 UEs capable of RACH adaptation because it may lead to coverage holes and goes against the intention of NES.

Proposal 5: RAN2 don’t introduce new legacy UE barring mechanism for RACH adaptation in time domain, i.e., Rel-19 NES cells support both Rel-19 UEs capable of RACH adaptation and legacy UEs.

### 2.2.2 Clarification on RACH framework

We have one general clarification question on the RACH framework to integrate RACH adaptation. In Rel-17, general RACH framework supporting multiple RACH partitions was introduced. As illustrated in Figure.3, a list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions (i.e. AdditionalRACH-Config) associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Figure.3 Illustration of RRC configuration of multiple RACH partitions

Observation 2: In Rel-17, general RACH framework supporting multiple RACH partitions was introduced. A list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Firstly, we discuss whether RACH adaptation is applicable to CBRA or CFRA. Because CFRA may use UE dedicated signaling, the benefit of RACH adaptation seems not quite clear. Thus, we propose to first study CBRA.

Proposal 6: RAN2 first study CBRA in RACH adaptation.

Secondly, even for CBRA, we think it is not clear what is the role of RACH adaptation in the general RACH framework. Specifically, there are two different understandings:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

In our understanding, it seems that RAN1 so far only discussed adaptation between rach-ConfigCommon and additional PRACH resources (i.e. Understanding 1). However, we also think it makes sense to support RACH adaptation in at least some RACH features or feature combination (e.g., redcap + additional PRACH resource), but some RACH feature may further discussion (e.g., msg1-repetition + additional PRACH resource). That is why we put an FFS in understanding 2.

We are open to either understanding, but RAN2 should make a conclusion about it from beginning because it will impact the workload.

Proposal 7: RAN2 discuss which is the correct understanding of RACH adaptation in time domain:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

### 2.2.3 Potential RAN2 work

Based on latest RAN1 progress, we think that at least the following RRC and MAC impacts are foreseen:

RRC impacts:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

- How to integrate additional PRACH resource set into general RACH framework, which couples with which understanding in Proposal 6 is agreed.

MAC impacts:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

  - For example, Rel-19 UE may always start from additional PRACH resource, considering legacy UE can only use legacy PRACH resource.

  - As another example, if target for RACH latency minimization, Rel-19 UE may prioritize legacy PRACH resource or additional PRACH resource depending on when preamble transmission is triggered (e.g. the UE may prioritize legacy resource when preamble transmission is triggered in T0, and the UE may prioritize additional PRACH resource when preamble transmission is triggered in T1 in Figure.4).

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

  - RAN2 need to discuss whether only the same PRACH resource set (i.e. legacy vs additional) as initial transmission can be selected.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

Figure. 4 Illustration of RO selection in RACH adaptation in time domain

Thus, we propose:

Proposal 8: To support additional PRACH resource adaptation, RRC spec impacts at least include:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

- How to integrate additional PRACH resource set into general RACH framework, which couples with which understanding in Proposal 6 is agreed.

Proposal 9: To support additional PRACH resource adaptation, MAC spec impacts at least include:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

### 2.2.4 Unified RAN2 design with SBFD RACH enhancement

Finally, we noticed that there are some similarities between RACH in NES and RACH in SBFD. We provide Figure.5 to illustrate the basic idea of NES RACH and SBFD RACH, where the RACH resources configured in the SBFD sub-band could provide more ROs for UE to perform the random access to reduce the random access latency, reduce the PRACH collision probability, and improve the PRACH coverage.

Figure.5 Illustration of similarities between RACH in NES and RACH in SBFD

The complete list of RAN1 agreements on SBFD RACH can be found in Appendix 2. And we provide our analysis on their similarities in Table.2.

|  | NES RACH | SBFD RACH |
| --- | --- | --- |
| How additional RO set is configured | Will down select among below alternatives: - Alt-1: via a dedicated prach-ConfigurationIndex - Alt-2: via legacy prach-ConfigurationIndex and additional mechanism to avoid overlapping. - Alt-3: Alt-1+ Alt-2 | - Option 1: legacy configuration is reused with new RO validation rules for SBFD-aware UE. - Option 2: the legacy table + dedicated prach-ConfigurationIndex is agreed. |
| Overlapping between additional RO set and legacy RO set? | Still under debating for below cases: - Case 1: always non-overlapping - Case 2: partial overlapping in time and/or frequency is allowed. | It is expected to be always non-overlapping because legacy set in non-SBFD symbols while additional set in SBFD symbols. |
| Validation rule of RO | - For non-overlapping case, it was agreed to follow legacy. - For partial overlapping case (if support), it is expected to introduce new rule to make overlapped ROs as invalid ROs. | For both option 1 and option 2, new RO validation rules are being discussed. But their details are not expected to impact RAN2 design. |
| SSB-RO mapping | - For non-overlapping case, it was agreed to follow legacy. - For partial overlapping case (if support), no conclusion for now but it is expected to introduce separate mapping parameters for non-overlapping ROs, to ensure overlapped ROs to be mapped to same SSB. | - For option 1, it is expected to follow legacy RO. - For option 2, it is expected to introduce separate mapping parameters for additional ROs in SBFD symbols. |
| Power control | It is expected to follow legacy because additional RO set is intended to only have different loading rather than radio condition. | It is expected to introduce separate configuration (i.e. target power and power ramping step) for additional RO set in SBFD symbols due to gNB-to-gNB interference. |

Table.2 Comparison between NES RACH and SBFD RACH

According to Table.2, we think both features introduce similar RACH enhancement. Essentially, both NES and SBFD introduces adaptation between two PRACH resource sets although their intention is different:

    - NES: one additional set of PRACH resource is introduced to dynamically adapt NES cell loading change.

- SBFD: one additional set of PRACH resource in SBFD symbol is introduced

  - In option 2, one additional set of PRACH resource is explicitly configured to cover ROs in SBFD symbols, which are invisible to legacy UEs.

  - In option 1, Rel-19 UE can also implicitly derive two sets of PRACH resource: one set of ROs in non-SBFD symbols and the other set of ROs in SBFD symbols with different interference (e.g. gNB-gNB interference).

Observation 3: In Rel-19, both NES and SBFD essentially introduced similar RACH enhancement via adaptation between two PRACH resource sets:

    - NES: one additional set of PRACH resource is introduced to dynamically adapt NES cell loading change.

- SBFD: one additional set of PRACH resource in SBFD symbol is introduced.

We think from RAN2 perspective, their main difference is on power control. Depending on whether the additional set of PRACH resource encounters different interference, separate power control parameters can be configured for additional PRACH resources.

- NES: separate power control parameters are not needed because of same inference between legacy PRACH resources and additional PRACH resources.

- SBFD: separate power control parameters are needed because of different inference between legacy PRACH resources and additional PRACH resources (i.e. gNB-to-gNB inference).

Observation 4: From RAN2 perspective, the main difference between NES and SBFD RACH is on power control. Depending on whether the additional set of PRACH resource encounters different interference, separate power control parameters (e.g. target power and power ramping step) can be configured for additional PRACH resources.

Since the difference of power control can be reflected in whether separate power control parameters (e.g. target power and power ramping step) are configured for additional PRACH resources, we think RAN2 can strive for a unified RRC/MAC design for both NES RACH and SBFD RACH. The details can be further discussed. Thus, we propose:

Proposal 10: As both NES and SBFD introduce similar RACH enhancement via adaptation between legacy PRACH resources and additional PRACH resources, RAN2 strive for a unified RRC/MAC design for them.

## 2.3 Adaptation of SSB in time domain

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

RAN1#116 discussed adaptation of SSB, and below agreement on adaptation mechanism was made [3]:

In above agreement, we think there are two groups of mechanisms which are highlighted as yellow and cyan respectively:

- The highlighted ones are essentially adaptation of SSB periodicity (e.g. ssb-PeriodicityServingCell in SIB1) and/or transmitted beam(s) in one SSB burst (i.e. ssb-PositionsInBurst in SIB1) within IE ServingCellConfigCommonSIB, as illustrated in Figure.6.

  - We think they may have RAN2 impacts related to L3 measurement, including RRM, RLM and BFD:

    - RRM: In legacy RRM based on SSB, the NW need to configure UEs SMTC and SSB_ToMeasure in MeasObjectNR for RRC_CONNECTED UEs and in SIB2/4 for RRC_IDLE/RRC_INACTIVE UEs. However, the legacy RRC reconfiguration of SMTC and/or SSB_ToMeasure may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing RRM.

    - RLM: In legacy RLM, several SSB(s) can be configured as RLM-RS in RRC. However, the legacy RRC reconfiguration of RadioLinkMonitoringConfig may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing RLM (e.g. one actually transmitted beam configured as RLM-RS is turned off after SSB adaptation).

    - BFD: In legacy BFD in SCell, several SSB(s) can be configured as BFD-RS in RRC. However, the legacy RRC reconfiguration of RadioLinkMonitoringConfig may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing BFD (e.g. one actually transmitted beam configured as BFD-RS is turned off after SSB adaptation).

Figure. 6 Illustration of SSB adaptation mechanisms

- The highlighted one is enhancement of Cell DTX for SSB adaptation.

  - Please note that Cell DTX is only applied to the UEs in CONNECTED state in Rel-18 NES, and Cell DTX is not in scope of WID. Thus, we think RAN2 can wait RAN1 input.

Observation 5: Adaptation of SSB periodicity and/or transmitted beam(s) in one SSB burst may lead to failure of UE performing L3 measurement, including RRM, RLM and BFD.

Observation 6: Although Cell DTX for SSB adaptation may have RAN2 impact, Cell DTX is only applied to the UEs in CONNECTED state and Cell DTX is not in scope of Rel-19 WID. Thus, RAN2 can wait RAN1 input.

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Meanwhile, RAN1#116b also made below agreement on applicable scenario of SSB adaptation [4]:

With this agreement, it is not clear for now whether SSB adaptation is only applicable to CONNECTED UE and whether it can be CD-SSB. We think one potential RAN2 impact is to introduce a new legacy UE barring mechanism if RAN1 agree SSB adaptation is applicable to IDLE / INACTIVE UEs and CD-SSB, i.e., Rel-19 NES cell can be configured to bar legacy UE meanwhile allow Rel-19 NES capable UEs to camp.

Observation 7: It is not clear whether SSB adaptation is only applicable to CONNECTED UE and whether it can be CD-SSB. If RAN1 agree SSB adaptation is applicable to IDLE / INACTIVE UEs and CD-SSB, one potential RAN2 impact is to introduce a new legacy UE barring mechanism.

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Finally, RAN1#116b made below agreement on signaling of SSB adaptation [4].

As usual, potential RAN2 work is to design MAC-CE format if RAN1 agree to use MAC-CE to notify UE SSB adaptation. For now, RAN2 can wait RAN1 input.

Based on above discussion, we identify below potential RAN2 work on SSB adaptation.

Proposal 11: On SSB adaptation, potential RAN2 work includes below 3 aspects:

- How the UE performs L3 measurement upon SSB adaptation, including RRM, RLM and BFD.

- If RAN1 agree it is applicable to IDLE / INACTIVE UEs and CD-SSB, RAN2 may need to introduce a new legacy UE barring mechanism, i.e., Rel-19 NES cell bars legacy UE but allows Rel-19 UEs to camp.

- If RAN1 agree to use MAC-CE to notify UE SSB adaptation, RAN2 design MAC-CE format.

# 3 Conclusion

In this contribution, we further discuss RAN2 work on common signal transmission adaptation. Our observations are:

Observation 1: It doesn’t make sense to deploy Rel-19 NES cells which only support Rel-19 UEs capable of paging adaptation especially at late release of NR because it may lead to coverage holes and it goes against the intention of introducing NES.

Observation 2: In Rel-17, general RACH framework supporting multiple RACH partitions was introduced. A list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Observation 3: In Rel-19, both NES and SBFD essentially introduced similar RACH enhancement via adaptation between two PRACH resource sets:

    - NES: one additional set of PRACH resource is introduced to dynamically adapt NES cell loading change.

- SBFD: one additional set of PRACH resource in SBFD symbol is introduced.

Observation 4: From RAN2 perspective, the main difference between NES and SBFD RACH is on power control. Depending on whether the additional set of PRACH resource encounters different interference, separate power control parameters (e.g. target power and power ramping step) can be configured for additional PRACH resources.

Observation 5: Adaptation of SSB periodicity and/or transmitted beam(s) in one SSB burst may lead to failure of UE performing L3 measurement, including RRM, RLM and BFD.

Observation 6: Although Cell DTX for SSB adaptation may have RAN2 impact, Cell DTX is only applied to the UEs in CONNECTED state and Cell DTX is not in scope of Rel-19 WID. Thus, RAN2 can wait RAN1 input.

Observation 7: It is not clear whether SSB adaptation is only applicable to CONNECTED UE and whether it can be CD-SSB. If RAN1 agree SSB adaptation is applicable to IDLE / INACTIVE UEs and CD-SSB, one potential RAN2 impact is to introduce a new legacy UE barring mechanism.

Based on observations, our proposals are:

Adaptation of paging occasions

Proposal 1: RAN2 don’t introduce new legacy UE barring mechanism for paging adaptation, i.e., Rel-19 NES cells provide separate paging resources for Rel-19 UEs capable of paging adaptation and legacy UEs.

Proposal 2: For both identified paging adaptation options, introduce an additional paging configuration for Rel-19 UEs different from PCCH-Config for legacy UEs, and RAN2 further discuss the legacy UE impacts.

Proposal 3: On paging adaptation Option a), RAN2 capture the following two detailed mechanisms of bundling paging frames for further study:

- Alt-1: via subgroup ID (SFN + PF_offset) mod T = (T div N)*(UE_ID SubGroup_ID mod N)

- Alt-2: via reducing interval of consecutive PFs (SFN + PF_offset) mod T = (T div N)*(UE_ID Mod N)

Proposal 4: RAN2 further compare between Option a) and Option b) of paging adaptation with Table.1 as starting point.

|  | Pros | Cons | Potential cross-WG impact |
| --- | --- | --- | --- |
| Option a): bundle PF | - More NES gain and more flexible to NW. - Extra UE power saving gain because NW may provide TRS right before the bundled PFs. | - Higher spec impacts (esp. formula change in TS 38.304). - It may result in RACH collision because the UEs are paged during a confined short period. | RAN1: support PEI with bundled PF. |
| Option b): extend value ranges of N, and increase Ns | Minor RAN2 spec impacts (i.e., extend candidate values of N and Ns in TS 38.331). | - Less flexible than option a). - Higher PO density (when Ns>4) may impact scheduling of other traffic, esp. SI update or PWS notifications. - It may also lead to RACH collision when Ns>4. | RAN1: support PEI when Ns>4* *Note: in legacy PEI, one PEI can map to 1,2,4 POs in one PF or 8 POs in two PFs. |

Table.1 Initial comparison between option a) and option b) of paging adaptation

Adaptation of PRACH in time domain

Proposal 5: RAN2 don’t introduce new legacy UE barring mechanism for RACH adaptation in time domain, i.e., Rel-19 NES cells support both Rel-19 UEs capable of RACH adaptation and legacy UEs.

Proposal 6: RAN2 first study CBRA in RACH adaptation.

Proposal 7: RAN2 discuss which is the correct understanding of RACH adaptation in time domain:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

Proposal 8: To support additional PRACH resource adaptation, RRC spec impacts at least include:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

- How to integrate additional PRACH resource set into general RACH framework, which couples with which understanding in Proposal 6 is agreed.

Proposal 9: To support additional PRACH resource adaptation, MAC spec impacts at least include:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

Proposal 10: As both NES and SBFD introduce similar RACH enhancement via adaptation between legacy PRACH resources and additional PRACH resources, RAN2 strive for a unified RRC/MAC design for them.

Adaptation of SSB in time domain

Proposal 11: On SSB adaptation, potential RAN2 work includes below 3 aspects:

- How the UE performs L3 measurement upon SSB adaptation, including RRM, RLM and BFD.

- If RAN1 agree it is applicable to IDLE / INACTIVE UEs and CD-SSB, RAN2 may need to introduce a new legacy UE barring mechanism, i.e., Rel-19 NES cell bars legacy UE but allows Rel-19 UEs to camp.

- If RAN1 agree to use MAC-CE to notify UE SSB adaptation, RAN2 design MAC-CE format.

# 4 References

[1] RP-240170, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#125b, Chair Notes.

[3] RAN1#116, Chair Notes.

[4] RAN1#116b, Chair Notes.

[5] RAN1#117, Chair Notes.

[6] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[7] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

# 5 Appendix 1: RAN1 agreements on NES RACH adaptation

## 5.1 RAN1#116

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

## 5.2 RAN1#116b

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

## 5.3 RAN1#117

Agreement

For adaptation of PRACH in time-domain, support at least the following case(s)

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

- FFS: whether additional conditions are needed to support the above cases

- FFS: Additional case whether full/partial overlap in both time and frequency is allowed

- Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least:

- a PRACH configuration index

- FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources

Study further the following

- When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource,

  - Additional parameter(s) for determining the additional PRACH resources e.g.

    - Scaled/adjusted PRACH configuration period

    - Additional timing offset

    - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Muting/masking ROs

- When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource

  - Additional mechanisms (if any) for determining the additional PRACH resources e.g.

    - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

- Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

Agreement

For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation

- Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

  - Strive to re-use existing DCI format(s)

- Option 3: Adaptation of PRACH transmission according to predefined condition(s)

  - FFS: details

- Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation

  - Strive to re-use existing DCI format(s)

- Option 5: Enhanced cell DRX

# 6 Appendix 2: RAN1 agreements on SBFD RACH

## 6.1 RAN1#116

Agreement

For random access operation for SBFD-aware UEs in RRC CONNECTED state, at least consider the following options:

- Option 1: Use one single RACH configuration with possible enhancement

  - The ROs within UL subband in SBFD symbols can be valid for SBFD-aware UE

  - FFS: Further details

- Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

  - The ROs within UL subband in SBFD symbols configured by the additional RACH configuration can be valid for SBFD-aware UE

  - FFS: Further details

Agreement

For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols.

- FFS Type-2 random access procedure (2-step RACH)

Agreement

For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols.

- FFS PRACH repetition in SBFD symbols.

- FFS PRACH repetition across SBFD symbols and non-SBFDs symbols.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, further study the following two options:

- Option 1: a valid RO can only be on SBFD symbols or on non-SBFD symbols

  - a configured RO across SBFD and non-SBFD symbols in the same slot or across slots is invalid

- Option 2: a valid RO can be across SBFD and non-SBFD symbols in the same slot or across slots

RAN1 to leverage the study in Rel-18 as baseline.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects:

- Msg2[/Msg4 PDSCH] reception in DL subband(s)

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping

- Msg3 repetition

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control

- FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE

Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

## 6.2 RAN1#116b

Agreement

Confirm the working assumption:

Working assumption:

For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols.

- Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon).

  - FFS the details

- FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s).

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, no separate prach-ConfigurationIndex to be configured in this option.

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, use existing random access configurations tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2 in TS38.211).

Agreement

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives:

- Alt 2-3:

  - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs.

  - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols

- Alt 2-4:

  - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs.

For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration),

- For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed.

- For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used

Agreement

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, consider from the following alternatives:

  - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

    - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

  - Alt 3: Introduce new entries on top of existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

- For FR1, consider from the following alternatives:

  - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

    - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

  - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)

  - Alt 3: Introduce new entries on top of existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state,

- no enhancements for the RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any).

  - FFS: the ROs in non-SBFD symbols that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

  - FFS: It’s up to network configuration to ensure the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, which are valid for non-SBFD aware UEs based on legacy RO validation rule, are also valid for SBFD aware UEs (i.e., the configured ROs in SBFD symbols, if configured as flexible by tdd-UL-DL-ConfigurationCommon, are within the UL usable PRBs)

- the RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least:

  - Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB

  - FFS: Other condition.

Note: For the case that all the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, there is no restriction that all the configured ROs in SBFD symbols should be within the UL usable PRBs.

## 6.3 RAN1#117

Agreement

A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default.

A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

Agreement

Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

Agreement

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider whether additional condition(s) on top of what was agreed in RAN1#116bis for RO validation are necessary. For example,

- Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol.

- Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB.

- Condition#3: A valid RO does not precede a SSB in the PRACH slot.

Agreement

Update the following agreement made in RAN1#116-bis meeting:

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, adopt the following:

  - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

    - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- For FR1, consider from the following alternatives:

  - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

    - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

  - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)