# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406672
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.1.2.2

Source:	Apple

Title:	Further discussion on LCM procedure of UE-sided model for AI/ML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The Rel-19 WID of AI/ML for air interface (WID RP-234039) was agreed in RAN#102 [1]. The WI objective on general LCM framework, including applicable conditions, is led by RAN2 which is copied below:

In RAN2#125b [2] and RAN2#126 [3], LCM procedure of UE-sided model of beam management was discussed, and many agreements were made. Meanwhile, RAN2 had post-meeting email discussion on LCM procedure with two phases:

- Phase 1: Agreeable definitions, whose summary report in [4]

- Phase 2: Reactive/Proactive framework for applicable functionality, whose summary report in [5]

In this contribution, we further discuss leftover issues of RAN2#125b/RAN2#126 and share our view on some key FFSs in email discussion reports [4][5].

- Key remaining issues

- Other important issues not discussed before

# 2 Discussion

## 2.1 Key remaining issues

Based on latest agreements, our understanding on the whole framework can be illustrated in Figure. 1 with below 2 highlights:

- We mark key FFSs of each step in Chair notes and [4][5].

- UE-sided data collection is out of scope of this contribution, but we assume that NW-side additional conditions (i.e. associated ID) is provided in training configuration because it is coupled with how the UE determines applicable functionalities (i.e. step 4a)

Figure.1: Latest understanding on LCM procedure of UE-sided model for beam management

### 2.1.1 Remaining issue in Step 2

In RAN2#125b, it was agreed that supported AI/ML-enabled Features/FGs and functionalities are included in UE capability. But one FFS is left on what functionality refers to.

Agreements for UE capability

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Then, email discussion summary [4] provides proposal on its definition:

Proposal 1: Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

We think the details of “functionality” should be determined by RAN1. Meanwhile, we also think RAN2 can assume one “functionality” may be per sub-use case (e.g. temporal prediction in beam management) for study purpose.

Proposal 1: The detail of “supported functionality” included in UE capability is left to RAN1. Meanwhile, RAN2 assume it may be per sub-use case (e.g. temporal prediction in beam management) for study purpose.

### 2.1.2 Remaining issues in Step 3

Email discussion summary [5] provides below proposal on Step 3.

Proposal 2: “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS the content of configuration.

#### 2.1.2.1 Option 1 (Joint decision by UE and NW) vs option 2 (UE decision)

We first discuss bullet 2):

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

Although it looks like a stage 3 issue, we think its actual intention is to cover both option 1 (Joint decision by UE and NW) and option 2 (UE decision), according to Rapporteur clarification in RAN2 reflector:

Bullet 2)

For network-side additional condition, as summarized in the paragraph above Rapporteur Summary 3, there are quite some companies think both Option 1 and Option 2 can work. The key difference between Option 1 and 2, in my understanding, is whether NW-side additional condition will be provided to UE or not in Step 3. Considering companies are ok with both options, it seems to me this information can be optionally signalled is equal to network “may” send this information. Whether to send NW-side additional condition can be up to network implementation. If it’s not provided by network, it is Option 1; otherwise, it’s Option 2. I think both options are considered in this proposal. But I’m ok to update it to remove “optional” per companies’ comments and also remove “check whether companies are ok to change it as mandatorily signalled” in this bullet.

Option 1: Joint decision by UE and NW

Option 2: UE decision

If associated ID is used as NW-side additional condition, we are not sure how option 1 works due to below concerns:

- UE reporting NW-side additional condition (in step 4) is not aligned with RAN1 understanding and out of scope of RAN1 discussion.

Below RAN1#117 agreement clearly indicated that it is the UE to use associated ID to determine whether NW-side additional condition is satisfied (i.e., consistency in case of same associated ID), instead of NW decision. But Option 1 is the NW to decide applicable functionality based on UE’s reporting of NW-side additional condition.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Furthermore, as highlighted below, Section 4.2 of TR 38.843 [6] only capture that the NW-sided additional condition is indicated to UE. RAN1 never agreed or study the NW-sided additional condition “from UE to gNB”.

For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary):

- Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

- Model training at NW and transfer to UE, where the model has been trained under the additional condition

- Information and/or indication on NW-side additional conditions is provided to UE

- Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

- Other approaches are not precluded

- Note: it does not deny the possibility that different approaches can achieve the same function

- It will introduce model ambiguity between UE and NW

Please note that one AI/ML functionality may include multiple NW-side additional conditions corresponding to multiple AI/ML models. Then, the UE may not know which model should be used for inference in option 1 because the NW doesn’t indicate the current NW-side additional condition to the UE. We provide an example in Figure.2, where the UE has model 1 (corresponding to associated ID 1) and model 2 (corresponding to associated ID 2) available for the same functionality A while the current NW condition is associated ID = 2. Then, after UE reports that the functionality is applicable under either associated ID=1 or ID=2 in step 4, the NW activates the functionality A in step 5, but the UE doesn’t know whether it should use model 1 or model 2 for inference (Note that NW-side additional condition is not indicated to the UE in Step 5 of Option 1 figure). If the UE chooses to use model 1, it will lead to non-consistency with current NW condition.

Figure.2: Issue illustration of model ambiguity between UE and NW in option 1 (joint UE and NW decision)

- It is not clear how the UE evaluates the applicability of functionality without any preliminary information from the NW. And it will lead to more frequent UE reporting, but benefit is not clear.

We are not sure how the UE evaluates the applicability of AI/ML functionality without any preliminary information (e.g. associated ID) from the NW. If the decision is based solely on the UE's internal conditions (i.e. UE-side additional condition) and the availability of AI/ML models, the reporting will be quite dynamic and more frequent. Therefore, the usefulness of such information for the NW to make a final determination on the applicability of AI/ML functionality is questionable.

Observation 1: Option 1 (joint decision of UE and NW) will lead to below issues:

- UE reporting NW-side additional condition is not aligned with RAN1 understanding and out of scope of RAN1 discussion.

- It will introduce model ambiguity between UE and NW

- It is not clear how the UE evaluates the applicability of functionality without any preliminary information from the NW. And it will lead to more frequent UE reporting, but benefit is not clear.

Thus, we propose that RAN2 focus on option 2. If majority prefer to pursue option 1, we think RAN1 needs to be consulted. Otherwise, it will cause misalignment between RAN1 and RAN2.

Proposal 2a: RAN2 focus on the solution of UE decision on applicability reporting (i.e. NW provides NW-side additional conditions to the UE, and the UE evaluates whether one functionality is applicable).

Proposal 2b: If majority support joint decision of UE and NW (i.e. NW doesn’t provide NW-side additional conditions to the UE, and the UE blindly reports NW-side additional conditions for NW to evaluate whether one functionality is applicable), RAN1 needs to be consulted how the UE evaluates the applicability without any preliminary information (e.g. associated ID) from the NW.

#### 2.1.2.2 Whether other inference configuration is needed besides associated ID in Step 3

Then, we discuss bullet 3):

3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS the content of configuration.

We understand the key point is, besides NW-side additional condition, whether other configuration is needed (e.g. resource of set A) for UE evaluating applicability. We think it depends on RAN1 design and it is possible that associated ID alone is sufficient. According to below RAN1#116b [7] agreement, CSI-ReportConfig is agreed to be used as inference configuration, and there are 4 alternatives on how to configure Set A.

Agreement For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded.

Please note:

- Alt-1 and Alt-4 implicitly configure Set A via mapping to resources of Set B, where the configuration of Set B is already provided to UE in training phase. Because associated ID can be used to link Set A and Set B, associated ID alone is sufficient to provide UE inference configuration in these two alternatives.

- Alt-3 explicitly configures Set A. Thus, if Alt-3 is agreed in RAN1, associated ID + configuration of Set A need to be provided to the UE in step 3 for evaluation.

Observation 2: According to RAN1#116b agreement, CSI-ReportConfig is used as inference configuration, and there are 4 alternatives on how to configure Set A, where Alt-1 and Alt-4 implicitly configure Set A via mapping to configuration of Set B. Because associated ID can be used to link Set A and Set B, associated ID alone is sufficient to provide UE inference configuration in these two alternatives.

Thus, we propose RAN2 to confirm this understanding and wait for RAN1 conclusion.

Proposal 3: Depending on RAN1 design on how to configure Set A, associated ID alone may be sufficient for the UE to evaluate whether one functionality is applicable (i.e. other configuration is not required in step 3). Thus, RAN2 wait for RAN1 conclusion.

#### 2.1.2.3 Whether to allow blind inference configuration in Step 3

During email discussion [5], another concern on step 3 is whether it implies blind inference configuration is allowed (i.e. inference configuration is provided before UE’s applicability reporting). As Observation 2 shows, if Alt-1 or Alt-4 on Set A configuration is agreed in RAN1, it means that associated ID alone already provides the UE full inference configuration in Step 3. It means that blind inference configuration has to be allowed.

Observation 3: If Alt-1 or Alt-4 on Set A configuration is agreed in RAN1, it means that associated ID alone already provides the UE full inference configuration in Step 3, which is blind inference configuration.

Thus, we propose RAN2 keep this topic open and discuss whether to allow blind inference configuration after RAN1 conclude the signaling of how to configure Set A.

Proposal 4: Because associated ID alone may provide the UE full inference configuration in some RAN1 design, RAN2 postpone the discussion whether to allow blind inference configuration after RAN1 conclude the signaling of how to configure Set A.

### 2.1.3 Remaining issues in Step 4a

We discuss the highlighted part in Proposal 3 in [5].

Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE.

According to email discussion Rapporteur clarification, this “if available” is also intended to cover option 1 (Joint decision by UE and NW) (i.e. when NW-side additional condition is not indicated by NW, it is option 1 and the UE only evaluates UE-side additional condition and model availability for applicability reporting). Due to same argument of our Proposal 2a/2b, we think this “if available” should be removed.

Proposal 5: Remove “(if available)” in Proposal 3 of email discussion [POST126][032] phase 2 summary.

### 2.1.4 Remaining issues in Step 4

Step 4 is corresponding to Proposal 4 in [5]:

Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

We discuss the FFS of bullet 2). We prefer to use RRCReconfigurationComplete to response NW due to below reasons:

- The NW may ignore the UE’s applicable functionality reporting if UAI is used

According to Section 7.9 of TS 38.300 [8], it is up to gNB whether to accommodate the request in UAI.

7.9 UE Assistance Information

…

In the second case, the UE can express a preference for temporarily reducing the number of maximum secondary component carriers, the maximum aggregated bandwidth and the number of maximum MIMO layers. In all cases, it is up to the gNB whether to accommodate the request.

Thus, we have concern that NW ignore the UE’s applicable functionality reporting in Step 4 and configures / activates functionality which is non-applicable for the UE.

- UAI don’t mandate the UE to report upon configuration

According to Section 5.7.4.2 of TS 38.331 [9], it is up to UE implementation whether / when to initiate UAI reporting because “may” is used in all UAI reporting initiation statements.

5.7.4.2	Initiation

A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference.

A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition.

…

Thus, it is possible that some UEs supporting AI/ML never report applicable functionalities (e.g. due to the UE’s preference not to use AI/ML). It may result in failure of LCM procedure in Rel-19.

- UAI may not work in some scenarios (e.g. Handover, neighbour measurement)

We take Handover as example. In legacy, target cell configuration is included in HO command and the UE response RRCReconfiguraitonComplete message to target cell. However, as UAI message can’t be sent directly to target cell, it means existing UAI framework can’t work for handover case.

- UAI will introduce extra signalling overhead

As applicable functionality reporting is configured via RRCReconfiguraiton message, the UE will always need to respond with RRCReconfiguraitonComplete message. It is unnecessary and redundant to use another RRC message (e.g. UAI) to report them.

Observation 4: As response to NW-side additional condition, RRCReconfigurationComplete is preferred over UAI due to below reasons:

- The NW may ignore the UE’s applicable functionality reporting if UAI is used

- UAI don’t mandate the UE to report upon configuration

- UAI may not work in some scenarios (e.g. Handover, neighbour measurement)

- UAI will introduce extra signalling overhead

Thus, we propose to use RRCReconfigurationComplete.

Proposal 6: As response to NW-side additional condition, RRCReconfigurationComplete is used to report applicable functionalities.

Meanwhile, we have concern on 1) even in proactive reporting via UAI. Thus, we propose that after the UE reports applicable functionalities in Step 4, it expects that NW will only configure / activate applicable functionalities in Step 5. And if NW configures non-applicable functionalities in Step 5, we think the UE behaviour should be clearly specified. In this case, we think the UE initiates RRC re-establishment procedure.

Proposal 7: Irrespective of which RRC message, after the UE reports applicable functionalities in Step 4, it expects that NW will only configure / activate applicable functionalities in Step 5. If NW still configures non-applicable functionalities in Step 5, the UE initiates RRC re-establishment procedure.

Finally, we discuss the following issue in summary report [4].

Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

In our understanding, the intention to introduce available functionality is to allow the UE to notify NW the case that one functionality has available model, but it is not applicable (e.g. due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition). However, we are not convinced it is necessary because the NW can’t differentiate whether it is due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition and thereby doesn’t have clue how to react.

Observation 5: Even if available functionality is introduced, the NW can’t differentiate whether it is due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition and thereby doesn’t have clue how to react.

Thus, we propose:

Proposal 8: RAN2 don’t introduce available functionalities separately from applicable functionalities.

### 2.1.5 Remaining issues in Step 6

We discuss the FFS in Proposal 6 in [5].

Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

In 3GPP, whether introducing L1/L2 activation signaling is generally decided by RAN1, which further depends on whether RAN1 identify requirement for dynamic activation, deactivation and switching. As usual, it is RAN1 decision and RAN2 can just wait for RAN1 conclusion. Please note that although MAC-CE format design is in RAN2, whether to introduce MAC-CE is still decided by RAN1.

Proposal 9: RAN2 leave it to RAN1 to decide whether introduce additional L1/L2 signaling for activation/deactivation.

Another FFS we want to discuss is below one in RAN2#126 agreement [3]:

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

We don’t think it is an essential issue. In our understanding, “functionality deactivation” just means one AI/ML functionality is disabled and fallback to legacy. According to our Proposal 1, one functionality can be a sub-use case. Then, it is confusing what is “functionality switching” means. For example, we think the switching from “temporal prediction in beam management” to “spatial prediction in beam management” will only introduce more confusions. Meanwhile, we think it is just a clarification and don’t think it needs to be captured in any specifications. Thus, we propose:

Proposal 10: RAN2 clarify “functionality deactivation” refers to “functionality is disabled and fallback to legacy”. And it doesn’t need to be captured in any specifications.

## 2.2 Other important issue

RAN1 discussed whether associated ID can be applicable to multiple cells, but they can’t conclude:

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

We think that it is an important issue to determine whether LCM procedure of UE-sided model is practical. So, we want to share our view from RAN2 perspective.

We think it should be PLMN unique because of below considerations:

- If associated ID is cell specific, it means that the UE needs to train cell specific AI/ML model because UE is not aware the underlaying NW implementation behind the ID (e.g. NW’s beam design). Thus, once the UE moves to another cell, it will have to re-initiate the training procedure of AI/ML model. It will make solution quite complicated and not scalable.

- At least in case 2b/3b of AI/ML based positioning, it is LMF to configure the data collection, and thereby the associated ID included in data collection configuration from LMF can only be PLMN unique.

Meanwhile, we also think it is feasible to specify PLMN unique associated ID from signaling perspective. 3GPP already specify similar signaling before. For example, Rel-16 RACS has introduced PLMN assigned UE Capability ID.

Thus, we propose:

Proposal 11: It is feasible to support PLMN unique associated ID from signaling perspective.

# 3 Conclusion

In this contribution, we share our view on LCM procedure of UE-sided model for beam management. Our observations are:

Observation 1: Option 1 (joint decision of UE and NW) will lead to below issues:

- UE reporting NW-side additional condition is not aligned with RAN1 understanding and out of scope of RAN1 discussion.

- It will introduce model ambiguity between UE and NW

- It is not clear how the UE evaluates the applicability of functionality without any preliminary information from the NW

Observation 2: According to RAN1#116b agreement, CSI-ReportConfig is used as inference configuration, and there are 4 alternatives on how to configure Set A, where Alt-1 and Alt-4 implicitly configure Set A via mapping to configuration of Set B. Because associated ID can be used to link Set A and Set B, associated ID alone is sufficient to provide UE inference configuration in these two alternatives.

Observation 3: If Alt-1 or Alt-4 on Set A configuration is agreed in RAN1, it means that associated ID alone already provides the UE full inference configuration in Step 3, which is blind inference configuration.

Observation 4: As response to NW-side additional condition, RRCReconfigurationComplete is preferred over UAI due to below reasons:

- The NW may ignore the UE’s applicable functionality reporting if UAI is used

- UAI don’t mandate the UE to report upon configuration

- UAI may not work in some scenarios (e.g. Handover, neighbour measurement)

- UAI will introduce extra signalling overhead

Observation 5: Even if available functionality is introduced, the NW can’t differentiate whether it is due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition and thereby doesn’t have clue how to react.

Based on observations, our proposals can be found below:

Proposal 1: The detail of “supported functionality” included in UE capability is left to RAN1. Meanwhile, RAN2 assume it may be per sub-use case (e.g. temporal prediction in beam management) for study purpose.

Proposal 2a: RAN2 focus on the solution of UE decision on applicability reporting (i.e. NW provides NW-side additional conditions to the UE, and the UE evaluates whether one functionality is applicable).

Proposal 2b: If majority support joint decision of UE and NW (i.e. NW doesn’t provide NW-side additional conditions to the UE, and the UE blindly reports NW-side additional conditions for NW to evaluate whether one functionality is applicable), RAN1 needs to be consulted how the UE evaluates the applicability without any preliminary information (e.g. associated ID) from the NW.

Proposal 3: Depending on RAN1 design on how to configure Set A, associated ID alone may be sufficient for the UE to evaluate whether one functionality is applicable (i.e. other configuration is not required in step 3). Thus, RAN2 wait for RAN1 conclusion.

Proposal 4: Because associated ID alone may provide the UE full inference configuration in some RAN1 design, RAN2 postpone the discussion whether to allow blind inference configuration after RAN1 conclude the signaling of how to configure Set A.

Proposal 5: Remove “(if available)” in Proposal 3 of email discussion [POST126][032] phase 2 summary.

Proposal 6: As response to NW-side additional condition, RRCReconfigurationComplete is used to report applicable functionalities.

Proposal 7: Irrespective of which RRC message, after the UE reports applicable functionalities in Step 4, it expects that NW will only configure / activate applicable functionalities in Step 5. If NW still configures non-applicable functionalities in Step 5, the UE initiates RRC re-establishment procedure.

Proposal 8: RAN2 don’t introduce available functionalities separately from applicable functionalities.

Proposal 9: RAN2 leave it to RAN1 to decide whether introduce additional L1/L2 signaling for activation/deactivation.

Proposal 10: RAN2 clarify “functionality deactivation” refers to “functionality is disabled and fallback to legacy”. And it doesn’t need to be captured in any specifications.

Proposal 11: It is feasible to support PLMN unique associated ID from signaling perspective.

# 4 References

[1] RP-234039, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#125b, Chair Notes.

[3] RAN2#125b, Chair Notes.

[4] R2-24xxxxx, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 1, Samsung.

[5] R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2, Intel Corporation.

[6] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.

[7] RAN1#116b, Chair Notes.

[8] TS 38.300-v18.0.0, NR and NG-RAN Overall Description; Stage 2, 2023-12.

[9] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.