# 3GPP TSG RAN WG2 Meeting #127      	                        R2-2406673
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.1.3

Source:	Apple

Title:	Further discussion on NW-sided data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The Rel-19 WID of AI/ML for air interface (WID RP-234039) was agreed in RAN#102 [1]. The WI objective on general LCM framework is led by RAN2. One of its sub-objectives is to specify data collection for NW-sided model, as highlighted copied below:

In RAN2#125b [2] and RAN2#126 [3], NW-sided data collection was discussed. It was agreed to design a unified measurement framework, and immediate MDT was agreed to support L1 measurement logging and reporting.

In this contribution, we further discuss NW-sided data collection from below aspects:

- NW-sided data collection for AI/ML based beam management

- NW-sided data collection for AI/ML based positioning

# 2 Discussion

## 2.1 NW-sided data collection for AI/ML based beam management

The following agreements were made with several FFSs in RAN2#125b [2] and RAN2#126 [3].

Agreements (RAN2#125b)

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

Agreements for beam management (RAN2#126)

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

Based on these agreements, our understanding on the whole procedure, including OAM centric and gNB centric data collection, is illustrated in Figure .1.

Figure. 1 Overall procedure of OAM centric and gNB centric NW-sided data collection

In the following, we discuss the below remaining issues based on Figure.1.

- Overall procedure

- AS procedure step 1: RRC configuration of L1 measurement logging

- AS procedure step 2: How UE logs L1 measurement

- AS procedure step 3: Trigger reporting of logged L1 measurement

- AS procedure step 4: Reporting of logged L1 measurement

### Overall procedure

As illustrated in Figure.1, our understanding is:

- CN procedure (i.e. procedure between RAN and CN/OAM/UDM) is labelled in blue font.

  - We think RAN2 can assume this procedure reuses existing procedure of immediate MDT.

  - One possible exception is that maybe a new server instead of TCE can be introduced to store training data, but it is up to RAN3/SA5 to decide.

- AS procedure (i.e. procedure between RAN and UE) is labelled in orange font.

  - This procedure includes 4 steps as illustrated in Figure. 1 and is common to both OAM centric and gNB centric data collection.

  - Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration.

Observation 1: Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration. Their AS procedure is same.

As we noticed that there were some misunderstandings on the overall procedure during online discussion of RAN2#126 [3], we suggest RAN2 to confirm the understanding.

Proposal 1: RAN2 assume the procedure between RAN and CN/OAM/UDM reuses existing procedure of immediate MDT, and the procedure between RAN and UE is common to both OAM centric and gNB centric data collection.

Meanwhile, we have UE privacy concern on gNB centric data collection because the collected data is generated by the UE which may include UE’s privacy sensitive information (e.g., the UE’s location information). Please note that the UE privacy requirement is already satisfied by OAM centric data collection because user consent for MDT is required before NW configures the UE to perform immediate MDT, according to TS 32.422 [4]:

4.9.1 Signalling based MDT

In case of signalling based MDT getting user consent before activating the MDT functionality is required because of privacy and legal obligations. It is the Operator responsibility to collect user consent before initiating an MDT for a specific IMSI, IMEI number or SUPI.

Collecting the user consent shall be done via customer care process. The user consent information availability should be considered as part of the subscription data and as such this shall be provisioned to the UDM database.

Observation 2: UE privacy requirement is preserved in OAM centric data collection due to UE consent mechanism in MDT, but it is not clear how to satisfy UE privacy requirement in gNB centric data collection.

Since RAN2 agreed to aim the same measurement framework for both gNB-centric data collection and OAM-centric data collection, we think similar UE consent mechanism in MDT should also be supported in gNB-centric data collection, i.e. user consent is required before NW configures the UE to gNB-centric data collection. The detailed signaling can be further discussed.

Proposal 2: To strive for a common measurement framework, similar UE consent mechanism in MDT is supported in gNB-centric data collection, i.e. UE consent is required before NW configures the UE gNB-centric data collection. FFS signaling details.

### 2.1.2 Step 1: RRC configuration of L1 measurement logging

In legacy immediate MDT and L3 RRM, measurement configuration (i.e. MeasConfig) is included in RRCReconfiguration message. And whether it is immediate MDT or L3 RRM is transparent to the UE. The existing RRC structure is illustrated in Figure.2, where each MeasID is linked with one MeasObjectNR and one ReportConfigNR.

Figure. 2 Illustration of existing RRC structure of L3 RRM and immediate MDT (note: some unrelated IEs, e.g., offsetMO, are omitted)

Figure. 3 Illustration of existing RRC structure of CSI framework (note: some unrelated IEs, e.g., csi-SSB-ResourceSet, are omitted)

According to below RAN1#116b agreement [9], RAN2 has agreed to reuse existing CSI framework for data collection of beam management.

Agreement

For network-sided AI/ML model for BM-Case1 and BM-Case2,

- support using existing CSI framework for configuration of Set A as the starting point

- support using existing CSI framework for configuration of Set B as the starting point

- Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

Thus, RAN2 should also integrate existing CSI framework into L3 measurement framework illustrated in Figure.2. The existing RRC structure of CSI framework is illustrated in Figure.3, where each csi-ResourceConfigId is linked with one csi-ResourceConfig and one csi-ReportConfig.

Based on analysis of existing RRC structure, our initial consideration on how to integrate L1 measurement logging into L3 measurement framework can be found in Figure.4 with below explanation:

- Configuration on CSI resource (e.g. reuse csi-ResourceConfig list of CSI-MeasConfig) and a new logging configuration (e.g. logging interval) are introduced within MeasObjectNR.

- A new L1 measurement reporting configuration (e.g. L1ReportConfig) is introduced within ReportConfigNR to indicate at least the reporting quantity configuration (e.g. L1 RSPR and/or beam index) and reporting type (FFS event trigger or NW request as reporting type).

Figure. 4 An example of how to integrate logged L1 measurement into L3 RRM framework (green modules are introduced for logged L1 measurement/reporting)

So, we propose:

Proposal 3: MeasConfig in RRCReconfiguration is reused to configure logged L1 measurements with below specification changes to integrate L1 measurement logging:

- Within MeasObjectNR: introduce configuration on CSI resource (e.g. reuse csi-ResourceConfig list of CSI-MeasConfig) and logging configuration (e.g. the logging interval).

- Within ReportConfigNR: introduce a L1 measurement reporting configuration (e.g. L1ReportConfig) to indicate at least the reporting quantity configuration (e.g. L1 RSPR and/or beam index) and reporting type (e.g. periodical reporting).

### 2.1.3 Step 2: How UE logs L1 measurement

To allow UE “reporting multiple instances of logged L1 measurement result from UE to gNB”, it requires the UE to store the logged multiple instances of L1 measurement results in an AS buffer. We need to determine the minimum AS layer memory size. In legacy NR, logged MDT and QoE measurements (including paused measurements and IDLE/INACTIVE measurements) had specified the minimum AS layer memory size, which are all 64kB according to TS 38.306 [6]:

Logged MDT (From Section 4.2.18 of TS 38.306) loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No. QoE (from Section 6 of TS 38.306) AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports".

Observation 3: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64kB according to TS 38.306.

Meanwhile, RAN1 Reply LS (R1-2310681) [8] provided typical data size for model training in beam management. According to the highlighted parts, we can estimate that the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) = 2800 bits=0.35KB.

Observation 4: Based on the typical training data size of beam management provided in RAN1 Reply LS (R1-2310681), the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) =0.35KB.

From RAN1 reply LS (R1-2310681) LCM purpose; UE-side/NW-side models; Data content; Typical data size (per data sample); Typical latency requirement; Notes. Training; UE-side, NW-side; L1-RSRPs and/or beam-IDs; See Note 1 for L1-RSRPs; Relaxed. ==omit table rows on “Inference” and “Monitoring” === Note 1: There is no agreement on the data size of L1-RSRPs for Set A or Set B, but the following typical data size is provided as guidance for RAN2 discussion. Based on existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. For BM Case 2, the data size L1-RSRPs for Set A and Set B represents the data size per predicted future time instance and per history measurement time instance, respectively. Payload size may not be fixed. Note 2: There are no agreements on the reporting type. Note 4: Feasibility and necessity of the monitoring schemes listed in the table are under discussion. Note 5: For BM Case 2, the typical value of the number of history measurement time instance used in evaluations is up to 8 and typical value of the number of predicted future time instance is 1~4.

Based on above analysis, we think it is sufficient to reuse the same minimum AS layer memory size as logged MDT and QoE. Meanwhile, to allow the NW to know when each sample of reported L1 measurement is measured, the UE can include the time stamp when the L1 measurement sample is logged. Thus, we propose:

Proposal 4a: Upon reception of configuration, the UE starts L1 measurements (L1 RSRP and/or beam index) and store the measurements with logging time stamp in an AS buffer.

Proposal 4b: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

Furthermore, the L1 measurement logging will increase UE’s power consumption. According to Section 7.2.1.3.1 of TR 38.843 [7], 3rd design principle of NW-sided data collection is to consider the UE energy consumption.

##### 7.2.1.3.1 Considerations for network-side data collection A set of general data collection principles is expected to be considered for network-side model training. These include: - UE to support data logging, - UE to report the collected data periodically, event-based, and on-demand, - The UE memory, processing power, energy consumption, signalling overhead should be considered. Note: The above principles can be revised depending on RAN1 requirements.

So, we have concern that the UE may endlessly perform L1 measurement logging. To resolve this issue, we think the UE can be configured with a logging duration and the UE is allowed to terminate the L1 measurement logging after the duration expires. Please note that similar mechanism was specified in logged MDT (i.e. the UE is allowed to stop logged MDT after T330 expires, according to TS 38.331 [5]).

Observation 5: TR 38.843 capture one design principle of NW side data collection is to consider UE energy consumption. And in logged MDT, the UE is allowed to stop logged MDT after T330 expires.

Thus, we propose to introduce similar duration mechanism for L1 measurement logging.

Proposal 5: Similar to T330 mechanism of logged MDT, the UE can be configured with a logging duration and the UE is allowed to terminate the L1 measurement logging after the configured duration expires.

### 2.1.4 Step 3: Trigger reporting of logged L1 measurement

In RAN2#126 [3], it was agreed to support periodical reporting and whether to support event-based reporting and network request reporting is FFS. We discuss them separately.

#### 2.1 4.1 Event-based reporting

We think at least the following 2 types of events are useful:

- AS buffer based event (e.g. when UE’s AS buffer to store L1 measurement logging is full)

- Radio condition based event (e.g. when UE’s RSRP > threshold)

For 1), please note that both logged MDT and QoE have specified the UE behavior when the AS buffer is full in TS 38.331 [5]:

#### UE behaviour when AS buffer for QoE is full #### 5.5b.1.2 Initiation While in RRC_INACTIVE and RRC_IDLE state, the UE shall: 1> store any previously or subsequently received application layer measurement report containers associated with the measConfigAppLayerId for which not all segments, or full message, has been submitted to lower layers for transmission; NOTE: The UE continues storing application layer measurement report containers for MBS broadcast service in RRC_CONNECTED as long as stored application layer measurement configurations/reports have not been retrieved. 1> if the memory reserved for storing application layer measurement report containers becomes full: 2> if the reports are associated with a configuration including appLayerMeasPriority: 3> discard reports in priority order where reports with the lowest priority are discarded first, and among reports with equal associated priority, discard reports in the order they were received, where older reports are discarded first; 2> else: 3> discard reports in the order they were received, where older reports are discarded first;

#### UE behaviour when AS buffer for logged MDT is full #### 5.5a.3.2 Initiation …omit unrelated part… 2> when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

The UE’s discarding rules are specified in logged MDT and QoE because the UE is in RRC_IDLE / RRC_INACTIVE state and thereby it doesn’t have CP tunnel to transmit the stored data to the NW. Different from logged MDT and QoE, the UE performing L1 measurement logging is in RRC_CONNECTED state and reporting can be triggered to refresh the buffer to store data instead of discarding stored data.

Observation 6: Both logged MDT and QoE have specified UE discarding behavior when the AS buffer is full because the UE is in RRC_IDLE/RRC_INACTIVE state and thereby it doesn’t have CP tunnel to transmit the stored data to the NW. Different from them, the UE performing L1 measurement logging is in RRC_CONNECTED state and reporting can be triggered to refresh the stored data.

For 2), please note that NW typically adopt different MIMO solutions when the UE is in cell edge (e.g. exploring robust) and cell center (e.g. exploring high throughput). Thus, we believe that NW may train different AI/ML models for cell center case and cell edge case, respectively. As a consequence, if the radio condition based event is not introduced, the UE may mix the data in cell center and cell edge together, which may make NW can’t train separate AI/ML models. Meanwhile please note that legacy A1/A2 event can be reused, and thereby RAN2 don’t need to introduce new trigger event.

Observation 7: NW typically adopt different MIMO solutions when the UE is in cell edge (e.g. exploring robust) and cell center (e.g. exploring high throughput). So, if without radio condition event, the UE may mix the data in cell center and cell edge together, which may make NW can’t train separate AI/ML models.

So, based on observation 6 and 7, we propose:

Proposal 6: Support the following event reporting of logged L1 measurement:

- AS buffer based event reporting (e.g. when UE’s AS buffer to store L1 measurement logging is full)

- Radio condition based event reporting (e.g. reuse legacy A1/A2 event)

#### 2.1.4.2 NW requested reporting

We think NW requested reporting is useful at least in below 2 scenarios:

- Scenario 1: when the serving cell determines to hand over the UE to another cell (e.g. due to radio condition) and the UE has logged L1 measurement stored in its AS buffer.

- Scenario 2: when the UE autonomously enters RRC_IDLE/RRC_INACTIVE state (e.g. expiry of T319) and has logged L1 measurement stored in its AS buffer.

In above 2 scenarios, we believe that NW may request the UE to immediately report the stored L1 measurements rather than waiting for expiry of configured periodicity. Similar to logged MDT, we can reuse the RRC message pair UEInformationRequest / UEInformationResponse.

Proposal 7: Support NW requested reporting of logged L1 measurement by reusing RRC message pair UEInformationRequest / UEInformationResponse.

To achieve Proposal 7, the UE needs to notify NW on the availability of stored L1 measurement when it returns to RRC_CONNECTED state.

Proposal 8: If the UE autonomously enters RRC_IDLE/RRC_INACTIVE state (e.g. expiry of T319) and has logged L1 measurement stored in its AS buffer, it notifies NW on the stored L1 measurement when it returns to RRC_CONNECTED state.

### 2.1.5 Step 4: Reporting of logged L1 measurement

In existing L3 RRM and immediate MDT, RRC message MeasurementReport is used for reporting in both periodical reporting and event reporting. And according to our Proposal 7, UEInformationResponse can also be used for NW requested reporting. Then, the only remaining issue is the below FFS made in RAN2#126 [3]:

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

According to our Proposal 4b, the minimum AS layer memory size for L1 measurement logging is 64KB. While the maximal payload size of one RRC message is 9KB without segmentation and up to 144KB with 16 segments. Thus, if RRC segmentation is allowed for logged L1 measurement reporting, it is sufficient to complete transmission of data stored in the AS buffer.

Observation 8: As the minimum AS layer memory size for L1 measurement logging is 64KB, single RRC message with segmentation (up to 144KB) is sufficient to complete transmission of data stored in AS buffer.

In existing TS 38.331 [5], it is specified that only UECapabilityInformation and MeasurementReportAppLayer support UL RRC message segmentation.

–	ULDedicatedMessageSegment

The ULDedicatedMessageSegment message is used to transfer segments of the UECapabilityInformation or MeasurementReportAppLayer message. SRB1 is used at transfer of segments of UECapabilityInformation and SRB4 or SRB5 is used at transfer of segments of MeasurementReportAppLayer.

Thus, MeasurementReport and UEInformationResponse also need to support UL segmentation.

Proposal 9: To allow reporting logged L1 measurements in a single RRC message, support UL segmentation for MeasurementReport and UEInformationResponse.

## 2.2 NW-sided data collection for AI/ML based positioning

In Section 7.2.1.3.1 of TR 38.843 [7], conclusion of NW-sided data collection for positioning is captured as illustrated below:

From Section 7.2.1.3.1 of TR 38.843

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Based on above conclusion, it seems to be straight forward to extend LPP for case 2b and NPPRa for case 3b. Thus, we propose:

Proposal 10: For NW-sided model of case 2b of AI/ML based positioning, extend LPP to report dataset from UE to LMF for model training.

Proposal 11: For NW-sided model of case 3b of AI/ML based positioning, extend NPPRa to report dataset from gNB to LMF for model training.

Finally, in case 3a, gNB performs inference based on new type of SRS transmitted by the UE, and then sends inference output (rather than dataset) to LMF. In another word, there is no dataset transfer between different entities.

Observation 9: In case 3a of AI/ML based positioning, gNB performs inference based on new type of SRS transmitted by the UE, and then sends inference output (rather than dataset) to LMF. So, there is no dataset transfer between different entities.

Thus, case 3a doesn’t need to enhance data collection. We suggest RAN2 to confirm this understanding.

Proposal 12: For NW-sided model of case 3a of AI/ML based positioning, it doesn’t need to enhance data collection because there is no dataset transfer between different entities.

# 3 Conclusion

In this contribution, we further discuss NW-sided data collection. Our Observation are:

Observation 1: Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration. Their AS procedure is same.

Observation 2: UE privacy requirement is preserved in OAM centric data collection due to UE consent mechanism in MDT, but it is not clear how to satisfy UE privacy requirement in gNB centric data collection.

Observation 3: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64kB according to TS 38.306.

Observation 4: Based on the typical training data size of beam management provided in RAN1 Reply LS (R1-2310681), the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) =0.35KB.

Observation 5: TR 38.843 capture one design principle of NW side data collection is to consider UE energy consumption. And in logged MDT, the UE is allowed to stop logged MDT after T330 expires.

Observation 6: Both logged MDT and QoE have specified UE discarding behavior when the AS buffer is full because the UE is in RRC_IDLE/RRC_INACTIVE state and thereby it doesn’t have CP tunnel to transmit the stored data to the NW. Different from them, the UE performing L1 measurement logging is in RRC_CONNECTED state and reporting can be triggered to refresh the stored data.

Observation 7: NW typically adopt different MIMO solutions when the UE is in cell edge (e.g. exploring robust) and cell center (e.g. exploring high throughput). So, if without radio condition event, the UE may mix the data in cell center and cell edge together, which may make NW can’t train separate AI/ML models.

Observation 8: As the minimum AS layer memory size for L1 measurement logging is 64KB, single RRC message with segmentation (up to 144KB) is sufficient to complete transmission of data stored in AS buffer.

Observation 9: In case 3a of AI/ML based positioning, gNB performs inference based on new type of SRS transmitted by the UE, and then sends inference output (rather than dataset) to LMF. So, there is no dataset transfer between different entities.

Based on observations, our proposals can be found below:

AI/ML based beam management

Proposal 1: RAN2 assume the procedure between RAN and CN/OAM/UDM reuses existing procedure of immediate MDT, and the procedure between RAN and UE is common to both OAM centric and gNB centric data collection.

Proposal 2: To strive for a common measurement framework, similar UE consent mechanism in MDT is supported in gNB-centric data collection, i.e. UE consent is required before NW configures the UE gNB-centric data collection. FFS signaling details.

Proposal 3: MeasConfig in RRCReconfiguration is reused to configure logged L1 measurements with below specification changes to integrate L1 measurement logging:

- Within MeasObjectNR: introduce configuration on CSI resource (e.g. reuse csi-ResourceConfig list of CSI-MeasConfig) and logging configuration (e.g. the logging interval).

- Within ReportConfigNR: introduce a L1 measurement reporting configuration (e.g. L1ReportConfig) to indicate at least the reporting quantity configuration (e.g. L1 RSPR and/or beam index) and reporting type (e.g. periodical reporting).

Proposal 4a: Upon reception of configuration, the UE starts L1 measurements (L1 RSRP and/or beam index) and store the measurements with logging time stamp in an AS buffer.

Proposal 4b: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

Proposal 5: Similar to T330 mechanism of logged MDT, the UE can be configured with a logging duration and the UE is allowed to terminate the L1 measurement logging after the configured duration expires.

Proposal 6: Support the following event reporting of logged L1 measurement:

- AS buffer based event reporting (e.g. when UE’s AS buffer to store L1 measurement logging is full)

- Radio condition based event reporting (e.g. reuse legacy A1/A2 event)

Proposal 7: Support NW requested reporting of logged L1 measurement by reusing RRC message pair UEInformationRequest / UEInformationResponse.

Proposal 8: If the UE autonomously enters RRC_IDLE/RRC_INACTIVE state (e.g. expiry of T319) and has logged L1 measurement stored in its AS buffer, it notifies NW on the stored L1 measurement when it returns to RRC_CONNECTED state.

Proposal 9: To allow reporting logged L1 measurements in a single RRC message, support UL segmentation for MeasurementReport and UEInformationResponse.

AI/ML based positioning

Proposal 10: For NW-sided model of case 2b of AI/ML based positioning, extend LPP to report dataset from UE to LMF for model training.

Proposal 11: For NW-sided model of case 3b of AI/ML based positioning, extend NPPRa to report dataset from gNB to LMF for model training.

Proposal 12: For NW-sided model of case 3a of AI/ML based positioning, it doesn’t need to enhance data collection because there is no dataset transfer between different entities.

# 4 References

[1] RP-234039, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#125b, Chair Notes.

[3] RAN2#126, Chair Notes.

[4] TS 32.422-v17.8.0, Trace control and configuration management, 2022-9.

[5] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[6] TS 38.306-v18.0.0, User Equipment (UE) radio access capabilities, 2023-12.

[7] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.

[8] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1.

[9] RAN1#116b, Chair Notes.