# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406674
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.1.4

Source:	Apple

Title:	Further discussion on UE-sided data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

The Rel-19 WID of AI/ML for air interface (WID RP-234039) was agreed in RAN#102 [1], the WI objective on UE-sided data collection is copied below:

This topic was discussed in RAN2#125b [2] and RAN2#126 [3]. However, this topic was still controversial. Only below agreements were made, and one table was agreed for further discussion in post-email discussion:

Agreements

- Solution 1a has no specification impact.

Agreements

For the options identified to realize the different levels of data content visibility

1.	Full visibility for standardized data content.

2.	Partial visibility for partially standardized data content.

3.	No standardized visibility

NOTE in the TR that RAN2 discussed that visibility can be achieved as per SLA only but is outside of the scope of our discussions.

Agreements

Capture table below in TR (Rapporteur will clean up and refine and review over email)

Capture the privacy concerns from different stakeholders as informative annexes in the TR. All the options (1a, 1b, 2, 3) should respect those privacy concerns. Details are up to TR rapporteur.

The table was further discussed in post-meeting email discussion, and the TR 38.843 was endorsed [4]. However, a lot of FFSs are left.

In this contribution, we further discuss UE-sided data collection, including the FFSs in the TR 38.843 [4] and RAN2 conclusion.

# 2 Discussion

We first discuss the remaining issues, especially the FFSs left in TR 38.843 [4], and then share our view on the study conclusion.

## 2.1 Remaining issues in TR 38.843

FFS#1: UP tunnel for option 2/3

We have 4 related FFSs in the table:

| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| --- | --- | --- | --- | --- |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |

We are not sure how UP solution works for both option 2 and 3. According to current NR definition of UP tunnel, it is between UE and UPF, i.e. the terminated entity is UPF in UL. It seems to be conflicted with option 2 (3GPP terminated entity is one NF in CN) and option 3 (3GPP terminated entity is OAM). We believe a new Network Architecture and new protocol stack is not in Rel-19 scope. Thus, we propose:

Proposal 1: On the “FFS: UP tunnel” for option 2 and 3, RAN2 confirm that UP solution does not work for option 2 and 3 (i.e., only CP tunnel is considered in option 2 and 3), and thereby remove the 4 FFSs.

FFS#2: LPP for option 2

We have one FFS in the table:

| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| --- | --- | --- | --- | --- |

We understand that LPP is one kind of CP tunnel solution, and the confusion is caused by the wording “NAS signaling”. Thus, we can modify “NAS signaling” to “NAS/LPP signaling” and thereby the FFS can be removed. Thus, we propose:

Proposal 2: On the “FFS: LPP” for option 2, modify “NAS signaling” to “NAS/LPP signaling” and thereby the FFS can be removed.

FFS#3: level of controllability for option 1b

We have one FFS in the table:

| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| --- | --- | --- | --- | --- |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |

As option 1b is agreed to use UP tunnel and application layer protocol, we don’t think option 1b has controllability in RAN level and it should be same as option 1a.

Observation 1: Because option 1b is agreed to use UP tunnel and application layer protocol, option 1b doesn’t have controllability in RAN level and it should be same as option 1a.

Thus, we propose to modify from “Controllability can be achieved.” to “No controllability in RAN”. In consequence, the “Example: per PDU sessions” in row of “Solution for network controllability” should also be removed.

Proposal 3: On the “FFS: level of controllability” for option 1b, as option 1b is agreed to use UP tunnel and application layer protocol, “Controllability can be achieved” should be modified to “No controllability in RAN”, and thereby the FFS and the “Example: per PDU sessions” in row of “Solution for network controllability” are removed.

FFS#4: impact to other layers for option 2

We have one FFS in the table:

| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| --- | --- | --- | --- | --- |

Please note that the table has agreed to capture a NOTE 4 to clarify that the impacts to higher layer depend on other WGs. Thus, this FFS can be removed.

Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion.

Observation 2: Because the table has agreed to capture a NOTE 4 to clarify that the impacts to higher layer depend on other WGs, all the FFSs related to higher layer impacts should be removed.

Meanwhile, similar to the discussion of Proposal 2, we think “Via NAS procedure” should be modified to “Via NAS/LPP procedure”. Thus, we propose:

Proposal 4: On the “FFS: impact to other layers” for option 2, remove the FFS and modify “Via NAS procedure” to “Via NAS/LPP procedure”.

FFS#5: Visibility of data content in MNO and Data format for option 1b

We have one FFS in the table:

| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| --- | --- | --- | --- | --- |

Please note that RAN2 only agreed below 3 levels of data content visibility and captured them in Note 3.

Agreements

For the options identified to realize the different levels of data content visibility

1.	Full visibility for standardized data content.

2.	Partial visibility for partially standardized data content.

3.	No standardized visibility

NOTE in the TR that RAN2 discussed that visibility can be achieved as per SLA only but is outside of the scope of our discussions.

As option 1b is agreed to use UP tunnel and application layer protocol, we think option 1b should be classified as “No standardized visibility” which is same as option 1a. During online discussion, there was some argument that option 1b has full data content visibility according to the definition of visibility in TS 26.532 [5]. However, TS 26.532 is specification about API whose definition of visibility should not be mixed with RAN2 defined 3 levels of data content visibility.

Observation 3: There was some argument that option 1b has full data content visibility according to the definition of visibility in TS 26.532. However, TS 26.532 is specification about API, and thereby the definition of visibility is from API perspective and should not be mixed with RAN2 defined 3 levels of data content visibility.

Meanwhile, please note that the table has agreed to capture a NOTE 4 to clarify that the impacts to higher layer depend on other WGs. Thus, the definition of visibility in TS 26.532 should not be discussed in RAN2.

Observation 4: As the table has agreed to capture a NOTE 4 to clarify that the impacts to higher layer depend on other WGs. Thus, the definition of visibility in TS 26.532 should not be discussed in RAN2.

Based on observation 3 and observation 4, we propose RAN2 to capture “No standardized visibility” for option 1b.

Proposal 5: On the FFS related to visibility of data content in MNO and Data format for option 1b, because the table has agreed to capture a NOTE 4 to clarify that the impacts to higher layer depend on other WGs, RAN2 capture “No standardized visibility” for option 1b.

FFS#6: Partial visibility of data for option 2 and 3

The related FFSs are highlighted below:

| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| --- | --- | --- | --- | --- |

In legacy NR, only standardized data / measurement is specified with clear RAN4 requirement. As we know, the content of standardized data is full visible to MNO, and non-standardized data is generally not in scope of 3GPP. So, the concept of “partial visibility” is confusing. According to offline / online discussion, we think there are two below solutions proposed to achieve “partial visibility”:

- Solution 1: introduce a container in RRC message to include non-standardized data

- Solution 2: introduce Service Level Agreement (SLA) between MNO and UE for non-standardized data

However, we are not convinced by either above solution due to below reasons:

- For both solutions, it is not clear which data is included in non-standardized data and whether these data include proprietary implementation and UE privacy information.

  - As both chipset vendor and OEM vendor, we have concern that non-standardized data include proprietary implementation and UE privacy information.

- In 3GPP, all standardized data / measurement is specified with clear RAN4 requirements to ensure the data reported by the UE is valid for the NW. However, we doubt how to specify RAN4 requirements for non- standardized data in both solutions.

- For solution 2, SLA is out of RAN2 scope as clarified in below agreement made in RAN2#126 [3]. Thus, RAN2 should not conclude that SLA can achieve “partial visibility”.

NOTE in the TR that RAN2 discussed that visibility can be achieved as per SLA only but is outside of the scope of our discussions.

Observation 5: As non-standardized data is generally not in scope of 3GPP, the concept of “partial visibility” is confusing, and it leads to below concerns:

- It is not clear which data is included in non-standardized data and whether these data include proprietary implementation and UE privacy information.

- It is not clear how to specify RAN4 requirements for non- standardized data in both solutions to ensure validity for NW.

- SLA is out of RAN2 scope as clarified in RAN2#126, and thereby RAN2 should not conclude that SLA can achieve “partial visibility”.

Based on above concern, we believe that it is premature to capture “partial visibility” in the table. So, we think it is sufficient to capture “Full visibility for standardized data content” for option 2 and 3 and remove all the FFSs on partial visibility.

Proposal 6: On the FFSs related to Partial visibility of data for option 2 and 3, it is sufficient to capture “Full visibility for standardized data content” for option 2 and 3 and all these FFSs are removed because the definition of partial visibility of data contents and how to achieve it in 3GPP protocol stack are not clear.

FFS#7: Whether SA2 need to be involved for option 3

The related FFS is highlighted below:

| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| --- | --- | --- | --- | --- |

As we propose to remove the FFS on UP solution for option 3 in Proposal 1, we think SA2 don’t need to be involved for option 3. Thus, we propose:

Proposal 7: On the FFSs whether to involve SA2 for option 3, if RAN2 agree to only consider CP tunnel for option 3, this FFS can be removed (i.e. SA2 do not need to be involved for option 3).

FFS#8: FFS in Note 1

The related FFS in Note 1 is highlighted below:

- Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed.

We didn’t see any valid proposal to modify the definition from other companies, and so think this FFS can be removed.

Proposal 8: Remove the FFS in Note 1 (i.e. “FFS if further refinements/modifications to this definition are needed”).

FFS#9: FFS in Note 2

The related FFS in Note 2 is highlighted below:

- Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data).

We think this FFS can be removed because the intention of MNO to modify the collected data is not clear.

Proposal 9: Remove the FFS in Note 2 (i.e. “FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data”).

Meanwhile, we think it is necessary to clarify that this definition of “visibility of data collection” is only for RAN2 study purpose. It should not be regarded as requirement for other WGs.

Proposal 10: Add below clarification for Note 2:

- Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. It is only for RAN2 study purpose. It should not be regarded as requirement for other WGs. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data).

FFS#10: FFS on the “need for control” as MNO concern

The related FFS is highlighted below:

MNO: - Network Information Disclosure: MNOs may inadvertently disclose sensitive network information, such as deployment strategies, network configurations, and performance metrics, to servers outside their network. - Data Transfer Risks: MNOs are obliged to protect private and/or undisclosed information about the network of users/customers when transferring data to external servers. This information may be disclosed/leaked without authorization. This could include subscriber identities, locations, website visited, phone calls, etc. - Regulatory Restrictions: Operators are bound by regulations which mandate the protection of customer data. Thus, any lack of control over data may lead to unwanted exposure of personal information. Non-compliance with regulatory guidelines due to improper data handling could result in significant fines or restrictions for the operators. - Need for Control: FFS.

We understand “Controllability of MNO on data transfer” is only one evaluation aspect of UE-sided data collection solution, but it is not a concern. Thus, we propose to remove this FFS.

Proposal 11: Remove the FFS on “need for control” in MNO concern because “Controllability of MNO on data transfer” is one evaluation aspect of UE-sided data collection solution but it is not a concern.

FFS#11: FFS whether according to RAN2 there might be 3GPP specification impact for option 1b/2/3

For option 1b and 2, we don’t foresee RAN impacts. Meanwhile, since the table has agreed to capture a NOTE 4 to clarify that the impacts to higher layer depend on other WGs, we think the FFS can be removed. For option 3, we think RAN2 may discuss whether data volume remains within the RRC signalling capacity. If RAN2 conclude it is within existing RRC signaling capability, we also don’t foresee RAN impacts.

Observation 6: No RAN impacts are foreseen for option 1b and 2. For option 3, if data volume remains within the RRC signalling capacity, No RAN impacts are foreseen as well.

Thus, we propose:

Proposal 12: On the FFS whether according to RAN2 there might be 3GPP specification impact for option 1b/2/3, RAN2 capture below conclusion in TR 38.843:

- No RAN impacts are foreseen for option 1b and 2.

- If data volume remains within the RRC signaling capacity, no RAN impacts are foreseen.

Based on discussion in this section, we provide the updated table in Appendix.

## 2.2 Discussion on study conclusion

Among all the identified 4 solutions, we think at least option 1a is feasible for Rel-19 because this solution has been widely deployed in industry.

Observation 7: On UE-sided data collection, option 1a (i.e. OTT 3GPP transparent) has been widely deployed in industry.

Thus, RAN2 can conclude option 1a is feasible.

Proposal 13: RAN2 conclude that option 1a (i.e. OTT 3GPP transparent) is feasible.

Then for option 1b/2/3, we are not sure why RAN2 need to enhance solution 1a with MNO controllability for training data transfer and visibility of data content. Our view is:

- For all the 3 identified AI/ML use cases identified by RAN1 (i.e. beam management, positioning, and CSI enhancement), we don’t see any blocking issue to use solution 1a to implement UE-sided training data collection.

- One may argue that 3rd party may help train UE-sided model (e.g. CN NF or sever from 3rd party). However, which model(s) UE can run is hardware/firmware dependent (e.g. power status, UE memory). At least in Rel-19, we believe only UE vendor can ensure the performance of UE-sided model because 3rd party is not aware of one UE’s specific hardware/firmware environment.

Observation 8: For all the 3 identified AI/ML use cases identified by RAN1 (i.e. beam management, positioning, and CSI enhancement), we don’t see any blocking issue to use solution 1a to implement UE-sided training data collection.

Observation 9: At least in Rel-19, only UE vendor can ensure the performance of UE-sided model because 3rd party is not aware of one UE’s specific hardware/firmware environment.

Based on above analysis, we think before making conclusion of option 1b/2/3, RAN2 need to first try to achieve consensus why UE-sided data collection needs to be enhanced with MNO controllability for training data transfer and visibility of data content.

Proposal 14: Before making conclusion of option 1b/2/3, RAN2 discuss why UE-sided data collection needs to be enhanced with MNO controllability for training data transfer and visibility of data content, including any blocking issue of solution 1a for RAN1 identified AI/ML use cases and how much system benefit the enhancement can achieve.

# 3 Conclusion

In this contribution, we further discuss UE-sided data collection. Our observations are:

Observation 1: Because option 1b is agreed to use UP tunnel and application layer protocol, option 1b doesn’t have controllability in RAN level and it should be same as option 1a.

Observation 2: Because the table has agreed to capture a NOTE 4 to clarify that the impacts to higher layer depend on other WGs, all the FFSs related to higher layer impacts should be removed.

Observation 3: There was some argument that option 1b has full data content visibility according to the definition of visibility in TS 26.532. However, TS 26.532 is specification about API, and thereby the definition of visibility is from API perspective and should not be mixed with RAN2 defined 3 levels of data content visibility.

Observation 4: As the table has agreed to capture a NOTE 4 to clarify that the impacts to higher layer depend on other WGs. Thus, the definition of visibility in TS 26.532 should not be discussed in RAN2.

Observation 5: As non-standardized data is generally not in scope of 3GPP, the concept of “partial visibility” is confusing, and it leads to below concerns:

- It is not clear which data is included in non-standardized data and whether these data include proprietary implementation and UE privacy information.

- It is not clear how to specify RAN4 requirements for non- standardized data to ensure validity for NW.

- SLA is out of RAN2 scope as clarified in RAN2#126, and thereby RAN2 should not conclude that SLA can achieve “partial visibility”.

Observation 6: No RAN impacts are foreseen for option 1b and 2. For option 3, if data volume remains within the RRC signalling capacity, No RAN impacts are foreseen as well.

Observation 7: On UE-sided data collection, option 1a (i.e. OTT 3GPP transparent) has been widely deployed in industry.

Observation 8: For all the 3 identified AI/ML use cases identified by RAN1 (i.e. beam management, positioning, and CSI enhancement), we don’t see any blocking issue to use solution 1a to implement UE-sided training data collection.

Observation 9: At least in Rel-19, only UE vendor can ensure the performance of UE-sided model because 3rd party is not aware of one UE’s specific hardware/firmware environment.

Based on above analysis, we have the following proposals we also provide the updated table in Appendix.

Remaining issues in TR 38.843

Proposal 1: On the “FFS: UP tunnel” for option 2 and 3, RAN2 confirm that UP solution does not work for option 2 and 3 (i.e., only CP tunnel is considered in option 2 and 3), and thereby remove the 4 FFSs.

Proposal 2: On the “FFS: LPP” for option 2, modify “NAS signaling” to “NAS/LPP signaling” and thereby the FFS can be removed.

Proposal 3: On the “FFS: level of controllability” for option 1b, as option 1b is agreed to use UP tunnel and application layer protocol, “Controllability can be achieved” should be modified to “No controllability in RAN”, and thereby the FFS and the “Example: per PDU sessions” in row of “Solution for network controllability” are removed.

Proposal 4: On the “FFS: impact to other layers” for option 2, remove the FFS and modify “Via NAS procedure” to “Via NAS/LPP procedure”.

Proposal 5: On the FFS related to visibility of data content in MNO and Data format for option 1b, because the table has agreed to capture a NOTE 4 to clarify that the impacts to higher layer depend on other WGs, RAN2 capture “No standardized visibility” for option 1b.

Proposal 6: On the FFSs related to Partial visibility of data for option 2 and 3, it is sufficient to capture “Full visibility for standardized data content” for option 2 and 3 and all these FFSs are removed because the definition of partial visibility of data contents and how to achieve it in 3GPP protocol stack are not clear.

Proposal 7: On the FFSs whether to involve SA2 for option 3, if RAN2 agree to only consider CP tunnel for option 3, this FFS can be removed (i.e. SA2 do not need to be involved for option 3).

Proposal 8: Remove the FFS in Note 1 (i.e. “FFS if further refinements/modifications to this definition are needed”).

Proposal 9: Remove the FFS in Note 2 (i.e. “FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data”).

Proposal 10: Add below clarification for Note 2:

- Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. It is only for RAN2 study purpose. It should not be regarded as requirement for other WGs. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data).

Proposal 11: Remove the FFS on “need for control” in MNO concern because “Controllability of MNO on data transfer” is one evaluation aspect of UE-sided data collection solution but it is not a concern.

Proposal 12: On the FFS whether according to RAN2 there might be 3GPP specification impact for option 1b/2/3, RAN2 capture below conclusion in TR 38.843:

- No RAN impacts are foreseen for option 1b and 2.

- If data volume remains within the RRC signaling capacity, no RAN impacts are foreseen.

Discussion on study conclusion

Proposal 13: RAN2 conclude that option 1a (i.e. OTT 3GPP transparent) is feasible.

Proposal 14: Before making conclusion of option 1b/2/3, RAN2 discuss why UE-sided data collection needs to be enhanced with MNO controllability for training data transfer and visibility of data content, including any blocking issue of solution 1a for RAN1 identified AI/ML use cases and how much system benefit the enhancement can achieve.

# 4 References

[1] RP-234039, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#125b, Chair Notes.

[3] RAN2#126, Chair Notes.

[4] R2-24xxxxx, RAN2 inputs to TR 38.843, Ericsson, 2024-6-8.

[5] TS 26.532, “Data Collection and Reporting; Protocols and Formats”.

# Appendix: updated TR 38.843 with our proposals

NEXT CHANGE

##### 7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). No RAN impacts are foreseen. FFS whether according to RAN2 there might be 3GPP specification impact.

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. No RAN impacts are foreseen. FFS whether according to RAN2 there might be 3GPP specification impact.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server. If data volume remains within the RRC signaling capacity, no RAN impacts are foreseen. FFS whether according to RAN2 there might be 3GPP specification impact.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc. The result of this analysis can be found below in Table 7.2.1.3.2-1.

It is worth noting that for the different options, the data content visibility was discussed. The different levels of data content visibility are captured in the Note 3 in the Table 7.2.1.3.2-1.

Note:	RAN2 discussed that, except for the case of standardized data content, the data content visibility could be achieved via SLA (Service Level Agreement). However, SLA is out of RAN2 scope.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS/LPP signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS/LPP layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. No controllability in RAN. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS/LPP procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS No standardized visibility | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. It is only for RAN2 study purpose. It should not be regarded as requirement for other WGs. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. |  |  |  |  |

Related to privacy, it has been stressed in RAN2 the importance that any potential mechanism to collect UE side data for model training purposes (including the options 1a, 1b, 2, 3 listed above) must comply with privacy protection regulations, requirements, laws and/or policies. An informative Annex is included at the end of this document capturing examples of privacy concerns for different stakeholders participating in the discussion.

NEXT CHANGE

Annex <Y>:
Informative Annex: Privacy concerns

This Annex compiles some examples of privacy concerns on data collection for UE-side model training raised by stakeholders during RAN2’s discussion.

MNO:

- Network Information Disclosure: MNOs may inadvertently disclose sensitive network information, such as deployment strategies, network configurations, and performance metrics, to servers outside their network.

- Data Transfer Risks: MNOs are obliged to protect private and/or undisclosed information about the network of users/customers when transferring data to external servers. This information may be disclosed/leaked without authorization. This could include subscriber identities, locations, website visited, phone calls, etc.

- Regulatory Restrictions: Operators are bound by regulations which mandate the protection of customer data. Thus, any lack of control over data may lead to unwanted exposure of personal information. Non-compliance with regulatory guidelines due to improper data handling could result in significant fines or restrictions for the operators.

- Need for Control: FFS.

Network Vendor:

- Sensitive Information Leakage: Network vendors may possess proprietary algorithms, system designs, and other intellectual property that are integral to their competitive edge. Unintentional disclosure of such information to third parties could undermine their market position and lead to potential legal issues.

- Implementation Details Exposure: The specific details of how network equipment is implemented, including software and hardware design, are crucial for maintaining the security and integrity of the network. If such information is disclosed, it could be exploited for malicious purposes or used by competitors to gain insights into the vendor's technology.

- Radio Topology and Settings Disclosure: Disclosing details such as radio topology and specific radio configurations should be prevented, because such information is sensitive and could affect operational security.

- Violation of user privacy regulation: The equipment of the network vendor may be used for collecting user’s data without getting approval/consent from the user in advance, and this behaviour may violate the local regulations and risks the sales of the equipment.

Chipset Vendor:

- Proprietary Technology Exposure: Chipset vendors develop specialized hardware and software that may contain trade secrets or patented technologies. There is a risk that the sensitive data could be exposed to a second vendor without the original chipset vendor's knowledge, which could compromise their competitive advantage and innovation.

- Respect for Implementation Secrecy: There is a universal understanding within the industry that chipset vendors often add proprietary layers on top of standardized specifications, and these unique implementations are critical for maintaining a diverse and successful ecosystem. The non-disclosure of such proprietary information is seen as essential for the continued success of industry standards.

OEM:

- User Information Disclosure: OEMs handle a vast amount of user data, including personal information, usage habits, and location data. There is a significant privacy concern if this information is disclosed to external entities without user consent, potentially violating privacy laws and damaging the OEM's reputation. OEMs are adamant that user data should not be shared with third-party entities without explicit and informed user consent, since a UE vendor might be legally bound by a data protection agreement with the end-user.

- Proprietary Technology Exposure: OEM vendors develop specialized hardware and software that may contain trade secrets or patented technologies. There is a risk that this information could be unintentionally disclosed to unauthorized parties, leading to privacy breaches. Another risk is that some sensitive data of an OEM vendor may be exposed to third parties without the knowledge of the OEM vendor.

END OF CHANGES