3GPP TSG-RAN WG2 Meeting #127	R2-2406675

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.7.1.1

Source:	Apple

Title:	On Responses to SA2 and RAN3 LS for XR

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

Both SA2 and RAN3 have sent LS with some questions that require RAN2 to reply, namely S2-2407351 [1] and R3-243957 [2] respectively. In this paper, we present our views on the possible responses from RAN2.

# Discussions

## Responses to SA2 LS (S2-2407351)

In [1], SA2 has asked RAN2, RAN3 and SA4 about the usefulness/feasibility of indicating dynamic changes of DL traffic periodicities:

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

We understand this is mainly related to DL traffic periodicity change. With such knowledge at the NG-RAN, we believe the NG-RAN can re-configure the DRX cycle accordingly to match the updated traffic periodicity, which allows the UE to wake up and sleep during the more precise intervals that reflect the DL traffic patterns. Hence, it seems to be beneficial from UE power saving point of view, as long as the gNB can use this information in a proper manner. However, whether this is feasible for the Application server to provide such information should be up to SA4.

Proposal 1: RAN2 should reply to SA2 that, in-band signaling for dynamic changes of traffic periodicity may be useful in terms of UE power saving, if the application server can provide the information and if NG-RAN can use such information properly.

Furthermore, SA2 has also conceived the option for the Application Server to provide the time to the next burst, and is seeking for the feedback from both SA4 and RAN2. For RAN2, SA2 is mainly asking about whether such information could be useful considering that there may be some jitter between NG-RAN and Application Server.

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

In general, we think more it could be useful for the NG-RAN to perform timely resource allocation, especially if the jitter is small (e.g. when the network loading is light). We believe that in certain network implementations, the impacts of jitter may be taken into account when using the information about the time to next burst. However, this is probably more relevant to RAN3 scope.

Proposal 2: RAN2 should reply to SA2 that, the information of the time to next burst may be useful. However, RAN3 is better positioned to evaluate whether jitter can be compensated by the NG-RAN when such information is used.

## Responses to RAN3 LS (R3-243957)

RAN3 has been discussing the feature of PSI-based SDU Discarding, which was defined by RAN2 during Rel-18. When UL congestion is detected, the gNB may send a MAC CE to the UE to activate PSI-based SDU Discarding on one or more uplink DRBs. In response, the UE applies different discard timers to packets belonging to PDU Set with different importances on these DRBs. Since this is controlled via MAC CE, RAN3 has questioned the expected UE behaviors in NR-DC scenarios where split bearer is configured and hence two MAC entities are involved:

RAN3 is currently discussing XR DC signaling enhancements for various bearer types. In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured. Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

In our view, split bearer itself is already a mean to cope with congestion. When the total data volume exceeds a threshold, the UE may submit PDCP PDU to either RLC entity corresponding to MN or SN, which enables offloading. Thus, when congestion is detected only on one leg (but not the other leg), the UE still have the chance to transmit packets on another leg that is not congested without applying a shorter discard timer. This would lead to a better user experience, as the UE does not discard packets early.

With this in mind, we think the UE should only activate PSI-based SDU discarding on a UL DRB, when both MN and SN have instructed it to activate such mechanism. If only one of the MN and SN has instructed the UE to activate PSI-based SDU discarding, the UE could continue to use the same discard timer for all packets.

Furthermore, a smart UE implementation may allow it to select the RLC entity for PDCP PDU submission when the data volume exceeds a threshold, based on which node has sent the MAC CE command to activate PSI-based discarding. For instance, if only MN has commanded the UE to activate PSI-based discarding, which implies congestion is present in MN, the UE may always submit PDCP PDU to the RLC entity corresponding to SN when data volume exceeds a threshold. Conversely, if only SN has commanded the UE to activate PSI-based discarding, which implies congestion is present in SN, the UE may always submit PDCP PDU to the RLC entity corresponding to MN when data volume exceeds a threshold. Hence, the impacts of congestion on either leg to the UL performance can be minimized.

Proposal 3: With Split Bearer, the UE should activate PSI-based discarding for a DRB, only when both MN and SN have instructed the UE to activate PSI-based discarding for this DRB (which implies UL congestion is present in both MN and SN).

# Conclusions

In this paper, we have presented our views on the LS from both SA2 and RAN3 for XR, and proposed the following:

Proposal 1: RAN2 should reply to SA2 that, in-band signaling for dynamic changes of traffic periodicity may be useful in terms of UE power saving, if the application server can provide the information and if NG-RAN can use such information properly.

Proposal 2: RAN2 should reply to SA2 that, the information of the time to next burst may be useful. However, RAN3 is better positioned to evaluate whether jitter can be compensated by the NG-RAN when such information is used.

Proposal 3: With Split Bearer, the UE should activate PSI-based discarding for a DRB, only when both MN and SN have instructed the UE to activate PSI-based discarding for this DRB (which implies UL congestion is present in both MN and SN).

# References

- S2-2407351, LS on FS_XRM Ph2, 3GPP SA WG2 Meeting #163, May 2024, Jeju, Korea.

- R3-243957, LS on UL PSI based PDU discarding in NR-DC, 3GPP TSG RAN WG3 Meeting #124, May 2024, Fukuoka, Japan.