3GPP TSG-RAN WG2 Meeting #127	R2-2406678

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.7.5

Source:	Apple

Title:	Views on RLC-AM Enhancements for Rel-19 XR

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For Rel-19 XR Phase 3, we have the following objective [1]:

-	Specify the following user plane enhancements [RAN2]

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

According to the agreements made in RAN2 #125bis, it is confirmed that RAN2 will consider the following directions:

RAN2 #125bis Agreements: RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

For each of these directions, we have some extensive discussions during RAN2 #126 and have reached the following agreements:

RAN2 #126 Agreements on Timely RLC Retransmissions: To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic RAN2 #126 Agreements on Avoidance of Unnecessary Retransmissions: For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

This paper aims to provide some of our views on the directions that RAN2 has identified for RLC-AM enhancements.

# Discussions

## Timely Retransmission

To achieve faster retransmission, several companies have proposed that transmitter RLC entity may autonomously perform retransmission even if no feedback is received from the receiver. From the latency perspective, such scheme may be beneficial as a RLC SDU can be retransmitted more proactively and quickly. However, there are also concerns on impacts to the system capacity especially in uplink scenarios, as the UE may not know the network status in general, blind retransmission may cause further congestions to other traffic flows and/or users.

In order to minimize the potential impacts of autonomous retransmission to the system capacity, we think RAN2 should only consider the enhancements of the existing mechanisms that are already well-established. It is undesirable and risky if autonomous retransmission is performed based on various types of new triggers that RAN2 has not extensively studied. Besides, given that the features of PDCP duplication and MAC/PHY repetitions are also available, this is questionable if there is a need to introduce any more brand-new mechanisms for such purposes.

We have noted that, the behaviours of autonomous retransmission (i.e. retransmission without feedback) based on the expiration of t-PollRetransmit has already been defined in TS 38.322:

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

It is clear that, the current specification has provided some guidelines about when a UE can trigger autonomous retransmission, and which RLC SDU (submitted to lower layer) can be selected for retransmission. Hence, in our opinions, enhancements for autonomous retransmission makes a lot of sense in terms of performance benefits and specification impacts. Nevertheless, there could be some counter-arguments to autonomous retransmission based on t-PollRetransmit, including:

- It may not be fast enough, as the transmitter still needs to wait for timer expiry.

- Reduce the timer value (e.g. based on network configuration) to trigger it more rapidly would lead to more frequent polling, which may result in a large number of STATUS PDU from the receiver that can also degrade capacity.

For (1), we must note that the value of t-PollRetransmit can be as short as 5ms, which should be sufficient for most cases, and introducing even smaller values does not incur too much specification complexity. For (2), we would like to clarify that, regardless of how many Polling are triggered and sent, the feedback of STATUS PDU is anyway constrained by t-StatusProhibit at the receiver side. According to TS 38.322, only one STATUS PDU will be sent even if it is triggered multiple times (e.g. by multiple polls) while t-StatusProhibit was running.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall: - if t-StatusProhibit is not running: - at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer. - else: - at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

With the considerations above, we think expiry of t-PollRetransmit should be the baseline for any further enhancement for autonomous retransmission triggering in Rel-19.

Proposal 1: The enhancements for autonomous retransmission should be based on the extensions of the existing trigger, i.e. the expiry of t-PollRetransmit.

Moreover, to make sure autonomous retransmission based on t-PollRetransmit can be triggered faster when there is a need, while minimizing autonomous retransmission that can unnecessarily impact capacity, we think RAN2 can consider adaptive t-PollRetransmit timer value to strike a balance. More specifically, the transmitter may apply different t-PollRetransmit timer values, in accordance with whether a delay-critical packet is present. If there is a packet that needs to be delivered urgently (e.g. the remaining of which is small), a t-PollRetransmit timer with a shorter duration can be used in a hope to trigger autonomous retransmission more rapidly. Conversely, if none of the packets in the queue (or waiting for acknowledgement) is considered delay-critical, the default t-PollRetransmit timer can be applied to minimize unnecessary triggering of autonomous retransmission.

Proposal 2: RAN2 can further discuss if different t-PollRetransmit timer values should be applied under different conditions relating to the presence of delay-critical RLC SDUs.

A key motivation to enable faster retransmission is to avoid discarding of packets with small delay budget, i.e. to ensure the packets can be delivered before expiry of their discard timer. Thus, we think it is reasonable to prioritize the RLC SDUs that are considered delay-critical (i.e. the remaining time till expiry of discard timer for the corresponding PDCP SDU has dropped below a threshold) when autonomous retransmission is triggered. For instance, when autonomous retransmission is triggered by the expiry of t-PollRetransmit, the UE should first consider delay-critical RLC SDUs that have been submitted to lower layers for retransmission.

Furthermore, if PSI-based discarding is activated, which implies that congestion is present, we think the UE may further take the importance of the corresponding PDCP SDU into account. That is, the UE may only consider the delay-critical RLC SDUs that belong to important PDU Sets for retransmission, when t-PollRetransmit is expired.

Proposal 3: Delay-critical RLC SDUs should be prioritized to be considered for retransmission upon expiry of t-PollRetransmit. FFS if PDU Set importance should also be taken into account when e.g. PSI-based discarding is activated.

## Avoidance/Minimization of Unnecessary Retransmission

Some RLC SDUs may be discarded after their initial transmission, and the transmitter may further consider these RLC SDUs for retransmission when e.g. negative acknowledgement for the RLC SDU is received, or when the t-PollRetransmit is expired. Apparently, the retransmission of these discarded RLC SDUs would not only result in radio resource wastage, but may also jeopardize the latency performance of other RLC SDUs that are still not in the transmission window. Some of these RLC SDUs may already be considered delay critical. To avoid unnecessary retransmission, RAN2 has identified both TX-initiated approach and RX-initiated approach.

Since discarding is performed by the PDCP layer of the transmitter, the TX-initiated approach is reasonable in a sense that the transmitter has full knowledge about when a packet is discarded, and so it can react in a timely manner. That is, the transmitter can immediately stop retransmission of any RLC SDU (or its segments) when the corresponding PDCP SDU is discarded, and the window can be advanced straight after the packets with SN inside the window are discarded. Thus, any unnecessary retransmission can be timely avoided, and so the latency performance is optimized. Furthermore, the transmitter can immediately trigger a notification to the receiver to move the receive window as well.

On the other hand, with the RX-initiated approach, the receiver may need to rely on some mechanisms similar to Reordering timer to determine whether a packet has been discarded at the transmitter. Since such a timer may start at the time where a SN gap is identified by the receiver, it cannot accurately reflect the timing of when the packet is discarded by the transmitter. Note that the receiver does not have the knowledge about when a packet arrives in the buffer, what discard timer value it is running on (e.g. discardTimer or discardTimerForLowImportance), and when it is discarded. Moreover, as the receiver needs to further indicate the abandoned SDUs to the transmitter, there is an additional signaling delay before the transmitter can react. Consequently, some of the unnecessary retransmission of a discarded SDU may have already been performed before the transmitter can advance the window, so the latency and resource wastage may not be prevented in time.

In our understanding, the objective of this WI aims to cope with packets with small delay budget, so we should try to minimize the latency. From this perspective, we think it makes more sense for RAN2 to focus on TX-initiated approach, which seems to be more optimal for the scenarios targeted by the Work Item objective.

Proposal 4: RAN2 should prioritize TX-initiated approach for avoidance of unnecessary retransmission.

Alternatively, when the remaining time budget for a RLC SDU is already quite small, it is unlikely that the number of retransmissions for this RLC SDU would reach maxRetxThreshold before its delay budget runs out. Hence, a different (smaller) threshold for maximum number of retransmission (i.e. maxRetxThreshold ) may be applied to such RLC SDUs (or segments) to reduce the number of unnecessary retransmission. In particular, when RETX_COUNT reaches the alternative maxRetxThreshold (which is smaller than the default value of maxRetxThreshold ) for a delay-critical RLC SDU, the transmitter may stop performing retransmission of this RLC SDU and may directly consider it as positively acknowledged.

Proposal 5: RAN2 can consider an alternative (smaller) value of maxRetxThreshold for RLC SDUs (or segments) with remaining time smaller than a threshold. The RLC SDU can be directly considered as positively acknowledged when its RETX_COUNT reaches this alternative maxRetxThreshold.

# Conclusions

In this paper, we have presented our views on RLC-AM enhancements and proposed the following:

Proposal 1: The enhancements for autonomous retransmission should be based on the extensions of the existing trigger, i.e. the expiry of t-PollRetransmit.

Proposal 2: RAN2 can further discuss if different t-PollRetransmit timer values should be applied under different conditions relating to the presence of delay-critical RLC SDUs.

Proposal 3: Delay-critical RLC SDUs should be prioritized to be considered for retransmission upon expiry of t-PollRetransmit. FFS if PDU Set importance should also be taken into account when e.g. PSI-based discarding is activated.

Proposal 4: RAN2 should prioritize TX-initiated approach for avoidance of unnecessary retransmission.

Proposal 5: RAN2 can consider an alternative (smaller) value of maxRetxThreshold for RLC SDUs (or segments) with remaining time smaller than a threshold. The RLC SDU can be directly considered as positively acknowledged when its RETX_COUNT reaches this alternative maxRetxThreshold.

# References

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN #103, Maastricht, Netherland, March 2024.

- 3GPP TS 38.322, V18.0.0, January 2024.

- 3GPP TS 38.323, V18.1.0, April 2024.