# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406679
Masstricht, Netherland, August 19-23, 2024

Agenda item:	7.9.3

Source:	Apple

Title:	Discussion on remaining RRC issues for NR Sidelink relay enhancements

WID/SID:	NR_SL_relay_enh-Core – Release 18

Document for:	Discussion and Decision

# 1 Introduction

After ASN.1 frozen, there are still some remaining RRC issues, which has no ASN,1 impact, not concluded by the end of RAN2#126 meeting [1]. In this paper, we discuss some of the remaining issues.

# 2 Discussion

2.1 SLRB index in SUI for L2 U2U relay

In RAN2#125bis meeting[2], there is one remaining issue in the following agreement:

For an E2E SLRB, source remote UE configures the same value of SLRB index to Relay UE and target Remote UE.  No change is made now in relation to allowing relay UE setting different value of SLRB index in SUI; whether to allow this, and if there would be any spec impact to do so, can be discussed next meeting based on contributions.

According to the agreement “For an E2E SLRB, source remote UE configures the same value of SLRB index to Relay UE and target Remote UE”, it is obvious that the L2 U2U relay UE should know the SLRB index used by remote UE to represent the e2e bearer. One of the agreements is to allow L2 U2U relay UE to knows the end-to-end QoS flow to end-to-end SLRB mapping by conveying this mapping in UEInformaitonRequestSidelink message, as shown in Figure 1 below based on TS 38.331 [3]):

UEInformationRequestSidelink-r18-IEs ::= SEQUENCE {

sl-E2E-QoS-InfoListPC5-r18         SEQUENCE (SIZE (1.. maxNrofSLRB-r16)) OF SL-E2E-QoS-InfoPC5-r18 OPTIONAL, -- Need N

lateNonCriticalExtension                 OCTET STRING                                                             OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                                                              OPTIONAL

}

SL-E2E-QoS-InfoPC5-r18 ::=         SEQUENCE {

sl-DestinationIdentityRemoteUE-r18   SL-DestinationIdentity-r16,

sl-E2E-SLRB-Index-r18                SLRB-PC5-ConfigIndex-r16,

sl-QoS-InfoList-r18                  SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16

}

Figure 1:	End-to-end bearer’s SLRB-PC5-ConfigIndex shared in UEInformationRequestSidelink

Then, according to the latest Stage-2 procedure for Layer-2 UE-to-UE relay operation in TS 38.300 [4] (as shown in Figure 2), the relay UE SidelinkUEInformationNR reporting happens right after QoS split procedure (Step 7 and Step 8 in Figure 2)

As SUI reporting occurs between step 8 and step 9b, we can see that when SUI is triggered, the end-to-end PC5 SLRB has already been established and the L2 U2U relay UE has already known the SLRB ID of each of established end-to-end SL DRB, as those has already been conveyed earlier in the UEInfromationRequestSidelink message.

Figure 2: : Procedure for L2 U2U Remote UE connection establishment

Observation 1	L2 U2U relay UE is aware of the SLRB index (SLRB-PC5-ConfigIndex ) used by Source remote UE for the end-t-end SL DRB, when SUI reporting is triggered.

After sending the SUI message, the L2 U2U Relay UE will receive the RRCReconfiguration which includes the SRAP-configuration (sl-SRAP-ConfigU2U) needed for U2U relay operation. This configuration indicates for each end-to-end bearer ID, which relay RLC channel shall be used for forwarding the SRAP data packet, as cited below in TS 38.351 [5].

#### 5.3a.3.3 Egress RLC channel determination For an U2U SRAP Data PDU to be transmitted, the SRAP entity shall: - If the U2U SRAP Data PDU is for SRB (i.e., the BEARER ID field is 0/1/2/3): - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-U2U-RLC as specified in TS 38.331 [3]. - Else if the U2U SRAP Data PDU is for DRB: - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to SRAP configuration indicated by upper layer for the concerned bearer as specified in TS 38.331 [3].

Ideally, this SRAP configuration configured by NW can be indicated to the SRAP entity w/o any processing in the RRC layer of the L2 U2U relay UE, if the SLRB ID used in the SRAP configuration is exactly same as the BEARER ID (i.e. SLRB-PC5-ConfigIndex) used in SRAP header of SRAP PDU.

Observation 2	SRAP entity of L2 U2U relay UE relies on RRC configuration to indicate SRAP mapping between e2e SLRB and egress PC5 Relay RLC channel.

Based on the above two observations, it is very unclear that why the L2 U2U relay UE want to choose a different SLRB ID to report to the gNB in SUI message, when it is triggered to report the per-SLRB QoS. Using a different SLRB ID means the relay UE will take two additional steps to convert SLRB ID of e2e bearer to a new ID value to be used in SUI message, and then convert it back to the prior value after receiving SRAP configuration, as shown in Figure 3 with the step-by-step processing flow describing the related relay UE behavior (i.e., between Step 7 and Step 9b in Figure 2).

Figure 3: : Processing flow for L2 U2U Relay UE in regards of SUI transmission

Note that the two steps in shaded dotted box above are completely unnecessary if the L2 U2U relay UE always reports the same SLRB index in SUI as it is received from PC5 interface from Source Remote UE. Therefore, we can see allowing relay UE setting different value of SLRB index in SUI adds unnecessary overhead and complexity in L2 relay UE. Moreover,

Observation 3	Allowing relay UE setting different value of SLRB index in SUI adds two unnecessary UE processing steps: converting BEAER ID into a new ID and then converting it back.

If relay UE is allowed to set different value of SLRB index in SUI message, then two above steps need to be clearly specified in Rel-18 RRC spec to avoid Relay UE implementation errors. On the contrary, if the relay UE always set the same value, then the above text in TS 38.351 [5] can be applied properly w/o additional changes in RRC specification.

Observation 4	Allowing relay UE setting different value of SLRB index in SUI has more spec impact than not allowing it.

Finally, the relay UE and remote UE cases are different. For the remote UE case, SLRB-Uu-ConfigIndex is created by gNB to provide SLRB configurations, before any SLRB is established via PC5 interface. According to R16 legacy procedure, the SLRB-PC5-CongfixIndex is allowed to be same or different, when a SLRB configuration from Uu RRC is used to configure SLRB in PC5-RRC message RRCReconfigurationSidelink. So, source remote UE simply reuse the same legacy procedure to covert between SLRB-Uu-ConfigIndex received from gNB and SLRB-PC5-ConfigIndex used in PC5. There is no any new R18 spec impact for L2 Remote UE. But for Relay UE case, the L2 U2U relay UE creating a new different index value to represent e2e SLRB in SUI is a completely new UE behavior, with both the above two steps in yellow-shade box are to be specified in RRC spec. ,

Observation 5	For Remote UE case, allowing different SLRB IDs are just legacy procedure w/o any new spec impact, whereas in Relay UE case, the SUI reporting of a new SLRB ID is a new UE behavior which needs to be specified.

The current procedure text for SUI transmission (section 5.8.3.3 of TS 38.331 [3]) only allows the same SLRB index to be set in SUI, as shown below:

#### 5.8.3.3 Actions related to transmission of SidelinkUEInformationNR message #### <text omitted> 5> include sl-U2U-InfoList and set its fields (if needed) for each entry as follows, to report the related information of the connected L2 U2U Remote UEs: 6> include the source L2 U2U Remote UE's L2 ID in sl-SourceUE-Identity; 6> include sl-PerSLRB-QoS-InfoList, with each entry including the per-SLRB second-hop QoS profile and the corresponding sl-RemoteUE-SLRB-Identity which is set to the same value as the SLRB-PC5-ConfigIndex received in RRCReconfigurationSidelink message from the L2 U2U Remote UE for the same end-to-end SLRB;

Based on the above analysis, we can see that the above procedure text in the current RRC CR makes sense, and has minimum spec impact. Therefore, we have the following proposal to close this issue:

Proposal 1	RAN2 concludes that L2 U2U Relay UE shall set the same value of SLRB index in SUI from what it received from remote UE. No more spec change is needed for this issue.

2.2 PC5 Relay RLC Channel & SRAP Mapping Setup

Based on the current text in TS 38.331 [3] below, the SRAP mapping creation for a DRB is included in the procedure text for “DRB addition/modification” procedure. For example, as shown below for the DRB addition case:.

5.8.9.1a.2.2 Sidelink DRB addition/modification operations For the sidelink DRB, whose sidelink DRB addition conditions are met as in clause 5.8.9.1a.2.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall: <text omitted> 1> for an end-to-end sidelink DRB (i.e. the UE is acting as L2 U2U Remote UE or L2 U2U Relay UE): 2> if the UE is in RRC_CONNECTED: 3> (re)configure the SRAP entity for the sidelink DRB, in accordance with the sl-SRAP-ConfigU2U included in sl-ConfigDedicatedNR, received from RRCReconfiguration; 2> else if the UE is in RRC_IDLE or RRC_INACTIVE, or out of coverage: 3> perform the PC5 Relay RLC channel addition/modification as specified in clause 5.8.9.7.2 if needed, according to the derived PC5 Relay RLC channel configuration as described in clause 5.8.9.11; 3> consider the PC5 Relay RLC channel applying the derived PC5 Relay RLC channel configuration as the egress PC5 Relay RLC channel; 3> configure the egress PC5 Relay RLC channel for this end-to-end sidelink DRB to SRAP; <text omitted>

According to the above text in current specification, upon DRB generation, the SRAP mapping for IDLE/INACTIVE/OCC UE is created after PC5 Relay RLC channel creation is done. However, this is not how UE implementation can be done because the UE need to trigger other procedures to meet the necessary conditions for SRAP generation. To be more specific, for an IDLE/INACTIVE/OOC UE, it need to do the following to satisfy the conditions corroding to the current spec:

- Trigger the UE information Transfer procedure specified in 5.8.9.11 to solicit relay UE to split QoS

- If necessary, triggers the PC5 Relay RLC channel addition/modification via RRCReconfigurationSidelink procedure, as shown in the text above.

Moreover, for a CONNETED L2 U2U remote UE or L2 U2U Relay UE, after QoS split, it needs to trigger SUI procedure to report the QoS and SLRB information to let NW to configures the SRAP mapping directly in RRCReconfiguration.

So, it is quite clear that the SRAP mapping cannot be configured when an end-to-end SL-DRB is added or modified.

Observation 6	There are extra Uu/PC5 signaling procedures to be executed between the time of end-to-end DRB addition/modification and the time when SRAP mapping generation is feasible.

Note that in Rel-17 U2N operation, the SRAP configuration from NW is applied to L2 U2N remote UE or L2 U2N relay UE directly without waiting for the creation of PC5/Uu Relay RLC channels. For example, as shown below for L2 Remote UE:

#### 5.3.5.16 L2 U2N or U2U Remote UE configuration The network configures the L2 U2N or U2U Remote UE with relay operation related configurations, e.g. SRAP configuration. The L2 U2N Remote UE shall: 1> if sl-L2RemoteUE-Config is set to setup or received from RRCSetup message: 2> if the sl-L2RemoteUE-Config contains the sl-SRAP-ConfigRemote: 3> if no SRAP entity has been established: 4> establish a SRAP entity as specified in TS 38.351 [66]; 3> configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRemote; <text omitted>

In lieu of this, we think the current text in 5.8.9.1a is not correct, it will create confusion about both the timing and condition of SRAP mapping generation in UE implementation. Logically, the L2 U2U remote UE or L2 U2U relay UE in CONNECTED state can apply SRAP configuration received from NW directly. Similarly, once the SL RLC channel configuration and its corresponding internal SL-RLC-channel-ID is derived based on SIB12 or preconfiguration and QOS requirements for each-hop, the SRAP mapping for an end-to-end DRB can also be established in IDLE/INACTIVE/OOC L2 U2U Remote/RelayUE, w/o waiting for the PC5 Relay RLC channel to be established by a PC5-RRC exchange.

Observation 7	The SRAP mapping generation for U2N/U2U UE can happen before PC5 Relay RLC channel addition/modification.

Therefore, we think the right procedure for SRAP configuration is done should be:

- 5.3.5.16 for RRC_CONNETED L2 U2U remote UE or L2 U2U Relay UE

- 5.8.9.11.4 for IDLE/INACTIVE/OCC L2 U2U remote UE and 5.8.9.11.3 for L2 U2U Relay UE.

Hence, we propose to correct the RRC spec to move the SRAP generation is to 5.8.9.11 for IDLE/INACTIVE (as the current text in 5.3.5.16 is sufficient for CONNECTED case).

Proposal 2	Specify the SRAP mapping configuration procedure for end-to-end SL-DRBs in clause 5.8.9.11 and 5.3.5.16 for IDLE/INACIVE/OCC case and CONNECTED case, respectively and remove corresponding text in 5.8.9.1a.2.

2.3 Processing of RRCReconfigurationCompleteSidelink

It is worth noting that the current procedure specified in clause 5.8.9.1.9 in TS 38.331[3]is quite insufficient if not completely useless, as shown below:

##### 5.8.9.1.9 Reception of an RRCReconfigurationCompleteSidelink by the UE The UE shall perform the following actions upon reception of the RRCReconfigurationCompleteSidelink: 1> stop timer T400 for the destination, if running; 1> consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied. 2> if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject: 3> consider no sidelink DRX to be applied for the corresponding sidelink unicast communication.

Although the UE shall “consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied” in this clause, the only specified processing in leve-2 bullet under this sentence is to process sl-DRX-ConfigReject. It gives wrong impression to readers that there is nothing significant to be triggered in the Tx UE side upon the reception of this message, except handing SL-DRX configuration reject.

Observation 8	The almost-empty clause 5.8.9.1.9 misleads the UE implementation in regards of processing of RRCReconfigurationCompleteSidelink message.

Nonetheless, there are many actions to be taken by the UE when the Tx UE of RRCReconfigruaitonSidleink message receives the response, such as:

- Addition/modification/release of SL-DRB

- Addition/modification/release of PC5 Relay RLC channel

- Triggering of SUI procedure to report relevant SL configurations to gNB

- Triggering the addition/modification/release of SRAP mappings for end-to-end SL DRBs.

- SL DRX configuration application

In the current RRC spec, the above Tx UE actions are spread in many different procedures, with the text like “ ….. after receiving the RRCReconfigurationCompleteSidelink message, …“ appearing in different subclauses, such as 5.8.9.1a.1, 5.8.9.1a.2, 5.8.9.1a.5, 5.8.9.1b and 5.8.9.7. In general, this is not a great way to draft high-standard specification. We suggest RAN2 to have a discussion on whether/how to consolidate the procedure texts from those sections into 5.8.9.1.9.

Proposal 3	RAN2 discuss whether/how to consolidate some procedure texts in 5.8.9.1a.1, 5.8.9.1a.2, 5.8.9.1a.5, 5.8.9.1b, 5.8.9.7 into 5.8.9.1.9 (Reception of RRCReconfigurationCompleteSidelink).

# 3 Conclusion

In this contribution, we discuss the remaining RRC issues, and have the following observations:

Observation 1	L2 U2U relay UE is aware of the SLRB index (SLRB-PC5-ConfigIndex ) used by Source remote UE for the end-t-end SL DRB, when SUI reporting is triggered.

Observation 2	SRAP entity of L2 U2U relay UE relies on RRC configuration to indicate SRAP mapping between e2e SLRB and egress PC5 Relay RLC channel.

Observation 3	Allowing relay UE setting different value of SLRB index in SUI adds two unnecessary UE processing steps: converting BEAER ID into a new ID and then converting it back.

Observation 4	Allowing relay UE setting different value of SLRB index in SUI has more spec impact than not allowing it.

Observation 5	For Remote UE case, allowing different SLRB IDs are just legacy procedure w/o any new spec impact, whereas in Relay UE case, the SUI reporting of a new SLRB ID is a new UE behavior which needs to be specified.

Observation 6	There are extra Uu/PC5 signaling procedures to be executed between the time of end-to-end DRB addition/modification and the time when SRAP mapping generation is feasible.

Observation 7	The SRAP mapping generation for U2N/U2U UE can happen before PC5 Relay RLC channel addition/modification.

Observation 8	The almost-empty clause 5.8.9.1.9 misleads the UE implementation in regards of processing of RRCReconfigurationCompleteSidelink message.

Then, we have the following proposal:

Proposal 1	RAN2 concludes that L2 U2U Relay UE shall set the same value of SLRB index in SUI from what it received from remote UE. No more spec change is needed for this issue.

Proposal 2	Specify the SRAP mapping configuration procedure for end-to-end SL-DRBs in clause 5.8.9.11 and 5.3.5.16 for IDLE/INACIVE/OCC case and CONNECTED case, respectively and remove corresponding text in 5.8.9.1a.2.

Proposal 3	RAN2 discuss whether/how to consolidate some procedure texts in 5.8.9.1a.1, 5.8.9.1a.2, 5.8.9.1a.5, 5.8.9.1b, 5.8.9.7 into 5.8.9.1.9 (Reception of RRCReconfigurationCompleteSidelink).

# 4 References

[1] Chairman’s Notes. RAN2#126

[2] Chairman’s Notes. RAN2#125bis

[3] TS 38.331 v18.2.0

[4] TS38.300 v18.2.0

[5] TS 38.351, v18.2.0