# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406681
Masstricht, Netherland, August 19-23, 2024

Agenda item:	8.2.3

Source:	Apple

Title:	Discussion on Ambient IoT Paging

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1 Introduction

The following agreements have been reached during RAN2#125bis [1]:

- Legacy paging message for device will not be supported. - Legacy paging occasion and legacy DRX for the device is not supported. This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well). - RAN2 assumes that the device will not support tracking/RAN area update procedure. - For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2

And the following agreements has been reached during RAN2#126 [2]:

- RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. - From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In this paper, we discuss the remaining issues on A-IoT paging design.

# 2 Discussion

### 2.1 Scope of A-IoT Paging in Stage 2 procedure

RAN2 has determined that for “inventory only” and “inventory + command” case, the procedure will start with paging (triggering message), but there is some controversy about so-called “command only” case. For this case, RAN2 will study the following two options:

- Option 1: Initial trigger message from the reader contains the command.  Final feasibility depends on SA2 and SA3 work/conclusions.

- Option 2: Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

For option 1, the so-called “trigger” message intends to deliver DL R2D data (“command”) directly to the device w/o paging it first, then we think this is more part of “data transmission” procedure rather than paging or inventory procedure. This is because paging is assumed equivalent to a mere trigger of device access. But “command/data” could be DT or DO-DTT. If this is DT, it does not necessarily trigger any device access. The device is only triggered if DL R2D data belongs to DO-DTT type, e.g., a “read” command which expects some data to be sent by A-IoT device as response.

To maintain clarity of terms used in A-IoT system design, we propose to first clarify the scope of paging design does not include the very first message used in “command only” Option 1. However, some mechanism designed for paging procedures can be also reused by “command + response” procedure with some necessary modifications.

Proposal 1	The scope of A-IoT paging procedure is limited to the inventory; “Command + response” transaction needs to be discussed under data transmission procedure.

### 2.2 D2R scheduling in Paging message

It has been agreed in RAN2#126 [2] that

AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

Regarding the FFS point above, we notice that RAN1 has a general agreement on D2R scheduling as below:

Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

Based on the above RAN1 agreement, we believe at least some scheduling information is to be provided by the R2D transmission. Given the options being discussed in RAN1 meeting [3], D2R transmission’s timing, in general, should be relative to the transmission timing of R2D message. Therefore, for D2R response scheduling, we need consider the following information:

- Resource type (exclusive or shared)

- Start time of (as relative to the R2D message)

- Time duration of the D2D transmission

- Frequency information (RBs, frequency offset) in PDRCH channel

The TB size of A-IoT is limited. It is worth nothing that the paging message itself may contain multiple identifiers as agreed in RAN2#126. Also, the length of a A-IoT device identifier can be as long as 128bit, and additional prefix/mask information may need to be included to identify a group of devices. Thus, there may not be enough room for a D2R transmission to contain too many R2D scheduling information in a DL MAC PDU. In particular, paging may trigger contention-based access (e.g., slotted aloha), which need multiple candidate D2R resources to be used by the intended paging recipients to choose from. So, we think it would be unreasonable to assume the scheduling of D2R transmissions are solely rely on the on-the-fly scheduling information in PDRCH transmission. Some of the scheduling information must be (pre-)configured instead of explicitly indicated in the paging message itself.

Proposal 2	RAN2 discuss which scheduling information should be (pre-)configured for D2R response scheduling instead of explicitly included in the paging message.

Regarding the resource type indication (exclusive vs. shared) for D2R scheduling in paging message, we think this can be indicated in an efficient manner by assuming each resource is shared by default. The reason for this design is that the contention-based RA is usually triggered by a paging message intends to reach a group of devices. As a result, more candidate “shared” resources needs to be scheduled in the paging message. If there is a way to avoid spending precious information bits to indicate the resource type, it would be beneficial. Thus, it is proposed to assume the resource scheduled in paging message by default is “shared” unless explicitly indicated as a contention-free resource exclusively to be used by one intended A-IoT device.

Proposal 3	D2R resource indicated in Paging message is by default shared resource, unless explicitly indicated as contention-free resource.

Finally, the paging message is not the only R2D message to be transmitted by the reader in PRDCH channel. It is also possible that some other R2D messages can contain corresponding D2R scheduling information. For example, A-IoT Msg2 in RA procedure may contain the scheduling information for A-IoT Msg3. We think the scheduling design for D2R transmissions in A-IoT system should follow a unified solution applicable to all R2D messages. There is no need to design different scheduling schemes for A-IoT paging and other A-IoT R2D message(s).

Proposal 4	D2R scheduling mechanism in PRDCH is common for both Paging message and other DL R2D messages.

### 2.3 Multiple Paging rounds

For A-IoT paging design, one thing to consider is that the “success” of A-IoT paging is not guaranteed, and the performance of paging could be very poor if the paging message is just transmitted one-shot. This is due to the fact that the reachability of the A-IoT devices are directly affected by the energy harvesting status of the devices. Also, the success ratio for contention-based access is relatively low (e.g., slotted aloha has a known efficiency of 0.36 at best and much worse when contention is higher). It is also worth noting that when paging is triggered (i.e., for inventory use case), the reader does not know the exact number of devices to be triggered in its coverage, so it is very hard to pre-allocate enough slotted resources for accommodate all access requests. As a result, some intended paging recipients may not be able to get the chance to respond to paging message.

To overcome this problem and bring the paging performance to an acceptable level, it is important to address this in AS layer design. To be more specific, RAN2 can consider to introduce a reader-initiated repetition of paging. With this paging method, a Paging procedure can be done in multiple rounds. During each round, the devices will be provided fresh D2R scheduling (e.g., resources) to access (again).

Proposal 5	An A-IoT Paging procedure involves multiple rounds of transmission of paging message.

First, to make the repeated paging scheme work, the system design need to make sure that not all the Ambient IoT devices will be triggered again and contend again in random access procedure. For example, the A-IoT device which has already responded to the same paging message transmitted earlier shall not try to respond again by a retransmission of the same paging message. For the sake of distinguishing repeated transmission of paging message, a paging transaction identifier is needed in the paging message, so that the devices at least can recognize the retransmission of the paging message with the same transaction identifier.

Proposal 6	If paging message is to be repeated, initial and retransmission of the paging message share the same transaction ID, which is to be included in each of the paging message transmissions.

Based on the agreements of CBRA (Contention-Based Random Access) procedure, the first two messages in CBRA are supposed to be used for contention resolution. Thus, the A-IoT device which has already responded to the same paging message transmitted earlier shall not try to respond again if the Msg1 & Msg 2 have been completed.

Proposal 7	A-IoT device has already completed Msg1 & Msg2 in a Paging procedure identified by transaction ID can skip responding to other repeated paging messages in the same procedure.

However, even with the above mechanism to suppress excessive random-access attempts, the initial round of access triggered by the initial paging message transmission may still suffer severe contention loss because all the target devices will attempt in this round. It is known that the performance of slotted-aloha access reaches its optimal efficiency as 36.8% when the offered access demand is equal to the number of slots. When system is overloaded, the performance will degrade rapidly. Therefore, it would be much better to divide the access attempts evenly in different paging/access rounds so that in each round, the number of access is limited to achieve the near-best performance of slotted aloha.

For example, Paging message and its response (access attempts) could be divided in the time domain as shown in Figure 1 below:

Figure 1: Repeated Paging with selective Responding devices

In the example above in Figure 1, the intended paging recipients (Tag 1, 2 or 3) are distributed to respond to different paging transmissions based on some selection mechanism. For each of such a paging transmission, new DL preamble in PRDCH would provide new timing synchronization, so a new “response window” during which the device can reliably Tx in sync is allowed. One issue is how the device(s) select the paging round to respond need to be studied. It is obvious that an A-IoT device can randomly select one or more rounds to access with a probability, but then the total number of rounds of the paging message transmission needs to be known by the A-IoT devices. Whether/how this is achieved can be further discussed by RAN2.

Proposal 8	RAN2 discuss mechanism to evenly distributed multi-access attempts in different paging rounds if paging is to be repeated in multiple rounds.

One consequence of the retransmissions of paging message is that the paging procedure would take a much longer time to complete, compared to a single-shot transmission of paging message in PRDCH channel. Therefore, whether the A-IoT reader can be involved in multiple overlapping paging procedures need to be discussed. If reader conducts multiple paging transactions at the same time, then an A-IoT device may be potentially paged by the same reader multiple times with different “paging records” (e.g. with different paging masks/filters), either in the same R2D transmission or multiple subsequent transmissions. Then, a couple of complicated questions needs to be answered. For example, whether the device need to respond to each of those separate paging transactions? And if not, how to select/prioritize the paging message to respond. It is worth noting that solving those issues would add complexity to the device side. So, RAN2 may prefer to have some restrictions on NW side to not cause such a scenario.

Proposal 9	RAN2 discuss whether/how A-IoT device(s) process multiple paging procedures initiated by the same reader simultaneously.

# 3 Conclusion

In this contribution, we discuss the paging issue for Ambient IoT, and have the following proposals:

Proposal 1	The scope of A-IoT paging procedure is limited to the inventory; “Command + response” transaction needs to be discussed under data transmission procedure.

Proposal 2	RAN2 discuss which scheduling information should be (pre-)configured for D2R response scheduling instead of explicitly included in the paging message.

Proposal 3	D2R resource indicated in Paging message is by default shared resource, unless explicitly indicated as contention-free resource.

Proposal 4	D2R scheduling mechanism in PRDCH is common for both Paging message and other DL R2D messages.

Proposal 5	An A-IoT Paging procedure involves multiple rounds of transmission of paging message.

Proposal 6	If paging message is to be repeated, initial and retransmission of the paging message share the same transaction ID, which is to be included in each of the paging message transmissions.

Proposal 7	A-IoT device has already completed Msg1 & Msg2 in a Paging procedure identified by transaction ID can skip responding to other repeated paging messages in the same procedure.

Proposal 8	RAN2 discuss mechanism to evenly distributed multi-access attempts in different paging rounds if paging is to be repeated in multiple rounds.

Proposal 9	RAN2 discuss whether/how A-IoT device(s) process multiple paging procedures initiated by the same reader simultaneously.

# 4 References

[1] Chairman’s Note, RAN2#125bis

[2] Chairman’s Note, RAN2#126

[3] Chairman’s Note, RAN1#117