# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406683
Masstricht, Netherland, August 19-23, 2024

Agenda item:	8.13.2

Source:	Apple

Title:	Relay discovery and selection for Multi-hop UE-to-NW Relay

WID/SID:	NR_SL_relay_enh2– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In this paper, we discuss the relay discovery and selection aspects for Multi-hop UE-to-NW relay.

# 2 Discussion

### 2.1 Scenario with Single Extra Hop

In 3GPP Rel-17, UE-to-NW relay has been specified with the support of both model-A and model-B relay discovery. However, the relay discovery operation between the remote UE and the relay UE in Rel-17 UE-to-NW scenario is across a single hop, where the remote UE and relay UE are within each other’s proximity. In Rel-18 NR SL relay enhancement WI, UE-to-UE relay has been introduced to allow a UE to discovery another UE two-hops away. There is quite a resemblance between this R18 U2U relay discovery requirement and Rel-19 Multi-hop SL relay discovery requirement.

Figure 1 has illustrated the 2-hop Multi-hop U2N scenario with one extra hop added on the top of R17 U2N relay scenario. For the sake of clarity on the terminologies used in our discussion, we use “Relay_last UE” to refer to the relay UE under the cell coverage to communicate with gNB directly. Relay_1st UE is the relay UE one-hop away from the remote UE, as shown in Figure 1. Similarly,  Relay_2nd UE is the name of the relay two hops away from the remote UE, when more hops are added to extend the topology.

Figure 1: MH U2N relay discovery (one extra hop) vs. U2U Relay discovery

Basically, as shown in the topology in Figure 1, the “Relay_last UE” and “Remote UE” in the topology are exactly two Sidelink UEs two-hops away, and the remote UE need find the relay_1st UE to reach the Relay_last UE. This is exactly the discovery between “Source Remote UE” and “Target Remote UE” in Rel-18 U2U discovery. Therefore, we think Re1-18 U2U relay discovery and (re)selection can be largely reused for Rel-19 work on multi-hop Sidelink UE-to-NW relay.

Proposal 1	Rel-18 UE-to-UE Relay Discovery & (re)selection can be considered as a baseline for Multi-hop Relay discovery for remote UE detecting relay UEs two hops away.

Then for the Relay_last UE itself, its relay discovery procedure can be exactly as same as Rel-17 design because it still acts as an in-coverage UE-to-NW relay and can simply follow the relay discovery design in Rel-17. For example, a Rel-19 Relay_last UE can announce it in Model A discovery with Rel-17 parameters included in PC5_DISCOVERY message and respond to model-B discovery request from the relay_1st UE as defined in Rel-17 specification. Of course, Relay_last UE may be also involved in some additional discovery procedure introduced in Rel-19, if necessary. But that can be further discussed.

Proposal 2	Rel-17 UE-to-NW Relay Discovery design for U2N relay UE is reused by the Relay_last UE.

Note that in Rel-18 UE-to-UE relay work, there are two approaches for U2U relay discovery [2]:

- stand-alone discovery procedure; and

- communication with integrated discovery procedure.

In the Rel-18 stand-alone U2U relay discovery procedure, both model-A and model-B are supported [2]. For model A relay UE announcement, a “discovery set” is announced to include all the neighbouring UEs in proximity. Similarly, in Rel-19 work, we can have relay_1st UE reuse this scheme to include all the “Relay_last UE” it has discovered in the discovery announcements. Note that there is no need for a relay_1st UE to announce the discovered remote UEs because relay_last UE will not need to actively detect the existence of remote UE(s).

Proposal 3	Model-A Multi-hop relay discovery is supported by letting a “Relay_1st UE” announcing the Relay_last UEs in proximity in its discovery message.

For model B discovery, we can also support similar scheme to let remote UE to triggering a request similar to source remote UE in U2U relay design, and eventually will result into the discovery of the “relay_last UE” 2-hops away via multiple query-response like exchanges. Note that for U2U discovery, the “User Info” of target UE may not be always unique. Thus, similarly in Rel-19 MH relay design, remote UE can also simply seek any target UE happens to be a Relay_last UE w/o specifying a unique “target user info ID”.

Proposal 4	Model-B Multi-hop relay discovery is supported by letting remote UE trigger discovery query for any relay_last UE.

Note that from U2N remote UE perspective, the relay (re-)selection procedure defined for R18 U2U source remote UE to select relay UE can be reused w/o change. If there are multiple relay_1st UE in proximity, it can follow the same R18 design to select one candidate relay_1st.

Proposal 5	R18 U2U relay (re-)selection criteria for source remote UE is reused for U2N remote UE in Rel-19 for the (re-)selection of Relay_1st UE.

Logically, the Rel-17 U2N relay UE selection criteria can also be used by remote UE to choose its Relay_last UE. However, there is one major difference between R19 requirements and Rel-17 requirements in regards of U2N relay UE selection. In Rel-19 multi-hop case, if there are multiple Relay_last UE detected as candidates (can be reached via the same relay_1st UE), it cannot simply follow Rel-17 relay selection by checking the PC5 link measurements, as there is no direct link between remote UE and Relay_last UE. RAN2 need to discuss how to select a Relay_last UE among discovered Relay_last UE by remote UE. Maybe some measurements about the last Uu hop can be considered as part of relay selection criteria..

Proposal 6	RAN2 discuss relay discovery and relay selection enhancements to allow remote UE select among multiple Relay_last UEs.

Certain information can help remote UE to choose a better Relay_last UE. One useful information is the RRC states of the Relay_last UE. This is because remote UE may prefer to choose a relay UE in RRC_CONNECTED state rather than an IDLE/INACTIVE relay UE for the concern of connection establishment latency. Therefore, we propose to add RRC State information in discovery message, so that it can be considered by remote UE in relay (re-)selection procedure.

Proposal 7	RRC state of Relay_last UE is included in its relay discovery message.

In the Rel-18 communication with integrated discovery procedure for UE-to-UE relay, DCR/DCA message are used to set-up per-hop PC5-S connection between Source remote UE and relay UE, and between relay UE and target remote UE.  Similarly, in Rel-19 work, this procedure can also be reused to setup PC5 links between the remote UE, Relay_1st UE and Relay_last UE .

Proposal 8	Rel-18 “Communication with integrated discovery procedure” is supported in Multi-hop U2N relay.

### 2.2 Forward-compatibility with More Extra Hops

It has been pointed out in the WID [1] that

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. <text omitted>

To consider the forward extensibility of relay discovery design, we consider the scenario with two hops as an example:

Figure 1: MH U2N relay discovery (two extra hop)

From Figure 2, we can see that the remote UE need discover the Relay_last UE 3-hops away in this topology, when the remote UE is 4-hops away from the gNB. In general. for a N-hop U2N relay discovery problem we need a N-1 hop U2U relay discovery solution. Therefore, RAN2 can easily extend the R18 U2U relay discovery solution to extra hops by add a few enhancements on top of Rel-18 U2U relay discovery design.

Proposal 9	RAN2 discuss how to extend R18 U2U relay discovery design to support at least one extra hop between two Sidelink UEs.

One way to ensure forward compatibility with future extensions for additional relays is to add the “number of hops away from the target UE” in the U2U relay discovery design. With this information, the UEs involved in relay discovery can easily detect which relay UE is relay_1st and which relay UE is relay_2nd.

Proposal 10	In order to ensure forward-compatibility, “number of hop” information is added in relay UE’s discovery messages.

# 3 Conclusion

In this contribution, we discuss the Relay discovery and selection for Multi-hop UE-to-NW Relay, and have the following proposals:

Proposal 1	Rel-18 UE-to-UE Relay Discovery & (re)selection can be considered as a baseline for Multi-hop Relay discovery for remote UE detecting relay UEs two hops away.

Proposal 2	Rel-17 UE-to-NW Relay Discovery design for U2N relay UE is reused by the Relay_last UE.

Proposal 3	Model-A Multi-hop relay discovery is supported by letting a “Relay_1st UE” announcing the Relay_last UEs in proximity in its discovery message.

Proposal 4	Model-B Multi-hop relay discovery is supported by letting remote UE trigger discovery query for any Relay_last UE.

Proposal 5	R18 U2U relay (re-)selection criteria for source remote UE is reused for U2N remote UE in Rel-19 for the (re-)selection of Relay_1st UE.

Proposal 6	RAN2 discuss relay discovery and relay selection enhancements to allow remote UE select among multiple Relay_last UEs.

Proposal 7	RRC state of Relay_last UE is included in its relay discovery message.

Proposal 8	Rel-18 “Communication with integrated discovery procedure” is supported in Multi-hop U2N relay.

Proposal 9	RAN2 discuss how to extend R18 U2U relay discovery design to support at least one extra hop between two Sidelink UEs.

Proposal 10	In order to ensure forward-compatibility, “number of hop” information is added in relay UE’s discovery messages.

# 4 References

[1] RP-241609: New WID on NR sidelink multi-hop relay

[2] TS 23.304 v18.3.0