# 3GPP TSG RAN WG2 Meeting #127      	                           R2-2406684
Masstricht, Netherland, August 19-23, 2024

Agenda item:	8.13.3

Source:	Apple

Title:	Control Plane Design for Multi-hop UE-to-NW Relay

WID/SID:	NR_SL_relay_enh2– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In this paper, we discuss the control plane design for Layer-2 Multi-hop UE-to-NW relay.

# 2 Discussion

According to WID[1], R19 work need consider the following two topologies for Multi-hop Sidelink UE-to-NW relay. In the first topology, there is one extra relay UE added. For the 2nd topology, there are two extra relays added, when compared to Rel-17 single-hop UE-to-NW relay.

Figure 1. Topologies for Rel-19 Multi-hop U2N relay

For the sake of clarity on the terminologies used in this discussion, we always use “Relay_last UE” to refer to the relay UE under the cell coverage to communicate with gNB directly. Relay_1st UE is the relay UE one-hop away from the remote UE, Relay_2nd UE is the relay two hops away from the remote UE. Both of Relay_1st UE and Relay_2nd UE are also referred as “intermediate relay UE”s.

For the control plan design of a Layer-2 Multi-hop Sidelink U2N relay, there are two distinctive design approaches.

The first approach keeps the “remote UE-relay UE” relationship in Rel-17 U2N relay design as same as the “remote UE – relay_last UE” relationship. The only change is that the link between two UEs in the latter case is now realized with a U2U relay instead of direct PC5 link.

The second approach is to model the remote UE - serving relay UE relationship based on the first hop between remote UE and relay_1st UE, and this relay_1st UE handles all the equivalent functions as Rel-17 Relay UE, although it may out of coverage and need use other relay UEs to reach the NW.

It is obvious that we cannot support both approaches in R19 work and RAN2 need decide on which approach to pursue. Hence, let us discuss the pros and cons of each approach.

### 2.1 U2U+U2N approach

In this approach, the single-hop remote UE-relay UE” relationship in Rel-17 U2N is inherited to represent the “remote UE – Relay_last UE” relationship because the Relay_last UE is same as the Rel-17 in-coverage Relay UE which is directly under NW control. Then the virtual PC5 connection between Remote UE and Relay_last UE is modelled as a Layer-2 U2U relay communication. For UL, the Relay_last UE is responsible for converting end-to-end U2U traffic received from remote UE into the format of end-to-end U2N traffic towards gNB. Similar operation can be done via DL in reverse direction.

Figure 2: U2U + U2N approach for 2-hop U2N Relay

Basically, from the above illustration, we can see that R18 U2U design can be largely reused to support the communication between Remote UE and Relay_last UE.

Observation 1: 	With U2U+U2N approach, Rel-18 U2U solution can be reused to support MH with single extra hop.

It is also evident that the remote UE plays a dual role in this approach. It is a L2 U2N remote UE in regards of its access to NW, but it is also a L2 U2U remote UE in regards of its communication towards its “U2N serving relay” UE, which is Relay_last UE.

Observation 2: 	With U2U+U2N approach, Rel-19 Remote UE has a dual role: L2 U2N remote UE towards NW via Relay_last UE and L2 Source Remote UE towards Relay_last UE.

With U2U+U2N approach, since the Relay_last UE is guaranteed to be an in-coverage UE, it will be able to monitor SIB and listen to paging in RRC_IDLE or RRC_INACTIVE state for the remote UE. Then, it is reasonable for the Relay_last UE to deliver/forward paging and SIB information via an end-to-end PC5-RRC message (SL-SRB3) UuMessageTransferSidelink.

As shown in Figure 3, the paging & SIB request and delivery reuse the existing PC5-RRC signalling procedures, which are transported via a L2 U2U relay UE (Relay_1st UE). It is also worth noting that those two end-to-end PC5-RRC messages are protected by PC5 security keys established between the remote UE and Relay_last UE, so that the Relay_1st UE cannot know the contents of the messages. And this kind of protection is applicable to the initial Uu RRC messages transmitted by the remote UE, so that the establishment cause can be hidden from the Relay_1st UE, too.

Figure 3: U2U + U2N approach for Paging/SIB forwarding

Observation 3: 	With U2U+U2N approach, end-to-end PC5-RRC signalling can be reused to deliver paging and SIB forwarding with privacy protected from the intermediate relays.

Finally, one of the major benefits of this approach is that the gNB control of intermediate relays are not necessary. Based on Rel-18 U2U relay design, a U2U relay UE can derive its PC5 Relay RLC channel configurations and SRAP mappings based on SIB or pre-configuration. This means, those intermediate relays are able to operate in IDLE/INACTIVE/OOC with minimum gNB involvement.

Observation 4: 	With U2U+U2N approach, Relay_1st UE or Relay_2nd UE does not need to be controlled by gNB and can operate solely as a L2 U2U relay UE with minimum gNB involvement.

### 2.2 Recursive U2N approach

In this approach, the remote UE - serving relay UE relationship is based on the first hop between remote UE and relay_1st UE, and this relay_1st UE handles all the equivalent functions as Rel-17 U2N Relay UE. Similarly, the relay_1st UE models its own access as a single-hop U2N relay via Relay_Last UE. Basically, this is a recursive scheme where the relay UE is acting as a U2N remote UE towards the next relay, as shown in Figure 4.

Figure 4: Recursive U2N approach for 2-hop U2N Relay

Observation 5: 	With Recursive U2N approach, each Relay UE (except Relay_last UE) need to act as both a L2 U2N remote UE (for upstream node) and a L2 U2N Relay UE (for downstream node).

One of the characteristics of recursive U2N approach is that it does not impose changes in Remtoe UE behaviour, as a OOC remote UE, it just chooses a single-hop U2N relay, and do not need to support U2U relay. However, whether a Rel-17 L2 U2N relay can be supported in Rel-19 work still depends on other aspects of design, such as Relay discovery/selection, SRAP header or service continuity mechanisms in Rel-19 MH Relay design.

Observation 6: 	With Recursive U2N approach, Rel-19 Remote UE has a single role: L2 U2N remote UE towards its immediate next relay UE relay_1st UE, FFS whether backward-compatible with Rel-17 U2N remote UE.

The main striking property of this approach is that each relay UE acts as a Rel-17 U2N relay UE, which means it has to be managed/configured by the NW via a PC5-RRC connection, this also requires all those relay UEs to be brought into RRC_CONNECTED state before an end-to-end Uu path from remote UE can be established. The SRAP configuration has to be provided by gNB for each of the relay UE, too.

Observation 7: 	With Recursive U2N approach, every Relay UE need to bring into “RRC_CONNECTED” state for NW configuration/control and SRAP mapping establishment.

With recursive U2N approach, when an intermediate relay UE receives the request from remote UE about paging and SIB forwarding, it cannot start monitoring SIB or listening to paging because it may be OOC. So, it has to relay this request to its upstream relay node.

Observation 8: 	With Recursive U2N approach, SIB and Paging forwarding has to be done on a hop-by-hop basis.

It also means the UE ID of remote UE for NW paging has to be shared hop by hop to every relay UEs in the recursive U2N scheme. This may posed as a security/privacy risk, which needs to be further studied in SA3.

Observation 9: 	With Recursive U2N approach, UE ID of remote UE is exposed to every relay UE.

It could be helpful for a recursive U2N approach to be modelled as a set of multiple “single-hop U2N relay” cases, but this is not entirely true. Actually, the relay UE still need model its downstream relay node as a “remote UE collection set” instead of a single Rel-17 L2 U2N “remote UE” because it needs to monitor all the paging identifiers under this downstream node, which include multiple paging identifiers (e.g., multiple remote UEs connected to a same downstream relay). This means the R17 signalling for paging solicitation (RemoteUEinformationSIdelink) cannot be reused.

Observation 10: With Recursive U2N approach, R17 L2 U2N CP signaling for paging forwarding has to be redesigned.

When the very first RRC message from the Remote UE is sent to the next relay UE, this relay UE could be able to deliver it to the immediate next relay UE via established PC5 link, but not able to deliver it to gNB “virtually” as it is in IDLE/INACTIVE state, as illustrate in Figure 5 below. This is because:

- Relay_1st UE is supposed to have a Uu Relay RLC channel as egress to relay this message. But this is not the case for this relay with no Uu connectivity in RLC layer.

- Also, the relay needs to enter RRC_CONNECTED state to receive proper configuration. But it cannot enter RRC_CONNECTED either because its next relay UE may also in IDLE/INACTIVE state, which is unable to support any CONNECTED state remote/relay UE under itself.

Figure 5: Design challenges in Remote UE end-to-end RRC Setup with Recursive U2N approach

This poses a design challenge for the L2 RRC Connection-establishment procedures via multiple relays. The problem is that the very first RRC message, if forwarded, can literally only trigger Relay_Last UE to enter RRC_CONNECTED. But if we allow that message delivered to gNB hop-by-hop, it will leave a broken and unsecure path for any DL e2e RRC message (e.g., RRCSetup or RRCResume) delivery or any more UL e2e RRC message transport. Hence, a new mechanism to trigger those relay UEs to enter RRC_CONNECTED before the end-to-end connection setup has to be introduced in Rel-19. For example, Relay_1st UE need to generate its own RRCSetupRequest to trigger a single-hop U2N CP procedure for connection setup first, which will add significant latency potentially.

Observation 11: With Recursive U2N approach, R17 L2 U2N CP procedure for end-to-end connection establishment has to be redesigned for IDLE/INACTIVE intermediate relay(s), with potentially significant latency added.

### 2.3 Proposed Way-forward

To evaluate and compare those two approaches, we consider the following aspects:

- Control plane complexity and overhead

- Security/Privacy

- Latency in CP Setup

Control Plane Complexity/Overhead

From the control plane perspective, the most important aspect is the complexity and associated overhead. The major concern of the Recursive U2N approach is that the control procedure and its overhand will be quite heavy. It need establish RRC connection to each of the UEs involved in the topology and configures them with RRCReconfiguration. It involves quite a lot of Uu and PC5-RRC control signalling towards each of the relay UEs such as forwarding SIB/paging solicitations and delivery of SIB and paging messages. It also incurs overhead for SRAP mapping configurations because gNB need also configure the underlying L2 relay path to each of the intermediate relay UE (by treating them as a U2N Remote UE with Local ID/BEAER ID usage) to deliver the control message to this relay UE. In overall, as NW is fully responsible for all the UEs involved, there is no autonomy existing in the system at all. Generally, this centralized control scheme is also not scalable.

But with U2U approach, the SIB and paging forwarding can simply be buried into end-to-end PC5-RRC messages and the intermediate relays do not need to store & process them. Also, those intermediate relays can work with minimum gNB involvements as U2U relay UE can configure PC5 Relay RLC channels based on pre-configuration.

Security/Privacy:

Another aspect to consider is about the security. With recursive U2N approach, remote UE’s UE ID has been exposed to every intermediate relay. But with U2U+U2N approach, the remote UE’s end-to-end Uu signalling and paging forwarding can all be protected with a PC5 security between the remote UE and Relay_last UE.

Latency in CP Setup:

The CP procedure to setup MH relay can be quite a time-consuming process with a Recursive U2N approach. With the recursive scheme, it requires a RRC establishment & full configuration of the relay UE which is nearest to gNB before the next-near relay can be setup & configured. However, the initiation of this access and the very first RRCSetupRequest, ironically, comes from the furthest UE in the topology. Therefore, this will undoubtedly result into a lot of latency before even the very first RRC message from remote UE can be delivered to the NW. But with U2U approach, the delivery of first RRC message is quite equivalent to the single-hop relay case, with just one or two extra hop of packet delivery delay.

Summary:

Based on the above observations and considerations, we think the correct way-forward is to adopt U2U+U2N-based approach for control plane design.

Proposal 1	U2U+U2N approach is adopted as baseline for Rel-19 MH U2N relay CP design.

# 3 Conclusion

In this contribution, we discuss the CP design for Multi-hop SL relay, and have the following observations:

Observation 1: 	With U2U+U2N approach, Rel-18 U2U solution can be reused to support MH with single extra hop.

Observation 2: 	With U2U+U2N approach, Rel-19 Remote UE has a dual role: L2 U2N remote UE towards NW via Relay_last UE and L2 Source Remote UE towards Relay_last UE.

Observation 3: 	With U2U+U2N approach, end-to-end PC5-RRC signalling can be reused to deliver paging and SIB forwarding with privacy protected from the intermediate relays.

Observation 4: 	With U2U+U2N approach, Relay_1st UE or Relay_2nd UE does not need to be controlled by gNB and can operate solely as a L2 U2U relay UE with minimum gNB involvement.

Observation 5: 	With Recursive U2N approach, each Relay UE (except Relay_last UE) need to act as both a L2 U2N remote UE (for upstream node) and a L2 U2N Relay UE (for downstream node).

Observation 6: 	With Recursive U2N approach, Rel-19 Remote UE has a single role: L2 U2N remote UE towards its immediate next relay UE relay_1st UE, FFS whether backward-compatible with Rel-17 U2N remote UE.

Observation 7: 	With Recursive U2N approach, every Relay UE need to bring into “RRC_CONNECTED” state for NW configuration/control and SRAP mapping establishment.

Observation 8: 	With Recursive U2N approach, SIB and Paging forwarding has to be done on a hop-by-hop basis.

Observation 9: 	With Recursive U2N approach, UE ID of remote UE is exposed to every relay UE.

Observation 10: With Recursive U2N approach, R17 L2 U2N CP signaling for paging forwarding has to be redesigned.

Observation 11: With Recursive U2N approach, R17 L2 U2N CP procedure for end-to-end connection establishment has to be redesigned for IDLE/INACTIVE intermediate relay(s), with potentially significant latency added.

Based on the discussion, we propose:

Proposal 1	U2U+U2N approach is adopted as baseline for Rel-19 MH U2N relay CP design.

# 4 References

[1] RP-241609: New WID on NR sidelink multi-hop relay