3GPP TSG-RAN WG2 Meeting #127	R2-2406693

# Maastricht, Netherlands, 19th – 23th August, 2024

Agenda Item: 	8.6.1

Source:	Apple Inc, China Telecom

Title:	Important topics for resolution for Rel-19 Mob Enh WI

Document for:		Discussion and Decision

# Introduction

We are entering the third meeting on R19 Mobility Enhancements in RAN2 and RAN3. While more work is to be done in measurements (event triggered as well CSI-RS based) and other objectives, the inter-CU LTM design has so far closely followed Rel-18 LTM as baseline with a few additions.

It would be good to progress into stage 3 details for inter-CU LTM soon (and in parallel to other stage-2 details on the other objectives) and to this effect, it is better to try and bring stage-2 aspects of inter-CU LTM (atleast nonDC case) to a final stage of sort.

In this paper we try to address the main open issues related to the WID objectives (from RAN2 perspective) to the effect of above.

# Inter-CU LTM

## Imp open items from general inter-CU LTM operation

To help with the discussion on what are the open items in the inter-CU LTM cell switch procedure, we have shown the general sequence of events and highlighted where further discussion is needed to conclude on stage-2 details.

It can be seen from the above that we need to resolve in this meeting atleast the below:

- Conclude on whether inter-CU LTM will be with Rel-18 based single reference configuration or multiple reference configurations.

- How the UE is signalled the LTM candidate configuration is also dependent on the decision from #1. So far, we have not yet concluded how the UE would know that the LTM switch it is asked to perform, would result in an inter-CU or intra-CU LTM. While part of this also depends on SA3 progress (whether MAC CE contains more info or not), it would be better if RAN2 concludes on whether it is RRC that provides the semi-static configuration of candidate association to CUs. Also RAN2 should try to design this to allow conditional inter-CU LTM to be able to use the signalling design.

- Related to the above is the flow sequence on how the signalling hand-shake would go during the LTM execution stage – specifically whether it is still the DU the decides on the LTM switch for inter-CU LTM as well. This can also help RAN3 in their work, and the earlier we conclude the better for the progress of this WI.

- Another important aspect is the discussion on what are use-cases that are supported for Rel-19 inter-CU LTM with DC. Needless to say, we have many permutations to consider and this is the first time RAN2 will likely start a discussion on inter-CU LTM with DC. And it is noteworthy that no other WG might start this. So it is critical to have a consensus on what is agreed and what is not in terms of usecases for DC and to inform SA3/RAN1/3/4 about this, as this will make their progress much more effective.

# Event triggered L1 objective

We can see that there are still a number of stage-2 aspects to be addressed for this objective including details on what are the LTM events, what is the evaluation procedure (and which layer/spec captures this), how the events are reported, and the details on the reporting format etc.

We think it is important for RAN2 to spend more time on this objective this meeting to make better progress, as there will be sort of re-alignment or re-conditioning that is likely to happen in Sep RAN-107, as the objectives would likely need to be revised to align with Rel-19 MIMO enhancements work item.

Since technical analysis is still to be done (as typically what the WGs do), we urge the companies to keep in mind the progress made on Rel-19 MIMO enh WI on TRP based beam switching which would very closely align with LTM mobility as long as the TRPs belong to different DUs or even different CUs. We would like to remind the below:

- From UE perspective and from physical layer perspective, the beam characteristics ( beam propagation/ fading and other wireless physical and RF aspects) do not change if the TRP belongs to another CU or the same CU/DU.

- MIMO has developed beam switching for also the cases where the TRPs are not co-located, and LTM mobility is no different from this!

- RAN2 should evaluate what technical merits a “new” design would bring (if it is different from MIMO design) if we are to include this in the Rel-19 Mob Enh WI for the same scenario where Rel-19 MIMO has already specified a design.

- We also urge companies to keep in mind that additional complexity in doing things multiple times in different way to achieve/satisfy the same lower layer requirement, and that 3GPP should device “sound technical” solutions that address technical problems and so should avoid situations which result in non-3GPP folks questioning why two different methods/solutions were agreed for the same requirement.

  - Needless to say, this is not a rhetorical question, as implementation teams across UE and NW vendors would analyze the specification which can result in the above question.

# Conclusion

Proposal 1: RAN2 to conclude in this meeting, whether to use multiple or single reference configuration for inter-CU LTM. If multiple reference configuration is to be adopted:

- Conclude on whether LTM-CSI-ResourceConfig will remain global across CUs or not

- Inform RAN3/RAN1 (CC RAN4) about this

Proposal 2: RAN2 to conclude on applicable use-cases for inter-CU LTM with DC for Rel-19 and inform SA3/RAN3/RAN1/RAN4

Proposal 3: Attempt to address important aspects of inter-CU LTM from stage-2 (preparation, early sync, execution and completion) and evaluate if stage-2 specification changes can be started.

Proposal 4: For event triggered L1 discussions, RAN2 should strive to evaluate on technical merits if the agreements are different from Rel-19 MIMO enhancements WI for the same requirements – to not re-invent the wheel for the same requirements if Rel-19 MIMO enh WI has already addressed this.