3GPP TSG-RAN WG2 Meeting #127	R2-2406694

# Maastricht, Netherlands, 19th – 23th August, 2024

Agenda Item: 	8.6.2

Source:	Apple

Title:	View on open issues in inter-CU LTM

Document for:		Discussion and Decision

# Introduction

Significant progress has been made on stage 2 details of inter-CU LTM in RAN2-125bis and RAN2-126. To make effective headway into this work-item, we should attempt on the completion of stage-2 aspects of inter-CU LTM and start capturing in a running CR (38.300) the details. This enables us to sort of conclude on stage-2 and dive into stage-3 details based on the foundation made on stage-2. In this paper we try to address the main open issues related to inter-CU LTM in an effort to bring stage-2 parts into a good shape.

# Open topics

## Items from general inter-CU LTM operation

To help with the discussion on what are the open items in the inter-CU LTM cell switch procedure, we have shown the general sequence of events and highlighted where further discussion is needed to conclude on stage-2 details.

## Multiple Reference Configurations

We had a brief discussion in RAN2-126 on the topic of multiple reference configurations as shown below:

- Reference configuration Single reference configuration as Rel-18 LTM (P3: R2-2404642: Apple, China Telecom) Proposal 3: A single reference config is used for inter-CU LTM, and this reference configuration is the same as Rel-18 reference configuration. RAN2 expects additions to the Rel-18 reference configuration structure for Rel-19 LTM. Multiple reference configuration, e.g. a reference configuration per CU (P5: R2-2404826: LG) Proposal 5. Multiple reference configurations are provided per CU for inter-CU LTM. [Ericsson]: Single reference configuration sounds simpler, but each CU may have different capabilities or default configurations. We need to see each solution’s pros and cons. [Rakuten]: Considering one CU covers many DUs, two reference configurations should be enough. [MediaTek]: Considering max 8 candidate cells, one reference configuration should be enough. [Qualcomm]: It is not easy to determine which one is better in the signalling overhead. It will be different case by case. [KDDI]: We don’t need coordinate with multiple CUs if we have multiple reference configurations, e.g. one reference per CU. [Huawei]: Don’t agree with KDDI. Even with single configuration, the source CU doesn’t need the coordination with others. [Nokia]: With the mixture of subsequent inter-CU and intra-CU LTM, a reference configuration per CU will reduce the signalling overhead in the end.

We acknowledge that a reference config for each CU would help with “localizing” the configuration of all candidate cells within a CU, and this also results in no need of exchange of configuration across CUs to a large extent. But we would first note that this advantage is only possible if we associate a particular reference config to a CU (i.e, the reference configuration is always per CU instead of having an arbitrary number of reference configurations).

Observation 1: In multiple reference configuration cases, as long as each reference configuration is always associated with one CU i.e., the number of reference configurations are not capped to an arbitrary number irrespective of number of CUs in the candidate LTM config, there are advantages of reducton of signaling amounting from the below:

      - No need to exchange the CU specific candidate configurations across CUs for most of the config (LTM-CSI resource information is an exception)

      - Each CU can create a reference configuration that suits to the candidates it configures

However, we want to point out that as per below agreements from RAN2-126, mediation and consolidation of configuration from the source is still needed.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. - In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

This implies that even with independent configurations that are limited to candidates belonging to a CU, cross-CU level configuration is still needed, for eg., to inform the UE about the neighbor beams to measure and report, and this can get more involved when event triggered measurements are configured/used.

Observation 2: Even with limiting the multiple reference configurations to per-CU, atleast the CSI resource information needs to consolidate by the source, as the UE needs to be configured and modified (via TCI) about the neighbor beams where some of them could from same CU while the others are not.

Otherwise, we need to start discussion about whether the global LTM-CSI-Resource configuration needs to be made per-CU as well. For ease of understanding, figure 2 below show the current Rel-18 LTM configuration structure.

Figure 2: Rel-18 LTM configuration structure

And if we start moving in the direction of separate LTM CSI resource configurations, then the UE needs to be informed about “which” CU related TCI config it is being provided for neighbor cell beam info and this needs to be extended when L1 event triggered measurement is handled. This requires changes to UE signalling (potentially) which is undesirable when avoidable.

Observation 3: If multiple LTM-CSI-Resource configurations are considered, the UE signalling aspects needs to be adapted (or changed) to let the UE know “from” which CU the signalled beam info is to be used (for eg TCI info belonging to which LTM-CSI-Resouce config). Such changes are not desirable.

And finally, we would like to point out that RAN2 spend considerable effort in stabilizing the Rel-18 reference configuration and this is already being used in CPAC features, where anyway intra-CU is not always the option.

Observation 4: Single reference config is still used in Rel-18 with S-CPAC feature where very likely inter-CU is involved. Also RAN2 spent a lot of effort in stabilizing the usage of reference configuration for Rel-18 LTM, and multiple reference configuration would lead to even more effort.

Based on all of the above, we think it’s better to stick to technically sound single reference configuration option for inter-CU LTM as well, while acknowledging that in some cases, the reference configuration is not very effective in inter-CU.

Proposal 1: Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations are supported.

## Identification of inter-CU LTM at the UE

An LTM configuration to the UE where the UE can find out if LTM cell switch is an intra-CU or inter-CU would be dependent on whether we have multiple or single reference configuration (i.e., re-use the existing signalling strucutures). This was postponed in the last meeting, to allow first the discussion of multiple reference configuration to be concluded.

In the above chapter we outlined on why we can continue with single reference configuration design, and once this is out of the way, the UE configuration to inform which candidate cell belongs with to which CU, can also go the similar way as in how the UE would be informed about whether it needs to do RLC reset or not.

Observation 5: If single reference configuration option is selected, then informing the UE about whether it needs to perform PDCP re-establishment as part of LTM Cell switch or not (inter-CU or not) can be with additional RRC signaling similar to NoResetID. This can be irrespective of whether the security key to be used is conveyed using MAC CE or not.

While we agree that a MAC CE informing the UE would let the UE know that the LTM switch from the MAC CE is inter-CU, it would be better in RAN2 to design a signaling framework that works with conditional LTM as well.

Proposal 2:  (To lay the framework for conditional LTM,) UE is pre-configured using RRC LTM configuration which will allow the UE to derive whether the LTM switch is inter-CU or intra-CU.

## Source DU in LTM execution decision

In the previous LTM discussions, one aspect of LTM execution was not fully concluded and that is whether the source DU continues to be the decision maker of the inter-CU LTM switch.

While this is primarily RAN3 topic, and as agreed in last RAN2-126 meeting that RAR based early-TA is not supported for inter-CU NW triggered LTM, it would be better to confirm the RAN3 understanding in RAN2 (and to allow RAN3 to progress) that we continue to allow the srouce DU to make the decision on whether the inter-CU LTM is triggered or not.

Observation 6: In RAN2-126, we agreed to not allow RAR based early TA acquisition. This would allow the source DU to “collect” TA info of potentially multiple candidate cells, but trigger LTM switch for the candidate of it’s choosing.

Observation 7: We also agreed that the UE can be configured with a mix of intra and inter-CU LTM candidates. And for intra-CU, it’s the source CU that makes the decision for LTM switch. It would be inefficient to now change this so that the decision making is only limited to intra-CU cases.

Also, RAN2 did go with such approach for Rel-18 LTM (as can be seen below) and there is no reason to deviate from this.

RAN2 has discussed the two execution approaches identified by RAN3 and this is the current RAN2 understanding on what the two approaches are: - Approach 1: the serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch. The gNB-CU then informs the target gNB-DU of the serving cell switch. - Approach 2: the serving gNB-DU first requests information from target gNB-DU (via the gNB-CU) before triggering LTM cell switch command to the UE. RAN2 thinks that Approach 1 shall be the baseline, while the support of Approach 2 could be up to the network implementation without any specification impact.

Based on all of the above, we propose that source DU continue to be the decision maker for LTM swtich for inter-CU as well.

Proposal 3: For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target CU of the target LTM cell.

## UE based TA for inter-CU

RAN2 could not spend time on the UE estimated TA for inter-CU in the previous meetings. We would like to conclude this topic atleast in the RAN2 WG.

Observation 8: We agreed that the UE can be configured with a mixture of intra and inter-CU LTM candidates. We do not see a practical case where candidate cells belonging to different CU are tightly synchronized, however this is more a NW deployment topic than standards.

Observation 9: Rel-18 allows UE estimated TA for synchronous deployments, and Rel-18 is the baseline for Rel-19 inter-CU. As long as the same requirements and conditions exist for UE estimated TA, inter-CU should not be an exception.

Observation 10: In the previous discussions, companies were wondering if the requirements would be different for UE estimated TA due to inter-CU. But this is not RAN2 domain, and interested companies can submit contributions to the interested WG, as per WID, Rel-18 LTM is the baseline for Rel-19 LTM (and UE estimated TA is part of Rel-18 LTM).

Based on the above, we think RAN2 should can continue with other aspects and make a decision if other WG decide not support the UE estimated TA for synchronized case.

Proposal 4: RAN2 will continue with using Rel-18 as the baseline for inter-CU (this includes the features from Rel-18 including UE estimated TA usecase from Rel-18), and will revisit this of other WGs think this is not feasible for inter-CU.

## Inter-CU LTM in DC

SA3 will start discussions on security topics for inter-CU LTM without DC. And it would help if RAN2 can converge on what are the allowed scenarios for inter-CU LTM with DC, as this information can help with SA3 discussions (for eg., to come up with solutions that can work for DC as well as non-DC cases where possible).

Also, since RAN1 and RAN4 are starting to take this WI for the first time, it would help them in their discussions if RAN2 can provide a sort of guidance on what DC scenarios are allowed and with what qualifications.

Needless to say, this is a big input to RAN3 in their discussions, as they have deferred DC cases to later meetings as well.

With that background, we tabulate below our analysis of the LTM with DC with technical analysis for each case.

Observation 11: Since SA3 will start discussions on security topics for inter-CU LTM without DC, a list of RAN2 usecases that are for inter-CU LTM with DC can help with SA3 discussions (for eg., to come up with solutions that can work for DC as well as non-DC cases where possible).

Observation 12 : a list of RAN2 usecases that are for inter-CU LTM with DC also helps with RAN3(who will take up DC in future meetings) / RAN4 (who would have potential SN specific requirements) and RAN1.

| Use-case | Before LTM switch |  | SN after LTM switch |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Type of LTM in MN | Type of LTM in SN |  | Analysis | Proposal |
| 2 | Inter-CU | No LTM | SN released | Similar to non-DC inter-CU LTM | Already being designed |
| 3 | Inter-CU | No LTM | SN not released | Inter-CU MN LTM change results in K-SN change as well if SN remains after LTM switch. Another case is that the candidate LTM has SN. In both cases, UE applied the sk-counter provided | Should be supported. |
| 4 | No LTM | Inter-CU LTM | SN with different CU | This is the case of sk-counter change, and NW should ensure that the candidate configuration has only SN (no MN change). If there is no change of sk-counter (PDCP is not re-established), then this is Rel-18 LTM | Should be supported |
| 5 | Intra-CU or no LTM | Intra-CU | SN present or absent | Rel-18 LTM | Baseline |
| 6 | Intra-CU or no LTM | Inter-CU | SN has to be present (from different CU) | Same as usecase 4. And the same requirement that PDCP has to be re-established for inter-CU | Should be supported. |
| 7 | Inter-CU | Intra-CU | LTM switch in SN ( no change in MN) | Same as rel-18 LTM where intra-CU LTM is in SN, with no change to MN | Rel-18 Baseline operation. |
| 8 | Inter-CU | Intra-CU | SN absent | This is the same as case 2. | Should be supported. |
| 9 | Inter-CU | Intra-CU | SN present | If the SN is present after inter-CU MN LTM switch, then this should be considered as inter-CU SN LTM as well and this is not allowed per WID. | Not allowed |
| 10 | Inter-CU | Inter-CU |  |  | Not supported based on WID |

Table 1: Rel-19 Inter-CU LTM with DC usecases

Based on the use-cases and analysis from the table above, we make the following proposals:

Proposal 5: RAN2 to agree on the usecases listed in table-1.

Proposal 6: RAN2 considers that the inter-CU LTM switch in SN requires a PDCP re-etablishment with a change of sk-counter.

Proposal 7: If MN has inter-CU LTM candidates configured, then the UE cannot be configured with inter-CU LTM in SN (as per WID)

Proposal 8: If MN has inter-CU LTM candidates configured, then the UE can be configured with intra-CU LTM in SN, but if the LTM switch in MN is inter-CU, then the UE should not have SN configured after the LTM switch.

Proposal 9: Send an LS to SA3/RAN1/RAN3/RAN4 about the DC use-cases and agreements.

# Conclusion

Observation 1: In multiple reference configuration cases, as long as each reference configuration is always associated with one CU i.e., the number of reference configurations are not capped to an arbitrary number irrespective of number of CUs in the candidate LTM config, there are advantages of reducton of signaling amounting from the below:

      - No need to exchange the CU specific candidate configurations across CUs for most of the config (LTM-CSI resource information is an exception)

      - Each CU can create a reference configuration that suits to the candidates it configures

Observation 2: Even with limiting the multiple reference configurations to per-CU, atleast the CSI resource information needs to consolidate by the source, as the UE needs to be configured and modified (via TCI) about the neighbor beams where some of them could from same CU while the others are not.

Observation 3: If multiple LTM-CSI-Resource configurations are considered, the UE signalling aspects needs to be adapted (or changed) to let the UE know “from” which CU the signalled beam info is to be used (for eg TCI info belonging to which LTM-CSI-Resouce config). Such changes are not desirable.

Observation 4: Single reference config is still used in Rel-18 with S-CPAC feature where very likely inter-CU is involved. Also RAN2 spent a lot of effort in stabilizing the usage of reference configuration for Rel-18 LTM, and multiple reference configuration would lead to even more effort.

Observation 5: If single reference configuration option is selected, then informing the UE about whether it needs to perform PDCP re-establishment as part of LTM Cell switch or not (inter-CU or not) can be with additional RRC signaling similar to NoResetID. This can be irrespective of whether the security key to be used is conveyed using MAC CE or not.

Observation 6: In RAN2-126, we agreed to not allow RAR based early TA acquisition. This would allow the source DU to “collect” TA info of potentially multiple candidate cells, but trigger LTM switch for the candidate of it’s choosing.

Observation 7: We also agreed that the UE can be configured with a mix of intra and inter-CU LTM candidates. And for intra-CU, it’s the source CU that makes the decision for LTM switch. It would be inefficient to now change this so that the decision making is only limited to intra-CU cases.

Observation 8: We agreed that the UE can be configured with a mixture of intra and inter-CU LTM candidates. We do not see a practical case where candidate cells belonging to different CU are tightly synchronized, however this is more a NW deployment topic than standards.

Observation 9: Rel-18 allows UE estimated TA for synchronous deployments, and Rel-18 is the baseline for Rel-19 inter-CU. As long as the same requirements and conditions exist for UE estimated TA, inter-CU should not be an exception.

Observation 10: In the previous discussions, companies were wondering if the requirements would be different for UE estimated TA due to inter-CU. But this is not RAN2 domain, and interested companies can submit contributions to the interested WG, as per WID, Rel-18 LTM is the baseline for Rel-19 LTM (and UE estimated TA is part of Rel-18 LTM).

Observation 11: Since SA3 will start discussions on security topics for inter-CU LTM without DC, a list of RAN2 usecases that are for inter-CU LTM with DC can help with SA3 discussions (for eg., to come up with solutions that can work for DC as well as non-DC cases where possible).

Observation 12 : a list of RAN2 usecases that are for inter-CU LTM with DC also helps with RAN3(who will take up DC in future meetings) / RAN4 (who would have potential SN specific requirements) and RAN1.

Proposal 1: Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations are supported.

Proposal 2:  (To lay the framework for conditional LTM,) UE is pre-configured using RRC LTM configuration which will allow the UE to derive whether the LTM switch is inter-CU or intra-CU.

Proposal 3: For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target CU of the target LTM cell.

Proposal 4: RAN2 will continue with using Rel-18 as the baseline for inter-CU (this includes the features from Rel-18 including UE estimated TA usecase from Rel-18), and will revisit this of other WGs think this is not feasible for inter-CU.

Proposal 5: RAN2 to agree on the usecases listed in table-1.

Proposal 6: RAN2 considers that the inter-CU LTM switch in SN requires a PDCP re-etablishment with a change of sk-counter.

Proposal 7: If MN has inter-CU LTM candidates configured, then the UE cannot be configured with inter-CU LTM in SN (as per WID)

Proposal 8: If MN has inter-CU LTM candidates configured, then the UE can be configured with intra-CU LTM in SN, but if the LTM switch in MN is inter-CU, then the UE should not have SN configured after the LTM switch.

Proposal 9: Send an LS to SA3/RAN1/RAN3/RAN4 about the DC use-cases and agreements.