3GPP TSG-RAN2 Meeting #127	R2-2406700

# Maastricht, NL, Aug 19th - 23rd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1889 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on legacy carrier selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-8-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support backward compatible feature, RAN2 makes following agreements: 1: When the upper layer provides multiple carriers in service to carrier mapping information to AS, we need TX profile extension to inform whether the transmisson corresponding the service is backward compatibile or not. If backward compatible is needed, only legacy carrier is used for transmission when PDCP duplication is not used. If PDCP duplication is used, at least legacy carrier is used. FFS whether to use PDCP duplication or not is up to UE implementation.. However, if more than one carriers including legacy carrier is indicated to MAC layer when duplication is used, carrier selection based on CBR threshold is applied. Then legacy carrier with CBR higher than the CBR threshold may not be selected, which violates the current RAN2’s agreement. According to current RRC specification, RRC layer will indicate the MAC layer to use legacy carrier when backwards compatible is needed and PDCP duplication is enabled 5.8.9.1a.6.2 Additional Sidelink RLC Bearer addition/modification operation ...... 4> indicate the allowed carriers for the two RLC bearers of the DRB, decided by UE implementation, to lower layer, where the carrier indicated in sl-FreqInfoList is used for the RLC bearer if the SL-TxProfile of at least one associated QoS flow for the sl-ServedRadioBearer indicates backwardsCompatible;. |
| Summary of change: |  | Clarify that the legacy carrier is selected without taking CBR threshold into consideration if legacy carrier is indicated to be used by RRC layer, when PDCP duplication is used. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality NR Sidelink CA Inter-operability: If the UE implements the CR but not the network, there is no inter-operability issue. If the network implements the CR but not the UE, there is no inter-operability issue. If one UE implements the CR but not the other UE, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Legacy carrier may not be used when duplication is enabled and backwards compatible is needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

5.22.1.11	TX carrier (re-)selection

The MAC entity shall consider a CBR of a carrier to be one measured by lower layers according to TS 38.215 [24] if CBR measurement results are available, or the corresponding sl-DefaultTxConfigIndex configured by upper layers if CBR measurement results are not available.

If the TX carrier (re-)selection is triggered for a Sidelink process according to clause 5.22.1.1, 5.22.1.2 or 5.22.1.3.3, the MAC entity shall:

1>	if there is no selected sidelink grant on any carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 38.331 [5] and TS 23.287 [19]):

2>	for each carrier configured by upper layers associated with the concerned sidelink logical channel:

3>	if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel:

NOTE 1:	In the case of multiple resource pools configured on a carrier, which specific resource pool is used to determine the CBR of this carrier is up to UE implementation, taking into account of sl-HARQ-FeedbackEnabled for the sidelink logical channel.

4>	consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel.

1>	else:

2>	for each sidelink logical channel, if any, where data is available and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1, if the CBR of the carrier is below sl-threshCBR-FreqKeeping associated with priority of the sidelink logical channel, for each sidelink logical channel, if any, where data is available and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1:

3>	select the carrier and the associated pool of resources.

2>	else:

3>	for each carrier configured by upper layers on which the sidelink logical channel is allowed, if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel:

4>	consider the carrier as a candidate carrier for TX carrier (re-)selection, for each carrier configured by upper layers on which the sidelink logical channel is allowed.

The MAC entity shall select a carrier on which the SL-CSI Request was received as a carrier for transmission of a Sidelink CSI Reporting MAC CE. The MAC entity shall for the Sidelink CSI Reporting MAC CE select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

The MAC entity shall select a carrier on which a Sidelink Inter-UE Coordination Request was received as a carrier for transmission of a Sidelink Inter-UE Coordination Information MAC CE. The MAC entity shall for the Sidelink Inter-UE Coordination Information MAC CE select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

The MAC entity shall:

1>	if one or more carriers are considered as the candidate carriers for TX carrier (re-)selection:

2>	if Tx carrier (re-)selection is triggered, for each sidelink logical channel allowed on the carrier where data is available:

3>	select one or more carrier(s) among the candidate carriers with increasing order of CBR from the lowest CBR or the carrier in sl-FreqInfoList, if indicated by upper layer to be used, and select the associated pool(s) of resources:

4>	if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel:

5>	select one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

4>	else:

5>	select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

NOTE 2:	It is left to UE implementation how many carriers to select based on UE capability.

NOTE 3:	It is left to UE implementation to determine the sidelink logical channels among the sidelink logical channels where data is available and that are allowed on the carrier for which Tx carrier (re-) selection is triggered.

NOTE 4:	It is left to UE implementation to determine whether the resource pool for CBR measurement is reused as the resource pool for SL grant creation.

NOTE 5:	For the transmission of Sidelink Inter-UE Coordination Request MAC CE, the MAC entity selects the TX pool of resource where the IUC resource set is required. For the transmission of Sidelink Inter-UE Coordination Information MAC CE, the MAC entity selects the TX pool of resource where the IUC resource set is located.

End of the change